PRZM-2, A Model for Predicting Pesticide Fate

in the Crop Root and Unsaturated Soil Zones:

Users Manual for Release 2.0

by

J.A. Mullins,1 R.F. Carsel,2

J.E. Scarbrough,1 and A.M. Ivery1

AScI Corporation1

Athens, GA 30605-2720

Environmental Research Laboratory2

U.S. Environmental Protection Agency

Athens, GA  30605-2720

ENVIRONMENTAL RESEARCH LABORATORY

OFFICE OF RESEARCH AND DEVELOPMENT

U.S. ENVIRONMENTAL PROTECTION AGENCY

ATHENS, GA  30605-2720

DISCLAIMER

The information in this document has been funded wholly or in part by the United States Environmental Protection Agency under Contract No. 68-C0-0054 to AScI.  It has been subject to the Agency's peer and administrative review, and it has been approved for publication as an EPA document.  Mention of trade names of commercial products does not constitute endorsement or recommendation for use by the U.S. Environmental Protection Agency.

FOREWORD

As environmental controls become more costly to implement and the penalties of judgment errors become more severe, environmental quality management requires more efficient analytical tools based on greater knowledge of the environmental phenomena to be managed.  As part of this Laboratory's research on the occurrence, movement, transformation, impact, and control of environmental contaminants, the Assessment Branch develops management or engineering tools to help pollution control officials reach decisions on the registration and restriction of pesticides used for agricultural purposes.

The pesticide regulatory process requires that the potential risk to human health resulting from the introduction or continued use of these chemicals be evaluated.  Recently much of this attention has been focused on exposure through leaching of pesticides to groundwater and subsequent ingestion of contaminated water.  To provide a tool for evaluating this exposure, the PRZM-2 model was developed.  PRZM-2 simulates the transport of field-applied pesticides in the crop root zone and the vadose zone taking into account the effects of agricultural management practices.  The model further provides estimates of probable exposure concentrations by taking into account the variability in the natural systems and the uncertainties in system properties and processes.

Rosemarie C. Russo, Ph.D.

Director

Environmental Research Laboratory

Athens, Georgia

ABSTRACT

This publication contains documentation for the PRZM-2 model.  PRZM-2 links two subordinate models--PRZM and VADOFT-- in order to predict pesticide transport and transformation down through the crop root, and unsaturated zone.  PRZM is a one-dimensional, finite-difference model that accounts for pesticide fate in the crop root zone.  This release of PRZM-2 incorporates several features in addition to those simulated in the original PRZM code--specifically, soil temperature simulation, volatilization and vapor phase transport in soils, irrigation simulation, microbial transformation, and a method of characteristics (MOC) algorithm to eliminate numerical dispersion.  PRZM is now capable of simulating transport and transformation of the parent compound and as many as two daughter species.  VADOFT is a one-dimensional, finite-element code that solves the Richard's equation for flow in the unsaturated zone.  The user may make use of constitutive relationships between pressure, water content, and hydraulic conductivity to solve the flow equations.  VADOFT may also simulate the fate of two parent and two daughter products.  The PRZM and VADOFT codes are linked together with the aid of a flexible execution supervisor that allows the user to build loading models that are tailored to site-specific situations.  In order to perform probability-based exposure assessments, the code is also equipped with a Monte Carlo pre- and post-processor.

TABLE OF CONTENTS

Page

Disclaimer ii

Foreword iii

Abstract iv

Figures x

Tables xiv

Acknowledgments xix

Section	Page

1.0	INTRODUCTION   1-1

1.1	Background and Objectives   1-1

1.2	Concept of Risk and Exposure Assessment   1-2

1.3	Overview of PRZM-2   1-7

1.3.1	Overview of PRZM   1-8

1.3.1.1	Features   1-8

1.3.1.2	Limitations   1-9

1.3.2	Overview of the Vadose Zone Flow and

Transport Model (VADOFT) 1-11

1.3.2.1	Features 1-11

1.3.2.2	Limitations 1-11

1.3.3	Overview of the Monte Carlo Simulation Module 1-12

1.3.4	Model Linkage 1-12

1.3.4.1	Temporal Model Linkage 1-12

1.3.4.2	Spatial Linkages 1-13

1.3.5	Monte Carlo Processor 1-13

1.3.6	Overview Summary 1-14

2.0	MODEL DEVELOPMENT, DISTRIBUTION, AND SUPPORT   2-1

2.1	Development and Testing   2-1

2.2	Distribution   2-2

2.3	Obtaining a copy of the PRZM-2 Model   2-3

2.3.1	Diskette   2-3

2.3.2	Electronic Bulletin Board System (BBS)   2-3

2.4	General/Minimum Hardware and Software

Installation and Run-Time Requirements   2-4

2.4.1	Installation Requirements   2-4

2.4.2	Run Time Requirements   2-4

2.5	Installation   2-4

2.6	Installation Verification and Routine Execution   2-5

2.7	Code Modification   2-5

2.8	Technical Help   2-5

2.9	Disclaimer2-7

TABLE OF CONTENTS (continued)

Section	Page

2.10	Trademarks   2-7

3.0	MODULES AND LOGISTICS   3-1

4.0	INPUT PARAMETERS FOR PRZM-2   4-1

4.1	Input File Descriptions   4-1

4.1.1	Meteorological File   4-2

4.1.2	Execution Supervisor File   4-2

4.1.2.1	Example Execution Supervisor   4-3

4.1.2.2	Example Execution Supervisor   4-4

4.1.2.3	Execution Supervisor Input Guide   4-5

4.1.3	PRZM Input File   4-7

4.1.3.1	Example PRZM Input File   4-7

4.1.3.2	PRZM Input Guide   4-8

4.1.4	VADOFT Input File 4-24

4.1.4.1	Example VADOFT Input File 4-24

4.1.4.2	VADOFT Input Guide for FLOW 4-25

4.1.4.3	VADOFT Input Guide for TRANSPORT 4-32

4.1.5	MONTE CARLO Input File 4-39

4.1.5.1	Example MONTE CARLO Input File 4-39

4.1.5.2	MONTE CARLO Input Guide 4-40

5.0	PARAMETER ESTIMATION   5-1

5.1	EXESUP (Execution Supervisor)   5-1

5.2	PRZM (Pesticide Root Zone Model)   5-2

5.3	VADOFT Parameters 5-63

6.0	PRZM CODE AND THEORY   6-1

6.1	Introduction and Background (PRZM)   6-1

6.1.1	Introduction   6-1

6.1.2	Background   6-2

6.2	Features and Limitations   6-2

6.2.1	Features   6-2

6.2.2	Limitations   6-4

6.3	Description of the Equations   6-6

6.3.1	Transport in Soil   6-6

6.3.2	Water Movement 6-14

6.3.2.1	Option 1 6-20

6.3.2.2	Option 2 6-20

6.3.3	Soil Erosion 6-21

TABLE OF CONTENTS (continued)

Section	Page

6.3.4	Volatilization 6-22

6.3.4.1	Soil Vapor Phase and Volat. Flux 6-23

6.3.4.2	Volat. Flux Through the Plant Canopy 6-25

6.3.4.3	Volat. Flux from Plant Surfaces 6-30

6.3.4.4	Soil Temperature Simulation 6-32

6.3.5	Irrigation Equations 6-41

6.3.5.1	Soil Moisture Deficit 6-42

6.3.5.2	Sprinkler Irrigation 6-42

6.3.5.3	Flood Irrigation 6-43

6.3.5.4	Furrow Irrigation 6-43

6.4	Numerical Solution Techniques 6-45

6.4.1	Chemical Transport Equations 6-45

6.4.2	Volatilization 6-47

6.4.3	Soil Temperature 6-50

6.4.4	Furrow Irrigation 6-51

6.5	Results of PRZM Testing Simulations 6-53

6.5.1	Transport Equation Solution Options 6-54

6.5.1.1	High Peclet Number 6-54

6.5.1.2	Low Peclet Number 6-54

6.5.2	Testing Results of Volatilization Subroutines 6-55

6.5.2.1	Comparison with Analytical Sol 6-55

6.5.2.2	Comparison with Field Data 6-62

6.5.2.3	Conclusions from Volatilization Model Testing 6-67

6.5.3	Testing Results of Soil Temp. Simulation Subroutine 6-70

6.5.4	Testing of Daughter Products Simulation 6-71

6.6	Biodegradation Theory and Assumptions 6-81

7.0	VADOFT CODE AND THEORY   7-1

7.1	Introduction   7-1

7.2	Overview of VADOFT   7-1

7.2.1	Features   7-1

7.2.1.1	General Description   7-1

7.2.1.2	Process and Geometry   7-1

7.2.1.3	Assumptions   7-1

7.2.1.4	Data Requirements   7-2

7.2.2	Limitations   7-2

7.3	Description of the Flow Module   7-3

7.3.1	Flow Equation   7-3

7.3.2	Numerical Solution   7-6

7.3.2.1	Numerical Appr. of the Flow Eq.   7-6

TABLE OF CONTENTS (continued)

Section	Page

7.3.2.2	General Guidance on Selection of Grid Spacings

and Time Steps, and the Use of

Solution Algorithms 7-15

7.4	Description of the Transport Module 7-15

7.4.1	Transport Equation 7-15

7.4.2	Numerical Solution of the Transport Equation 7-16

7.4.2.1	Numerical Appr. of the Transport

Equation 7-16

7.5	Results of VADOFT Testing Simulations 7-19

7.5.1	Flow Module 7-19

7.5.1.1	Transient Upward Flow 7-19

7.5.1.2	Steady Infiltration 7-19

7.5.2	Transport Module 7-20

7.5.2.1	Transport in a Semi-Infinite

Soil Column 7-20

7.5.2.2	Transport in a Finite Soil Column 7-20

7.5.2.3	Transport in a Layered Soil Column 7-20

7.5.3	Combined Nonlinear Flow and Transport Modules 7-32

7.5.3.1	Transport During Absorption of

Water in a Soil Tube 7-32

7.5.3.2	Transient Infiltration and Contam-

inant Transport in the Vadose Zone 7-32

8.0	UNCERTAINTY PREPROCESSOR   8-1

8.1	Introduction   8-1

8.2	Overview of the Preprocessor   8-1

8.2.1	Description of the Method   8-2

8.2.2	Uncertainty in the Input Variables   8-3

8.3	Description of Available Parameter Distributions   8-4

8.3.1	Uniform Distribution   8-4

8.3.2	Normal Distribution   8-5

8.3.3	Log-Normal Distribution   8-6

8.3.4	Exponential Distribution   8-6

8.3.5	The Johnson System of Distributions   8-7

8.3.6	Triangular Distribution   8-7

8.3.7	Empirical Distribution   8-8

8.3.8	Uncertainty in Correlated Variables   8-8

8.3.9	Generation of Random Numbers 8-13

8.4	Analysis of Output and Estimation of Distribution

Quantiles 8-13

8.4.1	Estimating Distribution Quantiles 8-14

8.4.2	Confidence of uρ 8-16

TABLE OF CONTENTS (continued)

Section	Page

9.0	REFERENCES   9-1

10.0	APPENDICES 10-1

10.1	Error Messages and Warnings 10-1

10.2	Variable Glossary 10-1

10.3	PRZM and VADOFT Example Input Files10-70

LIST OF FIGURES

Figure	Page

1.1	DECISION PATH FOR RISK ASSESSMENT1-3

1.2	TIME SERIES PLOT OF TOXICANT CONCENTRATIONS1-5

1.3	FREQUENCY DISTRIBUTION OF TOXICANT CONCENTRATIONS1-5

1.4	CUMULATIVE FREQUENCY DISTRIBUTION OF TOXICANT

CONCENTRATIONS1-5

1.5	TIME SERIES OF TOXICANT CONCENTRATIONS WITH MOVING

AVERAGE WINDOW OF DURATION TC1-6

1.6	LINKED MODELING SYSTEM CONFIGURATION1-6

5.1	PAN EVAPORATION CORRECTION FACTORS5-19

5.2	DIAGRAM FOR ESTIMATING SOIL EVAPORATION LOSS5-20

5.3	REPRESENTATIVE REGIONAL MEAN STORM DURATION VALUES

FOR THE UNITED STATES5-21

5.4	DIAGRAM FOR ESTIMATING SOIL CONSERVATION SERVICE SOIL

HYDROLOGIC GROUPS5-22

5.5	NUMERICAL DISPERSION ASSOCIATED WITH SPACE STEP5-23

5.6	PHYSICAL DISPERSION ASSOCIATED WITH ADVECTIVE TRANSPORT5-24

5.7	AVERAGE TEMPERATURE OF SHALLOW GROUNDWATER5-25

5.8	1/3-BAR SOIL MOISTURE BY VOLUME5-26

5.9	15-BAR SOIL MOISTURE BY VOLUME5-27

5.10	MINERAL BULK DENSITY5-28

5.11	ESTIMATION OF DRAINAGE RATE AD VERSUS NUMBER OF

COMPARTMENTS5-29

6.1	PESTICIDE ROOT ZONE MODEL6-3

LIST OF FIGURES

Figure	Page

6.2	SCHEMATIC REPRESENTATION OF A SINGLE CHEMICAL IN A SOIL

LAYER6-7

6.3	SCHEMATIC OF PESTICIDE VAPOR AND VOLATILIZATION PROCESSES6-24

6.4	VARIABILITY OF INFILTRATION DEPTHS WITHIN AN IRRIGATION

FURROW6-44

6.5	SCHEMATIC OF THE TOP TWO SOIL COMPARTMENTS AND THE

OVERLYING SURFACE COMPARTMENT6-49

6.6	COMPARISON OF SIMULATION RESULTS AT HIGH PECLET NUMBER6-56

6.7	COMPARISON OF SIMULATION RESULTS AT LOW PECLET NUMBER6-57

6.8	COMPARISON OF VOLATILIZATION FLUX PREDICTED BY PRZM AND

JURY'S ANALYTICAL SOLUTION: TEST CASES #1 AND #26-60

6.9	COMPARISON OF VOLATILIZATION FLUX PREDICTED BY PRZM AND

JURY'S ANALYTICAL SOLUTION: TEST CASES #3 AND #46-61

6.10	SENSITIVITY OF CUMULATIVE VOLATILIZATION FLUX TO

Kd AND DECAY RATE6-65

6.11	EFFECTS OF DELX ON VOLATILIZATION FLUX AND PESTICIDE

DECAY6-66

6.12	COMPARISON OF CONSTANT AND TWO-STEP DECAY RATES6-68

6.13	EFFECTS OF TWO-STEP DECAY RATES ON VOLATILIZATION

FLUX AND PESTICIDE DECAY6-69

6.14	COMPARISON OF SOIL TEMPERATURE PROFILES PREDICTED BY

ANALYTICAL AND FINITE DIFFERENCE SOLUTIONS (Time Step=1 HR)6-72

6.15	COMPARISON OF SOIL TEMPERATURE PROFILES PREDICTED BY

ANALYTICAL AND FINITE DIFFERENCE SOLUTIONS (Time Step=1 DAY)6-73

6.16	SCHEMATIC OF A SYSTEM OF PARENT AND DAUGHTER

PESTICIDE6-75

LIST OF FIGURES (Continued)

Figure	Page

6.17	CONVERSION OF C1 TO C2 TO C3 WITH NO ADSORPTION

AND NO DECAY 6-78

6.18	CONVERSION OF C1 TO C2 TO C3 WITH DECAY BUT NO

ADSORPTION 6-79

6.19	CONVERSION OF ALDICARB TO ALDICARB SULFOXIDE TO

ALDICARB SULFONE6-80

7.1	LOGARITHMIC PLOT OF CONSTITUTIVE RELATIONS FOR CLAY, CLAY

LOAM, AND LOAMY SAND7-7

7.2	LOGARITHMIC PLOT OF CONSTITUTIVE RELATIONS FOR SILT, SILTY

CLAY LOAM, SILTY CLAY, AND SILTY LOAM7-8

7.3	LOGARITHMIC PLOT OF CONSTITUTIVE RELATIONS FOR SANDY CLAY,

SANDY CLAY LOAM, SANDY LOAM, AND SAND7-9

7.4	STANDARD PLOT OF RELATIVE PERMEABILITY VS. SATURATION FOR

CLAY, CLAY LOAM, LOAM, AND LOAMY SAND7-10

7.5	STANDARD PLOT OF RELATIVE PERMEABILITY VS. SATURATION FOR

SILT, SILT CLAY LOAM, SILTY CLAY, AND SILTY LOAM7-11

7.6	STANDARD PLOT OF RELATIVE PERMEABILITY VS. SATURATION FOR

SANDY CLAY, SANDY CLAY LOAM, SANDY LOAM, AND SAND7-12

7.7	FINITE ELEMENT DISCRETIZATION OF SOIL COLUMN SHOWING NODE

AND ELEMENT NUMBERS7-13

7.8	SIMULATED PRESSURE HEAD PROFILES FOR THE PROBLEM OF

TRANSIENT UPWARD FLOW IN A SOIL COLUMN7-22

7.9	SIMULATED PROFILE OF WATER SATURATION FOR THE PROBLEM OF

TRANSIENT UPWARD FLOW IN A SOIL COLUMN7-23

7.10	SIMULATED PRESSURE HEAD PROFILES FOR FIVE CASES OF

THE PROBLEM OF STEADY INFILTRATION IN A SOIL COLUMN7-25

LIST OF FIGURES (Continued)

Figure	Page

7.11	SIMULATED PROFILES OF WATER SATURATION FOR FIVE CASES

OF THE PROBLEM OF STEADY INFILTRATION IN A SOIL COLUMN7-26

7.12	SIMULATED CONCENTRATION PROFILES FOR THE PROBLEM OF SOLUTE

TRANSPORT IN A SEMI-INFINITE SOIL COLUMN7-27

7.13	SIMULATED CONCENTRATION PROFILES FOR TWO CASES OF

THE PROBLEM OF SOLUTE TRANSPORT IN A SOIL COLUMN OF

FINITE LENGTH7-30

7.14	SIMULATED OUTFLOW BREAKTHROUGH CURVE FOR CASE 1

OF THE PROBLEM OF SOLUTE TRANSPORT IN A LAYERED

SOIL COLUMN7-33

7.15	SIMULATED OUTFLOW BREAKTHROUGH CURVE FOR CASE 2

OF THE PROBLEM OF SOLUTE TRANSPORT IN A LAYERED

SOIL COLUMN7-34

7.16	ONE-DIMENSIONAL SOLUTE TRANSPORT DURING ABSORPTION

OF WATER IN A SOIL TUBE7-39

7.17	SIMULATED PROFILES OF WATER SATURATION DURING ABSORP-

TION OF WATER IN A SOIL TUBE7-40

7.18	SIMULATED CONCENTRATION PROFILES FOR THE PROBLEM OF

ONE-DIMENSIONAL SOLUTE TRANSPORT DURING ABSORPTION

OF WATER IN A SOIL TUBE7-41

7.19	PROBLEM DESCRIPTION FOR TRANSIENT INFILTRATION AND

TRANSPORT IN THE VADOSE ZONE7-42

7.20	INFILTRATION RATE VS. TIME RELATIONSHIP USED IN

NUMERICAL SIMULATION7-43

7.21	SIMULATED WATER SATURATION PROFILES7-44

7.22	SIMULATED PRESSURE HEAD PROFILES7-45

7.23	SIMULATED VERTICAL DARCY VELOCITY PROFILES7-46

7.24	SIMULATED SOLUTE CONCENTRATION PROFILES7-47

8.1	TRIANGULAR PROBABILITY DISTRIBUTION8-9

LIST OF TABLES

Table	Page

3-1	LIST OF SUBROUTINES AND FUNCTIONS AND A BRIEF DESCRIPTION OF

THEIR PURPOSE3-2

3-2	LIST OF ALL PARAMETER FILES, PARAMETER DIMENSIONS, AND

A BRIEF DESCRIPTION3-6

4-1	VARIABLE DESIGNATIONS FOR PLOTTING FILES4-21

4-2	MONTE CARLO INPUT AND OUTPUT LABELS4-23

5-1	TYPICAL VALUES OF SNOWMELT (SFAC) AS RELATED

TO FOREST COVER5-30

5-2	MEAN DURATION (HOURS) OF SUNLIGHT FOR LATITUDES 0° TO 50° IN

THE NORTHERN AND SOUTHERN HEMISPHERES5-30

5-3	INDICATIONS OF THE GENERAL MAGNITUDE OF THE

SOIL/ERODIBILITY FACTOR, K5-31

5-4	INTERCEPTION STORAGE FOR MAJOR CROPS5-31

5-5	VALUES OF THE EROSION EQUATION'S TOPOGRAPHIC FACTOR, LS,

FOR SPECIFIED COMBINATIONS OF SLOPE LENGTH AND STEEPNESS5-32

5-6	VALUES OF SUPPORT-PRACTICE FACTOR, P5-33

5-7	GENERALIZED VALUES OF THE COVER AND MANAGEMENT FACTOR, C,

IN THE 37 STATES EAST OF THE ROCKY MOUNTAINS5-34

5-8	MEAN STORM DURATION (TR) VALUES FOR SELECTED CITIES5-37

5-9	AGRONOMIC DATA FOR MAJOR AGRICULTURAL CROPS IN THE

UNITED STATES5-38

5-10	RUNOFF CURVE NUMBERS FOR HYDROLOGIC SOIL-

COVER COMPLEXES5-39

5-11	METHOD FOR CONVERTING CROP YIELDS TO RESIDUE5-40

5-12	RESIDUE REMAINING FROM TILLAGE OPERATIONS5-40

5-13	REDUCTION IN RUNOFF CURVE NUMBERS CAUSED BY

CONSERVATION TILLAGE AND RESIDUE MANAGEMENT5-41

LIST OF TABLES (Continued)

Table	Page

5-14	VALUES FOR ESTIMATING WFMAX IN EXPONENTIAL

FOLIAR MODEL5-41

5-15	PESTICIDE SOIL APPLICATION METHODS AND DISTRIBUTION5-42

5-16	MAXIMUM CANOPY HEIGHT AT CROP MATURATION5-42

5-17	DEGRADATION RATE CONSTANTS OF SELECTED PESTICIDES

ON FOLIAGE5-43

5-18	ESTIMATED VALUES OF HENRY'S CONSTANT FOR SELECTED

PESTICIDES5-44

5-19	PHYSICAL CHARACTERISTICS OF SELECTED PESTICIDES

FOR USE IN DEVELOPMENT OF PARTITION COEFFICIENTS

AND REPORTED DEGRADATION RATE CONSTANTS IN SOIL

ROOT ZONE5-45

5-20	OCTANOL WATER DISTRIBUTION COEFFICIENTS AND SOIL

DEGRADATION RATE CONSTANTS FOR SELECTED CHEMICALS5-48

5-21	ALBEDO FACTORS OF NATURAL SURFACES FOR SOLAR

RADIATION5-50

5-22	EMISSIVITY VALUES FOR NATURAL SURFACES AT NORMAL

TEMPERATURES5-51

5-23	COEFFICIENTS FOR LINEAR REGRESSION EQUATIONS FOR

PREDICTION OF SOIL WATER CONTENTS AT SPECIFIC

MATRIC POTENTIALS5-51

5-24	THERMAL PROPERTIES OF SOME SOIL AND REFERENCE

MATERIALS5-52

5-25	HYDROLOGIC PROPERTIES BY SOIL TEXTURE5-53

5-26	DESCRIPTIVE STATISTICS AND DISTRIBUTION MODEL FOR

FIELD CAPACITY5-54

5-27	DESCRIPTIVE STATISTICS AND DISTRIBUTION MODEL FOR

WILTING POINT5-55

LIST OF TABLES (Continued)

Table	Page

5-28	CORRELATIONS AMONG TRANSFORMED VARIABLES OF ORGANIC

MATTER, FIELD CAPACITY, AND WILTING POINT5-56

5-29	MEAN BULK DENSITY FOR FIVE SOIL TEXTURAL

CLASSIFICATIONS5-57

5-30	DESCRIPTIVE STATISTICS FOR BULK DENSITY5-57

5-31	DESCRIPTIVE STATISTICS AND DISTRIBUTION MODEL FOR

ORGANIC MATTER5-58

5-32	ADAPTATIONS AND LIMITATIONS OF COMMON IRRIGATION

METHODS5-59

5-33	WATER REQUIREMENTS FOR VARIOUS IRRIGATION AND

SOIL TYPES5-59

5-34	REPRESENTATIVE FURROW PARAMETERS DESCRIBED IN THE

LITERATURE5-60

5-35	FURROW IRRIGATION RELATIONSHIPS FOR VARIOUS SOILS,

SLOPES, AND DEPTHS OF APPLICATION5-60

5-36	SUITABLE SIDE SLOPES FOR CHANNELS BUILT IN VARIOUS

KINDS OF MATERIALS5-61

5-37	VALUE OF "N" FOR DRAINAGE DITCH DESIGN5-61

5-38	REPRESENTATIVE PERMEABILITY RANGES FOR SEDIMENTARY

MATERIALS5-62

5-39	VALUES OF GREEN-AMPT PARAMETERS FOR SCS HYDROLOGIC

SOIL GROUPS5-62

5-40	DESCRIPTIVE STATISTICS FOR SATURATED HYDRAULIC

CONDUCTIVITY5-65

5-41	DESCRIPTIVE STATISTICS FOR VAN GENUCHTEN WATER

RETENTION MODEL PARAMETERS, α, β, γ5-67

5-42	DESCRIPTIVE STATISTICS FOR SATURATION WATER CONTENT

(θs) AND RESIDUAL WATER CONTENT (θr)5-68

LIST OF TABLES (Continued)

Table	Page

5-43	STATISTICAL PARAMETERS USED FOR DISTRIBUTION

APPROXIMATION5-69

5-44	CORRELATIONS AMONG TRANSFORMED VARIABLES PRESENTED

WITH THE FACTORED COVARIANCE MATRIX5-71

6-1	SUMMARY OF SOIL TEMPERATURE MODEL CHARACTERISTICS6-34

6-2	INPUT PARAMETERS FOR THE TEST CASES - ANALYTICAL SOLUTION6-59

6-3	TRIFLURALIN VOLATILIZATION LOSSES, AMOUNTS REMAINING IN

SOIL, AND ESTIMATED LOSSES VIA OTHER PATHWAYS FOR THE 120-

DAY FIELD TEST6-64

6-4	INPUT PARAMETERS FOR THE TEST CASES - WATKINSVILLE SITE6-64

6-5	SIMULATION RESULTS OF USING DIFFERENT COMPARTMENT DEPTH6-67

6-6	SIMULATED SOIL TEMPERATURE PROFILE AFTER ONE DAY FOR

DIFFERENT COMPARTMENT THICKNESSES6-74

7-1	SOIL PROPERTIES AND DISCRETIZATION DATA USED IN SIMULATING

TRANSIENT FLOW IN A SOIL COLUMN7-21

7-2	SOIL PROPERTIES USED IN SIMULATING STEADY-STATE

INFILTRATION7-21

7-3	ITERATIVE PROCEDURE PERFORMANCE COMPARISON7-24

7-4	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA USED

IN SIMULATING ONE-DIMENSIONAL TRANSPORT IN A SEMI-INFINITE

SOIL COLUMN7-24

7-5	CONCENTRATION PROFILE CURVES AT t = 25 hr AND t = 50 hr

SHOWING COMPARISON OF THE ANALYTICAL SOLUTION AND RESULTS

FROM VADOFT7-28

7-6	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA USED

IN SIMULATING ONE-DIMENSIONAL TRANSPORT IN A FINITE

SOIL COLUMN7-29

LIST OF TABLES (Concluded)

Table	Page

7-7	CONCENTRATION PROFILE CURVES SHOWING COMPARISON OF THE

ANALYTICAL SOLUTION AND VADOFT7-31

7-8	VALUES OF PHYSICAL PARAMETERS USED IN THE SIMULATION OF

TRANSPORT IN A LAYERED SOIL COLUMN7-35

7-9	BREAKTHROUGH CURVES COMPUTED USING THE ANALYTICAL SOLUTION

AND VADOFT (CASE 1)7-36

7-10	BREAKTHROUGH CURVES COMPUTED USING THE ANALYTICAL

SOLUTION AND VADOFT (CASE 2)7-37

7-11	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION

DATA USED IN SIMULATING TRANSPORT IN A VARIABLY

SATURATED SOIL TUBE7-38

7-12	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA

USED IN SIMULATING TRANSIENT INFILTRATION AND

CONTAMINANT TRANSPORT IN THE VADOSE ZONE7-38

10-1	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING

APPROACHES10-2

10-2	EXESUP PROGRAM VARIABLES10-13

10-3	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE

DESIGNATION10-16

10-4	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND

VARIABLE DESIGNATIONS10-55

10-5	MONTE-CARLO PROGRAM VARIABLES10-68

ACKNOWLEDGMENTS

A number of individuals contributed to this effort.  Their roles are acknowledged in the following paragraphs.

Several components of PRZM-2 were excerpted from the RUSTIC model.  The following contributors to the RUSTIC model are acknowledged:  Mr. K.A. Voos of Woodward-Clyde Consultants (WCC) programmed the execution supervisor and linked the models.  The linkage was conceived by Mr. J.D. Dean and Dr. Atul Salhotra of WCC and Dr. P.S. Huyakorn of HydroGeologic.  Dr. Huyakorn and his staff wrote the time/space bridging subroutines for the linkage.  Mr. R.W. Schanz (WCC) and Ms. Meeks (WCC) wrote the irrigation and MOC algorithms.  The volatilization routines were written by Dr. J. Lin and Dr. S. Raju of Aqua Terra Consultants.  Mr. Dean wrote the daughter products algorithms that were implemented by Dr. Lin.  Mr. J.L. Kittle implemented modifications to allow multiple segment (zone) simulation capability.

The original VADOFT code was written and documented by Dr. Huyakorn,

Mr. H. White, Mr. J. Buckley, and Mr. T. Wadsworth of HydroGeologic.  The Monte Carlo pre- and post-processors were written by Dr. Salhotra, Mr. P. Mineart, and Mr. Schanz of WCC.  Modifications were developed and implemented by the authors of this document.

Final assembly of the model code, documentation and model testing were performed by AScI.  The support of the graphics staff of AScI is appreciated.  Also a special thanks to Ms. T. Robinson (AScI) and Ms. S. Tucker (CSC) for their typing of text, equations, and tables.

The authors would like to acknowledge Mr. D.S. Brown, Chief, Assessment Branch, ERL-Athens, for his suggestions, input, and helpful comments.

SECTION 1

INTRODUCTION

This publication contains documentation for a linked groundwater loading model, known as PRZM-2, for organic chemical contaminant transport down through the crop in root and vadose zones.  A brief section on background and objectives for the model development effort follows in this introduction (Section 1.1).  Section 1.2 gives a synopsis of risk and exposure assessment concepts.  The reader who has sufficient background in these concepts may proceed to Section 1.3, which provides an overview of the PRZM-2 modeling system, including major features and limitations.

1.1  BACKGROUND AND OBJECTIVES

The U.S. Environmental Protection Agency is continually faced with issues concerning the registration and restriction of pesticides used for agricultural purposes.  Each of these regulatory processes requires that the potential risk to human health resulting from the introduction or continued use of such chemicals be evaluated.  Recently, much of this attention has been focused on exposure through leaching of pesticides to groundwater and subsequent ingestion of contaminated water.

The capability to simulate the potential exposure to pesticides via this pathway has two major facets:

o	Prediction of the fate of the chemical, after it is applied, as it is transported by water down through the crop root and soil vadose zones.

o	Evaluation of the probability of the occurrence of concentrations of various magnitudes at various depths.

Several models are capable of simulating the transport and transformation of chemicals in the subsurface and in the root zone of agricultural crops.  However, none of these models have been linked together in such a way that a complete simulation package, which takes into account the effects of agricultural management practices on contaminant fate is available for use either by the Agency or the agricultural chemical industry to address potential groundwater contamination problems.  Without such a package, the decision maker must rely on modeling scenarios that are either incomplete or potentially incorrect.  Each time a new scenario arises, recurring questions must be answered:

o	What models should be used?

o	How should mass transfer between models be handled?

The resolution of these issues for each scenario is both expensive and time consuming.  Furthermore, it precludes consistency of approach to evaluation of contamination potential for various scenarios.

The modeling package described in this report seeks to overcome these problems by providing a consistent set of linked unsaturated zone models that have the flexibility to handle a wide variety of hydrogeological, soils, climate, and pesticide scenarios.  However, the formulation of the risk analysis problem requires more than a simple, deterministic evaluation of potential exposure concentrations.  The inherent variability of force, capacitance and resistance in natural systems, combined with the inability to exactly describe these attributes of the system, suggests that exposure concentrations cannot be predicted with certainty.  Therefore, the uncertainty associated with the predictions must be quantified.  Consequently, this simulation package also seeks to provide this capability by utilizing Monte Carlo simulation techniques.

Stated more concisely, the objectives of this model development effort were to provide a simulation package that can:

o	Simulate the transport and transformation of field-applied pesticides in the crop root zone and the underneath unsaturated zone taking into account the effects of agricultural management practices

o	Provide probabilistic estimates of exposure concentrations by taking into account the variability in the natural systems and the uncertainty in system properties and processes

Furthermore, it was desirable that the simulation package be easy to use and parameterize, and execute on IBM or IBM-compatible PCs and the Agency's DEC/VAX machines.  As a result, considerable effort has gone into providing parameter guidance for both deterministic and probabilistic applications of the model and software development for facile model implementation.

1.2  CONCEPT OF RISK AND EXPOSURE ASSESSMENT

Exposure assessment, as defined in the Federal Register (1984) for human impacts, is the estimation of the magnitude, frequency, and duration at which a quantity of a toxicant is available at certain exchange boundaries (i.e., lungs, gut, or skin) of a subject population over a specified time interval.  Exposure assessment is an element of the larger problems of risk assessment and risk management, as demonstrated in Figure 1.1.  The concentration estimates generated during an exposure assessment are combined with demographic and toxicological information to evaluate risk to a population--which can be used, in turn, to make policy decisions regarding the use or disposal of the chemical.

Major components of risk assessment are indicated below.  Of these, the first three constitute the important steps for exposure assessment and are discussed in detail here.

o	Characterization and quantification of chemical sources

o	Identification of exposure routes

o	Quantification of contaminant movement through the exposure routes to the receptor population/location

o	Characterization of the exposed population

o	Integration of quantified environmental concentrations with the characteristics of the exposed populations to yield exposure profiles

Characterization of sources(s) requires in a broad sense the estimation of the loading of a chemical into various environmental media.  For the groundwater contamination problem, on a regional scale, this requires data on chemical uses and distribution of those uses (spatially and temporally).  It also requires information on the crops being grown, registered or proposed chemical uses of those crops, and regional management practices.  For a specific field-scale area, similar data would be needed to support an assessment; however, greater detail may be necessary.

The identification of exposure pathways involves a qualitative (or semiquantitative) assessment of how the chemical is thought to move from the source to the exposed population.  Important fate processes that may serve to reduce the concentration of the chemical(s) along various pathways in different environmental media are also identified.  For the case of groundwater exposure, important pathways and processes are predefined to a large extent in the models to be used.  The quantification of concentrations in a medium, given the source strength, pathways, and attenuation mechanisms along each pathway, is the next step, and is the major benefit of using models such as PRZM-2.  The guidelines are very specific in the requirement that concentrations be characterized by duration and frequency as well as magnitude.  These characteristics can be determined through the analysis of time series exposure data generated by the model.

PRZM-2 produces time series of toxicant concentrations such as appears in Figure 1.2.  Each time series can be compared to a critical value of the concentration y.  This type of analysis easily shows whether the criterion is exceeded and gives a qualitative feel for the severity of the exceedance state.  If we determine how often a contaminant is at a particular level or within a specified range, a frequency distribution of the values of y (Figure 1.3) can be created.  If, in addition, we choose any value of y in Figure 1.2 and determine the area under the curve to the right of that value, we can plot Figure 1.4, which is a cumulative frequency distribution of the toxicant concentration.  The cumulative frequency distribution shows the chance that any given value y that we select will be exceeded.  If the example time series is long enough, then the "chance" approaches the true "probability" that y will be exceeded.

Thus far, only the concentration to which the organism will be exposed has been discussed and nothing has been said concerning the duration of the event.  If we take the same time series and impose a window of length "t" on it at level yc (Figure 1.5), and move it incrementally forward in time, we can make a statement concerning the toxicant concentration within the duration window.  Normally, the average concentration within the window is used.  The resulting cumulative frequency distribution shows the chance that the moving average of duration tc will exceed the critical value of y, yc.

The moving average window should be the same length as that specified for yc.  For instance, in the case of cancer risk, a 70-year (lifetime) window is normally used to average the data in the simulated time series.  The use of the moving window for averaging the time series allows us to compare both the concentration and duration against the standard.  The chance or probability that the moving average concentration exceeds the standard is the essence of the exposure assessment.  This type of information provides a precursor to the estimates of risk taken in using this chemical under the conditions of the model simulation.  The use of models like PRZM-2 that provide data in environmental concentrations, duration and probability of occurrence ends here.

The next step in exposure assessment involves the characterization of the exposed population.  Such factors as habits, age, sex, and location with respect to the source are of importance.  The integration of concentration estimates and population characteristics makes possible the counting of the conditional events of concentration in an environmental medium and the opportunity for the population to be exposed to these concentrations.  The exposure assessment ends at this point.  The actual intake of chemicals, their fate within the human body (e.g., pharmacokinetics, toxicology), and their effects on the exposed population are not considered.  These, however, are also elements of the risk assessment.

Although the concepts underlying an exposure assessment are relatively simple, the actual application of these concepts is complicated because of large variations in source-specific and environment-specific characteristics and the necessity to integrate specialized knowledge from a number of different fields.  This variability underscores the need to use a model such as PRZM-2 in the evaluation of exposure concentrations.

1.3  OVERVIEW OF PRZM-2

This section gives an overview of the PRZM-2 model highlighting the features and limitations of the simulation package as a whole, and the component models PRZM and VADOFT.  The PRZM-2 code was designed to provide state-of-the-art deterministic simulation of the fate of pesticides, applied for agricultural purposes, both in the crop root zone and the underlying vadose zone.  The model is capable of simulating multiple pesticides or parent/daughter relationships.  The model is also capable of estimating probabilities of concentrations or fluxes in or from these various media for the purpose of performing exposure assessments.

To avoid writing an entirely new computer code, it was decided to make use of existing codes and software to the extent possible.  Thus, due to its comprehensive treatment of important processes, its dynamic nature, and its widespread use and acceptability to the Agency and the agricultural chemical industry, the Pesticide Root Zone model (PRZM) (Carsel et al. 1984) was selected to simulate the crop root zone.

Having selected PRZM, two options were evaluated for developing the PRZM-2 linked model to meet the objectives stated in Section 1.1.  The first involved use of PRZM only.  In this configuration, PRZM would be used to simulate both the root zone and the vadose zone.  This option was rejected because the assumptions of the elementary soil hydraulics in PRZM (i.e., drainage of the entire soil column to field capacity in 1 day) were considered inadequate for simulating flow in a thick vadose zone.  The second option involved PRZM linked to a to be determined unsaturated zone model.  The option finally selected is depicted in Figure 1.6.  In this configuration, an enhanced version of PRZM is linked to a one-dimensional vadose zone flow and transport model.  Both the vadose and PRZM models simulate water flow and solute transport.  Subsequently, a new code (VADOFT) was written to perform the flow and transport simulation in the vadose zone.

1.3.1	Overview of PRZM

1.3.1.1	Features--

The Pesticide Root Zone Model (PRZM) is a one-dimensional, dynamic, compartmental model that can be used to simulate chemical movement in unsaturated soil systems within and immediately below the plant root zone.  It has two major components--  hydrology (and hydraulics) and chemical transport.  The hydrologic component for calculating runoff and erosion is based on the Soil Conservation Service curve number technique and the Universal Soil Loss Equation.  Evapotranspiration is estimated either directly from pan evaporation data, or based on an empirical formula.  Evapotranspiration is divided among evaporation from crop interception, evaporation from soil, and transpiration by the crop.  Water movement is simulated by the use of generalized soil parameters, including field capacity, wilting point, and saturation water content.  The chemical transport component can simulate pesticide application on the soil or on the plant foliage.  With a newly added feature, biodegradation can also be considered in the root zone.  Dissolved, adsorbed, and vapor-phase concentrations in the soil are estimated by simultaneously considering the processes of pesticide uptake by plants, surface runoff, erosion, decay, volatilization, foliar washoff, advection, dispersion, and retardation.  Two options are available to solve the transport equations:  (1) the original backwards-difference implicit scheme that may be affected by excessive numerical dispersion at high Peclet numbers, or (2) the method of characteristics algorithm that eliminates numerical dispersion while slightly increasing model execution time.

PRZM has the capability to simulate multiple zones.  This allows PRZM and VADOFT to combine different root zone and vadose zone characteristics into a single simulation.  Zones can be visualized as multiple land segments joined together in a horizontal manner.  There are three reasons a user may choose for implementing multiple zones:

1)		to simulate heterogenous PRZM root zones with a homogeneous vadose zone

2)		to simulate a homogeneous root zone with heterogenous vadose zones

3)		to simulate multiple homogeneous root zones with multiple homogeneous vadose zones

Weighing multiple zones together and their use are discussed in detail in Section 5.

Another added feature is the ability to simulate as many as three chemicals simultaneously as separate compounds or as a parent-daughter relationship.  This gives the user the option to

observe the effects of multiple chemicals without making additional runs or the ability to enter a mass transformation factor from a parent chemical to one or two daughter products.

Predictions are made on a daily basis.  Output can be summarized for a daily, monthly, or annual period.  Daily time series values of various fluxes or storages can be written to sequential files during program execution for subsequent analysis.

1.3.1.2	Limitations--

There were significant limitations in the original (Release I) version of PRZM.  A few were obvious to the developers; others were pointed out subsequently by model users.  These are broken into four categories:

o		Hydrology

o		Soil hydraulics

o		Method of solution of the transport equation

o		Deterministic nature of the model

The Release II version of PRZM has been suitably modified to overcome many of these limitations.

Hydrologic and hydraulic computations are still performed in PRZM on a daily time step even though, for some of the processes involved (evaporation, runoff, erosion), finer time steps might be used to ensure greater accuracy and realism.  For instance, simulation of erosion by runoff depends upon the peak runoff rate, which is in turn dependent upon the time base of the runoff hydrograph.  This depends to some extent upon the duration of the precipitation event.  PRZM retains its daily time step primarily due to the relative availability of daily versus shorter time step meteorological data.  This limitation has  been mitigated, in part, by enhanced parameter guidance.

In PRZM, Release I, the soil hydraulics were simple--all drainage to field capacity water content was assumed to occur within 1 day.  (An option to make drainage time dependent also was included, but there is not much evidence to suggest that it was utilized by model users to any great extent.)  This had 1-day drainage assumption the effect, especially in deeper soils, of inducing a greater-than-anticipated movement of chemical through the profile.  While this representation of soil hydraulics has been retained in PRZM, the user has the option of coupling PRZM to VADOFT.  PRZM is then used to represent the root zone, while VADOFT, with a more rigorous representation of unsaturated flow, is used to simulate the thicker vadose zone.  The VADOFT code is discussed in more detail in a subsequent section.  For short distances from the soil surface to the water table, PRZM can be used to represent the entire vadose zone without invoking the use of VADOFT as long as no layers that would restrict drainage are present.

The addition of algorithms to simulate volatilization has brought into focus another limitation of the soil hydraulics representation.  PRZM simulates only advective, downward movement of water and does not account for diffusive movement due to soil water gradients.  This means that PRZM is unable to simulate the upward movement of water in response to gradients induced by evapotranspiration.  This process has been identified by Jury et al. (1984) as an important one for simulating the effects of volatilization.  However, the process would seem less likely to impact the movement of chemicals with high vapor pressures.  For these chemicals, vapor diffusion would be a major process for renewing the chemical concentration in the surface soil.

Another limitation of the Release I model was the apparent inadequacy of the solution to the transport equation in advection-dominated systems.  The backward difference formulation of the advection term tends to produce a high degree of numerical dispersion in such systems.  This results in overprediction of downward movement due to smearing of the peak and subsequent overestimation of loadings to groundwater.  In this new release, a new formulation is available for advection-dominated systems.  The advective terms are decoupled from the rest of the transport equation and solved separately using the method of characteristics (MOC).  The remainder of the transport equation is then solved as before, using the fully implicit scheme.  This approach effectively eliminates numerical dispersion with only a small increase in the computation time.  In low-advection systems, the MOC approach reduces to the original PRZM solution scheme, which becomes exact as velocities approach zero.

The final limitation is the use of field-averaged water and chemical transport parameters to represent spatially heterogeneous soils.  Several researchers have shown that this approach produces slower breakthrough times than are observed using stochastic approaches.  This concern has been addressed by adding the capability to run PRZM-2 in a Monte Carlo framework.  Thus, distributional, rather than field-averaged, values can be utilized as inputs that will produce distributional outputs of the relevant variables (e.g., flux to the water table).

The Special Actions option in PRZM-2 allows the user to output soil profile pesticide concentrations at user-specified times during the simulation period and to change selected model parameters to better represent chemical behavior and the impacts of agricultural management practices.  The required input format and parameters are specified in Section 4.

By using the 'SNAPSHOT' capability of Special Actions, the user can output the pesticide concentration profile, i.e., the total concentration in each soil compartment, for any user-specified day during the simulation period.  In this way, the user can run PRZM-2 with only monthly or annual output summaries and still obtain simulation results for selected days when field data were collected.  There is no inherent limit to the number of SNAPSHOTs that can be requested in a single run.  When more than one chemical is being simulated, the concentration profiles are provided by the order of the chemical number, i.e., NCHEM.

To better represent the expected behavior of the chemical being simulated, or the impacts of tillage or other agricultural practices, the following parameters can be reset to new values at any time during the simulation period:

Solution Decay Rate (DWRATE)

Sorbed Decay Rate (DSRATE)

Partition Coefficient (KD)

Bulk Density (BD)

Curve Number (CN)

USLE Cover Factor (USLEC)

Thus, for chemicals that demonstrate seasonal decay rates or partition coefficients, or different values for the period following application compared to later in the crop season, the appropriate parameters can be changed at user-specified times to mimic the observed, or expected, behavior of the compound.

Similarly, for agricultural practices or specific tillage operations that affect the soil bulk density, curve number, or cover factor, these parameter values can be altered during the simulation in an attempt to better represent their impacts.  The parameter guidance in Section 5 may help the user in determining adjustments for these parameters.  Users should note that adjustments to the bulk density, and possibly the partition coefficient, may affect the pesticide balance calculation.

1.3.2	Overview of the Vadose Zone Flow and Transport Model (VADOFT)

VADOFT is a finite-element code for simulating moisture movement and solute transport in the vadose zone.  It is the second part of the two-component PRZM-2 model for predicting the movement of pesticides within and below the plant root zone and assessing subsequent groundwater contamination.  The VADOFT code simulates one-dimensional, single-phase moisture and solute transport in unconfined, variably saturated porous media.  Transport processes include hydrodynamic dispersion, advection, linear equilibrium sorption, and first-order decay.  The code predicts infiltration or recharge rate and solute mass flux entering the saturated zone.  The following description of VADOFT is adapted from Huyakorn et al. (1988a).

1.3.2.1	Features--

The code, which employs the Galerkin finite-element technique to approximate the governing equations for flow and transport, allows for a wide range of nonlinear flow conditions.  Boundary conditions of the variably saturated flow problems may be specified in terms of prescribed pressure head or prescribed volumetric water flux per unit area.  Boundary conditions of the solute transport problem may be specified in terms of prescribed concentration or prescribed solute mass flux per unit area.  All boundary conditions may be time dependent.  An important feature of the algorithm is the use of constitutive relationships for soil water characteristic curves based on soil texture.

1.3.2.2	Limitations--

Major assumptions of the flow model are that the flow of the fluid phase is one-dimensional, isothermal and governed by Darcy's law and that the fluid is slightly compressible and homogeneous.  Hysteresis effects in the constitutive relationships of relative permeability versus water saturation, and water saturation versus capillary pressure head, are assumed to be negligible.

Major assumptions of the solute transport model are that advection and dispersion are one-dimensional and that fluid properties are independent of contaminant concentrations.  Diffusive/dispersive transport in the porous-medium system is governed by Fick's law.  The hydrodynamic dispersion coefficient is defined as the sum of the coefficients of mechanical dispersion and molecular diffusion.  Adsorption and decay of the solute is described by a linear equilibrium isotherm and a lumped first-order decay constant.  Parent/daughter chemical relationships may be simulated.

The code handles only single-phase flow (i.e., water) and ignores the presence of a second phase--i.e., air.  The code does not take into account sorption nonlinearity or kinetic sorption effects that, in some instances, can be important.  The code considers only single-porosity (granular) soil media.  It does not simulate flow or transport in fractured porous media or structured soils.

1.3.3  Overview of the Monte Carlo Simulation Module

MCARLO performs all the functions necessary to execute a Monte Carlo simulation.  It reads special data for parameters to be varied (e.g., distribution types and moments) and output variables to be observed, generates random numbers, correlates them and performs transformations, exchanges these generated values for PRZM-2 parameters, performs statistical analysis on the output variables, and writes out statistical summaries for the output variables.

The MCARLO module makes use of an input and output file.  Inputs to the MCARLO module are discussed in Section 4.  The user should be aware that many of the parameters entered in the Monte Carlo input file once designated as constants will be used in lieu of that same parameter value entered in the standard input file.

The final limitation is that only a small number of input variables may be changed at random by invoking the Monte Carlo routines.  It is not difficult to add additional variables, however.

1.3.4	Model Linkage

One of the more challenging problems in this model development effort was the temporal and spatial linkage of the component models.  In the section which follows, these linkages are discussed.

1.3.4.1	Temporal Model Linkage--

The resolution of the temporal aspects of the two models was straightforward.  PRZM runs on a daily time step.  The time step in VADOFT is dependent upon the properties of soils and the magnitude of the water flux introduced at the top of the column.  In order for the nonlinear Richards' equation to converge, VADOFT may sometimes require time steps on the order of minutes.

For the linkage of PRZM-2, through VADOFT the resolution of time scales is also straightforward.  VADOFT is prescribed to simulate to a "marker" time value, specifically to the end of a day.  The last computational time step taken by VADOFT is adjusted so that it coincides with

the end of the day.  PRZM's daily water fluxes are used as input to VADOFT.  VADOFT utilizes this flux as a constant over the day and adjusts its internal computational time step in order to converge.

1.3.4.2	Spatial Linkages--

The spatial linkages utilized for the models are more complex.  The principal problem is the presence of a fluctuating water table.  A second problem is that of the incompatibility between the hydraulics in PRZM and VADOFT.  Of course, any linking scheme utilized must provide a realistic simulation of the flow of water and transport of solutes at the interfaces and must ensure mass balance.

The major problem with the interfacing of these two models is that while VADOFT solves the Richards' equation for water flow in a variably saturated medium, PRZM uses simple "drainage rules" to move water through the soil profile.  Because of this incompatibility, there may be times when PRZM produces too much water for VADOFT to accommodate within one day.  This is very likely to happen in agricultural soils, where subsoils are typically of lower permeability than those of the root zone, which have been tilled and perforated by plant roots and soil biota.  The result of this would be water ponded at the interface which would belong neither to PRZM or VADOFT.

The solution was to prescribe the flux from PRZM into VADOFT so that VADOFT accommodates all the water output by PRZM each day.  This eliminates the problem of ponding at the interface.  However, it does force more water into the vadose zone than might actually occur in a real system, given the same set of soil properties and meteorological conditions.  The consequence is that water and solute are forced to move at higher velocities in the upper portions of the vadose zone.  If the vadose zone is deep, then this condition probably has little impact on the solution.  If it is shallow, however, it could overestimate loadings to groundwater, especially if chemical degradation rates are lower in the vadose zone than in the root zone.

1.3.5	Monte Carlo Processor

PRZM-2 can be run in a Monte Carlo mode so that probabilistic estimates of pesticide loadings to the saturated zone from the source area can be made.  The input preprocessor allows the user to select distributions for key parameters from a variety of distributions; the Johnson family (which includes the normal and lognormal), uniform, exponential and empirical.  If the user selects distributions from the Johnson family, he or she may also specify correlations between the input parameters.  The Monte Carlo processor reads the standard deterministic input data sets for each model, then reads a Monte Carlo input file that specifies which parameters are to be allowed to vary, their distributions, the distribution parameters, and correlation matrix.  The model then executes a prespecified number of runs.

The output processor is capable of preparing statistics of the specified output variables including mean, maximum values and quantiles of the output distribution.  The output processor also can tabulate cumulative frequency histograms of the output variables and send them to a line printer for plotting.

1.3.6	Overview Summary

A modeling system (PRZM-2) has been developed for the U.S. Environmental Protection Agency that is capable of simulating the transport and transformation of pesticides, following application, down through the crop root zone and underlying vadose zone.  The modeling system was designed to handle a variety of geometries likely to be encountered in performing evaluations for pesticide registration or special reviews.  A major objective was to keep the model simple and efficient enough so that it could be operated on an IBM-PC or IBM-compatible PC and used in a Monte Carlo mode to generate probabilistic estimates of pesticide loadings or water concentrations.  The model consists of two major computational modules--PRZM, which performs pollutant fate calculations for the crop root zone and is capable of incorporating the effects of management practices and VADOFT, which simulates one-dimensional transport and transformation within the vadose zone.

Linkage of these models is accomplished through the use of simple bridging algorithms that conserve water and solute mass.

SECTION 2

MODEL DEVELOPMENT, DISTRIBUTION, AND SUPPORT

NOTE:  Refer to the READ.ME file for the latest supplemental information, changes, and/or additions to the PRZM-2 model documentation.  A copy of the READ.ME file is included on each distribution diskette set or it can be down loaded from the Center for Exposure Assessment Modeling (CEAM) electronic bulletin board system (BBS).  It can be installed on a hard disk using the INSTALL (diskette) or INSTALP2 (BBS) program.  It is an ASCII (non-binary) text file that can be displayed on the monitor screen by using the DOS TYPE command (e.g., TYPE READ.ME) or printed using the DOS PRINT command (e.g., PRINT READ.ME).

The READ.ME file contains a section entitled File Name and Content that provides a brief functional description of each PRZM-2 file by name or file name extension type.  Other sections in this document contain further information about

o	system development tools used to build the microcomputer release of the PRZM-2 model system

o	recommended hardware and software configuration for execution of the model and all support programs

o	routine program execution

o	minimum file configuration

o	run time and performance

o	program modification

o	technical help contacts

2.1  DEVELOPMENT AND TESTING

The PRZM-2 model system was developed and tested on a Digital Equipment Corporation (DEC) VAX 6310 running under version 5.4-2 of the VMS operating system (OS) and version 5.5-98 of VAX VMS FORTRAN-77, and an Advanced Logic Research (ALR) 486/25 microcomputer running under version 4.00 of IBM PC DOS and version 2.51 of Salford FORTRAN (FTN77/486).

The following FORTRAN tools also were used to perform static evaluations of the PRZM-2 FORTRAN code on an IBM PS/2 Model 8085-071 running under version 3.3 of IBM PC DOS, MICRO EXPRESS (ME) 486/25 and 486/33 systems running under version 5.00 of Microsoft (MS) DOS, and a Sun SPARCstation 1+GX running version 4.1.1 of UNIX/SunOS:

o Lahey				- F77L, F77L-EM/32 versions 5.01,

4.02 (DOS, ext DOS)

o Microsoft			- MSFORT version 5.00 (OS/2, DOS)

o Ryan-McFarland	- RMFORT versions 2.45, 3.10.01 (DOS)

o Salford				- FTN77/386 version 2.50 (DOS,

extended DOS)

o Silicon Valley

Systems		- SVS-77/386 version  2.81 (DOS,

extended DOS)

o Sun				- (UNIX/SunOS, version 1.4)

o Waterloo			- Watcom-77/386 version  8.5E (DOS,

extended DOS)

In addition to the VAX and ALR systems, PRZM-2 has also been successfully executed on a PRIME 50 Series minicomputer running under PRIMOS, the Sun SPARCstation, and the IBM PS/2 Model 8085-071.

The distribution version of the PRZM-2 model system is built with the Lahey FORTRAN (F77L-EM/32) extended (i.e., protected) mode FORTRAN compiler and link editor, version 5.01.  Refer to section 2.4.2 for specific hardware and software run time requirements for the host system for the PRZM-2 model system.

2.2  DISTRIBUTION

The PRZM-2 model system and all support files and programs are available on diskette from CEAM, located at the U.S. EPA Environmental Research Laboratory, Athens, Georgia, at no charge.  The CEAM has an exchange diskette policy.  It is preferred that diskettes be received before sending a copy of the model system (refer to section 2.3, Obtaining a Copy of the PRZM-2 Model System).

Included in a distribution diskette set are

o	PRZM-2 general execution and user support guide (READ.ME) file

o	interactive installation program (refer to section 2.5, Installation)

o	test input and output files for installation verification

o	executable task image file for the PRZM-2 model system

o	FORTRAN source code files

o	command and/or "make" files to compile, link, and run the task image file (PRZM2.EXE)

A FORTRAN compiler and link editor are NOT required to execute any portion of the model.  If the user wishes to modify the model, it will be up to the user to supply and/or obtain

o	an appropriate text editor that saves files in ASCII (non-binary) text format

o	FORTRAN development tools to recompile and link edit any portion of the model

CEAM cannot support, maintain, and/or be responsible for modifications that change the function and/or operational characteristics of the executable task image, MAKE, or DOS command files supplied with this model package.

The microcomputer release of the PRZM-2 model is a full implementation of the VAX/VMS version.  The microcomputer implementation of this model performs the same function as the U.S. EPA mainframe/minicomputer version.

2.3  OBTAINING A COPY OF THE PRZM-2 MODEL

NOTE:  k=1,024; m=1,048,576; b=1 byte

2.3.1 Diskette

To obtain a copy of the PRZM-2 distribution model system on diskette, send

o	the appropriate number of double-sided, double-density (DS/DD 360kb) 5.25 inch, or double-sided, high-density (DS/HD 1.44mb) 3.5 inch error-free diskettes

NOTE:	To obtain the correct number of diskettes, contact CEAM at 706/546-3549.

o	a cover letter, with a complete return address, requesting the PRZM-2 model to:

Model Distribution Coordinator

Center for Exposure Assessment Modeling

Environmental Research Laboratory

U.S. Environmental Protection Agency

960 College Station Road

Athens, GA  30606-2720

Program and/or user documentation, or instructions on how to order documentation, will accompany each response.

2.3.2  Electronic Bulletin Board System (BBS)

To down load a copy of the PRZM-2 model system, or to check the status of the latest release of this model or any other CEAM software product, call the CEAM BBS 24 hours a day, 7 days a week.  To access the BBS, a computer with a modem and communication software is needed.  The phone number for the BBS is 706/546-3402.  Communication parameters for the BBS are

o	300/1200/2400/9600 baud rate

o	8 data bits

o	no parity

o	1 stop bit

2.4  GENERAL/MINIMUM HARDWARE AND SOFTWARE INSTALLATION AND RUN TIME REQUIREMENTS

NOTE:  Refer to the READ.ME file for the latest supplemental and more complete information, changes, and/or additions concerning specific hardware and software installation and run time requirements.

2.4.1 Installation Requirements

o	3.5 inch, 1.44mb diskette drive, or 5.25 inch, 360kb diskette drive

o	hard disk drive

o	approximately 4.5mb free hard disk storage

2.4.2 Run Time Requirements

o	386 or 486 compatible microcomputer

o	MS or PC DOS version 3.30 or higher

o	640k base memory

o	4mb of extended (XMS) memory

o	4.5mb free hard disk storage

Refer to READ.ME file for suggested modification of the CONFIG.SYS and/or

AUTOEXEC.BAT DOS system configuration and start-up files.

2.5  INSTALLATION

To install the PRZM-2 model system and/or related support files on a hard disk, insert the first distribution diskette in a compatible diskette drive (refer to section 2.4).  Then type

A:\INSTALL or B:\INSTALL

at the DOS system prompt and press the <Enter> key.  Then follow instructions and respond to prompts presented on the monitor screen by the interactive installation program.  Complete installation instructions are also printed on each external diskette label.

NOTE:  To install the PRZM-2 model system and/or related support files on a hard disk from an interactive, self-extracting installation program down loaded from the CEAM BBS or through Internet, type

INSTALP2

at the DOS system prompt then press the <Enter> key.  This assumes that the current default drive and sub-directory is the same as the drive and sub-directory where the file INSTALP2.EXE is stored.  Then follow instructions and respond to prompts presented on the monitor screen by the interactive installation program.

The PRZM-2 distribution diskette sets and BBS files implement software product installation standards to insure the most error-free, maintainable, and user-acceptable distribution of CEAM products.  It has a unique menu option, command, full-screen (interactive), diagnostic, error-recovery, help, and selective installation capabilities using state-of-the-art human-factors engineering practices and principles.

NOTE:  The contents of the distribution diskettes can be copied to another set of "backup" diskettes using the DOS DISKCOPY command.  Refer to the DOS Reference Manual for command application and use.  The "backup" diskettes must be the same size and storage density as the original source diskettes.

2.6  INSTALLATION VERIFICATION AND ROUTINE EXECUTION

Refer to the following sections in the READ.ME file for complete instructions concerning installation verification and routine execution of the PRZM-2 model:

o	File Name and Content

o	Routine Execution

o	Run Time and Performance

o	Minimum File Configuration

2.7  CODE MODIFICATION

Included in the diskette set are

o	an executable task image file for the PRZM-2 model system

o	FORTRAN source code files

o	command and/or "make" files to compile, link, and run the task image file (PRZM2.EXE)

If the user wishes to modify the model or any other program, it will be up to him or her to supply and/or obtain

o	an appropriate text editor that saves files in ASCII (non-binary) text format

o	FORTRAN development tools to recompile and link edit any portion of the model

CEAM cannot support, maintain, and/or be responsible for modifications that change the function of any executable task image (*.EXE), DOS batch command (*.BAT), and/or "make" utility file(s) supplied with this model package.

2.8  TECHNICAL HELP

For questions and/or information concerning

o	installation and/or testing of the PRZM-2 model system and/or support programs or files, call 706/546-3590 for assistance

o	PRZM-2 model and/or program content, application, and/or theory, call 706/546-3210 for assistance

o	use of the CEAM electronic bulletin board system (BBS), contact the BBS system operator (SYSOP) at 706/546-3590

o	CEAM software and distribution Quality Assurance and Control, call 706/546-3125

o	other environmental software and documentation distributed through CEAM, contact the Model Distribution Coordinator at 706/546-3549

o	other support available through CEAM, contact Mr. Dermont Bouchard, CEAM Manager

o	by mail at the following address

Center for Exposure Assessment Modeling (CEAM)

Environmental Research Laboratory

U.S. Environmental Protection Agency

960 College Station Road

Athens, Georgia  30605-2720

o	by telephone at 706/546-3130

o	by fax at 706/546-2018

o	through the CEAM BBS message menu and commands.  The CEAM BBS communication parameters and telephone number are listed above (section 2.3.2).

To help technical staff provide better assistance, write down a response to the following topics before calling or writing.  If calling, be at the computer, with the computer on, and in the proper sub-directory (e.g., \PRZM2) when the call is placed.

o	program information:

-	describe the problem, including the exact wording of any error and/or warning message(s)

-	list the exact steps, command(s), and/or keyboard key sequence that will reproduce the problem machine information:

o	machine information:

-	list computer brand and model

-	list available RAM (as reported by DOS CHKDSK command)

-	list extended memory present and free (XMS)

-	list name and version of extended memory (XMS) manager (i.e., HIMEM, VDISK, RAMDRIVE, etc.)

-	list available hard disk space (as reported by DOS CHKDSK command)

-	list the brand and version of DOS (as reported by DOS VER command)

-	list the name of any memory resident (TSR) program(s) installed

-	printer brand and model

-	monitor brand and model

NOTE: If contacting CEAM by mail, fax, or BBS, include responses to the above information in your correspondence.

2.9  DISCLAIMER

Mention of trade names or use of commercial products does not constitute endorsement or recommendation for use by the United States Environmental Protection Agency.

Execution of the PRZM-2 model system, and modifications to the DOS system configuration files (i.e., \CONFIG.SYS and \AUTOEXEC.BAT) must be used and/or made at the user's own risk.  Neither the U.S. EPA nor the program authors can assume responsibility for model and/or program modification, content, output, interpretation, or usage.

CEAM software products are built using FORTRAN-77, assembler, and operating system interface command languages.  The code structure and logic of these products is designed for single-user, single-tasking, non-LAN environment and operating platform for microcomputer installations (i.e., single user on dedicated system).

A user will be on their own if he/she attempts to install a CEAM product on a multi-user, multi-tasking, and/or LAN based system (i.e., Windows, DESQview, any LAN).  CEAM cannot provide installation, operation, and/or general user support under any combination of these configurations.  Instructions and conditions for proper installation and testing are provided with the product in a READ.ME file.  While multiuser/multitasking/LAN installations could work, none of the CEAM products have been thoroughly tested under all possible conditions.  CEAM can provide scientific and/or application support for selected products if the user proves that a given product is installed and working

correctly.

2.10  TRADEMARKS

o	F77L is a registered trademark of Lahey Computer Systems, Inc.  All other Lahey products are trademarks of Lahey Computer Systems, Inc.

o	IBM, Personal Computer/XT (PC/XT), Personal Computer/AT (PC/AT), PC DOS, VDISK, and Personal System/2 (PS/2) are registered trademarks of International Business Machines Corporation

o	DESQview is a trademark of Quarterdeck Office Systems, Inc.

o	Sun and SunOS are registered trademarks of Sun Microsystems, Inc.

o	SPARC is a registered trademark of SPARC International, Inc.

o	UNIX is a registered trademark of American Telephone and Telegraph

o	SVS FORTRAN-77 is a trademark of Silicon Valley Software

o	PRIME and PRIMOS are trademarks of Prime Computers, Inc.

o	Microsoft, RAMDRIVE, HIMEM, MS, and MS-DOS are registered trademarks of Microsoft Corporation

o	Windows is a trademark of Microsoft Corporation

o	RM/FORTRAN is a trademark of Language Processors, Inc.

o	DEC, VAX, VMS, and DCL are trademarks of Digital Equipment Corporation

o	386 is a trademark of Intel Corporation

o	US Robotics is a registered trademark and Courier HST is a trademark of U.S.Robotics, Inc.

SECTION 3

MODULES AND LOGISTICS

The PRZM-2 model consists of four major modules.  These are:

o	EXESUP, which controls the simulation

o	PRZM, which performs transport and transformation simulations for the root zone

o	VADOFT, which performs transport and transformation simulations for the vadose zone

o	MONTE CARLO, which performs sensitivity analysis by generating random inputs

In this section, Table 3-1 gives a listing of all subroutines and functions organized by module calling routines.  Table 3-2 gives a listing of all parameter files and their dimensions.  A brief description for each listing is also given.

TABLE 3-1.	LIST OF SUBROUTINES AND FUNCTIONS AND A BRIEF DESCRIPTION OF THEIR PURPOSE.

MODULE	SUBROUTINE

CALLING	    or

ROUTINE	FUNCTION		PURPOSE

EXESUP

INIT			initilizes common block CONST.INC

ECHOF		echo names of files opened.

ENDDAY	used to determine julian day and simulation progress.

FILOPN		opens and assigns file unit numbers.

ECHOGD		echoes global data input.

DONBAR		calculates percent complete bar.

ADDSTR		add string to end of existing string.

INPREA		reads and initializes program input.

BMPCHR		converts character to uppercase.

CENTER		centers string message on screen.

COMRD		checks input for end of file.

COMRD2		checks input for comment lines.

COMRD3		checks input for END statement.

DISPLAY		display data to echo file and screen.

ECHORD		echoes line numbers read from input.

ELPSE		add trailing string and fill middle.

ERRCHK		write error messages.

EXPCHK	check argument for exponential limits.

FILCLO		closes open files.

OPECHO		flags the printing utility.

RELTST		checks argument as a real number.

SQRCHK	gives square root with error checking.

SUBIN		tracks entry into a subroutine.

SUBOUT		tracks exit from a subroutine.

TRCLIN		writes subroutine tracking to screen.

SCREEN		controls display to screen.

LFTJUS		left justifies a character string.

LNCHK		takes natural log of a number.

LNGSTR		returns length of a character string.

LOGCHK	takes base 10 logarithm of a number with error checking 			provided.

NAMFIX	left justifies and capitalizes a string.

CLEAR		clears the display screen.

FILCHK		checks that necessary files are open.

EXESUP	controls calls to PRZM, VADOFT ,and MONTE

CARLO.

INITEM		determines global data.

FILINI		initializes file unit numbers.

PRZM2		controls model calling routines.

LSUFIX		performs internal reads.

PRZM

BIODEG	perform time dependant solution for microbiodegradation.

SLPST1	set up coefficient matrix for the solution of pesticide

TABLE 3-1.  (Continued)

MODULE	SUBROUTINE

CALLING	    or

ROUTINE	FUNCTION		PURPOSE

transport.

PRZMRD		reads PRZM input file.

HYDR2		perform soil hydraulic calculations.

PLGROW	determines plant growth parameters for use in other subroutines.

FARM

insures pesticide application is applied during adequate 	moisture conditions.

INIDAT	provides common block CMISC.INC values.

TRDIA1		solves tridiagonal maxtrix.

HYDROL	calculates snowmelt, crop interception, runoff, and infiltra			tion.

HFINTP	determines boundary for head, concentration or flux.

PESTAP	computes amount of pesticide application.

PLPEST	determines amount of pesticide which disappears by first 			order decay and pesticide washoff.

SLPST0	sets up the matrix for transport of pesticide.

CANOPY	calculates the overall vertical transport resistance.

MOC		solves the advection component of the pesticide transport 			process.

MASBAL	calculates mass balance error terms for both flow and transport.

PSTLNK	provides linkage for transformation and source terms of 			parent/daughter.

OUTCNC	prints daily, monthly, and annual pesticide concentration 			profiles.

TRDIAG		solves tridiagonal matrix.

OUTRPT	prints daily, monthly, and annual concentration profiles plus 			snapshots.

VALDAT	checks simulation dates against calendar dates.

XPRZM		performs PRZM execution calls.

INITDK		initializes amount of pesticide decay each chemical which 			could have daughter products.

OUTPST	prints daily, monthly, and annual pesticide flux profiles.

INITL			initializes PRZM arrays.

OUTTSR	prints daily, monthly, and annual time series data.

OUTHYD		accumulates summaries for water flow.

HYDR1	performs hydraulic calculations assuming a uniform soil 			profile.

PRZECH		echoes PRZM input to files.

RSTPUT		writes PRZM input to a restart file.

RSTGET		reads PRZM input from a restart file.

RSTPT1		writes PRZM input to a restart file.

RCALC		function to compute biodegradation.

RSTGT1		reads PRZM input from a restart file.

TABLE 3-1.  (Continued)

MODULE	SUBROUTINE

CALLING	    or

ROUTINE	FUNCTION		PURPOSE

PRZEXM		creates input file for EXAMS model.

PRZDAY	transfers start and end dates to common block.

THCALC		computes moisture for PRZM.

INIACC		initializes PRZM storage arrays.

KDCALC		computes KD.

MCPRZ	computes MONTE CARLO inputs for PRZM.

FNDCHM		function to find a chemical number.

FNDHOR		function to find a horizon number.

PZCHK	checks horizonal values for consistency.

KHCORR		corrects Henry's law constant.

ACTION		performs special actions.

GETMET		reads in meteorological data.

IRRIG			performs irrigation algorithm.

FURROW		computes furrow irrigation.

INFIL			computes Green-Ampt infiltration.

EVPOTR		computes evapotranspiration.

EROSN		computes erosion losses.

SLTEMP		calculates soil temperatures.

PRZM			performs calls to PRZM routines.

TDCALC	calculates total days in a simulation.

VADOFT

VADCAL	calls relevant subroutines to compute nodal head and 			concentration.

BALCHK		mass balance calculation.

READTM		reads in HVTM, TMHV, QVTM from input.

VADINP		reads in flow and transport input.

TRIDIV		performs tridiagonal matrix solution.

VADOFT	saves information between flow and transport.

IRDVC		reads in integer vectors.

VSWCOM	computes nodal values of water saturation and Darcy 			velocities.

VADCHM	transfers chemical specific data to VADOFT variables.

INTERP	performs linear interpolation using tabulated data of 			relative permeability versus water saturation.

SWFUN	computes water saturation values for grid element.

PKWFUN		computes relative permeability.

DSWFUN		computes moisture capacity.

XTRANS		controls transport calling routines.

RDPINT		reads non-default nodes data.

VARCAL	computes nodal head and concentration values.

ASSEMF		assembly routine for flow.

VADPUT		writes VADOFT input to restart file.

VADGET	reads VADOFT input from a restart file.

ASSEMT		assembly routine for transport.

XFLOW		controls flow calling routines.

TABLE 3-1.  (Continued)

MODULE	SUBROUTINE

CALLING	    or

ROUTINE	FUNCTION		PURPOSE

MCVAD	determines MONTE CARLO variables for VADOFT.

READVC		reads in vectors.

CONVER	computes the limiting values of water saturation for each 			material.

MONTE CARLO

MTPV			calculates vectors.

OUTPUT		write summary statistics.

INITMC	initializes statistical summation arrays.

DECOMP	decomposes the matrix BBT (N by N) into a lower triangular 			form.

RANDOM		controls random numbers generation.

NMB		generates normal (0-1) random numbers.

UNIF	generates uniform random numbers.

EXPRN	generates exponentially distributed random numbers.

EMPCAL	generates values from empirical distributions.

TRANSM	converts normally distributed correlated vectors to the 			parameter set returned to the model.

TRANSB	transforms variables from normal space to SB space or vice-			versa.

OUTFOR	writes tables and plots of cumulative distribution.

STOUT	initializes the amount of pesticide decay.

FRQTAB		prints tabular frequency output.

FRQPLT		plots cumulative distributions.

MCECHO		echoes MONTE CARLO input.

READM		reads in MONTE CARLO input.

MAXAVG	computes maximum daily average output.

STATIS		performs summations for MONTE CARLO.

TABLE 3-2.	LIST OF ALL PARAMETERS FILES, PARAMETER DIMENSIONS, AND A BRIEF DESCRIPTION.

FILE	PARAMETER			DESCRIPTION

CTRACE.INC	MAXSUB=50	maximum number of subroutines.

MAXLIN=10	maximum number of lines for trace option.

PMXMAT.INC	MXMAT=5	maximum number of VADOFT materials.

PMXNLY.INC	MXNLAY=20		maximum number of layers in VADOFT.

PMXPRT.INC	MXPRT=100		maximum number of VADOFT observation 			nodes.

PMXTIM.INC	MXTIM=31	maximum number of VADOFT iterations 			allowed.

PMXTMV.INC	MXTMV=31		maximum number of VADOFT time interpola			tion values.

PMXVDT.INC	MXVDT=31

maximum number of VADOFT time steps.

PCMPLR.INC	REALMX=1.0D+30		maximum real number.

REALMN=1.0D-30		minimum real number.

MAXINT=2147483647		maximum integer value.

MAXREC=512			maximum record length.

EXNMX=-53.0	maximum negative exponential number.

EXPMN=REALMN	minimum exponential real number.

EXPMX=53.0	maximum positive exponential number.

WINDOW=.TRUE.	allows screen window on or off.

PCASCI=.TRUE.	allows attributes for PC's for displays.

NONPC=.FALSE.	allows attributes for non-PC's for displays.

PMXNOD.INC	MXNOD=100			maximum number of VADOFT nodes 								allowed.

PMXZON.INC	MXZONE=10		maximum number of PRZM zones.

PPARM.INC	NCMPTS=100	maximum number of compartments in PRZM.

NAPP=50		maximum number of applications in PRZM.

NC=5			maximum number of crops allowed in PRZM.

NPII=800		maximum number of PRZM particles in MOC.

NCMPP2=NCMPTS+2	maximum number of compartments plus 2 for top and bottom ends.

MXCPD=100		maximum number of cropping periods.in 			PRZM.

PIOUNI.INC	KUOUT=6			screen unit number.

NMXFIL=99			maximum number of file units open.

FILBAS=30			base file unit number.

PMXNSZ.INC	MXNSZO=10		maximum number of VADOFT zones allowed.

CMCRVR.INC	MCMAX=50		maximum number of random input variables.

NMAX=10		maximum number of summary output 			variables.

NCMAX=10			maximum number of CDF's.

NRMAX=1000	maximum number of MONTE CARLO runs.

NEMP=20		maximum number of empirical distributions.

MCSUM=MCMAX+NMAX	maximum number of random input and output variables.

NPMAX=5	maximum length of MONTE CARLO averaging periods.

SECTION 4

INPUT PARAMETERS FOR PRZM-2

This section describes the development of the input data files used in the Execution Supervisor (PRZM2.RUN), PRZM, VADOFT, and MONTE CARLO.  All of these files, except for the meteorological file, may have embedded comment lines.  A comment line is any line beginning with three asterisks (***).  These lines are ignored by the code during execution.  For best accuracy and process time, a text or line editor is recommended for inputing file records.  To better understand record formats used in model input, an example record format statement appears below:

FORMAT	3I2,2X,F8.0,E10.3,1X,2(I5,1X,F8.0)

where input would look like:

010181     0.340   2.40E00     1    0.340    1    0.340

The format identifier, 3I2, specifies there are three integers with 2 columns each.  The format identifier, 2X, specifies there are two blank spaces.  The format identifier, F8.0, specifies there is one floating point field with eight columns and also a decimal point with no precision (although up to 7 seven of these columns may be points of precision with the eighth column being the decimal point since this is a FORTRAN read statement).  The format identifier, E10.3, specifies there is one field of ten columns that may include an exponential suffix.  The format identifier, 2(I5,1X,F8.0), specifies that there are two sequential sets of I5,1X,F8.0 entered. All format specifiers should be right justified so that unused columns in a field are assumed to be zeros by the code.

4.1  INPUT FILE DESCRIPTIONS

The Execution Supervisor (PRZM2.RUN) is used to define:  1) which modules are chosen for simulation; 2) the number of zones used in a simulation; 3) input, output, and scratch file names with optional path statements; 4) the starting and ending date of a simulation; 5) the number of chemicals (either separate or daughter); 6) weighting parameters between PRZM and VADOFT zones; 7) and global echo and trace levels during execution.

PRZM, VADOFT, and MONTE CARLO input files consist of various title and FORTRAN-formatted records.  Each of these module files along with their examples are discussed in the following pages.  For further descriptions, see Section 5 on parameter estimation.

4.1.1  Meteorological File

The PRZM-2 model requires use of a meteorological file that is specified in the execution supervisor.  Information on daily precipitation, pan evaporation, temperature, wind speed, and solar radiation is included in each record of the meteorological file.  These files are from the National Oceanic and Atmospheric Administration (NOAA) and are available from the Athens-ERL.  An example file format is shown below:

RECORD FORMAT:	1X,3I2,5F10.0

READ STATEMENT:	MM, MD, MY, PRECIP, PEVP, TEMP, WIND, SOLRAD

where

MM		= meteorological month

MD	= meteorological day

MY	= meteorological year

PRECIP	= precipitation (cm day-1)

PEVP	= pan evaporation data (cm day-1)

TEMP	= temperature (celsius)

WIND	= wind speed (cm sec-1)

SOLRAD	= solar radiation (Langleys)

4.1.2  Execution Supervisor File (PRZM2.RUN)

The PRZM-2 model requires existence of a control file (PRZM2.RUN) also known as the execution supervisor file.  This file specifies options by the user to control the overall (global) parameters during model execution.  The file must always be resident in the current directory where the execution is performed.  On the following pages are examples of the execution supervisor input file.

4.1.2.1  Example Execution Supervisor (PRZM2.RUN) input file

ONE ZONE

*** option records

PRZM		ON

VADOFT		ON

MONTE CARLO		OFF

TRANSPORT SIMULATION	ON

*** zone records

PRZM ZONES		1

VADOFT ZONES		1

ENDRUN

*** input file records

PATH                   		D:\PRZM2\INPUT\

MCIN		MC.INP

METEOROLOGY	1	MET.INP

PRZM INPUT	1	PRZM3.INP

VADOFT INPUT	1	VADF3.INP

*** output file records

PATH		D:\PRZM2\OUTPUT\

TIME SERIES	1	TIMES.OUT

PRZM OUTPUT	1	PRZM.OUT

VADOFT OUTPUT	1	VADF.OUT

MCOUT		MC.OUT

MCOUT2		MC2.OUT

*** scratch file records

PRZM RESTART	1	RESTART.PRZ

VADOFT FLOW RS	1	VFLOW.RST

VADOFT TRANS RST	1	VTRANS.RST

VADOFT TAPE10	1	VADF.TAP

ENDFILES

*** global records

START DATE		010181

END DATE		311283

NUMBER OF CHEMICALS	3

PARENT OF 2		1

PARENT OF 3		2

ENDDATA

*** display records

ECHO		4

TRACE		OFF

NOTE:  Three asterisks (***) denote a comment line and are ignored by the program.

4.1.2.2  Example Execution Supervisor (PRZM2.RUN) input file

TWO ZONES WITH MONTE CARLO OPTION

***Options

PRZM		ON

VADOFT		ON

MONTE CARLO		ON

TRANSPORT SIMULATION	ON

PRZM ZONES		2

VADOFT ZONES		2

ENDRUN

***Input files

MCIN		MC.INP

METEOROLOGY	1	MET.INP

METEOROLOGY	2	METx.INP

PRZM INPUT	1	PRZM.INP

PRZM INPUT	2	PRZMx.INP

VADOFT INPUT	1	VADF.INP

VADOFT INPUT	2	VADFx.INP

***Output files

TIME SERIES	1	TIMES.OUT

TIME SERIES	2	TIMESx.OUT

PRZM OUTPUT	1	PRZM.OUT

PRZM OUTPUT	2	PRZMx.OUT

VADOFT OUTPUT	1	VADF.OUT

VADOFT OUTPUT	2	VADFx.OUT

MCOUT		MC.OUT

MCOUT2		MC2.OUT

***Scratch files

PRZM RESTART	1	RESTART.PRZ

PRZM RESTART	2	RESTARTx.PRZ

VADOFT FLOW RST	1	VFLOW.RST

VADOFT FLOW RST	2	VFLOWx.RST

VADOFT TRANS RST	1	VTRANS.RST

VADOFT TRANS RST	2	VTRANSx.RST

VADOFT TAPE10	1	VADF10.TAP

VADOFT TAPE10	2	VADF10x.TAP

ENDFILES

START DATE		010181

END DATE		311281

NUMBER OF CHEMICALS	3

PARENT OF 2		1

PARENT OF 3		2

WEIGHTS

1.0          0.0

0.0          1.0

ENDDATA

ECHO					ON

TRACE

OFF

NOTE:  Three asterisks (***) denote a comment line and are ignored by the program.

4.1.2.3  Execution Supervisor (PRZM2.RUN) Input Guide

RECORD 1 - OPTIONS	FORMAT	A18,6X,A56

LABEL (Col. 1-18)		EXECUTION STATUS (Col. 25-78)

PRZM				ON or OFF	(the root zone model execution)

VADOFT			ON or OFF	(the vadose zone model execution)

MONTE CARLO		ON or OFF	(Monte Carlo execution)

TRANSPORT			ON or OFF	(vadose zone transport execution)

RECORD 2 - ZONES	FORMAT	A18,6X,I2

LABEL (Col. 1-18)		ZONE NUMBER (Col. 25-78)

PRZM ZONES			1 to 10	(total number of PRZM land zones)

VADOFT ZONES		1 to 10	(total number of VADOFT land zones)

ENDRUN			-------	(specifies end of OPTIONS and ZONE records)

RECORD 3 - INPUT FILES	FORMAT	A18,1X,I2,3X,A56

LABEL (Col. 1-18)		ZONE NUMBER (Col. 20-21)	NAME (Col. 25-78)

PATH				-------			directory (optional)

METEOROLOGY		1 to 10			filename

PRZM INPUT			1 to 10			filename

VADOFT INPUT		1 to 10			filename

MCIN				-------			filename

RECORD 4 - OUTPUT FILES	FORMAT	A18,1X,I2,3X,A56

LABEL (Col. 1-18)		ZONE NUMBER (Col. 20-21)	NAME (Col. 25-78)

PATH				-------			directory (optional)

TIME SERIES			1 to 10			filename

PRZM OUTPUT		1 to 10			filename

VADOFT OUTPUT		1 to 10			filename

MCOUT			1 to 10			filename

MCOUT2			1 to 10			filename

RECORD 5 - SCRATCH FILES    FORMAT	A18,1X,I2,3X,A56

LABEL (Col. 1-18)		ZONE NUMBER (Col. 20-21)	NAME (Col. 25-78)

PATH				-------			directory (optional)

PRZM RESTART		1 to 10			filename

VADOFT FLOW RESTART	1 to 10			filename

VADOFT TRANS RESTART	1 to 10			filename

VADOFT TAPE		1 to 10			filename

ENDFILES			------- (specifies end of file name records)

RECORD 6 - GLOBAL RECORDS   FORMAT	A18,1X,3I2

LABEL (Col. 1-18)	VALUE (Col. 20-25)

START DATE		ddmmyy	(starting day, month, year)

END DATE		ddmmyy	(ending day, month, year)

NUMBER OF CHEMICALS	1 to 3	(number of chemicals)

PARENT OF 2		1		(parent of the second chemical if TRANSPORT=ON and if more than one chemical)

PARENT OF 3		1 or 2	(parent of third chemical if TRANSPORT=ON and if more than one chemical)

WEIGHTS			------	(indicates next values are weights)

NOTE:	enter next lines only if PRZM or VADOFT have multiple zones.  Enter a line for every increasing PRZM zone containing a fractional weight to each VADOFT zone.  FORMAT: 10(F8.2)

1.0		0.0	(PRZM zone 1 weight to VADOFT zone 1 and 2)

0.0		1.0	(PRZM zone 2 weight to VADOFT zone 1 and 2)

ENDDATA			------	(specifies end of GLOBAL data)

RECORD 7 - DISPLAY RECORDS   FORMAT	A18,6X,A56

LABEL (Col. 1-18)		VALUE (Col. 25-78)

ECHO				1 to 9	(amount output increasingly displayed to the screen and to files)

TRACE				ON or OFF	(tracking of subroutines for debugging)

EFFECT OF THE ECHO LEVEL ON MODEL OUTPUT

ECHO LEVEL	1	2	3	4	5	6	7	8	9

Percent bar graph	X	X	X	X	X	X	X	X	X

Simulation status to screen		X	X	X	X	X	X	X	X

Simulation status to files			X	X	X	X	X	X	X

Subroutine trace available				X	X	X	X	X	X

Warnings displayed					X	X	X	X	X

Results of linkage routines						X	X	X	X

Detailed water/solute data							X	X	X

Detailed head/concentration data							X	X

Echo of line being read from input							X	X

Echo of image being read from input								X

4.1.3  PRZM Input File

The PRZM-2 model requires a PRZM input file if the PRZM option is specified "ON" in the execution supervisor file.  The following page shows an example PRZM input file with various options implemented as a reference.

4.1.3.1 Example PRZM.INP input file for PRZM-2

3 CHEMICALS, 2 HORIZONS, EROSION, IRRIGATION, PRZM INPUT FOR ZONE 1

HYDROLOGY PARAMETERS (CROP DATA FROM USDA NO.283 HANDBOOK)

0.72	0.00	2	15.000	1	3

9.6	9.7	12.2	13.61	5.4	1	5.5

15.7	14.5	12.5	11.3	9.5	9.0

1

0.15	0.14	1.0	2.0	5.8

1

1	0.15	15.0	80.000	3	86	78	82	0.1	0.1	0.1	60.0

1

110582	300982	151082	1

PESTICIDE TRANSPORT AND TRANSFORMATION AND APPLICATION PARAMETERS

2	3	0

ALDICARB            ATRAZINE            CARBOFURAN

120582	0	2.5	2.5	2.5	1.00	1.00	1.00

120682	0	2.5	2.5	2.5	1.00	1.00	1.00

1	1

SOILS PARAMETERS

45.0	0.3	0	0	0	0	0	1		1	1	0

4.3E3	0.0E00	2.5E-7	0.0E00	5.5E-7	5.5E-3	5.5E-7

3	0.25	0.55	.78

0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.97 10.0

8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3

2

1	15.0	1.45	0.233	 0.0	0.0	0.0	0.0

0.020	0.000	0.000	0.014	0.014	0.000	0.023	0.230	0.000

0.5	.233	.050	1.0	.1	1.	.3

8.3	10.0	60.0	0.0	0.0

0.000	0.000	0.000

2	30.0	1.45	0.233	0.0	0.0	0.0 	0.0

0.020	0.000	0.000	0.007	0.007	0.000	0.023	0.023	0.000

2.5	.233	.050	0.1	0.	.1	0.

8.3	10.0	60.0	0.0	0.0

0.000	0.000	0.000

0	0

WATR	YEAR	1	PEST	YEAR	1		CONC			YEAR	1

2	YEAR

RFLX1	TSER	1.0E05

INFL	TCUM	31

SPECIAL ACTIONS

010782 SNAPSHOT

4.1.3.2  PRZM input guide for PRZM-2

RECORD 1	FORMAT	A78

col: 1-78	TITLE:		label for simulation title.

RECORD 2	FORMAT	A78

col: 1-78	HTITLE:		label for hydrology information title.

RECORD 3	FORMAT	2F8.0,I8,F8.0,2I8

col: 1-8	PFAC:		pan factor used to estimate daily evapotranspiration.

col: 9-16	SFAC:		snowmelt factor in cm/degrees celsius above freezing.

col: 17-24	IPEIND:		pan factor flag.  0 = pan data read, 1 = temperature 	data read, 2 = either available used.

col: 25-32	ANETD:		minimum depth of which evaporation is extracted 	(cm).

col: 33-40	INICRP:		flag for initial crop if the simulation date is before 	the emergence date. (see record 10).  1 = yes, 0 = no.

col: 41-48	ISCOND:	surface condition of initial crop if INICRP = 1.  1 = fallow, 2 = cropping, 3 = residue.

RECORD 4	Only if IPEIND = 1 or 2 (see record 3).

FORMAT	6F8.0

col: 1-48	DT:		monthly daylight hours for January - June.

RECORD 5	Only if IPEIND = 1 or 2 (see record 3).

FORMAT	6F8.0

col: 1-48	DT:		monthly daylight hours for July - December.

RECORD 6	FORMAT	I8

col: 1-8	ERFLAG:	flag to calculate erosion. 1 = yes, 0 = no.

RECORD 7	Only if ERFLAG = 1 (see record 6).

FORMAT	5F8.0

col: 1-8	USLEK:		universal soil loss equation (K) of soil erodibilty.

col: 9-16	USLELS:	universal soil loss equation (LS) topographic factor.

col: 17-24	USLEP:		universal soil loss equation (P) practice factor.

col: 25-32	AFIELD:		area of field or plot in hectares.

col: 33-40	TR:		average duration of rainfall produced by storms (hrs).

RECORD 8	FORMAT	I8

col: 1-8	NDC:		number of different crops in the simulation (1 to 5).

RECORD 9	Repeat this record up to NDC (see record 8).

FORMAT	I8,3F8.0,I8,3(1X,I3),3(1X,I3),2F8.0

col: 1-8	ICNCN:		crop number of the different crop.

col: 9-16	CINTCP:		maximum interception storage of the crop (cm).

col: 17-24	AMXDR:		maximum rooting depth of the crop (cm).

col: 25-32	COVMAX:	maximum areal coverage of the canopy (percent).

col: 33-40	ICNAH:		surface condition of the crop after harvest date (see 	record 11).  1 = fallow, 2 = cropping, 3 = residue.

col: 42-52	CN:		runoff curve numbers of antecedent moisture condition II for fallow, cropping, residue (3 values).

col: 54-64	USLEC:		universal soil loss cover management factors for 	fallow, cropping ,residue (C value). Required if ERFL	AG = 1 (see record 6) else set to 0.0 (3 values).

col: 65-72	WFMAX:		maximum dry weight of the crop at full canopy (kg 	m-2).  Required if FAM = 3 (see record 16) else set to 	0.0 .

col: 73-80	HTMAX:		maximum canopy height at maturation date (cm) (see 	record 11).

RECORD 10	FORMAT	I8

col: 1-8	NCPDS:		number of cropping periods (sum of NDC for all

cropping dates in record 11).

RECORD 11	Repeat this record up to NCPDS (see record 10).

FORMAT	2X,3I2,2X,3I2,2X,3I2,I8

col: 3-4	EMD:		integer day of crop emergence.

col: 5-6	EMM:		integer month of crop emergence.

col: 7-8	IYREM:		integer year of crop emergence.

col: 11-12	MAD:		integer day of crop maturation.

col: 13-14	MAM:		integer month of crop maturation.

col: 15-16	IYRMAT:	integer year of crop maturation.

col: 19-20	HAD:		integer day of crop harvest.

col: 21-22	HAM:		integer month of crop harvest.

col: 23-24	IYRHAR:	integer year of crop harvest.

col: 25-32	INCROP:	crop number associated with NDC (see record 8).

RECORD 12	FORMAT	A78

col: 1-80	PTITLE:		label for pesticide title.

RECORD 13	FORMAT	3I8

col: 1-8	NAPS:		total number of pesticide applications occuring at different dates (1 to 50).  Note: if two or more pesticides are applied on the same date then NAPS = 1 for that day.

col: 9-16	NCHEM:		number of pesticide(s) in the simulation.  This value should equal the number in the execution supervisor file (1 to 3).

col: 17-24	FRMFLG:	flag for testing of ideal soil moisture conditions for the application of pesticide(s) relative to the target date (see record 15 for target date information).  1 = yes, 0 = no.

RECORD 14	FORMAT	3A20

col: 1-60	PSTNAM:	names of pesticide(s) for output titles.

RECORD 15	Repeat this record up to NAPS (see record 13).

FORMAT	2X,3I2,I8,6F8.0

col: 3-4	APD:		integer target application day.

col: 5-6	APM:		integer target application month.

col: 7-8	IAPYR:		integer target application year.

col: 9-16	WINDAY:	number of days in which to check soil moisture values following the target date for ideal pesticide(s) applications.  Required if FRMFLG = 1 else set to 0.

col: variable	DEPI:		depth of the pesticide(s) application (cm).  Note:  DEPI should be entered in the same order as in PSTNAM (record 14) if NCHEM is greater than one.

col: variable	TAPP:		total application of the pesticide(s) (kg ha-1).  Note:  TAPP should be entered in the same order as in PSTNAM (record 14) if NCHEM is greater than one.

RECORD 16	FORMAT	2I8,F8.0

col: 1-8	FAM:		foliar application model flag.  1 = pest. application to soil only, 2 = linear pesticide foliar application based on crop canopy, 3 = pesticide foliar application using nonlinear exponential filtration.

col: 9-16	IPSCND:		condition for disposition of foliar pesticide after harvest.  1 = surface applied, 2 = complete removal, 3 = left alone.  Required if FAM=2 or 3.

col: 17-24	FILTRA:		filtration parameter.  Required if FAM = 3 else set to 0.0.

RECORD 17	Only if FAM=2 or 3, repeat this record up to NCHEM.

FORMAT	3F8.0

col: 1-8	PLVKRT:	pesticide volatilization decay rate on plant foliage (days-1).

col: 9-16	PLDKRT:	pesticide decay rate on plant foliage (days-1).

col: 17-24	FEXTRC:	foliar extraction coefficient for pesticide washoff per centimeter of rainfall.

RECORD 18	FORMAT	A78

col: 1-78	STITLE:		label for soil properties title.

RECORD 19	FORMAT	2F8.0,9I4

col: 1-8	CORED:		total depth of soil core in cm. (must be sum of all horizons thicknesses (THKNS)in record 33 and at least as deep as the root depth in record 9).

col: 9-16	UPTKF:		plant uptake factor.  1 = uptake is equal to transpiration * dissolved phase concentration, 0 = no uptake is simulated.  .001 to .99 = uptake is a fraction of transpiration * dissolved phase concentration.

col: 17-20	BDFLAG:	bulk density flag.  1 = mineral value entered, 0 = apparent bulk density known and entered in record 33.

col: 21-24	THFLAG:	field capacity and wilting point flag.  1 = calculated by the model, 0 = water contents are entered.

col: 25-28	KDFLAG:	soil/pesticide adsorption coefficient. 1 = calculated by the model, 0 = KD value entered in record 36.

col: 29-32	HSWZT:		drainage flag.  1 = restricted, 0 = free draining.

col: 33-36	MOC:		method of characteristics flag.  1 = yes, 0 = no.

col: 37-40	IRFLAG:		irrigation flag. 0 = no, 1 = year round, 2 = during cropping period only.

col: 41-44	ITFLAG:		soil temperature simulation flag.   1 = yes, 0 = no.

col: 45-48	IDFLAG:		thermal conductivity and heat capacity flag.  1 = yes, 0 = no.

col: 49-52	BIOFLG:		biodegradation flag.  1 = yes, 0 = no.

RECORD 20	Only if BIOFLG = 1 (see record 19).

FORMAT	5F8.0

col: 1-8	AM:		maintenance coefficient of the metabolizing Xm population (day-1).

col: 9-16	AC:		maintenance coefficient of the co-metabolizing Xc population (day-1).

col: 17-24	AS:		maintenance coefficient of the sensitive Xs population (day-1).

col: 25-32	AR:		maintenance coefficient of the non-sensitive Xr population (day-1).

col: 33-40	KE:		average enzyme content of the Xc population (dimensionless).

RECORD 21	Only if BIOFLG = 1 (see record 19).

FORMAT	7F8.0

col: 1-8	KSM:		saturation constant of the metabolizing Xm population with respect to pesticide concentration.

col: 9-16	KCM:		saturation constant of the metabolizing Xm population with respect to carbon concentration.

col: 17-24	KC:		saturation constant of the co-metabolizing Xc population.

col: 25-32	MKS:		saturation constant of the sensitive Xs population.

col: 33-40	KR:		saturation constant of the non-sensitive Xr population.

col: 41-48	KIN:		inhibition constant (mg g-1 dry soil).

col: 49-56	KSK:		carbon solubilization constant (day-1).

RECORD 22	Only if BIOFLG =1 (see record 19).

FORMAT	6F8.0

col: 1-8	KLDM:		death rate of the metabolizing Xm population (day-1).

col: 9-16	KLDC:		death rate of the co-metabolizing Xc population

(day-1).

col: 17-24	KLDS:		death rate of the sensitive Xs population (day-1).

col: 25-32	KLDR:		death rate of the non-sensitive Xr population (day-1).

col: 33-40	KL1:		second order death rate of the Xs population (mg g-1 day-1).

col: 41-48	KL2:		dissociation constant of the enzyme substrate complex (day-1).

RECORD 23	Only if BIOFLG = 1 (see record 19).

FORMAT	5F8.0

col: 1-8	USM:		growth rate of the metabolizing Xm population with respect to pesticide concentration (day-1).

col: 9-16	UCM:		specific growth rate of the metabolizing Xm population with respect to carbon concentration

(day-1).

col: 17-24	MUC:		specific growth rate of the co-metabolizing Xc population (day-1).

col: 25-32	US:		specific growth rate of the sensitive Xs population (day-1).

col: 33-40	UR:		specific growth rate of the non-sensitive Xr population.

RECORD 24	Only if BIOFLG = 1 (see record 19).

FORMAT	5F8.0

col: 1-8	YSM:		true growth yield of the metabolizing Xm population with respect to pesticide concentration (mg(dry wt.)/mg).

col: 9-16	YCM:		true growth yield of the metabolizing Xm population with respect to carbon concentration (mg(dry wt.)/mg).

col: 17-24	YC:		true growth yield of the co-metabolizing Xc population (mg(dry wt.)/mg).

col: 25-32	YS:		true growth yield of the sensitive Xs population (mg(dry wt.)/mg).

col: 33-40	YR:		true growth yield of the non-sensitive Xr population (mg(dry wt.)/mg).

RECORD 25	FORMAT	7F8.0

col: 1-8	DAIR:		diffusion coefficient for the pesticide(s) in the air.  Only required if HENRYK is greater than 0 else set to 0.0

col: variable	HENRYK:	henry's law constant of the pesticide(s) for each NCHEM.

col: variable	ENPY:		enthalpy of vaporization of the pesticide(s) for each NCHEM.

RECORD 26	Only if IRFLAG = 1 or 2.

FORMAT	I8,3F8.0

col: 1-8	IRTYP:		type of irrigation.  1 = flood irrigation, 2 = furrow, 3 = over canopy, 4 = under canopy sprinkler.

col: 9-16	FLEACH:	leaching factor as a fraction of irrigation water depth.

col: 17-24	PCDEPL:	fraction of water capacity at which irrigation is applied.

col: 25-32	RATEAP:	maximum rate at which irrigation is applied (cm

hr-1).

RECORD 27	Only if IRFLAG = 1 or 2 and IRTYP = 2.

FORMAT	7F8.0

col: 1-8	Q0:		flow rate of water entering the heads of individual furrows (m3 s-1).

col: 9-16	BT:		bottom width of the furrows (m).

col: 17-24	ZRS:		slope of the furrow channel walls (horizontal/vertical).

col: 25-32	SF:		slope of the furrow channel bottom (vertical/horizontal).

col: 33-40	EN:		Manning's roughness coefficient for the furrow.

col: 41-48	X2:		length of the furrow (m).

col: 49-56	XFRAC:		location in furrow where PRZM infiltration calculations are performed, as a fraction of the furrow length (X2).  If XFRAC = -1, average depths are used in PRZM.

RECORD 28	Only if IRFLAG = 1 or 2 and IRTYP = 2.

FORMAT	2F8.0

col: 1-8	KS:		saturated hydraulic conductivity of the soil in which furrows are dug (m s-1).

col: 9-16	HF:		green-amp infiltration suction parameter (m).

RECORD 29	Only if KDFLAG = 1 (see record 19).

FORMAT	I8,3F8.0

col: 1-8	PCMC:		flag for which model is used to estimate KD (see record 36).  1 = mole fraction, 2 = mg liter-1, 3 = micromoles liter-1, 4 = KOC entered (dimensionless).

col: variable	SOL:		pesticide(s) solubility entered according to PCMC flag above for each NCHEM.

RECORD 30	Only if ITFLAG = 1 (see record 19).

FORMAT	14I5

col: 1-60	ALBEDO:	monthly values of soil surface albedo (12 values).

col: 61-65	EMMISS:	reflectivity of soil surface to longwave radiation (fraction).

col: 66-70	ZWIND:		height of wind speed measurement above the soil surface (m).

RECORD 31	Only if ITFLAG = 1 (see record 19).

FORMAT	12I5

col: 1-60	BBT:		average monthly values of bottom boundary soil temperatures in degrees celsius (12 values).

RECORD 32	FORMAT	I8

col: 1-8	NHORIZ:	total number of horizons (minimum of 1).

RECORD 33	Repeat records 33-38 in data sets up to NHORIZ.

FORMAT	I8,7F8.0

col: 1-8	HORIZN:	horizon number in relation to NHORIZ.

col: 9-16	THKNS:		thickness of the horizon.

col: 17-24	BD:		bulk density if BDFLAG = 0 or mineral density if BDFLAG = 1.

col: 25-32	THETO:		initial soil water content in the horizon (cm3 cm-3).

col: 33-40	AD:		soil drainage parameter if HSWZT = 1, else set to 0.0 (day-1).

col: 41-48	DISP:		pesticide(s) hydrodynamic solute dispersion coefficient for each NCHEM.

RECORD 34	Only if BIOFLG = 1 (see record 19).

FORMAT	8X,5F8.0

col: 9-16	Q:		average carbon content of the population.

dimensionless.

col: 17-24	CM1:		mineralizable carbon (mg g-1).

col: 25-32	Y1:		concentration of metabolizing microbial population (mg g-1).

col: 33-40	Y2:		concentration of co-metabolizing microbial population (mg g-1).

col: 41-48	Y3:		concentration of sensitive microbial population (mg

g-1).

col: 49-56	Y4:		concentration of non-sensitive microbial population (mg g-1).

RECORD 35	FORMAT	8X,9F8.0

col: variable	DWRATE:	dissolved phase pesticide(s) hydrolysis decay rate for each NCHEM (day-1).

col: variable	DSRATE:	adsorbed phase pesticide(s) hydrolysis decay rate for each NCHEM (day-1).

col: variable	DGRATE:	vapor phase pesticide(s) decay rate for each NCHEM (day-1).

Note:  set DWRATE and DSRATE equal to simulate lumped first-order degradation.

RECORD 36	FORMAT	8X,7F8.0

col: 9-16	DPN:		thickness of compartments in the horizon (cm).

col: 17-24	THEFC:		field capacity in the horizon (cm3 cm-3).

col: 25-32	THEWP:		wilting point in the horizon (cm3 cm-3).

col: 33-40	OC:		organic carbon in the horizon (percent).

col: variable	KD:		pesticide(s) partition coefficient for each NCHEM.  Required if KDFLAG = 0, else set to 0.0 (cm-3 g-1).

RECORD 37	Only if ITFLAG = 1 (see record 19).

FORMAT	8X,5F8.0

col: 9-16	SPT:		initial temp. of the horizon (celsius).

col: 17-24	SAND:		sand content in the horizon.  Required if THFLAG = 1, else set to 0.0 (percent).

col: 25-32	CLAY:		clay content in the horizon.  Required if THFLAG = 1, else set to 0.0 (percent).

col: 33-40	THCOND:	thermal conductivity of the horizon (cm-1 day-1).  Required if IDFLAG = 0, else set to 0.0.

col: 41-48	VHTCAP:	heat capacity per unit volume of the soil horizon (cm-3 celsius-1).  Required if IDFLAG = 0, else set to 0.0.

RECORD 38	Only if NCHEM greater than 1.  Note: this record is used for parent/daughter relationship.  Set to zero for simulating independent parent chemicals.

FORMAT	8X,3F8.0

col: 9-16	DKRT12:		transformation rate for chemical 1 to 2.

col: 17-24	DKRT13:		transformation rate for chemical 1 to 3.  If NCHEM = 2, set to 0.0 .

col: 25-32	DKRT23:		transformation rate for chemical 2 to 3.  If NCHEM = 2, set to 0.0 .

RECORD 39	FORMAT	2I8

col: 1-8	ILP:		flag for initial pesticide(s) levels before simulation start date.  1 = yes, 0 = no.

col: 9-16	CFLAG:		conversion flag for initial pesticide(s) levels.  0 = mg/kg-1, 1 = kg/ha-1.  Leave blank if ILP = 0.

RECORD 40	Only if ILP = 1 (see record 39).  NOTE:  number of lines = THKNS(I) divided by DPN(I) where I = HORIZN.  Maximum of 8 values per line.  Enter this record in data sets for each NCHEM.

FORMAT	8F8.0

col: 1-80	PESTR:		initial pesticide(s) levels.

RECORD 41	FORMAT	3(4X,A4,4X,A4,I8),I4

col: 5-8	ITEM1:		hydrologic hardcopy output flag.  WATR is inserted or leave blank.

col: 13-16	STEP1:		time step of hydrologic output.  DAY = daily, MNTH = monthly, YEAR = yearly.

col: 17-24	LFREQ1:	frequency of hydrologic output given by a specific compartment number.

col: 29-32	ITEM2:		pesticide flux output flag.  PEST is inserted or leave blank.

col: 37-40	STEP2:		same as STEP1.

col: 41-48	LFREQ2:	same as LFREQ1.

col: 53-56	ITEM3:		pesticide concentration output flag.  CONC is inserted or leave blank.

col: 61-64	STEP3:		same as STEP1.

col: 65-72	LFREQ3:	same as LFREQ1.

col: 73-76	EXMFLG:	flag for reporting output to file for EXAMS model.  1 = yes, 0 = no.  If ERFLAG = 0, EXMFLG is automatically set to 0.

RECORD 42	FORMAT	I8,4X,A4

col: 1-8	NPLOTS:	number of times series plots (max. of 7).

col: 13-16	STEP4:		Time step of output.  This option outputs pesticide runoff and erosion flux and pesticide leaching below core depth.  Three options are available: DAY for daily, MNTH for monthly, YEAR for yearly.

RECORD 43	Only if NPLOTS is greater than 0 and ECHOLV greater than 2.

NOTE:  repeat this record up to NPLOTS.

FORMAT	4X,A4,A1,3X,A4,I8,F8.0

col: 5-8	PLNAME:	name of plotting variable (see Table 4-1 on page 4-23).

col: 9-9	INDX:		index to identify which pesticide if applicable.  1 = first chemical, 2 = second chemical, 3 = third chemical.

col: 13-16	MODE:		plotting mode.  enter TSER (daily) or TCUM (cumulative) to plot to times series file.

col: 17-24	IARG:		argument value for PLNAME (see table 4-1).

col: 25-32	CONST:		constant with which to multiply for unit conversion.  Leave blank for default to 1.0 .

RECORD 44	Only if special actions are desired (see record 45).

FORMAT	A78

col: 1-78	ATITLE:		label for special actions title.

RECORD 45	Only if special actions are desired.  Repeat this record for each special action required (up to 7).

FORMAT	2X,3I2,1X,A8,1X,I3,3F8.0

col: 3-4	SADAY:		day of special action.

col: 5-6	SAMON:		month of special action.

col: 7-8	SAYR:		year of special action.

col: 10-17	SPACT:		special action variable (see below).

col: 19-21	NACTS:		horizon or crop number affected by special actions (see below).

col: variable	SPACTS:		new value(s) for the special action (see page 4-23).

SPACT            NACTS	SPACTS

BD		HORIZON NO.	NEW VALUE(S) (F8.0)

CN		CROP NO.	NEW VALUES   (3I8)

DSRATE		HORIZON NO.	NEW VALUE(S) (3F8.0)

DWRATE		HORIZON NO.	NEW VALUE(S) (3F8.0)

KD		HORIZON NO.	NEW VALUE(S) (3F8.0)

SNAPSHOT*	-----------	--------------------

USLEC	CROP NO.		NEW VALUE(S) (3F8.0)

*  Used to display pesticide concentration profile.

TABLE 4-1. VARIABLE DESIGNATIONS FOR PLOTTING FILES

Variable										Arguments

Designation	FORTRAN							Required

(PLNAME)	Variable	Description		Units		(IARG)

Water Storage

INTS		CINT	Interception sto	cm		None

rage on canopy

SWTR	SW	Soil water storage	cm		1-NCOM2

SNOP	SNOW	Snow pack storage	cm		None

THET	THETN	Soil water content	cm cm-1	1-NCOM2

Water Fluxes

PRCP		PRECIP	Precipitation		cm day-1	None

SNOF	SNOWFL	Snowfall		cm day-1	None

THRF	THRUFL	Canopy throughfall	cm day-1	None

INFL		AINF	Percolation into	cm day-1	1-NCOM2

each soil com-

partment

RUNF	RUNOF	Runoff depth		cm day-1	None

CEVP	CEVAP	Canopy evaporation	cm day-1	None

SLET		ET	Actual evapotrans-	cm day-1	1-NCOM2

piration from each

compartment

TETD	TDET	Total daily actual	cm day-1	None

evapotranspiration

Sediment Flux

ESLS		SEDL	Event soil loss	Tonnes	None

day-1

Pesticide Storages

FPST		FOLPST	Foliar pesticide	g cm-2	None

storage

TPST		PESTR	Total soil pesticide	g cm-3	1-NCOM2

storage in each soil

compartment

TABLE 4-1.  VARIABLE DESIGNATIONS FOR PLOTTING FILES (continued)

Variable										Arguments

Designation	FORTRAN							Required

(PLNAME)	Variable	Description		Units		(IARG)

SPST		SPESTR	Dissolved pesticide	g cm-3	1-NCOM2

storage in each soil

compartment

Pesticide Fluxes

TPAP		TAPP	Total pesticide	g cm-2	None

application		day-1

FPDL		FPDLOS	Foliar pesticide	g cm-2	None

decay loss		day-1

WFLX	WOFLUX	Foliar pesticide	g cm-2	None

washoff flux		day-1

DFLX	DFFLUX	Individual soil	g cm-2	1-NCOM2

compartment pesticide	day-1

net diffusive flux

AFLX		ADFLUX	Pesticide advective	g cm-2	1-NCOM2

flux from each soil	day-1

compartment

DKFX	DKFLUX	Pesticide decay flux	g cm-2	1-NCOM2

in each soil compart-	day-1

ment

UFLX	UPFLUX	Pesticide uptake	g cm-2	1-NCOM2

flux from each soil	day-1

compartment

Pesticide Fluxes

RFLX		ROFLUX	Pesticide runoff flux	g cm-2	None

day-1

EFLX		ERFLUX	Pesticide erosion flux	g cm-2	None

RZFX		RZFLUX	Net pesticide flux	g cm-2	None

past the maximum root	day-1

depth

TUPX	SUPFLX	Total pesticide uptake	g cm-2	None

flux from entire soil	day-1

profile

TABLE 4-1.  VARIABLE DESIGNATIONS FOR PLOTTING FILES (concluded)

Variable										Arguments

Designation	FORTRAN			 				Required

(PLNAME)	Variable	Description		Units		(IARG)

TDKF	SDKFLX	Total pesticide decay	g cm-2	None

flux from entire profile 	day-1

PCNC	TCNC	Pesticide concentration	g cm-3	None

in canopy

VFLX		PVFLUX	Soil pesticide 	g cm-2	None

volatilization flux	day-1

FPVL		FPVLOS	Foliar pesticide	g cm-2	None

volatilization flux	day-1

Soil Temperature

STMP	SPT	Soil temperature in	°C	1-NCOM2

each soil compartment

Canopy Height

CHGT	HEIGHT	Canopy height	cm		None

4.1.4  VADOFT Input File

The PRZM-2 model requires a VADOFT flow input file if VADOFT is specified "ON" in the execution supervisor (PRZM2.RUN) file.  Also if TRANSPORT SIMULATION is specified "ON", VADOFT transport input must follow.

4.1.4.1  Example of VADOFT FLOW and TRANSPORT input file for PRZM-2

***********************************FLOW**************************************

3 CHEMICAL, 2 HORIZON, 1 MATERIAL, VADOSE ZONE FLOW SIMULATION FOR ZONE 1

61	1	1	1	1	1	1	1	0	0

20	2	1		.01

1	1	1	1	0	1	2	1	0

0.0		1.0		1.0		1.0

1		0.0		1.0

2

1	20	1		50.0

2	40	1		80.0

0.0E00	0

0	1		0.0		0.0E00	0	0	0	0

7.12E02    		.43E00	0.0E00	0.0E00

0.045E00		-1.0E00	0.145E00	2.68E00	0.626E00                                 5   10

YEAR

**********************************TRANSPORT**********************************

3 CHEMICAL, 2 HORIZON, 1 MATERIAL, VADOSE TRANSPORT SIMULATION FOR ZONE 1

61	1	1	1	0	1

0	1	1	0	0	1	2	1

0.0		1.0		1.0		1.0

1	0.0			1.0

2

1	20	1	50.0

2	40	1	80.0

0.0E00	0	0.0E00	0	0.0E00	0

0	0		0.0		0.0	0	0	0	0

1.30E01	.43E00

1.00E00	1.01E00	1.00E00	0.0E00	0.0E00	0.0E00

1		0.0		1.0		0.0E00

1	2.000E-2	0.00E00   	7.00E-3	0.00E00	2.30E-2	0.0E00

1	1

5	10

YEAR

4.1.4.2  VADOFT Input Guide for Flow

RECORD 1	FORMAT	A80

col: 1-80	TITLE:	label for flow simulation title.

RECORD 2		FORMAT	10I5

col: 1-5	NP:	total number of Vadoft nodal points (max of 100).

col: 6-10	NMAT:	total number of different porous materials (maximum of 5).

col: 11-15	NONU:	flag to indicate if initial condition is non-uniform.  1 = yes, 0 = no.

col: 16-20	ITRANS:	flag to indicate if running in transient or steady-state.  Must be set to 1 if PRZM is ON.  1 = transient, 0 = steady-state.

col: 21-25	IMODL:	flag to indicate if running flow or transport model.  1 = flow, 0 = transport.  Set to 1 here.

col: 26-30	IKALL:	time stepping index.  1 = backward difference, 0 = central difference.  This flag is automatically set to 1 in FLOW.

col: 31-35	IMBAL:	flag to indicate if mass balance computation is required.  1 = yes, 0 = no.

col: 36-40	INTSPC:	flag to indicate initial conditions for head values.  1 = hydraulic head, 0 = pressure head.

col: 41-45	IHORIZ:	flag to indicate if flow direction is horizontal.  1 = yes, 0 = no.  Set to 0 if PRZM is ON.

col: 46-50	ICHAIN:	flag to indicate if daughter products are used.  1 = yes, 0 = no.  Automatically set to 0 for flow.

RECORD 3	FORMAT	3I5,E10.3

col: 1-5	NITMAX:	maximum number of iterations per time step.  Suggested value of 20.

col: 6-10	INEWT:		flag to indicate nonlinear iterative procedure for solving saturated flow equation.  0 = Picard, 1 = standard Newton-Raphson, 2 = modified Newton-Raphson.  Suggested value of 2 if PRZM is ON.

col: 11-15	IRESOL:		maximum number of refinements each time step if solution does not converge.  Suggested value of 1.

col: 16-25	HTOL:		head tolerance for the nonlinear solution (length).  Suggested value of 0.01.

RECORD 4	FORMAT	8I5

col: 1-5	KPROP:		flag to indicate relationship between relative permeability versus saturation and pressure head versus saturation.  1 = functional parameters supplied in record 15, 0 = model calculated.

col: 6-10	ITSGN:		flag to indicate if output time values are to be model calculated.  1 = yes, 0 = no.

col: 11-15	ITMARK:	flag to indicate if output time values differ from computational time values (see records 6 and 7).  1 = yes, 0 = no.

col: 16-20	NSTEP:		value of which time step to output nodal values from.  When NSTEP = n, then output is printed.  Must be from 1 up to 31 (days).

col: 21-25	NVPR:		value of which time step to output nodal velocities.  When NVPR = n, then output is printed.  Must be from 1 up to 31 (days).

col: 26-30	IOBSND:	flag to indicate if values are printed at certain observation nodes.  1 = yes, 0 = no.  NOTE:  Echo level must be greater than or equal to 6 in PRZM2.RUN file.

col: 31-35	NOBSND:	number of observation node(s) to be printed.  NOBSND must not be greater than NP (see record 2).  If IOBSND = 0 then set NOBSND = 0.

col: 36-40	IPRCHK:	flag to indicate if detailed information is generated in the flow matrix.  1 = yes, 0 = no.

RECORD 5	Only if ITRANS = 1 (see record 2).

FORMAT	4E10.3

col: 1-10	TIMA:		initial time value (t).  Suggested value if PRZM is ON: 0.0

col: 11-20	TIN:		initial time step value(t).  Suggested value if PRZM is ON: 1.0.  Omit if ITSGN = 0.

col: 21-30	TFAC:		time step multiplier. Suggested value if PRZM is ON: 1.0.  Omit if ITSGN = 0.

col: 31-40	TMAX:		maximum time step value allowed (t).  Suggested value if PRZM is ON: 1.0  Omit if ITSGN = 0.

RECORD 6	Only if ITGSN = 0 (see record 4) and ITRANS = 1.

FORMAT	8E10.3

col: 1-80	TMVEC(I):	time values corresponding to the number of time steps where I = 1...31 (t).  Input up to 8 values per line.

RECORD 7	Only if ITMARK = 1 and ITRANS = 1

FORMAT	I5,2E10.3

col: 1-5	ITMGEN:	flag to indicate if backup file marker time values are used.  1 = yes, 0 = no.

col: 6-15	STMARK:	starting marker time value (t).  If PRZM and TRANSPORT are ON, set to 0.0.

col: 16-25	DTMARK:	marker time value increment (t).  If PRZM and TRANSPORT are ON, set to 1.0.

RECORD 8	Only if ITRANS = 1, ITMARK = 1 and ITMGEN = 0.

FORMAT	8E10.3

col: 1-80	TMFOMT:	output marker file time values (t) corresponding to TMVEC(I) (see record 6).  Input up to 8 values per line.

RECORD 9	FORMAT	I5

col: 1-5	NLAYRG:	number of soil horizons to be discretized.

RECORD 10	Repeat this record up to NLAYRG (see record 9).

FORMAT	3I5,E10.3

col: 1-5	ILAYR:		horizon number in relation to NLAYRG.

col: 6-10	NELM:		number of finite elements in ILAYR.

col: 11-15	IMATL:		porous material number related to NMAT (see record 2) in ILAYR.

col: 16-25	THL:		thickness of the horizon (ILAYR).

RECORD 11	FORMAT	E10.3,I5

col: 1-10	CHINV:		default initial values of pressure (l) or hydraulic head (m l3) for nodes in the matrix.

col: 11-15	CNPIN:		number of non-default nodes in the matrix related to the default initial values (CHINV) if NONU = 1 (see record 2), else set to 0.

RECORD 12	FORMAT	2I5,2E10.3,2I5,2E10.3

col: 1-5	IBTND1:		type of boundary condition for the first node.  1 = pressure head, 0 = water flux.

col: 6-10	IBTNDN:	type of boundary condition for the last node.  1 = pressure head, 0 = water flux.

col: 11-20	VALND1:	value of the pressure head or water flux at the first node.  The value should be positive for influx and negative for efflux.  Set to 0.0 if PRZM is ON.

col: 21-30	VALNDN:	value of the pressure head or water flux at the last node.  The value should be positive for influx and negative for efflux.  Set to 0.0 if fluid is exiting the last node.

col: 31-35	ITCND1:		flag to indicate if the boundary condition at the first node is transient.  1 = yes, 0 = no.  Automatically set to 0 if PRZM is ON.

col: 36-40	ITCNDN:	flag to indicate if the boundary condition at the last node is transient.  1 = yes, 0 = no.  Automatically set to 0 if PRZM is ON.

col: 41-50	FLX1:		fluid flux injected into the first node (l3 t).  Automatically set to 0.0 for FLOW if PRZM is ON.

col: 51-60	FLXN:		fluid flux injected into the last node (l3 t).  Automatically set to 0.0 for FLOW if PRZM is ON.

RECORD 13	Repeat this record up to NMAT (see record 2).

FORMAT	4E10.3

col: 1-10	PROP1:		saturated hydraulic conductivity of the material (use cm day-1 if PRZM is ON).

col: 11-20	PROP2:		effective porosity of the material.

col: 21-30	PROP3:		specific storage of the material.  For unsaturated flow, set to 0.0.

col: 31-40	PROP4:		air entry pressure head of the material.

RECORD 14	Omit for FLOW simulation.

RECORD 15	Repeat this record up to NMAT if KPROP = 1.

FORMAT	5E10.3

col: 1-10	FVAL1:		residual water phase saturation of the material (residual water content / saturated water content).

col: 11-20	FVAL2:		parameter n of the relative permeability versus saturation relationship.  Suggested value of 0.0 or negative value.

col: 21-30	FVAL3:		leading coefficient of the saturation versus capillary head relationship (alpha).

col: 31-40	FVAL4:		power index of the saturation versus capillary head relationship (beta).

col: 41-50	FVAL5:		power index of the saturation versus capillary head relationship (gamma).  Suggested value of 1.- (1./FVAL4).

RECORD 16	Repeat records 16-19 in data sets up to NMAT if KPROP = 0.

FORMAT	I5

col: 1-5	NUMK:		number of entry pairs of relative permeability and saturation of the material.

RECORD 17	Only if KPROP = 0.

FORMAT	8E10.3

col: 1-10	SMV1:		value of water phase saturation for point 1 of the entry pairs related to NUMK.

col: 11-20	PKRW1:		value of relative permeability (l2) for point 1 of the entry pairs related to NUMK.

col: 21-30	SMV2:		etc.

col: 31-40	PKRW2:		etc.

RECORD 18	Only if KPROP = 0.

FORMAT	I5

col: 1-5	NUMP:		number of entry pairs of pressure head versus saturation values for the material.

RECORD 19	Only if KPROP = 0.

FORMAT	8E10.3

col: 1-10	SSWV1:		value of water phase saturation for point 1 of the entry pairs related to NUMP.

col: 11-20	HCAP1:		value of the pressure head  (l) for point 1 of the entry pairs related to NUMP.

col: 21-30	SSWV2:		etc.

col: 31-40	HCAP2:		etc.

RECORD 20	Only if NONU = 1.

NOTE: enter next two variables sequentially for every non-default node (CNPIN).

FORMAT	5(I5,E10.3)

col: 1-5	N:		non-default node number relative to CNPIN (see record 11).

col: 6-15	PINT:		non-default initial value of pressure head (l) or hydraulic head (m l3) of the node number (n).

RECORD 21	Omit for FLOW simulation.

RECORD 22	Omit for FLOW simulation.

RECORD 23	Omit for FLOW simulation.

RECORD 24	Only if ITCND1 = 1 and PRZM is OFF.

FORMAT	I5

col: 1-5	NTSNDH1:	number of selected time values of pressure head or water flux for transient simulation at first node.

RECORD 25	Only if ITCND1 = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	TMHV1:		time values in relation to NTSNDH1 at the first node for pressure head or water flux (t).  Enter up to 8 values per line up to NTSNDH1 lines.

RECORD 26	Only if ITCND1 = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	HVTM1:		values of pressure head or water flux corresponding to TMHV1 at the first node (length).  Enter up to 8 values per line up to NTSNDH1 lines.

RECORD 27	Omit for FLOW simulation.

RECORD 28	Only if ITCNDN =1 and PRZM is OFF.

FORMAT	I5

col: 1-5	NTSNDH2:	number of selected time values of pressure head or water flux for transient simulation at the last node.

RECORD 29	Only if ITCNDN = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	TMHV2:		time values in relation to NTSNDH2 at the last node for pressure head or water flux (t).  Enter up to 8 values per line up to NTSNDH2 lines.

RECORD 30	Only if ITCNDN = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	HVTM2:		values of pressure head or water flux corresponding to TMHV2 at the last node (length).  Enter up to 8 values per line up to NTSNDH2 lines.

RECORD 31	Omit for FLOW simulation.

RECORD 32	Only if IOBSND = 1.

FORMAT	16I5

col: 1-80	NDOBS:		increasing sequential numbers of observation nodes.  Enter up to 16 per line up to NOBSND (see record 4).

RECORD 33	FORMAT	A4

col: 1-4	OUTF:		output time step for printing.  Enter DAY for daily, MNTH for monthly, YEAR for yearly.

4.1.4.3  VADOFT Input Guide for TRANSPORT

RECORD 1	FORMAT	A80

col: 1-80	TITLE:		label for transport simulation title.

RECORD 2	FORMAT	10I5

col: 1-5	NP:		total number of Vadoft nodal points.

col: 6-10	NMAT:		total number of different porous materials (maximum of 5).

col: 11-15	NONU:		flag to indicate if initial condition is non-uniform.  1 = yes, 0 = no.

col: 16-20	ITRANS:		flag to indicate if running in transient or steady-state.  Must be set to 1 if PRZM is ON.  1 = transient, 0 = steady-state.

col: 21-25	IMODL:		flag to indicate if running flow or transport model.  1 = flow, 0 = transport.  Set to 0 here.

col: 26-30	KALL:		time stepping index.  1 = backward difference, 0 = central difference.  This flag is automatically set to 1 for steady-state simulation.

col: 31-35	IMBAL:		flag to indicate if mass balance computation is required.  1 = yes, 0 = no.

col: 36-40	INTSPC:		flag to indicate initial conditions for head values.  1 = hydraulic head, 0 = pressure head.  Automatically set to 0 for transport.

col: 41-45	IHORIZ:		flag to indicate if flow direction is horizontal.  1 = yes, 0 = no.  Set to 0 if PRZM is ON.

col: 46-50	ICHAIN:		flag to indicate if daughter products are used.  1 = yes, 0 = no.

RECORD 3	Omit for transport simulation.

RECORD 4	FORMAT	8I5

col: 1-5	KPROP:		flag to indicate relationship between relative permeability versus saturation and pressure head versus saturation.  Set to 0 for Transport simulation.

col: 6-10	ITSGN:		flag to indicate if output time values are to be model calculated.  1 = yes, 0 = no.

col: 11-15	ITMARK:	flag to indicate if output time values differ from computational time values (see records 6 and 7).  1 = yes, 0 = no.

col: 16-20	NSTEP:		value of which time step to output nodal values from.  When NSTEP = n, then output is printed.  Must be from 1 up to 31 (days).

col: 21-25	NVPR:		value of which time step to output nodal velocities.  When NVPR = n, then output is printed.  Must be from 1 up to 31 (days).

col: 26-30	IOBSND:	flag to indicate if values are printed at certain observation nodes.  1 = yes, 0 = no.  NOTE:  Echo level must be greater than or equal to 6 in PRZM2.RUN file.

col: 31-35	NOBSND:	number of observation node(s) to be printed.  NOBSND must not be greater than NP (see record 2).  If IOBSND = 0 then set NOBSND = 0.

col: 36-40	IPRCHK:	flag to indicate if detailed information is generated in the flow matrix.  1 = yes, 0 = no.

RECORD 5	Only if ITRANS = 1 (see record 2).

FORMAT	4E10.3

col: 1-10	TIMA:		initial time value (t).  Suggested value if PRZM is ON: 0.0

col: 11-20	TIN:		initial time step value(t).  Suggested value if PRZM is ON: 1.0.  Omit if ITSGN = 0.

col: 21-30	TFAC:		time step multiplier. Suggested value if PRZM is ON: 1.0.  Omit if ITSGN = 0.

col: 31-40	TMAX:		maximum time step value allowed (t).  Suggested value if PRZM is ON: 1.0  Omit if ITSGN = 0.

RECORD 6	Only if ITGSN = 0 (see record 4) and ITRANS = 1.

FORMAT	8E10.3

col: 1-80	TMVEC(I):	time values corresponding to the number of time steps where I = 1...31 (t).  Input up to 8 values per line.

RECORD 7	Only if ITMARK = 1 and ITRANS = 1

FORMAT	I5,2E10.3

col: 1-5	ITMGEN:	flag to indicate if backup file marker time values are used.  1 = yes, 0 = no.

col: 6-15	STMARK:	starting marker time value (t).  If PRZM and TRANSPORT are ON, set to 0.0.

col: 16-25	DTMARK:	marker time value increment (t).  If PRZM and TRANSPORT are ON, set to 1.0.

RECORD 8	Only if ITRANS = 1, ITMARK = 1 and ITMGEN = 0.

FORMAT	8E10.3

col: 1-80	TMFOMT:	output marker file time values (t) corresponding to TMVEC(I) (see record 6).  Input up to 8 values per line.

RECORD 9	FORMAT	I5

col: 1-5	NLAYRG:	number of soil horizons to be discretized.

RECORD 10	Repeat this record up to NLAYRG (see record 9).

FORMAT	3I5,E10.3

col: 1-5	ILAYR:		horizon number in relation to NLAYRG.

col: 6-10	NELM:		number of finite elements in ILAYR.

col: 11-15	IMATL:		porous material number related to NMAT (see record 2) in ILAYR.

col: 16-25	THL:		thickness of the horizon (ILAYR).

RECORD 11	FORMAT	E10.3,I5  Repeat for each NCHEM.

col: 1-10	CHINV:		default initial values of concentration (m l3) for nodes in the matrix.

col: 11-15	CNPIN:		number of non-default nodes in the matrix related to the default initial values (CHINV) if NONU = 1 (see record 2), else set to 0.

RECORD 12	FORMAT	2I5,2E10.3,2I5,2E10.3

col: 1-5	IBTND1:		type of boundary condition for the first node.  1 = concentration, 0 = solute flux.

col: 6-10	IBTNDN:	type of boundary condition for the last node.  1 = concentration, 0 = solute flux.

col: 11-20	VALND1:	value of the concentration or solute flux at the first node.  The value should be positive for influx and negative for efflux.  Set to 0.0 if PRZM is ON.

col: 21-30	VALNDN:	value of the concentration or solute flux at the last node.  The value should be positive for influx and negative for efflux.  Set to 0.0 if fluid is exiting the last node.

col: 31-35	ITCND1:		flag to indicate if the boundary condition at the first node is transient.  1 = yes, 0 = no.  Automatically set to 0 if PRZM is ON.

col: 36-40	ITCNDN:	flag to indicate if the boundary condition at the last node is transient.  1 = yes, 0 = no.  Automatically set to 0 if PRZM is ON.

col: 41-50	FLX1:		fluid flux injected into the first node (l3 t).  Automatically set to 0.0 if PRZM is ON.

col: 51-60	FLXN:		fluid flux injected into the last node (l3 t).  Automatically set to 0.0 if PRZM is ON.

RECORD 13	Repeat records 13-14 in data sets up to NMAT.

FORMAT	2E10.3

col: 1-10	CPROP1:	longitudinal dispersivity of the material.

col: 11-20	CPROP2:	effective porosity of the material.

RECORD 14	FORMAT	3(2E10.3)

col: variable	CPROP3:	retardation coefficient for the material.  Enter this value up to NCHEM.

col: variable	CPROP4:	molecular diffusion for the material.  Enter this value up to NCHEM.

RECORD 15	Omit for TRANSPORT

RECORD 16	Omit for TRANSPORT

RECORD 17	Omit for TRANSPORT

RECORD 18	Omit for TRANSPORT

RECORD 19	Omit for TRANSPORT

RECORD 20	Only if NONU = 1.  Repeat this record up to NCHEM.

NOTE: enter next two variables sequentially for every non-default node (CNPIN).

FORMAT	5(I5,E10.3)

col: 1-5	N:		non-default node number relative to CNPIN (see record 11).

col: 6-15	PINT:		non-default initial value of concentration (m l3) of the node number (n).

RECORD 21	Repeat records 21-22 in data sets up to NMAT.

FORMAT	I5,3E10.3

col: 1-5	I:		porous material number in relation to NMAT.

col: 6-15	VDFI:		default value of darcy velocity.

col: 16-25	SWDFI:		default value of water saturation.

col: 26-35	UWFI:		value of upstream weighting factor.  Set to 0.0 if no upstream weighting is desired.

RECORD 22	FORMAT	I5,6E10.3

col: 1-5	I:		porous material number in relation to NMAT.

col: variable	CLAMDI:	decay coefficient of the material.  Enter this value up to NCHEM.

col: variable	CRACMP:	transformation mass fraction of the material.  Enter this value up to NCHEM.

RECORD 23	FORMAT	2I5

col: 1-5	NVREAD:	flag to indicate if darcy velocities will be read from internal scratch files.  If PRZM and TRANSPORT are ON, but not FLOW, then NVREAD is set to 1.  1 = yes, 0 = no.

col: 6-10	IVSTED:		flag to indicate if the velocities are at steady-state.  This implies steady-state within each day, not the entire simulation.  1 = yes , 0 = no.  If PRZM is ON then IVSTED is set to 1.

RECORD 24	Only if ITCND1 = 1 and PRZM is OFF.

FORMAT	I5

col: 1-5	NTSNDH1:	number of selected time values of concentration or solute flux for transient simulation at first node.

RECORD 25	Only if ITCND1 = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	TMHV1:		time values in relation to NTSNDH1 at the first node for pressure head or water flux (t).  Enter up to 8 values per line up to NTSNDH1 lines.

RECORD 26	Only if ITCND1 = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	HVTM1:		values of concentration or solute flux corresponding to TMHV1 at the first node (length).  Enter up to 8 values per line up to NTSNDH1 lines.

RECORD 27	Only if IBTND1 = 0 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	QVTM1:		volumetric fluxes corresponding to TMHV1 at the first node. Enter 8 values per line up to NTSNDH1.

RECORD 28	Only if ITCNDN =1 and PRZM is OFF.

FORMAT	I5

col: 1-5	NTSNDH2:	number of selected time values of concentration or solute flux for transient simulation at the last node.

RECORD 29	Only if ITCNDN = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	TMHV2:		time values in relation to NTSNDH2 at the last node for concentration or solute flux (t).  Enter up to 8 values per line up to NTSNDH2 lines.

RECORD 30	Only if ITCNDN = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	HVTM2:		values of pressure head or water flux corresponding to TMHV2 at the last node (length).  Enter up to 8 values per line up to NTSNDH2 lines.

RECORD 31	Only if ITCNDN = 1 and PRZM is OFF.

FORMAT	8E10.3

col: 1-80	QVTM2:		volumetric fluxes corresponding to TMHV2 at the last node.  Enter 8 values per line up to NTSNDH2.

RECORD 32	Only if IOBSND = 1.

FORMAT	16I5

col: 1-80	NDOBS:		increasing sequential numbers of observation nodes.  Enter up to 16 per line up to NOBSND (see record 4).

RECORD 33	FORMAT	A4

col: 1-4	OUTT:		output time step for printing.  Enter DAY for daily, MNTH for monthly, YEAR for yearly.

4.1.5  MONTE CARLO Input File

The PRZM-2 model requires a Monte Carlo input file when MONTE CARLO is specified "ON" in the execution supervisor file.  The following is an example Monte Carlo input file.

4.1.5.1  Example MONTE CARLO input file for PRZM-2

***Title

MONTE CARLO TEST INPUT

***Number of runs and confidence level

100		90.0

***Monte Carlo inputs

KOC 1				1		800. 1400.		10.10000.	1.

FIELD CAPACITY		1	1	.316	.130		0.05	0.60	5.

WILTING POINT		1	1	.150	.066		0.03	0.30	5.

ORGANIC CARBON		1	1	1.30	.870		0.01	5.00	1.

FIELD CAPACITY		2	1	.288	.110		0.04	.540	5.

WILTING POINT		2	1	.143	.076		0.03	.030	5.

ORGANIC CARBON		2	1	.110	.070		0.01	1.00	1.

DISPERSION 1			1	1	50.0	15.0		10.0	90.0	7.

***Empirical Distribution Data

4

89.7		0.10

82.9		0.20

76.1		0.30

69.3		0.40

***Monte Carlo outputs

INFILTRATION		1	1			CDF		WRITE		1

DISPERSION 1		1	1			CDF		WRITE		1

END

***Correlations

FIELD CAPACITY	1	1	WILTING POINT		1		1	0.757

FIELD CAPACITY	1	1	ORGANIC CARBON		1		1	0.609

FIELD CAPACITY	2	1	WILTING POINT		2		1	0.757

FIELD CAPACITY	2	1	ORGANIC CARBON		2		1	0.170

END

NOTE:	The above Monte Carlo input file contains lines beginning with three asterisks (***).  These are considered comment lines and will be ignored by the program.

4.1.5.2  MONTE CARLO Input Guide

RECORD 1	FORMAT	A80

col: 1-80	TITLE:		label for Monte Carlo simulation title.

RECORD 2	FORMAT	I5,F10.0

col: 1-5	NRUN:		number of Monte Carlo runs (1 to 1000).

col: 6-15	PALPH:		confidence level for percentile confidence bounds.  Entered as a percent(%).  Default of 90.

RECORD 3	Repeat this record for number of inputs desired up to 50 records.

FORMAT	A20,2I5,5F10.0

col: 1-20	PNAME:		Monte Carlo input variable name (up to 20 characters).  See Table 4-2 on page 4-47.

col: 21-25	IND1:		integer index for horizon, application, or material.  See Table 4-2 on page 4-47.

col: 26-30	INDZ:		zone number (1 to 10).

col: 31-40	VAR1:		the mean value of the distribution variable.

col: 41-50	VAR2:		the standard deviation of the distribution variable.

col: 51-60	VAR3:		the minimum value for the variable.

col: 61-70	VAR4:		the maximum value for the variable.

col: 71-80	VAR5:		flag to indicate the type of the variable distribution.

0 = constant, 1 = normal

2 = log-normal, 3 = exponential

4 = uniform

5 = Johnson SU

6 = Johnson SB

7 = empirical, entered in record 4

8 = triangular

RECORD 4	FORMAT	A3

col: 1-3	ENDIT:		enter "END" to indicate end of record 3

RECORD 5	only if VAR5 = 7 (see record 3).

FORMAT	I5

col: 1-5	NDAT:		number of data pairs in empirical cumulative distribution (1 to 20).

RECORD 6	only if VAR5 = 7 (see record 3).  Note: repeat record 5 for every time VAR5 =7.

FORMAT	2F10.0

col: 1-10	DIST1:		value of quantile for data pair I where I = 1....NDAT.

col: 11-20	DIST2:		cumulative probability for data pair I where I = 1....NDAT.

RECORD 7	repeat this record for number of outputs desired up to 10 records.

FORMAT	A20,2I5,2(A20),I5

col: 1-20	SNAME:		Monte Carlo output variable name.  See Table 4-2 on page 4-47.

col: 21-25	IND1:		integer index for horizon, application, or material number.  See Table 4-2 on page 4-47.

col: 26-30	INDZ:		zone number (1 to 10).

col: 31-50	SNAME2:	enter "CDF" to indicate if cumulative distributions are plotted.

col: 51-70	SNAME3:	enter "WRITE" to indicate if values are written as output for each Monte Carlo run (NRUN).

col: 71-75	NAVG:		length of the averaging period (in days) for output variables (1 to 5).

RECORD 8	FORMAT	A3

col: 1-3	ENDIT:		enter "END" to indicate end of output variables.

RECORD 9	only if VAR5 = 1, 2, 5, or 6   note: this record may be repeated up to half of the number of inputs in record 3 if correlation is desired.

FORMAT	A20,2I5,A20,2I5,F10.0

col: 1-20	NAME1:		variable (PNAME) in record 3 to be correlated.

col: 21-25	IND1:		integer index for horizon, application, or material number (1 to 10).

col: 26-30	INDZ:		zone number (1 to 10).

col: 31-50	NAME2:		variable (PNAME) in record 3 to be correlated with NAME1.

col: 51-55	IND1:		same as IND1 above.

col: 56-60	INDZ:		same as INDZ above.

col: 61-70	CORR:		the value of the correlation coefficient for NAME1 and NAME2.

RECORD 10	FORMAT	A3

col: 1-3	ENDIT:		enter "END" to indicate end of correlation inputs.

TABLE 4-2.  MONTE CARLO INPUT AND OUTPUT LABELS

Parameter		Monte Carlo Label	Index

Random PRZM Model Inputs

Soil Bulk Density (g/cm3)	BULK DENSITY	Horizon

Wilting Point (cm3/cm3)	WILTING POINT 	Horizon

Field Capacity (cm3/cm3)	FIELD CAPACITY	Horizon

Organic Carbon Content (%)	ORGANIC CARBON	Horizon

Application Mass, Chem 1(kg/ha)	APPLICATION 1	App.

Application Mass, Chem 2(kg/ha)	APPLICATION 2	App.

Application Mass, Chem 3(kg/ha)	APPLICATION 3	App.

Dispersion Coeff., Chem 1(cm2/day)	DISPERSION 1		Horizon

Dispersion Coeff., Chem 2(cm2/day)	DISPERSION 2		Horizon

Dispersion Coeff., Chem 3(cm2/day)	DISPERSION 3		Horizon

Decay Rate in Water, Chem 1(days-1)	WATER DECAY 1		Horizon

Decay Rate in Water, Chem 2(days-1)	WATER DECAY 2		Horizon

Decay Rate in Water, Chem 3(days-1)	WATER DECAY 3		Horizon

Decay Rate in Vapor, Chem 1(days-1)	VAPOR DECAY 1		Horizon

Decay Rate in Vapor, Chem 2(days-1)	VAPOR DECAY 2		Horizon

Decay Rate in Vapor, Chem 3(days-1)	VAPOR DECAY 3		Horizon

Decay Rate of Sorbed, Chem 1(days-1)	SORBED DECAY 1	Horizon

Decay Rate of Sorbed, Chem 2(days-1)	SORBED DECAY 2	Horizon

Decay Rate of Sorbed, Chem 3(days-1)	SORBED DECAY 3	Horizon

Henry's Constant, Chem 1	HENRY'S CONSTANT 1	-----

Henry's Constant, Chem 2	HENRY'S CONSTANT 2	-----

Henry's Constant, Chem 3	HENRY'S CONSTANT 3	-----

Irrigation Moisture Level (Fraction)	IRRIG LEVEL			-----

Application Year		APP YEAR			App.

Julian Application Year	APP DAY			App.

Soil Water Content (cm3/cm3)	THETA				Comp.

Total Soil Pesticide,Chem 1(kg/ha)	SOIL PESTICIDE 1  	Comp.

Total Soil Pesticide,Chem 2(kg/ha)	SOIL PESTICIDE 2  	Comp.

Total Soil Pesticide,Chem 3(kg/ha)	SOIL PESTICIDE 3  	Comp.

Infiltration Depth (cm)	INFILTRATION		-----

Runoff Depth (cm)	RUNOFF			-----

Precipitation (cm)	PRECIPITATION		-----

Evapotranspiration	EVAPOTRANSPIRATION	Comp.

Flood or Furrow Irrigation Depth	IRREG DEPTH			-----

Runoff Flux, Chem 1 (kg/ha/day)	RUNOFF FLUX 1		-----

Runoff Flux, Chem 2 (kg/ha/day)	RUNOFF FLUX 2		-----

Runoff Flux, Chem 3 (kg/ha/day)	RUNOFF FLUX 3		-----

Erosion Flux, Chem 1 (kg/ha/day)	EROSION FLUX 1		-----

Erosion Flux, Chem 2 (kg/ha/day)	EROSION FLUX 2		-----

Erosion Flux, Chem 3 (kg/ha/day)	EROSION FLUX 3		-----

Decay Flux, Chem 1 (kg/ha/day)	DECAY FLUX 1		-----

Decay Flux, Chem 2 (kg/ha/day)	DECAY FLUX 2		-----

Decay Flux, Chem 3 (kg/ha/day)	DECAY FLUX 3		-----

Volat. Flux, Chem 1 (kg/ha/day)	VOLAT. FLUX 1		-----

Volat. Flux, Chem 2 (kg/ha/day)	VOLAT. FLUX 2		-----

TABLE 4-2.  MONTE CARLO INPUT AND OUTPUT LABELS (conclude)

Parameter		Monte Carlo Label	Index

Random VADOFT Model Inputs

Volat. Flux, Chem 3 (kg/ha/day)	VOLAT. FLUX 3		-----

Plant Flux, Chem 1 (kg/ha/day)	PLANT FLUX 1		Comp.

Plant Flux, Chem 2 (kg/ha/day)	PLANT FLUX 2		Comp.

Plant Flux, Chem 3 (kg/ha/day)	PLANT FLUX 3		Comp.

Root Zone Flux, Chem 1 (kg/ha/day)	ROOT FLUX 1			-----

Root Zone Flux, Chem 2 (kg/ha/day)	ROOT FLUX 2			-----

Root Zone Flux, Chem 3 (kg/ha/day)	ROOT FLUX 3			-----

Hydraulic Conductivity	HYDRAULIC CONDUC		Material

Residual Saturation	RESIDUAL SATURATION	Material

Van-Genuchten Alpha	V-G ALPHA			Material

Van-Genuchten N	V-G POWER N			Material

Decay Rate Chemical 1	VADOFT DECAY 1		Material

Decay Rate Chemical 2	VADOFT DECAY 2		Material

Decay Rate Chemical 3	VADOFT DECAY 3		Material

Dispersion Coefficient, Chemical 1	VAD DISPC 1			Material

Dispersion Coefficient, Chemical 2	VAD DISPC 2			Material

Dispersion Coefficient, Chemical 3	VAD DISPC 3			Material

Retardation, Chemical 1	VAD RETARD 1		Material

Retardation, Chemical 2	VAD RETARD 2		Material

Retardation, Chemical 3	VAD RETARD 3		Material

Random VADOFT Model Outputs

Total Water Flux		VAD WATER FLUX		-----

Advection Flux, Chemical 1	VAD ADVECTION 1		-----

Advection Flux, Chemical 2	VAD ADVECTION 2		-----

Advection Flux, Chemical 3	VAD ADVECTION 3		-----

Dispersion Flux, Chemical 1	VAD DISPERSION 1		-----

Dispersion Flux, Chemical 2	VAD DISPERSION 2		-----

Dispersion Flux, Chemical 3	VAD DISPERSION 3		-----

Decay Flux, Chemical 1	VAD DECAY FLUX 1		-----

Decay Flux, Chemical 2	VAD DECAY FLUX 2		-----

Decay Flux, Chemical 3	VAD DECAY FLUX 3		-----

Concentration, Chemical 1	VAD CONC 1			Node

Concentration, Chemical 2	VAD CONC 2			Node

Concentration, Chemical 3	VAD CONC 3			Node

SECTION 5

PARAMETER ESTIMATION

This section describes estimation of the parameters established in Section 4 to provide the user with an aid in inputing records for EXESUP, PRZM, and VADOFT modules.  For convenience to the user, all variables (or parameters) from Section 4 are categorized by module name and alphabetized to ensure quick reference.

5.1  EXESUP (Execution Supervisor)

The Execution Supervisor generally consists of labels and options; therefore, only parameters of obscure definitions are defined.

ECHO - This value can be entered as an integer value (1-9) to control the amount of display sent to the screen and output files.  Also entering "ON" or "OFF" rather than an integer value defaults the echo level to 5 (ON) or a minimal display of 1 (OFF).  For MONTE CARLO simulations, the echo level defaults to 1 automatically to prevent excessive output.

ENDDATE - A valid calendar date that specifies the day at which all of the simulation processes stop.  The user must choose this date with respect to meteorological file dates to ensure adequate weather data exist for the total elapsed time (STARTDATE to ENDDATE) of the simulation.

NUMBER OF CHEMICALS - This value (1-3) controls the number of pesticides being simulated.  As many as three separate chemicals are allowed per simulation.  Whether these multiple chemicals have a parent-daughter relationship depends upon transformation mass fractions entered in the PRZM and VADOFT input files.

PARENT OF 2 - This value implies the NUMBER OF CHEMICALS is greater than 1 and that a possible parent-daughter relationship exists.

PARENT OF 3 - This value implies the NUMBER OF CHEMICALS is greater than 2 and that a possible parent-daughter relationship exists.

PATH - A computer-specific drive and directory statement allowing any proceeding file names to be read or written in this area.

STARTDATE - A valid calendar date that specifies the day at which all simulation processes begin.  The user must choose this date with respect to meteorological file dates to ensure adequate weather data exists from this date forward to the ENDDATE.

TRACE - Primarily a tool for code debugging.  By entering "ON" or "OFF", the user has the option to track subroutine calling processes during a simulation.

WEIGHTS - Values entered that specify a fractional percent of fluxes between PRZM and VADOFT zones.  These values are ordered into a matrix with a sum of 1.0 for each PRZM zone.

5.2 PRZM (Pesticide Root Zone Model)

AC - Maintenance coefficient of the co-metabolizing Xc population.  This value specifies the amount of energy required to maintain co-metabolizing (inhibited growth) microorganisms.

AD - Soil water drainage rate.  This value is required if HSWZT = 1.  It is an empirical constant and dependent on both soil type and the number of compartments (DPN(I)/THKNS(I), where I = number of horizons) to be simulated.  Although there is limited experience using this option, three soils were evaluated for testing AD.  The analysis was performed by comparing the storage of water in the soil profile following the infiltration output from SUMATRA-1 (van Genuchten 1978).  Each soil had a profile depth of 125 cm.  The amount of water moving out of the profile changed by only 1 to 2% over the range of compartments (15-40).  Calibrating PRZM by comparison was accomplished and estimates of AD calculated.  Suggested values of AD for clay loam, loamy sand, and sand as a function of the number of compartments are given in Figure 5.11.

AFIELD - This is the erosion area or plot size in hectares.

ALBEDO - Soil surface albedo.  To simulate soil temperatures, ALBEDO values for each must be specified for each month.  As the surface condition changes, the ALBEDO values change accordingly.  Values for some natural surface conditions are provided in Table 5-21.

AM - Maintenance coefficient of the metabolizing Xm population.  This parameter is used in biodegradation processes to express the amount of energy required to maintain metabolizing (enhanced) microorganism growth rates.

AMXDR - The maximum active rooting depth of crops.  PRZM requires this parameter in centimeters to estimate the measurement of root depth from the land surface.  For ranges on specific root depths, consult the USDA Handbook No. 283 (Usual Planting and Harvesting Dates), or the local Cooperative Extension Service.  For general information,  Table 5-9 shows the ranges for major crops.

ANETD - This value represents soil evaporation moisture loss during a fallow, dormant period.  Evaporation is initially assumed to occur in the top 10 cm of soil with remaining moisture losses occurring below 10 cm up to the maximum rooting depth.  Values for ANETD apply when there is no growing season, allowing a reduced level of moisture loss

through evaporation.  For soils with limited drainage, set ANETD to 10 cm.  Values for free drainage soils are shown in Figure 5.2.

AR - Maintenance coefficient of the non-sensitive Xr population.  This parameter specifies the energy to sustain non-sensitive (indifferent) microorganisms.

AS - Maintenance coefficient of the sensitive Xs population.  This parameter specifies the value of energy required to sustain sensitive (lethally affected) microorganisms.

BD - Soil bulk density.  This value is required in the basic chemical transport equations of PRZM and is also used to estimate moisture saturation values.  Two methods are provided for estimating BD if site data are not available.  Method one requires percent sand, clay and organic matter.  The procedure from Rawls (1983) is used to estimate BD in (5.1):

Method 1

100.0

BD =	                                                      	(5.1)

%OM	  +	100.0 - %OM

OMBD		    MBD

where

BD = soil bulk density, g cm-3

OM = organic matter content of the soil, %

OMBD = organic matter bulk density of the soil, g cm-3

MBD = mineral bulk density, g cm-3

Step 1.	Locate the percent sand along bottom of Figure 5.10.

Step 2.	Locate the percent clay along side of Figure 5.10.

Step 3.	Locate the intersection point of the two values and read the mineral bulk density.

Step 4.	Solve the Rawls equation for BD.

Method 2

Step 1.	Use Table 5-29 to locate the textural class.

Step 2.	Read mean BD for the general soil texture.

Table 5-30 shows distributional properties of BD information.

BBT - Bottom boundary soil temperatures.  BBT values for each month must be specified.  The BBT soil temperature for shallow core depths may vary significantly with time throughout the year.  For deep cores, BBT will be relatively constant.  BBT can be estimated from NOAA data reports, Department of Commerce.  Depending on core depth used in the simulation, the average temperature of shallow groundwater, as shown in Figure 5.7, may be used to estimate BBT.

BDFLAG - Flag to indicate bulk density calculation.

BIOFLG - Biodegradation flag.  This flag allows the user to simulate the degradation of pesticides by microorganisms in the root zone.  Parameters associated with biodegradation

are very specific and may be difficult to obtain for soil conditions.  As an alternative, estimates of biological parameters can be found in literature on kinetics of microbial growth in liquid culture.

BT - Bottom width of the furrows.  BT will depend mostly upon the type of equipment used to dig the furrow channels and the spacing between the furrows.

CFLAG - Conversion flag for initial pesticide levels.  This flag is valid when ILP = 1.  If CFLAG = 0, then initial pesticide levels (PESTR) are in units of mgkg-1.  If CFLAG = 1, then initial pesticide levels (PESTR) are in units of kgha-1.  Leave CFLAG blank if ILP = 0.

CINTCP - The maximum interception storage of the crop (cm).  This parameter estimates the amount of rainfall that is intercepted by a fully developed plant canopy and retained on the plant surface.  A range of 0.1 to 0.3 for a dense crop canopy is reported by USDA (1980).  Values for several major crops are provided in Table 5-4.

CM - Mineralizable carbon (mg g-1).  This value represents the carbon substrate in the soil solution originating from a fraction of the carbon compounds of the solid phase.

CN - Runoff curve numbers of antecedent moisture condition II.  The interaction of hydrologic groups (Figure 5.4) and land use treatment (cover) is accounted for by assigning a runoff curve number (CN) for average soil moisture condition (AMC II) to important soil cover complexes for fallow, cropping, and residue parts of a growing season.  Tables 5-10 through 5-14 can be used to help estimate the correct curve numbers.

CORED - The total depth of the soil core in centimeters.  This value specifies the maximum depth in which PRZM simulates vertical movement.  CORED must be greater or equal to the active crop root depth (AMXDR).  For simulation using PRZM and VADOFT, the core depth (CORED) is usually equal to the root zone (AMXDR).

COVMAX - This is the maximum areal crop coverage.  PRZM estimates crop ground cover to a maximum value, COVMAX, by linear interpolation between emergence and maturity dates.  As a crop grows, its ground cover increases thus influencing the mass of pesticide that reaches the ground from an above surface application event.  For most crops, the maximum coverage will be on the order of 80 to 100 percent.

DAIR - Vapor phase diffusion coefficient.  When Henry's law constant (HENRYK) is greater than zero, vapor phase diffusion is used to calculate equilibrium between vapor and solution phases.  Fick's first law defines the diffusion coefficient as the proportionality between the chemical flux and the spatial gradient in its concentration (Nye 1979).  In soil, vapor phase diffusion occurs in the soil air space.  Each chemical will have its own characteristic diffusion coefficient depending on its molecular weight, molecular volume, and shape (Streile 1984).  Jury et al. (1983) has concluded that the diffusion coefficient will not show significant variations for different pesticides at a given temperature; they recommend using a constant value of 0.43 m2 day-1 for all pesticides.  This value is recommended unless other chemical-specific data are available.  Note that DAIR is entered in cm2 day-1.  The user should be sure to convert the above recommended value.

DEPI - The depth(s) of pesticide incorporation.  This variable is only needed if soil application of a chemical is specified (FAM=1).  Typical depths are 5 to 10 centimeters.  Representative values for several soil application methods are given in Table 5-15.

DGRATE - Vapor phase degradation rate constant(s).  Pesticides are degraded by different mechanisms, and at different rates, depending upon whether they are in vapor, liquid or absorbed phase (Streile 1984).  A lumped first-order rate is assumed for DGRATE.  In general, a zero value of DGRATE is recommended, unless chemical-specific data are available to justify a non-zero value.  For example, if the user is calibrating for a highly volatile and/or photo-sensitive chemical, vapor phase attenuation processes in the upper 1 to 2 mm of the soil surface may be very important.  Field studies have shown that photo chemical loss of organic chemicals may be rapid and substantial immediately following application to the land surface, especially in the case of hydrophobic or cationic organics that sorb to soil particles (Miller et al. 1987).

DISP - Dispersion of pesticide(s).  The dispersion or "smearing out" of the pesticide as it moves down in the profile is attributed to a combination of molecular diffusion and hydrodynamic dispersion.  Molecular diffusion, Dm, in soils will be lower than free-water diffusion and has been estimated by Bresler (1973)

Dm = Dw aebθ	(5.2)

where

Dw = molecular diffusion in free water, cm2 day-1

a  = soil constants having a range of 0.001 to 0.005

b  = soil constants having an approximate value of 10

θ  = volumetric water content, cm3 cm-3

Hydrodynamic dispersion is more difficult to estimate because of its site-soil specificity and its apparent strong dependence upon water velocity.  Most investigators have established an effective diffusion or dispersion coefficient that combines molecular and hydrodynamic terms.  Most notable among these is

D = 0.6 + 2.93 v1.11	(5.3)

where

D = effective dispersion coefficient, cm2 day-1

v = pore water velocity, cm day-1

by Biggar and Nielsen (1976).  Note in equation 5.3 that D is a time and depth varying function since v is both time and depth-varying.  The problem remains to estimate the assumed constant for DISP, the effective dispersion coefficient.  As noted earlier, the backward difference numerical scheme in PRZM produces numerical dispersion.  This dispersion is also related to the magnitude of the velocity term.  Other variables that influence the truncation error include the time and space steps.  A sensitivity test was performed to examine the influence of the spatial step, ▵x.  Results are given in Figure 5.5.  For these runs, the DISP parameter was set to 0.0.  The influence of DISP superimposed on the numerical dispersion created by the model at a ▵x value of 5.0 cm is shown in Figure 5.6.  A number of studies were performed to investigate the impact of model parameters other than DISP on the apparent dispersion.  From these, the following guidance is offered:

1)	A spatial step or compartment size of 5.0 cm will mimic the observed field effective dispersion quite well and should be used as an initial value.

2)	No fewer than 30 compartments should be used in order to minimize mass balance errors created by numerical dispersion.

3)	The DISP parameter should be set to 0.0 unless field data are available for calibration.

4)	If DISP calibration is attempted, the compartment size should be reduced to 1.0 cm to minimize numerical dispersion.

5)	The Biggar and Nielsen (1976) equation previously noted can be used to bound the values only should the need arise to increase dispersion beyond that produced by the numerical scheme.

If the user chooses the MOC algorithm to simulate advection transport, then numerical dispersion will be eliminated and a typical value for field-observed data dispersion should be entered.  Use of the MOC algorithm will result in increased model execution time.

DKRT12,DKRT13,DKRT23 - Transformation rate from a parent chemical (1 or 2) to a daughter chemical (2 and/or 3).  When multiple chemicals are specified in PRZM2.RUN, either a parent/daughter relationship exists or the chemicals are independent (chosen by the user).  For a parent/daughter relationship, DKRTxx is the mass fraction degrading from parent x to daughter x.  By setting DKRTxx to 0.0, the user is specifying that the multiple chemicals (xx) are independent parents.

DPN - Thickness of the compartments in the horizon.  The DPN parameter allows the user to specify a different layer depth for each soil horizon.  The value of each DPN can be divided by each horizon thickness (THKNS) to obtain the total number of compartments in PRZM.  In general, a smaller DPN will generate more accurate results and provide greater spatial resolution, but will also consume more CPU time.  From a volatilization viewpoint, a smaller DPN in the top horizon is required for better estimation of the volatilization flux from the soil surface.  In addition, since pesticide runoff is calculated from the surface layer, a smaller layer depth allows a better representation of surface-applied chemicals.  For the surface horizon, DPN values in the range of 0.5 to 2.0 cm are recommended; a 1.0 cm value for DPN is commonly used.  Smaller values down to 0.1 cm can be used for highly volatile compounds where volatilization is a major loss mechanism.  For subsurface soil horizons, DPN values in the range of 5.0 to 30.0 cm are recommended depending on the spatial resolution needed at lower depths.

DSRATE - Absorbed phase degradation rate constant(s).  See DWRATE for guidance.

DT - Daylight hours for each month in relation to latitude. These values are used to calculate total potential ET if daily pan evaporation data do not exist.  Table 5.2 lists monthly daylight hours for the northern hemisphere.

DWRATE - Solution phase degradation rate constant(s).  This rate constant contributes to the disappearance of pesticide(s) through decay.  For most cases, the same values should be used for solution (DWRATE) and adsorbed (DSRATE) phases for all depths.  This will allow a lumped first-order degradation rate constant.  The dissipation rate of pesticides below the root zone, however, is virtually unknown.  Several studies have suggested the rate of dissipation decreases with depth; however, no uniform correction factor was suggested between surface/subsurface rates.  First-order dissipation rates for selected pesticides in the root zone were tabulated in Tables 5-19 and 5-20.

EMMISS - Infrared Emissivity.  Most natural surfaces have an infrared emissivities lying between 0.9 and 0.99.  Values for all natural surfaces are not well known, but it is usually close to unity.  Specific values of EMMISS for some natural surfaces are given in Table 5-22.

EN - Manning's roughness coefficient.  This is a well known measure of the resistance of open channels to flow.  Chow (1959) suggests the values of EN range from 0.016 to 0.033 in excavated or dredged earth channels.  EN values for the furrows listed in Table 5-34  range from 0.01 to 0.048.  Table 5-37 lists the values of EN suggested by the USDA Soil Conservation Service for drainage ditches with various hydraulic radii (defined as the flow area divided by the wetted perimeter).

ENPY - Enthalpy of vaporization.  This parameter is used in the temperature correction equation for Henry's Law constant.  In a limited literature search, we could find only two pesticides for which ENPY values reported: 18.488 kcal mole-1 for lindane and 20.640 kcal mole-1 for napropamide (Streile 1984).  Chemical-specific values are needed for ENPY, but it appears that a value of 20 kcal mole-1 is a reasonable first guess.

ERFLAG - Erosion flag used to determine whether erosion losses are to be calculated during a simulation.  The total mass of pesticide loss by erosion is determined using the chemicals affinity for soil.  The amount of pesticide loss by these means is quite small for highly soluble pesticides.  If the apparent distribution coefficient is less than or equal to 5.0, erosion can usually be neglected.  For a compound having a greater distribution coefficient, erosion losses should be estimated (ERFLAG = 1).

EXMFLG - Flag for reporting output into the EXAMS model file format.  This flag allows a user to create an input file for the EXAMS model through PRZM output if so desired.  The EXAMS input file created has the name PRZM2EXA.Dxx where xx is the year of PRZM simulation.

FAM - Foliar application model flag.  This flag specifies how the pesticide is applied to foliage (if FAM = 2 or 3).

FEXTRC - Foliar washoff extraction coefficient.  Washoff from plant surfaces is modeled using a relationship among rainfall, foliar mass of pesticide, and an extraction coefficient.  The parameter (FEXTRC) is the required input parameter to estimate the flux of pesticide washoff.  Exact values are varied and depend upon the crop, pesticide properties, and application method.  Smith and Carsel (1984) suggest that a value of 0.10 is suitable for most pesticides.

FILTRA - The filtration parameter of initial foliage to soil distribution.  This parameter relates to the equation for partitioning the applied pesticide between foliage and the ground.  Lassey (1982) suggests values in the range of 2.3 to 3.3 m2 kg-1.  Miller (1979) suggested a value of 2.8 m2 kg-1 for pasture grasses.  Most of the variation appears to be due to the vegetation and not the aerosol.  FILTRA only applies if FAM=3.

FLEACH - The leaching factor as a fraction of irrigation water depth.  This factor is used to specify the amount of water added by irrigation to leach salts from saline soil and is defined as a fraction of the amount of water required to meet the soil water deficit.  For instance, a value of 0.25 indicates that 25 percent extra water is added to meet the soil water deficit.

FRMFLG - Flag for testing of ideal soil moisture conditions.  This flag specifies whether to check preceding days (WINDAY) after the target application date (APD) for moisture levels being ideal for pesticide application.  If a preceding date has adequate moisture levels and the target date does not, then the application date is changed automatically.  If the soil moisture after a specified number of days (WINDAY) fails to meet ideal conditions, execution is halted.

HENRYK - Henry's constant is a ratio of a chemical's vapor pressure to its solubility.  It represents the equilibrium between the vapor and solution phases.  It is quite common to express HENRYK as a dimensionless number.  Specific values for HENRYK for selected pesticides can be found in Table 5-18.

HF - Suction parameter.  HF represents water movement due to suction in unsaturated soils, and has units of length (meters).  As with KS, HF has been correlated with SCS hydrologic soil groups (Brakensiek and Rawls 1983) and are shown in Table 5-39.

HORIZN - Horizon number.  The horizon number in relation to the total number of horizons (NHORIZ) must be specified when inputing parameters for each of the PRZM horizons.

HSWZT - Flag to indicate soil water drainage calculation.  The HSWZT flag indicates which drainage model is invoked for simulating the movement of recharging water.  Drainage model 1 (HSZWT = 0) is for freely draining soils; drainage model 2 (HSZWT = 1) is for more poorly drained soils and requires the user to enter a soil water drainage rate (AD).

HTMAX - Maximum canopy height of the crop at maturation in centimeters.  Canopy height increases during crop growth resulting in pesticide flux changes in the plant compartment.  Users should have site-specific information on HTMAX since it varies with climate, crop species, and environmental conditions.  General ranges for different crops are listed in Table 5-16.

ICNAH - This is the surface condition after crop harvest.  Three values are allowed-- fallow, cropping, and residue (foliage remains on ground).

ICNCN - The crop number of the different crop.  This value is in relation to NDC (number of different crops).  This allows separate crop parameters to be specified for each different crop in a simulation.

IDFLAG - Thermal conductivity and volumetric heat capacity flag.  This flag allows a user to simulate soil temperature profiles.  If IDFLAG = 0, the user must enter thermal conductivity (THCOND) and volumetric heat capacity (VHTCAP).  If IDFLAG = 1, the model automatically simulates soil temperature profiles.

ILP - Initial pesticide levels flag.  ILP should be set to 1 when evidence of pesticide is present before the simulation start date (STARTDATE).  See also CFLAG and PESTR.

INCROP - The crop number associated with the number of different crops (NDC).  INCROP should be an increasing integer from the first different crop to the last different crop grown.

INICRP - Initial crop flag.  This flag indicates that before the simulation date occurs, a previous crop existed.

IRFLAG - Flag to simulate irrigation.  If irrigation is desired, the user has a choice of applying water for the whole year or during a cropping period whenever a specified deficit exists.

IRTYP - Specifies the type of irrigation used.  See Table 5-32.

IPEIND - Pan Factor flag.  When this flag is set to 0, daily pan evaporation is read from the meteorological file.  When this flag is set to 1, pan data are calculated from daylight hours according to latitude.  When this flag is set to 2, pan data are calculated through either the met file or daylight hours according to availability.

IPSCND - Flag indicating the disposition of pesticide remaining on foliage after harvest.  This flag only applies if FAM = 2 or 3.  If IPSCND = 1, pesticide remaining on foliage is converted to surface application to the top soil layer.  If IPSCND = 2, remaining pesticide on foliage is completely removed after harvest.  If IPSCND = 3, remaining pesticide on foliage is retained as surface residue and continues to undergo decay.

ISCOND - The surface condition for the initial crop if applicable.

ITFLAG - Flag for soil temperature simulation.  This flag allows a user to specify soil temperatures (BBT) for shallow core depths.  For deep cores (CORED), temperatures will remain relatively constant.

KC - Saturation constant of the co-metabolizing Xc population.  See KSM and KCM for further explanation.

KCM - Saturation constant of the metabolizing Xm population with respect to carbon concentration.  This value represents an inhibition of growth rate in relation to soil carbon.  Lower saturation constants result in decreased carbon content consequently resulting in a lower growth rate.

KE - Average enzyme content of the Xc population.  This parameter specifies the amount of the enzyme necessary to allow the population to break a pesticide down.

KD - Pesticide soil-water distribution coefficient.  The user can enter KD directly if KDFLAG = 0 (see PCMC and SOL) or allow the model to calculate KD automatically (KDFLAG = 1).

KDFLAG - Flag to indicate soil/pesticide adsorption coefficient.  A user may choose to enter KD by setting this flag to 0 else the model automatically calculates the adsorption coefficient.

KIN - Inhibition constant of the Xi population.  Evolution of the population requires a finite value controlling growth.  KIN accounts for natural variations found in metabolic activities affecting growth rates.

KL1 - Second-order death rate of the Xi population.

KL2 - Dissociation constant of the enzyme substrate complex.

KLDC - Death rate of the co-metabolizing Xc population.

KLDM - Death rate of the metabolizing Xm population.

KLDR - Death rate of the non-sensitive Xr population.

KLDS - Death rate of the sensitive Xs population.

KR - Saturation constant of the non-sensitive Xr population.  See KSM and KCM for further explanation.

KS - Saturated hydraulic conductivity.  This parameter represents the limiting infiltration rate when the soil column is saturated and suction pressure is no longer important.  KS depends upon soil mineralogy, texture, and degree of compaction.  Ranges for various unconsolidated materials are given in Table 5-38.  KS has also been correlated with SCS hydrologic soil groups (Brakensiek and Rawls 1983) shown in Table 5-39.

KSK - Carbon solubilization constant.

KSM - Saturation constant of the metabolizing Xm population with respect to pesticide concentration.  This value represents an inhibition of growth rate.  Lower saturation constants result in lower bacteria rates, consequently resulting in lower growth rates.  Higher saturation constants increase bacteria growth, resulting in higher growth rates.

MKS - Saturation constant of the sensitive Xs population.  See KSM and KCM for further explanation.

MOC - Flag to indicate method of characteristics calculation.  The MOC algorithm is a two-pass solution technique used to simulate advection and dispersion.  The solution technique reduces truncation error.  Because of the 24 hour time step in PRZM, this method can lead to significant losses of mass under high velocity (greater than 120 cm per day) conditions.

MUC - Specific growth rate of the co-metabolizing Xc population.

NAPS - Number of pesticide applications.  This is the total number of application dates specified during the simulation.  It is possible to apply up to three chemicals on the same application date, but for PRZM this still constitutes one application.

NCHEM - Number of chemicals in the simulation.  PRZM and VADOFT allow up to three chemicals to be specified.  Using more than one chemical (i.e., NCHEM=3) indicates either a parent-daughter relationship or multiple separate chemicals (determined by transformation mass fractions).  NCHEM should be consistent with the number of chemicals specified in the Execution Supervisor file.

NCPDS - Number of cropping periods.  This is entered as a sum of all cropping dates from the beginning simulation date to the ending simulation date.

NDC - The number of different crops in the simulation.  This value determines how many separate crops will be grown during a simulation.  If only one type of crop is grown (ex: corn), then NDC = 1.  This includes the crop type of the initial crop also (INICRP).

NHORIZ - Total number of horizons.  PRZM allows the user to specify how many horizons are to be simulated within the core depth (CORED).  The horizon should serve as a distinct morphologic zone generally described by layers (i.e., surface, subsurface, substratum) according to soil pedon descriptions or soil interpretation records, if available.

NPLOTS - Number of time series plots.  PRZM can report several output variables (PLNAME) to a time series file.  NPLOTS specifies how many are written in a single simulation.

OC - Percent of soil organic carbon.  OC is conventionally related to soil organic matter as %OC = %OM/1.724.  Guidance on estimating OM is found in Table 5-31.  Information is categorized by hydrologic soil group and by depth.  Also shown are coefficients of variation for each soil group and depth.  Carsel et al. (1988) determined that the Johnson SB distribution provides the best fit to this data.  Rao and Wagenet (1985) and Nielsen et al. (1983) have reported that these values are often normally distributed.  Carsel et al. (1988) noted that organic carbon is weakly correlated with field capacity and wilting point water content with the correlation coefficients ranging from 0.1 to 0.74.  Strength of correlation decreases with depth, as shown previously in Table 5-28.

PCDEPL - Fraction of available water capacity where irrigation is triggered.  The moisture level where irrigation is required is defined by the user as a fraction of the available water capacity.  This fraction will depend upon the soil-moisture holding characteristics, the type of crop planted, and regional agricultural practices.  In general, PCDEPL should range between 0.0 and 0.6, where a value of 0.0 indicates that irrigation begins when soil moisture drops to wilting point, and 0.6 indicates the more conservative practice of irrigating at 60 percent of the available water capacity.  Schwab et al. (1966) recommend values between 0.45 and 0.55.

PCMC - Flag for estimating distribution coefficients (KD).  PRZM allows the user to estimate the KD by multiplying the organic carbon partition coefficient (Koc) derived from the solubility (SOL).  PCMC is the flag for using one of four different models for estimating Koc.  The four models are:

PCMC1 Log Koc = (-0.54 * Log SOL) + 0.44

Koc = organic carbon distribution coefficient

where SOL = water solubility, mole fraction

PCMC2 Log Koc = 3.64 - (0.55 * Log SOL)

where SOL = water solubility, mg l-1

PCMC3 Log Koc = 4.40 - (0.557 * Log SOL)

where SOL = water solubility, micromoles l-1

PCMC4 Koc = SOL

where SOL = Koc, dimensionless

PESTR - Initial pesticide(s) levels.  PESTR levels are required if evidence of pesticide(s) is present before the simulation start date (ILP = 1).  PESTR is entered in units specified by CFLAG for each compartment in each horizon and for all chemicals (NCHEM).

PFAC - The pan factor is a dimensionless number used to convert daily pan evaporation to daily potential evapotranspiration (ET).  Pan factor general ranges are between 0.60 to 0.80.  See Figure 5.1 for specific regions of the United States.

PLDKRT - Foliage pesticide first-order decay rate.  Pesticide degradation rates on plant leaf surfaces is represented as a first-order process controlled by PLDKRT.  The user must be consistent in specifying PLDKRT and PLVKRT rates.  If PLDKRT includes volatilization processes, then PLVKRT should be zero.  If PLVKRT is non-zero then PLDKRT should include all attenuation processes except volatilization.  Recent information (Willis and McDowell 1987) is available for estimating degradation rates of pesticides on plant foliage.  In the work cited above, observed half-lives (days) were grouped by chemical family.  These were:

o	Organochlorine  5.0 ± 4.6

o	Organophosphorus  3.0 ± 2.7

o	Carbamate  2.4 ± 2.0

o	Pyrethroid  5.3 ± 3.6

These mean half-lives correspond to degradation rates of 0.14,0.23,0.29, and 0.13 day-1, respectively.  These are in reasonable agreement with values in Table 5-17.

PLNAME - Name of plotting variable.  When creating a time series plot, PLNAME specifies the variable in Table 4.1 for which that output data are written.

PLVKRT - Foliage pesticide first-order volatilization rate.  Pesticide volatilization from plant leaf surfaces is represented as a first-order process controlled by PLVKRT.  For organophosphate insecticides, Stamper et al. (1970) has shown that the disappearance rate from leaf surfaces can be estimated by a first-order kinetic approach.  Similar observations for first-order kinetics were found for volatilization of 2,4-D iso-octyl ester from leaf surfaces by Grover et al. (1985).  Volatilization losses of toxaphene and DDT from cotton plants decreased exponentially with time and were linearly related to the pesticide load on these plants (Willis et al. 1983).  Table 5-17 shows disappearance rates for selected pesticides on plant foliage.  These rates are applicable to estimation of PLVKRT since the overall decay rate (PLDKRT) includes loss associated with volatilization.

PSTNAM - Pesticide(s) name.  This is a label used to identify pesticide output.  Pesticide names should be placed in order of chemical 1, chemical 2, and chemical 3 if applicable (NCHEM=3).

Q - Average carbon content of the Xi population.

Q0 - Flow rate into a single furrow.  Q0 is defined as the volume of water entering the furrow per unit time.  Flow rates are usually set so that sufficient water reaches the end of the furrow without causing excessive erosion.  Table 5-35 lists the maximum non-erosion flow rates for various furrow channel slopes.

RATEAP - Maximum sprinkler application rate.  RATEAP is used to limit sprinkler applications to volumes that the sprinkler system is capable of delivering per time step.  This value is defined as a maximum depth (cm) of water delivered per hour.  Table 5-33 lists sprinkler rates.

SF - Channel slope.  SF is determined by regional topography and the design grades of the furrows, and is defined as vertical drop in elevation per horizontal distance of the bed.  Furrows are usually used only in relatively level terrain, with slopes no greater than 0.03 (Todd 1970).  A few representative slopes are listed in Table 5-34.

SFAC - The snowmelt factor is a used to calculate snowmelt rates in relation to temperature.  Snow is considered any precipitation that falls when the air temperature is below 0 degrees celsius.  In areas where climatology prevents snow fall, SFAC should be set to 0.0.  Typical ranges for SFAC are provided in Table 5-1.

SOL - Pesticide water solubility.  By specifying a water solubility (SOL) for pesticides, the model can calculate the Koc and KD by using one of the models specified for PCMC.  SOL must be entered according to the PCMC model selected. Table 5-19 on page 5-45 provides pertinent values for selected pesticides for obtaining SOL.  Methods are also available to calculate Koc (SOL if PCMC=4).  The octanol-water distribution coefficient can be used for calculating Koc with a relationship to organic carbon (OC).  Karickhoff et al. (1979) proposed a relationship between Kow and Koc given by

Log Koc = 1.00 (Log Kow) - 0.21	(5.4)

where

Kow = octanol-water distribution coefficient (cm3 g-1)

Koc = organic carbon distribution coefficient (cm3 g-1)

Selected pesticides having properties suitable for use with the octanol-water distribution model by Karickhoff are provided in Table 5-20.

SPACT - Special action variable.  During the course of a PRZM simulation, there may be a change in chemical behavior or agricultural management practices.  SPACT allows the user to specify a special action variable from section 4 and change its value at a user-specified time (SADAY,SAMON,SAYR).  Also the SPACT variable 'SNAPSHOT' can allow a user to output soil profile pesticide concentrations at a user-specified time during the simulation.

SPT - Initial soil temperature profile.  To simulate the soil temperature profile, initial SPT values for each soil horizon must be specified.  Since PRZM is often used for long periods of simulation, the initial temperature profile will not have any significant effect on the predicted temperature profile after a few days or weeks of simulation unless the core depth (CORED) is deep.  Lower horizons in the core should be assigned values corresponding approximately to the bottom boundary temperature (BBT).

TAPP - Total pesticide(s) application.  For each pesticide and each application date, the amount of pesticide is entered in kg-active ingredient ha-1.  Typical rates are included on the product's registration label.

THCOND,VHTCAP - Thermal conductivity and volumetric heat capacity of soil horizon.  If the user chooses to have the model simulate the soil temperature profile and sets the IDFLAG flag to zero, then the thermal conductivity (THCOND) and heat capacity (VHTCAP) must be specified.  Representative values for some soil types are given in Table 5-24.  Note that the value of THCOND is entered in PRZM in units of cal cm-1 ∘C-1 day-1; therefore, the values in Table 5-24 should be multiplied by 86,400.  If IDFLAG = 1, then THCOND and VHTCAP are calculated by the model from %sand, %clay, and %OC, based on the method in de Vries (1963).

THEFC,THEWP - Field capacity and wilting point.  Often these soil-water properties have been characterized and can be found from soil data bases.  Where such data are not available, one of three following estimation methods can be used.  Method one requires the textural properties (percent sand, silt, and clay), organic matter content (%), and bulk density (g cm-3) of a specific soil.  Method two utilizes a soil texture matrix for estimating soil water content if only the sand (%) and clay (%) contents are known.  Method three provides mean field capacity and wilting points if only soil texture is known.

Method 1 - Rawls and Brakensiek (1982)	(5.5)

⊝x =	a + [b * SAND(%)] + [c * CLAY(%)] + [d * ORGANIC MATTER(%)] + [e * BULK DENSITY(g cm-3)]

where

⊝x =	water retention cm3 cm-3 for a given matric potential (field capacity = -0.33 bar and wilting point = -15.0 bar)

a-e = regression coefficients

Step 1.	From Table 5-23 find the matric potential for field capacity and

wilting point.

Step 2.	For each matric potential, find the regression coefficient (a-e) that are in the Rawls and Brakensiek equation.

Step 3.	For any given soil solve the equation for the -0.33 and -15.0 potential.

Method 2

Use Figure 5.8 for estimating the field capacity and Figure 5.9 for estimating the wilting point, given the percent sand and clay.

Method 3

Use Table 5-25 to locate the textural class of the soil of choice.  After locating the textural class, read the mean field capacity and wilting point potentials (cm3 cm-3), to the right of the textural class.

Guidance for estimating distributional properties for THEFC and THEWP is given in Tables 5-26 and 5-27.

THETO - Initial water content of the soil.  This value provides the model with a starting calculation for moisture.  If site-specific data are not available, field capacity value is recommended for THETO.

THEWP - See THEFC for guidance.

THFLAG - Flag to indicate field capacity and wilting point calculation.

THKNS - Thickness of the horizon.  This value is the depth (cm) of the horizon specified (HORIZN) in relation to core depth (CORED).

TR - Storm duration peak runoff rate.  TR is entered as an average, although in reality this parameter changes seasonally as well as with each storm type.  This value represents the time period when storms occur producing peak runoff over a short duration.  Table 5-8 provides estimates for TR for selected locations in the U.S. for both mean summer and annual time periods while Figure 5.3 provides regionalized values for different areas in the United States.

UPTKF - Plant uptake efficiency factor.  This value provides for removal of pesticides by plants.  It is also a function of the crop root distribution and the interaction of soil, water, and the pesticide.  Several approaches to modeling the uptake of nutrients/pesticides have been proposed ranging from process models that treat the root zone system as a distribution sink of known density or strength to empirical approaches that assume a relationship to the transpiration rate.  Dejonckheere et al. (1983) reported the mass of uptake into sugarbeets for the pesticides aldicarb and thiofanox for three soils (sandy loam, silt loam, and sandy clay loam).  Mass removal expressed as a percentage of applied material for aldicarb on sandy loam, silt loam, and clay loam ranged from 0.46 to 7.14%, 0.68 to 2.32%, and 0.15 to 0.74%, respectively.  For thiofanox, 2.78 to 20.22%, 0.81 to 8.70%, and 0.24 to 2.42% removals were reported for the respective soils.  Other reviews have suggested ranges from 4 to 20% for removal by plants.  Sensitivity tests conducted with PRZM indicate an increase in the uptake by plants as the crop root zone (AMXDR) increases and the partition coefficient (KD) decreases.  For highly soluble pesticides and for crop root zones of greater than 120 cm, values of greater than 20% were simulated.  For initial estimates, a value of 1.0 for UPTKF is recommended.

USLEC - The universal soil loss cover management factor (C value).  Values for USLEC are dimensionless and range from 0.001 (well managed) to 1.0 (fallow or tilled condition).  One value for each of the three growing periods (fallow, cropping, and residue) is required.  Specific values can be calculated by Wischmeier and Smith (1978) or obtained from a local SCS office.  Generalized values are provided in Table 5-7.

USLEK - The universal soil loss equation (K) of soil erodibilty.  This is a soil-specific parameter developed by the USDA.  Specific values can be obtained from the local SCS office.  Approximate values are listed in Table 5-3.

USLELS - The universal soil loss equation (LS) topographic factor.  This is a slope length and steepness parameter developed by the USDA.  The value is dimensionless and can be estimated from Table 5-5.

USLEP - The universal soil loss equation (P) practice factor.  This value is developed by the USDA to describe conservative agricultural practices.  Values are dimensionless and range from 0.10 (extensive practices) to 1.0 (no supporting practices).  Specific values can be estimated in Table 5-6.

UCM - Specific growth rate of the metabolizing Xm population with respect to carbon concentration.

UR - Specific growth rate of the non-sensitive Xr population.

US - Specific growth rate of the sensitive Xs population.

USM - Specific growth rate of the metabolizing Xm population with respect to pesticide concentration.

VHTCAP - See THCOND for guidance.

WINDAY - An integer number of days.  This specifies the number of days after the target date (APD) that the code checks for ideal moisture conditions.  For this value to be valid, FRMFLG must equal 1.  WINDAY should be less than the difference of the target date (APD) to the next chronological target date.

WFMAX - The maximum dry foliar weight.  This value is used only if a user desires to have the model estimate the distribution between plants and the soil by an exponential function when a pesticide is applied.  WFMAX of the plant above ground (kg m-2) is the exponent used in the exponential foliar pesticide application model.  Estimates of WFMAX for several crops are given in Table 5-14.

X2 - Length of the furrow.  X2 will depend upon the size of the field and the local topography.  Table 5-35 lists maximum furrow lengths for various slope textures, irrigation application depths, and furrow slopes.

XFRAC - Location of the furrow.  XFRAC is a fraction of furrow length (X2) that specifies where PRZM infiltration calculations are performed.  To use the average depth of furrow infiltration depths, set XFRAC to -1.

Y1 - Metabolizing (Xm) microbial population.

Y2 - Co-metabolizing (Xc) microbial population.

Y3 - Sensitive (Xs) microbial population.

Y4 - Non-sensitive (Xr) microbial population.

YC - True growth yield of the co-metabolizing Xc population.

YCM - True growth yield of the metabolizing Xm population with respect to carbon concentration.

YR - True growth yield of the non-sensitive Xr population.

YS - True growth yield of the sensitive Xs population.

YSM - True growth yield of the metabolizing Xm population with respect to pesticide concentration.

ZRS - Side slope of the furrows.  This parameter is defined as the slope of the channel walls, horizontal distance/vertical distance.  ZRS will depend upon the cohesiveness of soils and the type of equipment used to dig the furrows.  Table 5-36 lists the suitable side slopes for different types of soils, with values ranging from 1.5 to 3.0 for unconsolidated materials.

ZWIND - Height of wind speed measuring instrument.  The wind speed anemometer is usually fixed at 10 meters (30 feet) above the ground surface.  This height may differ at some weather stations such as at a class A station where the anemometer may be attached to the evaporation pan.  The correct value can be obtained from the meteorological data reports for the station whose data are in the simulation.

Figure 5.1

Figure 5.2

Figure 5.3

Figure 5.4

Figure 5.5

Figure 5.6

Figure 5.7

TABLE 5-1.	TYPICAL VALUES OF SNOWMELT (SFAC) AS RELATED TO FOREST COVER

Snowmelt Factor, (cm ∘C-1 day-1)

FOREST COVER		MINIMUM	MAXIMUM

Coniferous - quite dense	0.08 - 0.12	0.20 - 0.32

Mixed forest - coniferous,

deciduous, open		0.10 - 0.16	0.32 - 0.40

Predominantly deciduous forest	0.14 - 0.20	0.40 - 0.52

Open areas		0.20 - 0.36	0.52 - 0.80

Source:	Anderson, E.A., "Initial Parameter Values for the Snow Accumulation and Ablation Model", Part IV.2.2.1, National Weather Service River Forecast System - User's Manual, NWS/NOAA, U.S. Dept. of Commerce, Silver Springs, MD., March 31, 1978.

TABLE 5-2.	MEAN DURATION (HOURS) OF SUNLIGHT FOR LATITUDES IN THE NORTHERN AND SOUTHERN HEMISPHERESa

Days In					Latitude North*

Month	Month		00	10	20	30	35	40	45	50

Jan		31		12.1	11.6	11.0	10.5	10.1	 9.8	 9.3	 8.6

Feb		28		12.1	11.7	11.6	11.2	10.9	10.7	10.4	10.0

Mar		31		12.1	12.0	12.0	12.0	12.0	12.0	11.8	11.8

Apr		30		12.1	12.4	12.6	13.0	13.1	13.3	13.6	13.8

May		31		12.1	12.5	13.1	13.7	14.1	14.4	14.9	15.4

Jun		30		12.1	12.7	13.3	14.0	14.5	15.0	15.5	16.3

Jul		31		12.1	12.5	13.2	13.9	14.3	14.7	15.2	15.9

Aug		31		12.1	12.4	12.9	13.2	13.5	13.7	14.1	14.5

Sep		30		12.1	12.2	12.2	12.4	12.4	12.5	12.5	12.7

Oct		31		12.1	11.8	11.6	11.4	11.3	11.1	10.9	10.7

Nov		30		12.1	11.8	11.2	10.7	10.3	10.0	 9.5	 9.1

Dec		31		12.1	11.5	10.9	10.2	 9.9	 9.4	 8.7	 8.1

a - Criddle, W.D. Methods of Computing Consumptive Use of Water, Proceedings ASCE. 84(IR 1). 1958

* - Values for the southern hemisphere were assumed equal to the northern hemisphere lagged by six months, e.g., the duration for January in the northern hemisphere is the same as July in the southern hemisphere.

TABLE 5-3.	INDICATIONS OF THE GENERAL MAGNITUDE OF THE SOIL/ERODIBILITY FACTOR, Ka

Organic Matter Content

Texture Class			<0.5%			2%			4%

Sand					0.05			0.03			0.02

Fine sand			0.16			0.14			0.10

Very Fine Sand			.42			.36			.28

Loamy Sand			.12			.10			.08

Loamy Fine Sand			.24			.20			.16

Loamy Very Fine Sand	.44			.38			.30

Sandy Loam			.27			.24			.19

Fine Sandy Loam			.35			.30			.24

Very Fine Sandy Loam	.47			.41			.33

Loam					.38			.34			.29

Silt Loam			.48			.42			.33

Silt					.60			.52			.42

Sandy Clay Loam			.27			.25			.21

Clay Loam			.28			.25			.21

Silty Clay Loam			.37			.32			.26

Sandy Clay			.14			.13			.12

Silty Clay			.25			.23			.19

Clay								0.13-0.29

a The values shown are estimated averages of broad ranges of specific-soil values.  When a texture is near the borderline of two texture classes, use the average of the two K values.  For specific soils, Soil Conservation Service K-value tables will provide much greater accuracy.  (Control of Water Pollution from Cropland, Vol. I, A Manual for Guideline Development.  U.S. Environmental Protection Agency, Athens, GA. EPA-600/2-75-026a).

TABLE 5-4.	INTERCEPTION STORAGE FOR MAJOR CROPS

Crop					Density			CINTCP (cm)

Corn					Heavy			0.25 - 0.30

Soybeans				Moderate		0.20 - 0.25

Wheat				Light			0.0  - 0.15

Oats					Light			0.0  - 0.15

Barley				Light			0.0  - 0.15

Potatoes				Light			0.0  - 0.15

Peanuts				Light			0.0  - 0.15

Cotton				Moderate		0.20 - 0.25

Tobacco				Moderate		0.20 - 0.25

TABLE 5-5.	VALUES OF THE EROSION EQUATION'S TOPOGRAPHIC FACTOR, LS, FOR SPECIFIED COMBINATIONS OF SLOPE LENGTH AND STEEPNESSa

Slope Length (feet)

%

Slope		25	50	75	100	150	200	300	400	500	600	800

0.5		.07	.08	.09	.10	.11	.12	.14	.15	.16	.17	.19

1		.09	.10	.12	.13	.15	.16	.18	.20	.21	.22	.24

2		.13	.16	.19	.20	.23	.25	.28	.30	.33	.34	.38

3		.19	.23	.26	.29	.33	.35	.40	.44	.47	.49	.54

4		.23	.30	.36	.40	.47	.53	.62	.70	.76	.82	.92

5		.27	.38	.46	.54	.66	.76	.93	1.1	1.2	1.3	1.4

6		.34	.48	.58	.67	.82	.95	1.2	1.4	1.5	1.7	1.9

8		.50	.70	.86	.99	1.2	1.4	1.7	2.0	2.2	2.4	2.8

10		.69	.97	1.2	1.4	1.7	1.9	2.4	2.7	3.1	3.4	3.9

12		.90	1.3	1.6	1.8	2.2	2.6	3.1	3.6	4.0	4.4	5.1

14		1.2	1.6	2.0	2.3	2.8	3.3	4.0	4.6	5.1	5.6	6.5

16		1.4	2.0	2.5	2.8	3.5	4.0	4.9	5.7	6.4	7.0	8.0

18		1.7	2.4	3.0	3.4	4.2	4.9	5.7	6.4	7.0	8.0	9.0

20		2.0	2.9	3.5	4.1	5.0	5.8	7.0	8.2	9.1	10.0	12.0

25		3.0	4.2	5.1	5.9	7.2	8.3	10.0	12.0	13.0	14.0	17.0

30		4.0	5.6	6.9	8.0	9.7	11.0	14.0	16.0	18.0	20.0	23.0

40		6.3	9.0	11.0	13.0	16.0	18.0	22.0	25.0	28.0	31.0	--

50		8.9	13.0	15.0	18.0	22.0	25.0	31.0	--	--	--	--

60		12.0	16.0	20.0	23.0	28.0	--	--	--	--	--	--

a	Values given for slopes longer than 300 feet or steeper than 18% are extrapolations beyond the range of the research data, and therefore, less certain than others.  (Control of Water Pollution from Cropland, Vol. I, A Manual for Guideline Development.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/2-75-026a).

TABLE 5-6.	VALUES OF SUPPORT-PRACTICE FACTOR, Pa

Land Slope (percent)

Practice	1.1-2.0	2.1-7.0	7.1-12.0	12.1-18.0	18.1-24.0

(Factor P)

Contouring (Pc)	0.60	0.50	0.60	0.80	0.90

Contour Strip

cropping (Psc)b

R-R-M-M	0.30	0.25	0.30	0.40	0.45

R-W-M-M	0.30	0.25	0.30	0.40	0.45

R-R-W-M	0.45	0.38	0.45	0.60	0.68

R-W	0.52	0.44	0.52	0.70	0.90

R-O	0.60	0.50	0.60	0.80	0.90

Contour listing

or ridge

planting (Pcl)	0.30	0.25	0.30	0.40	0.45

Contour terracing

(Pt)c	d0.6/√n	0.5/√n	0.6√n	0.8/√n	0.9/√n

No support practice	1.0	1.0	1.0	1.0	1.0

a	Control of Water Pollution from Cropland, Vol. I, A Manual for Guideline Development.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/2-75-026a.

b	R = rowcrop, W = fall-seeded grain, O = spring-seeded grain, M = meadow.  The crops are grown in rotation and so arranged on the field that rowcrop strips are always separated by a meadow or winter-grain strip.

c	These Pt values estimate the amount of soil eroded to the terrace channels and are used for conservation planning.  For prediction of off-field sediment, the Pt values are multiplied by 0.2.

d	n = number of approximately equal-length intervals into which the field slope is divided by the terraces.  Tillage operations must be parallel to the terraces.

TABLE 5-7.	GENERALIZED VALUES OF THE COVER AND MANAGEMENT FACTOR, C, IN THE 37 STATES EAST OF THE ROCKY MOUNTAINSa,b

Productivity Leveld

Line Crop, Rotation, and Managementc		High       Mod.

No.				C Value

Base value: continuous fallow, tilled up and down		1.00	1.00

Corn

1	C, RdR, fall TP, conv (1)		0.54	0.62

2	C, RdR, spring TP, conv (1)		.50	.59

3	C, RdL, fall TP, conv (1)		.42	.52

4	C, RdR, wc seeding, spring TP, conv (1)		.40	.49

5	C, RdL, standing, spring TP, conv (1)		.38	.48

6	C, fall shred stalks, spring TP, conv (1)		.35	.44

7	C(silage)-W(RdL, fall TP) (2)		.31	.35

8	C,RdL, fall chisel, spring disk, 40-30%rc(1)		.24	.30

9	C(silage),W wc seeding, no-till pl in c-k(1)		.20	.24

10	C(RdL)-w)RdL, spring TP) (2)		.20	.28

11	C, fall shred stalks, chisel pl, 40-30%rc(1)		.19		.26

12	C-C-C-W-M, RdL, TP for C, disk for W (6)		.17		.23

13	C, RdL, strip till row zones, 55-40% rc (1)		.16		.24

14	C-C-C-W-M-M, RdL, TP for C, disk for W (6)		.14		.20

15	C-C-W-M, RdL, TP for C, disk for W (4)		.12		.17

16	C, fall shred, no-till pl, 70-50% rc (1)		.11		.18

17	C-C-W-M-M, RdL, TP for C, disk for W (5)		.087		.14

18	C-C-C-W-M, RdL, no-till pl 2nd & 3rd C (5)		.076		.13

19	C-C-W-M, RdL, no-till pl 2nd C (4)		.068		.11

20	C, no-till pl in c-k wheat, 90-70% rc (1)		.062	.14

21	C-C-C-W-M-M, no-till pl 2nd & 3rd C (6)		.061	.11

22	C-W-M, RdL, TP for C, disk for W (3)		.055		.095

23	C-C-W-M-M, RdL, no-till pl 2nd C (5)		.051		.094

24	C-W-M-M, RdL, TP for C, disk for W (4)		.039		.074

25	C-W-M-M-M, RdL, TP for C, disk for W (5)		.032		.061

26	C, no-till pl in c-k sod, 95-80% rc (1)		.017		.053

Cottone

27	Cot, conv (Western Plains) (1)		0.42	0.49

28	Cot, conv (South) (1)		.34	.40

Meadow

29	Grass & Legume mix		.004	0.01

30	Alfalfa, lespedeza or Sericia		.020

31	Sweet clover		.025

Sorghum, grain (Western Plains)e

32	RdL, spring TP, conv (1)		0.43	0.53

33	No-till pl in shredded 70-50% rc		.11		.18

TABLE 5-7.	GENERALIZED VALUES OF THE COVER AND MANAGEMENT FACTOR, C, IN THE 37 STATES EAST OF THE ROCKY MOUNTAINSa,b

Productivity Leveld

Line Crop, Rotation, and Managementc		High       Mod.

No.				C Value

Base value: continuous fallow, tilled up and down		1.00	1.00

Soybeanse

34	B, RdL, spring TP, conv (1)		0.48		0.54

35	C-B, TP annually, conv (2)		.43		.51

36	B, no-till pl		.22		.28

37	C-B, no-till pl, fall shred C stalks (2)		.18		.22

Wheat

38	W-F, fall TP after W (2)		0.38

39	W-F, stubble mulch, 500 lbs rc (2)		.32

40	W-F, stubble mulch, 1000 lbs rc (2)		.21

41	Spring W, RdL, Sept TP, conv (N&S Dak) (1)		.23

42	Winter W, RdL, Aug TP, conv (Kansas) (1)		.19

43	Spring W, stubble mulch, 750 lbs rc (1)		.15

44	Spring W, stubble mulch, 1250 lbs rc (1)		.12

45	Winter W, stubble mulch, 750 lbs rc (1)		.11

46	Winter W, stubble mulch, 1250 lbs rc (1)		.10

47	W-M, conv (2)		.054

48	W-M-M, conv (3)		.026

49	W-M-M-M, conv (4)		.021

a	This table is for illustrative purposes only and is not a complete list of cropping systems or potential practices.  Values of C differ with rainfall pattern and planting dates.  These generalized values show approximately the relative erosion-reducing effectiveness of various crop systems, but locationally derived C values should be used for conservation planning at the field level.  Tables of local values are available from the Soil Conservation Service.

b	Control of Water Pollution from Cropland, Vol. I, A Manual for Guideline Development.  U.S. Environmental Protection Agency, Athens, GA. EPA-600/3-75-026a.

c	Numbers in parentheses indicate number of years in the rotation cycle.  No. (1) designates a continuous one-crop system.

d	High level is exemplified by long-term yield averages greater than 75 bu. corn or 3 tons grass-and-legume hay; or cotton management that regularly provides good stands and growth.

e	Grain sorghum, soybeans, or cotton may be substituted for corn in lines 12, 14, 17-19, 21-25 to estimate C values for sod-based rotations.

TABLE 5-7.	GENERALIZED VALUES OF THE COVER AND MANAGEMENT FACTOR, C, IN THE 37 STATES EAST OF THE ROCKY MOUNTAINSa,b

Productivity Leveld

Line Crop, Rotation, and Managementc		High       Mod.

No.				C Value

Base value: continuous fallow, tilled up and down		1.00	1.00

Abbreviations defined:

B - soybeans	F - fallow

C - Corn	M - grass & legume hay

c-k - chemically killed	pl - plant	conv - conventional

W - wheat	cot - cotton

we - cover

lbs rc -	pounds of crop residue per acre remaining on surface after new crop seeding

% rc -	percentage

7-50% rc -	70% cover for C values in first column; 50% for second column

RdR -	residues (corn stover, straw, etc.) removed or burned

RdL -	all residues left on field (on surface or incorporated)

TP -	turn plowed (upper 5 or more inches of soil inverted, covering residues

TABLE 5-8.	MEAN STORM DURATION* (TR) VALUES FOR SELECTED CITIES

Storm Duration (hrs)			Storm Duration (hrs)

Summer				Summer

Mean	(June-		Mean	(June-

Annual	Sept)	Location	Annual	Sept)

Great Lakes			Southeast

Champaign-Urbana,IL	6.1	4.6	Greensboro, NC	5.0 	3.6

Chicago, IL	5.7	4.5	Columbia, SC	4.5	3.5

Davenport, IA	6.6	5.3	Atlanta, GA	8.0	6.2

Detroit, MI	4.4	3.1	Birmingham, AL	7.2 	5.0

Louisville, KY	6.7	4.5	Gainesville, FL	7.6	6.6

Minneapolis, MN	6.0	4.5	Tampa, FL	3.6	3.1

Stubenville, OH	7.0	5.9

Toledo, OH	5.0	3.7	Rocky Mountains

Zanesville, OH	6.1	4.3

Lansing, MI (30 Yr)	5.6	4.2	Denver, CO (8 Yr)	4.3	3.2

Lansing, MI (21 Yr)	6.2	5.1	Denver, CO (25 Yr)	4.8 	3.2

Denver, CO (24 Yr)	9.1	4.4

Lower Mississippi Valley			Rapid City, SD	8.0	6.1

Salt Lake City, UT	4.5	2.8

Memphis, TN	6.9	4.7	Salt Lake City, UT	7.8	6.8

New Orleans, LA	6.9	5.0

Shreveport, LA (17)	7.8	5.3	California

Lake Charles, LA	7.7	5.9

Oakland, CA	4.3	2.9

Texas and Southwest			San Francisco, CA	5.9	11.2

Abilene, TX	4.2	3.3	Northeast

Austin, TX	4.0	3.3

Brownsville, TX	3.5	2.8	Caribou, ME	5.8	4.4

Dallas, TX	4.2	3.2	Boston, MA	6.1	4.2

El Paso, TX	3.3	2.6	Lake George, NY	5.4	4.5

Waco, TX	4.2	3.3	Kingston, NY	7.0	5.0

Phoenix, AZ	3.2	2.4	Poughkeepsie, NY	6.9	4.9

New York City, NY	6.7	4.8

Northwest			Mineola, LI, NY (2)	5.6	4.0

Upton LI, NY	6.3	4.6

Portland, OR (25yr)	5.4	4.5	Wantagh, LI, NY (2)	5.6	4.0

Portland, OR (10yr)	15.5	9.4	Long Island, NY	4.2	3.4

Eugene, OR	29.2	15.0	Washington, DC	5.9	4.1

Seattle, WA	21.5	12.7	Baltimore, MD	6.0	4.2

Source:	Woodward-Clyde Consultants, "Methodology for Analysis of Detention Basins for Control of Urban Runoff Quality", prepared for U.S. EPA, Office of Water, Nonpoint Source Division, 1986.

* -	These values may be misleading in arid regions or regions with pronounced seasonal rainfall patterns.

TABLE 5-9.  AGRONOMIC DATA FOR MAJOR AGRICULTURAL CROPS IN THE UNITED STATES

Representative			Crop      	Crop      	             	          	Range of

States of		Planting Window,	Emergency	Maturity	Harvest Window,	Average	Active Plant

Major States	Month, Day	(Days from	(Days from	Month, Day	Yield/Acre	Rooting Depth

Crop		Productiona		(Julian Day)b	Planting)	Planting)	(Julian Day)b	1977-1979c	(cm)

Corn	IA, IL, IN	April 25 (115)	5-15	110-130	Sept. 25 (268	110 bu	60-120

NE, OH	to June 15 (166)	5-15	110-130	to Dec. 10 (344)

Soybeans	IA, IL, IN,	May 1 (121) to	5-15	110-130	Sept. 15 (258)	35 bu	30-60

MS,OH	June 25 (176)			to Dec. 10 (344)

Cotton	TX, MS, CA,	March 1 (60) to	5-15	110-130	Sept. 1 (244) to	670 lbs	30-90

AZ, AR	May 25 (145)			Jan. 15 (015)

[TX to June 20			[TX Aug. 1 (213)

(171)]				to Dec. 20 (354)]

Wheat	KS, OK, CA,	Aug. 15 (227) to	5-15	200-225	June 15 (166) to	40 bu	15-30

ND, MT, WA,	Oct. 25 (298)			Sept. 20 (263)

MN, ID	[WA to Nov. 20 (324),

CA to Feb. 15 (046)]

Potatoes	Long Island	April 1 (091) to	5-15	150-170	Sept. 1 (244) to	335 cwt	15-45

NY, ME, ID,	May 1 (121)			Oct. 1 (274)

WA, CA, OR

Peanuts	GA, TX, AL,	April 5 (095) to	5-15	150-175	Aug. 10 (222) to	2550 lbs	30-60

NC, VA	June 5 (156)			Dec. 15 (349)

[TX Mar. 31 (090)

to July 20 (201)]

Tobacco	NC, SC, TN,	April 5 (095) to	Planted in	120-150	July 1 (182) to	2000 lbs	30-60

KY, VA	June 20 (171)	Field as		Oct. 1 (274)

Seedling

Grain	TX, KS, NE	TX Mar. 1 (060) to	5-15	120-150	TX July 1 (182)	62 bu	15-30

Sorghum		July 1 (182)			to Nov. 20 (324)

KS, NE May 5 (125)			KS, NE Sept. 20

to July 1 (182)			(263) to Dec. 1 (335)

aBay, D.M. and Bellinghausen, R.P.  Missouri Department of Agriculture.  May 1979.

bBurkhead, B.E., Max, R.C. Karnes, R.B., and Neid, E.  Usual Planting and Harvesting Dates.  USDA, Agricultural Handbook No. 283.  1972.

cKirkbride, J.W. (Ed.).  Crop Production Annual Summary.  USDA, Crop Reporting Board Publication CrPr 2-1.  1980.

TABLE 5-10.	RUNOFF CURVE NUMBERS FOR HYDROLOGIC SOIL-COVER COMPLEXESa (ANTECEDENT MOISTURE CONDITION II, AND Ia = 0.2 S)

Cover

Treatment	Hydrologic  	Hydraulic Soil Group   Land Use	or Practice	Condition	A	B	C	D

Fallow	Straight Row	---	77	86	91	94

Row crops	Straight Row	Poor	72	78	85	91

Straight row	Good	67	78	85	89

Contoured	Poor	70	79	84	88

Contoured	Good	65	75	82	86

Contoured and terraced	Poor	66	74	80	82

Contoured and terraced	Good	62	71	78	81

Small	Straight row	Poor	65	76	84	88

grain	Straight row	Good	63	75	83	87

Contoured	Poor	63	74	83	87

Contoured	Good	61	73	81	84

Contoured and terraced	Poor	61	72	79	82

Contoured and terraced	Good	59	70	78	81

Close-	Straight row	Poor	66	77	85	89

seeded	Straight row	Good	58	72	81	85

legumesb	Contoured	Poor	64	75	83	85

or rota-	Contoured	Good	55	69	78	83

tion	Contoured and terraced	Poor	63	73	80	83

meadow	Contoured and terraced	Good	51	67	76	80

Pasture		Poor	68	79	86	89

or range		Fair	49	69	79	84

Good	39	61	74	80

Contoured	Poor	47	67	81	88

Contoured	Fair	25	59	75	83

Contoured	Good	 6	35	70	79

Meadow		Good	30	58	71	78

Woods		Poor	45	66	77	83

Fair	36	60	73	79

Good	25	55	70	77

Farmsteads		----	59	74	82	86

Roads		----	72	82	87	89

(dirt)c		----	74	84	90	92

(hard surface)c

a Mockus, 1972.

b Close-drilled or broadcast.

c Including right-of-way.

TABLE 5-11.  METHOD FOR CONVERTING CROP YIELDS TO RESIDUEa

Straw/Grain	Bushel

Cropb	Ratio	Weight

(lbs)

Barley	1.5	48

Corn	1.0	56

Oats	2.0	32

Rice	1.5	45

Rye	1.5	56

Sorghum	1.0	56

Soybeans	1.5	60

Winter wheat	1.7	60

Spring Wheat	1.3	60

a	Crop residue = (straw/grain ratio) x (bushel weight in lb/bu) x (crop  yield in bu/acre).

b	Knisel, W.G. (Ed.).  CREAMS:  A Field-Scale Model for Chemicals, Runoff, and Erosion from Agricultural Management Systems.  USDA, Conservation Research Report No. 26, 1980.

TABLE 5-12.  RESIDUE REMAINING FROM TILLAGE OPERATIONSa

Residue

Tillageb	Remaining

Operation	(%)

Chisel Plow	65

Rod weeder	90

Light disk	70

Heavy disk	30

Moldboard plow	10

Till plant	80

Fluted coulter	90

V Sweep	90

a	Crop residue remaining = (crop residue from Table 10) x (tillage factor(s).

b	Knisel, W.G. (Ed.).  CREAMS:  A Field-Scale Model for Chemicals, Runoff, and

Erosion from Agricultural Management Systems.  USDA, Conservation Research

Report No. 26, 1980.

TABLE 5-13.	REDUCTION IN RUNOFF CURVE NUMBERS CAUSED BY CONSERVATION TILLAGE AND RESIDUE MANAGEMENTa

Large	Medium	Surface	Reductive

Residue	Residue	Covered	in Curve

Cropb	Cropb	by Residue	Numberd

(lb/acre)	(lb/acre)	(%)	(%)

0	0	0	0

400	150	10			0

700	300	19			2

1,100	450	28			4

1,500	700	37			6

2,000	950	46			8

2,500	1,200	55			10

6,200	3,500	90			10

a	Knisel, W.G. (Ed.).  CREAMS:  A Field-Scale Model for Chemicals, Runoff, and Erosion from Agricultural Management Systems.  USDA, Conservation Research Report No. 26, 1980.

b	Large-residue crop (corn).

c	Medium residue crop (wheat, oats, barley, rye, sorghum, soybeans).

d	Percent reduction in curve numbers can be interpolated linearly.  Only  apply 0 to 1/2 of these percent reductions to CNs for contouring and terracing practices when they are used in conjunction with conservation tillage.

TABLE 5-14.	VALUES FOR ESTIMATING WFMAX IN EXPONENTIAL FOLIAR MODEL

Bushela		Units

Yielda	dry wt.	Straw/Grain	Conversion

Crop	(Bu/Ac)	(lbs/Bu)	Ratio	Factor	WFMAX

Corn	110	56	1.0	1.1214 x 10-4	1.38

Sorghum	 62	56	1.0	1.1214 X 10-4	0.78

Soybeans	 35	60	1.5	1.1214 x 10-4	0.59

Winter

wheat 	40	60	1.7	1.1214 x 10-4	0.72

a 10-year average

TABLE 5-15.	PESTICIDE SOIL APPLICATION METHODS AND DISTRIBUTION

Method of

Application	Common Procedure	Distribution	DEPI

Broadcast	Spread as dry granules	Remains on the	0.0

or spray over the whole	soil surface

surface

Disked-in	Disking after broadcast	Assume uniform	10.0

application	distribution to

tillage depth

(10 cm)

Chisel-plowed	Chisel plowing after	Assume linear	15.0

broadcast	distribution to

tillage depth

(15 cm)

Surface banded	Spread as dry granules	Remains on soil	0.0

or a spray over a fraction	surface

of the row

Banded -	Spread as dry granules	Assume uniform	5.0

incorporated	or a spray over a	distribution to

fraction of the row	depth of incor-

and incorporated in	poration (5 cm)

planting operation

TABLE 5-16.	MAXIMUM CANOPY HEIGHT AT CROP MATURATION

Crop	Height (cm)	Reference

Barley	20 - 50		A

Grain Sorghum	90 - 110		B

Alfalfa	10 - 50		A

Corn	80 - 300		A

Potatoes	30 - 60		A

Soybeans	90 - 110		B

Sugarcane	100 - 400		A

References:

A.  Szeicy et al. (1969)

B.  Smith et al. (1978)

TABLE 5-17.	DEGRADATION RATE CONSTANTS OF SELECTED PESTICIDES ON FOLIAGEa

Class		Group			Decay Rate (days-1)

Organochlorine		Fast			0.231 - 0.1386

(aldrin, dieldrin, ethylan,

heptachlor, lindane,

methoxychlor).

Slow			0.1195 - 0.0510

(chlordane, DDT, endrin,

toxaphene).

Organophosphate		Fast				0.2772 - 0.3013

(acephate, chlorphyrifos-methyl,

cyanophenphos, diazinon, depterex,

ethion, fenitrothion, leptophos,

malathion, methidathion, methyl

parathion, phorate, phosdrin,

phosphamidon, quinalphos, alithion,

tokuthion, triazophos, trithion).

Slow			0.1925 - 0.0541

azinphosmethyl, demeton, dimethoate,

EPN, phosalone).

Carbamate		Fast			0.630

(carbofuran)

Slow			0.1260 - 0.0855

(carbaryl)

Pyrethroid	(permethrin)			0.0196

Pyridine	(pichloram)			0.0866

Benzoic acid	(dicamba)			0.0745

a	Knisel, W.G. (Ed.).  CREAMS:  A Field-Scale Model for Chemicals, Runoff, and Erosion from Agricultural Management Systems.  USDA, Conservation Research Report No. 26, 1980.

TABLE 5-18.	ESTIMATED VALUES OF HENRY'S CONSTANT FOR SELECTED PESTICIDES

Compound	Henry's Constant

(dimensionless)	References

Alachlor	1.3E-06	A

Aldrin	6.3E-04	D

Anthracene	4.4E-05	D

Atrazine	2.5E-07	A

Bentazon	2.0E-10	A

Bromacil	3.7E-08	C

Butylate	3.3E-03	A

Carbaryl	1.1E-05	A

Carbofuran 	1.4E-07	A

Chlorpyrifos	1.2E-03	A

Chrysene	4.7E-05	D

Cyanazine	1.2E-10	A

DDT	2.0E-03	C

Diazinon	5.0E-05	C

Dicamba	3.3E-08	A

Dieldrin	6.7E-04	C

Diuron	5.4E-08	C

Endrin	1.8E-05	D

EPTC	5.9E-04	C

Ethoprophos	6.0E-06	C

Fenitrothion	6.0E-06	B

Fonofos	2.1E-04	A

Heptachlor	1.7E-02	D

Lindane	1.3E-04	B

Linuron	2.7E-06	A

Malathion	2.4E-06	B

Methomyl	4.3E-08	A

Methyl Parathion	4.4E-06	A

Metolachlor	3.8E-07	A

Metribuzin	9.8E-08	A

Monuron	7.6E-09	C

Napropamide	7.9E-07	C

Parathion	6.1E-06	C

Permethrin	6.2E-05	A

Picloram	1.9E-08	B

Prometryne	5.6E-07	C

Simazine	1.3E-08	A

Terbufos	1.1E-03	A

Toxaphene	2.3E+00	A

Triallate	7.9E-04	C

Trichlorfon	1.5E-09	B

Trifluralin	6.7E-03	A

2,4-D (acid)	5.6E-09	A

2,4,5-T (acid)	7.2E-09	B

References:	A. Donigian et al. (1986)  B. Spencer et al. (1984) C. Jury et al. (1984)  D. Schnoor et al. (1987)

TABLE 5-19.	PHYSICAL CHARACTERISTICS OF SELECTED PESTICIDES FOR USE IN DEVELOPMENT OF PARTITION COEFFICIENTS (USING WATER SOLUBILITY) AND REPORTED DEGRADATION RATE CONSTANTS IN SOIL ROOT ZONE (Continued)

Mode of Action							Degradation

Solubility							Mole-			Partitioning Model		Rate Constant

in water		In-			Nema-		cular		PCMC1	PMCM2	PCMC3	in Soil

Common	20-25°C)	Refer-	sect-	Herb-	Fungi-	to-	Acar-	weight	Refer-	(mole	(mg/l)	(µm/l)	Root Zone	Refer-

Chemical	Name	(mg/l)	ence	icide	icide	cide	cide	icide	(g)	ence	fraction)			(days-1)	ence

Actellic	pirimiphosmethyl	5	a	X					274	b	3.28x10-7	5	18

Alachlor	alachlor	220	b		X				269.9	b	1.47x10-5	220	815	.0384	f

Antor	diethatyl ethyl	105	a		X				311.5	c	6.07x10-6	105	337	.0099-.0173	g

Aresin	monol inuron	735	a		X				214.6	b	6.17x10-6	735	3430

Balan	benefin	70	b		X				335.3	b	3.76x10-6	70	209	0.3349	f

Basalin	fluchloralin	0.7	b		X				355.7	b	3.55x10-8	0.7	2	0.0169	f

Baygon	propoxur	2000	a	X					209	b	1.72x10-4	2000	9600

Baygon Meb	plifenate	50	a	X		 			336.2	d	2.68x10-6	50	149

Bayleton	triadimefon	70	a			X			267.45	d	4.72x10-6	70	262

Baythion	phoxim	7	b	X					298	b	4.23x10-7	7	24

Baythion C	chlorphoxim	1.7	a	X					301.45	d	1.02x10-7	1.7	5.6

Betasan	bensulide	25	c		X				397.5	b	1.13x10-6	25	63

Bromophos	bromophos	40	a	X					366	b	1.97x10-6	40	109	.0198	f

Butachlor	butachlor	23	a		X				312	e	1.33x10-6	23	74

Bux	bufencarb	1	b	X					221.3	b	8.14x10-8	1.0	5

Carbamult	promecarb	92	a	X					207	d	8.01x10-6	92	444

Carbyne	barban	11	c		X				258.1	b	7.70x10-7	11	43	.0347	g

Chlordimeform	chlordimeform	250	a	X				X	196.7	b	2.30x10-5	250	1270

Chlorfenvin-

phos	chlorfenvinphos	110	a	X					359.5	b	5.51x10-6	110	306	.0055	f

Chloro IPC	chlorpropham	108	b		X				213.7	b	9.11x10-6	108	505	.0058-.00267	g

Chlorpyrifos	chlorpyrifos	2	b	X					350.5	b	1.03x10-7	2.0	6

Co-Ral	coumaphos	1.5	b	X					362.8	b	7.45x10-8	1.5	4

Counter	terbufos	15	a	X			X		288	d	9.38x10-7	15	52

DNOC	DNOC	130	a	X	X	X			198.1	b	1.18x10-5	130	656

Dichlorprop	dichlorprop	350	a		X				235	b	2.68x10-5	350	1490	.0578-.0866	f

Dimetan	dimetan	30000	b	X					197.3	b	2.74x10-3	30000	152000

Dimethoate	dimethoate	X=25000	a	X					229.1	b	1.97x10-3	25000	109000	.0057

Dinitramine	dinitroamine	1	a		X				322.2	c	5.60x10-8	1	3	.0193-.0856	f

Dinoseb	dinoseb	52	c		X				240.2	b	3.90x10-6	52	217	.0462-.0231	g

Dazomet	dazomet	1200	b		X	X	X		162.3	b	1.33x10-4	1200	7390

Devrinol	napropamide	73	a		X				271.36		4.85x10-6	73	269

Elocron	dioxacarb	6000	a	X					223	b	4.85x10-4	6000	26900	.3465-.0248	f

Evik	ametryn	185	a		X				227	b	1.47x10-5	185	815	.0231-.0077	g

Far-Go	triallate	4	b			X			304.6	b	2.37x10-7	4	13	.0231-.0713	g

Fongarid	furalaxyl	230	a			X			301	d	1.38x10-5	230	764

TABLE 5-19.	PHYSICAL CHARACTERISTICS OF SELECTED PESTICIDES FOR USE IN DEVELOPMENT OF PARTITION COEFFICIENTS (USING WATER SOLUBILITY) AND REPORTED DEGRADATION RATE CONSTANTS IN SOIL ROOT ZONE (Continued)

Mode of Action							Degradation

Solubility							Mole-			Partitioning Model		Rate Constant

in water		In-			Nema-		cular		PCMC1	PMCM2	PCMC3	in Soil

Common	20-25°C)	Refer-	sect-	Herb-	Fungi-	to-	Acar-	weight	Refer-	(mole	(mg/l)	(µm/l)	Root Zone	Refer-

Chemical	Name	(mg/l)	ence	icide	icide	cide	cide	icide	(g)	ence	fraction)			(days-1)	ence

Fornothion	fornothion	2600	a	X				X	257	b	1.82x10-4	2600	10100

Fuji-one	isoprothiolane	48	a	X		X			290	d	2.98x10-6	48	166

Gardona	tetrachlorvin-

phos	11	b	X					366	b	5.42x10-7	11	30	.1732-1386

Gesaran	methoprotryne	320	a		X				271	b	2.13x10-5	320	1180

Goal	oxyfluorfen	0.1	c		X				361.7	c	4.98x10-9	0.1	0.3	.0231-.0173	c

Guthion	azinphos-methyl	29	a	X					317.3	b	1.65x10-5	29	91	.0533-.0014	f

Hoelon	diclofop methyl	30	a		X				340.9	d	1.59x10-6	30	88

Imidan	phosmet	25	b	X					317.3	b	1.42x10-6	25	79

IPC	propham	250	b		X				179.2	b	2.51x10-5	250	1400	.0347-.0116	g

Linuron	linuron	75	a		X				249.1	b	5.42x10-6	75	300	.0280-.0039	f

Malathion	malathion	145	a	X					330.4	b	7.91x10-6	145	439	2.91-.4152	f

Mecoprop	mecoprop	620	a		X				214.6	b	5.21x10-5	620	2890

MEMC	MEMC	50000	a			X			295	d	3.05x10-3	50000	169000

Merpelan AZ	isocarbamid	13000	a		X				185	d	1.27x10-3	13000	70300

Mesoranil	aziprotryn	75	b		X				225	b	6.01x10-6	75	333

Mesurol	mercaptodi-

methur	2.7x107	a	X					225.3	b	2.16	2.7x107	1.2x108

Methomyl	methomyl	58000	a	X					162.2	b	6.44x10-3	58000	358000

Methoxychlor	methoxychlor	0.1	b	X					345.7	b	5.21x10	0.1	0.3	.0046-.0033	f

Meth-Para-	methyl Para-

thion	  thion	 = 57.5	a	X					263.2	b	3.94x10-6	57.5	219	.2207	f

Nemacur	fenamiphos	400	a			X			300	b	2.38x10-5	400	1320

Nortron	ethofumesate	110	a		X				286	d	6.93x10-6	110	385

Orthene	acephate	6.5x105	b	X					183.2	b	0.06 6.5x105 	650000 	3550000

Oxamyl	oxamyl	2.8x105	a	X			X	X	219	b	0.023 2.8x105	280000	1280000	.0354-.0646	f

Parathion	parathion	24	b	X					291.3	b	1.48x10-6	24	82	.2962-.0046	f

Patoran	metabromuron	330	a		X				258.9	d	2.30x10-5	330	1280	.0234	f

Phorate	phorate	50	b	X					260.4	b	3.46x10-6	50	192	.0363-.0040	f

Propachlor	propachlor	580	c		X				211.7	b	4.94x10-5	580	2740	.0231-.0139	g

Propanil	propanil	500	c		X				218	b	4.13x10-5	500	2290	.693-.231	g

Prowl	pendimethalin	0.5	c		X				281.3	c	3.20x10-8	0.5	1.8

Prynachlor	prynachlor	500	a		X				221.7	b	4.06x10-5	500	2260

Quinalphos	quinalphos	22	a	X				X	298	d	1.33x10-6	22	74

Ronstar	oxadiazon	0.7	b		X				345.23	b	3.65x10-8	0.7	2.0

Sancap	dipropetryn	16	a		X				255.4	b	1.13x10-6	16	63

Semeron	desmetryn	580	a		X				213	b	4.91x10-5	580	2720

TABLE 5-19.	PHYSICAL CHARACTERISTICS OF SELECTED PESTICIDES FOR USE IN DEVELOPMENT OF PARTITION COEFFICIENTS (USING WATER SOLUBILITY) AND REPORTED DEGRADATION RATE CONSTANTS IN SOIL ROOT ZONE (Continued)

Mode of Action							Degradation

Solubility							Mole-			Partitioning Model		Rate Constant

in water		In-			Nema-		cular		PCMC1	PMCM2	PCMC3	in Soil

Common	20-25°C)	Refer-	sect-	Herb-	Fungi-	to-	Acar-	weight	Refer-	(mole	(mg/l)	(µm/l)	Root Zone	Refer-

Chemical	Name	(mg/l)	ence	icide	icide	cide	cide	icide	(g)	ence	fraction)			(days-1)	ence

Supracide	methidathion	240	a	X				X	302	b	1.43x10-5	240	795	.0495-.0108	f

Tachigareu	hymexazol	85000	a			X			99.05	b	0.02	85000	858000

Temik	aldicarb	6000	a	X			X	X	190.3	b	5.68x10-4	6000	31500	.0322-.0116	f

Tolban	profluralin	0.1	a		X				347.3	c	5.19x10-9	0.1	0.3	.0049	f

Trifluralin	trifluralin	24	b		X				335.3	b	1.29x10-6	24	71	.0956-.0026	f

Tsumacide	MTMC	2600	a	X					165	d	2.84x10-4	2600	15800

Tordon	picloram	430	c		X				241.5	b	3.21x10-5	430	1780	.0354-.0019	f

Toxaphene	toxaphene	3	b	X					413	b	1.31x10-7	3	7	.0046	f

Trichlorfon	trichlorfon	120000	a	X					257.35	d	8.40x10-3	120000	466000

Calculations for the Karickhoff and Chiou partitioning equations are:

PCMC1:	ppm solubility        =  millimole solubility (MMS);  MMS  = molar solubility; (MS);                   MS                  = mole fraction

molecular weight (g)                                              103                                         55.5 (molar conc. water)

Chiou:	ppm solubility        =  millimole solubility (MMS);  MMS  * 106 = µm/l

molecular weight (g)                                               103

References:

a	Farm Chemical Handbook, Meister Publishing Company, Willoughby, OH (1981).

b	Pesticide Manual, issued by the British Crop Protection Council, 1968.

c	Herbicide Handbook of the Weed Science Society of America, 4th ed. 1979.

d	Calculations based on information from Farm Chemical Handbook, 1981.

e	Analytical Reference Standards and Supplemental Data for Pesticides and Other Organic Compounds, U.S. EPA-600/2-81-011, 1981.

f	Nash, R.G. 1980. Dissipation Rate of Pesticides from Soils.  Chapter 17.  IN CREAMS:  A field Scale Model for Chemicals, Runoff, and Erosion from Agricultural Management Systems.  W.G. Knisel, ed. USDA Conservation Research Report No. 26. 643 pp.

g	Control of Water Pollution from Cropland, Vol. I, a manual for guideline development, EPA-600/2-75-026a.1

TABLE 5-20.	OCTANOL WATER DISTRIBUTION COEFFICIENTS (log Kow) AND SOIL DEGRADATION RATE CONSTANTS FOR SELECTED CHEMICALS

Degradation Rate

Chemical Name	Log Kowb	Constant (days-1)	Reference

Alachlor		2.78	0.0384			A

Aldicarb		0.70	0.0322 - 0.0116		A

Altosid		2.25

Atrazine		2.45	0.0149 - 0.0063		A

Benomyl		2.42	0.1486 - 0.0023		A

Bifenox		2.24	0.1420			A

Bromacil		2.02

Captan		2.35

Carbaryl		2.56	0.1196 - 0.0768		A

Carbofuran		2.44	0.0768 - 0.0079		A

Chloramben		1.11

Chlordane		4.47	0.0020 - 0.0007

Chloroacetic Acid		-0.39

Chloropropham		3.06	0.0058 - 0.00267		D

Chloropyrifos		4.97

Cyanazine		2.24	0.0495			C

Dalapon		0.76	0.0462 - 0.0231		D

Dialifor		4.69

Diazinon		3.02	0.0330 - 0.0067		A

Dicamba		0.48	0.2140 - 0.0197		A

Dichlobenil		2.90	0.0116 - 0.0039

Dichlorofenthion		5.14

2,4,-Dichlorophenoxy-

acetic Acid		2.81	0.0693 - 0.0231		D

Dichloropropene		1.73

Dicofol		3.54

Dinoseb		2.30	0.0462 - 0.0231		D

Diuron		2.81	0.0035 - 0.0014		D

Endrin		3.21

Fenitrothion		3.36	0.1155 - 0.0578		A

Fluometuron		1.34	0.0231			C

Linuron		2.19	0.0280 - 0.0039		A

Malathion		2.89	02.91 - 0.4152		A

Methomyl		0.69

Methoxychlor		5.08	0.0046 - 0.0033		A

Methyl Parathion		3.32	0.2207			A

Monolinuron		1.60

Monuron		2.12	0.0046 - 0.0020		D

MSMA		-3.10

Nitrofen		3.10

Parathion		3.81	0.2961 - 0.0046		A

TABLE 5-20.	OCTANOL WATER DISTRIBUTION COEFFICIENTS (log Kow) AND SOIL

DEGRADATION RATE CONSTANTS FOR SELECTED CHEMICALS (concluded)

Degradation Rate

Chemical Name	Log Kowb	Constant (days-1)		Reference

Permethrin		2.88	0.0396			E

Phorate		2.92	0.0363 - 0.0040		A

Phosalone		4.30

Phosmet		2.83

Picloram		0.30	0.0354 - 0.0019		A

Propachlor		1.61	0.0231 - 0.0139		D

Propanil		2.03	0.693  - 0.231		D

Propazine		2.94	0.0035 - 0.0017		D

Propoxur		1.45

Ronnel		4.88

Simazine		1.94	0.0539 - 0074		A

Terbacil		1.89

Terbufos		2.22

Toxaphene		3.27	0.0046			E

Trifluralin		4.75	0.0956 - 0.0026		A

Zineb		1.78	0.0512			A

A	Nash, R. G. 1980.  Dissipation Rate of Pesticides from Soils.  Chapter 17.

IN CREAMS:  A Field Scale Model for Chemicals, Runoff, and Erosion from Agricultural Management Systems.  W. G. Knisel, ed.  USDA Conservation Research Report No. 26.  643pp.

B	Smith, C. N.  Partition Coefficients (Log Kow) for Selected Chemicals.

Athens Environmental Research Laboratory, Athens, GA.  Unpublished report, 1981.

C	Herbicide Handbook of the Weed Science Society of America, 4th ed.  1979.

D	Control of Water Pollution from Cropland, Vol. I, a manual for guideline 	development, EPA-600/2-75-026a.

E	Smith, C. N. and R. F. Carsel.  Foliar Washoff of Pesticides (FWOP) Model:

Development and Evaluation.  Accepted for publishing in Journal of Environmental Science and Health - Part B.  Pesticides, Food Contaminants, and Agricultural Wastes, B 19(3), 1984.

TABLE 5-21.	ALBEDO FACTORS OF NATURAL SURFACES FOR SOLAR RADIATION*

Surface		Reflectivity

Fresh Dry Snow		0.80-0.90

Clean, Stable Snow Cover		0.60-0.75

Old and Dirty Snow Cover		0.30-0.65

Dry Salt Cover		0.50

Lime		0.45

White Sand, Lime		0.30-0.40

Quartz Sand		0.35

Granite		0.15

Dark Clay, Wet		0.02-0.08

Dark Clay, Dry		0.16

Sand, Wet		0.09

Sand, Dry		0.18

Sand, Yellow		0.35

Bare Fields		0.12-0.25

Wet Plowed Field		0.05-0.14

Newly Plowed Field		0.17

Grass, Green		0.16-0.27

Grass, Dried		0.16-0.19

Grass, High Dense		0.18-0.20

Prairie, Wet		0.22

Prairie, Dry		0.32

Stubble Fields		0.15-0.17

Grain Crops		0.10-0.25

Alfalfa, Lettuce, Beets, Potatoes		0.18-0.32

Coniferous Forest		0.10-0.15

Deciduous Forest		0.15-0.25

Forest with Melting Snow		0.20-0.30

Yellow Leaves (fall)		0.33-0.36

Desert, Dry Soils		0.20-0.35

Desert, Midday		0.15

Desert, Low Solar Altitude		0.35

Water (0 to 30°)a		0.02

Water (60°)a		0.06

Water (85°)a		0.58

*	References:

Van Wijk, W.R.  1963.  Physics of Plant Environment, p. 87.  North-	Holland Publishing

Co., Amsterdam.

Brutsaert, W.  1982.  Evaporation into the Atmopshere:  Theory, History, 	and

Applications.  D. Reidel Publishing Co., Dordrecht, Holland.

a	angle of solar incidence.

TABLE 5-22.	EMISSIVITY VALUES FOR NATURAL SURFACES AT NORMAL TEMPERATURES*

Surface		Emissivity

Sand (dry-wet)		0.95-0.98

Mineral Soil (dry-wet)		0.95-0.97

Peat (dry-wet)		0.97-0.98

Firs		0.97

Tree Vegetation		0.96-0.97

Grassy Vegetation		0.96-0.98

Leaves		0.94-0.98

Water		0.95

Snow (old)		0.97

Snow (fresh)		0.99

*	References

Van Wijk, W.R.  1963.  Physics of Plant Environment, p. 87.  North-Holland Publishing

Co., Amsterdam.

Brutsaert, W.  1982.  Evaporation into the Atmosphere:  Theory, History, and Applications, D. Reidel Publishing Co., Dordrecht, Holland.

TABLE 5-23.	COEFFICIENTS FOR LINEAR REGRESSION EQUATIONS FOR PREDICTION OF SOIL WATER CONTENTS AT SPECIFIC MATRIC POTENTIALSa

Organic	Bulk

Sand	Clay	Matter	Density

Matric	Intercept	(%)	(%)	(%)	(g cm-3)	R2

Coefficient	   a	 b	 c	 d	   e

-0.20	0.4180	-0.0021	0.0035	0.0232	-0.0859	0.75

-0.33	0.3486	-0.0018	0.0039	0.0228	-0.0738	0.78

-0.60	0.2819	-0.0014	0.0042	0.0216	-0.0612	0.78

-1.0	0.2352	-0.0012	0.0043	0.0202	-0.0517	0.76

-2.0	0.1837	-0.0009	0.0044	0.0181	-0.0407	0.74

-4.0	0.1426	-0.0007	0.0045	0.0160	-0.0315	0.71

-7.0	0.1155	-0.0005	0.0045	0.0143	-0.0253	0.69

-10.0	0.1005	-0.0004	0.0044	0.0133	-0.0218	0.67

-15.0	0.0854	-0.0004	0.0044	0.0122	-0.0182	0.66

a  Rawls, W. J., U.S. Department of Agriculture, Agricultural Research

Service, Beltsville, MD. Personal Communication.

TABLE 5-24.	THERMAL PROPERTIES OF SOME SOIL AND REFERENCE MATERIALS*

Water Content	Heat Capacity		Thermal Cond.

Material	    (%)	(cal cm-3°C-1)	(cal cm-1 °C-1 sec-1)

Clay		1.44			0.00288

Light Soil w/Roots		0.09			0.00027

Wet Sandy Soil		0.64			0.0064

Dead Air		0.000312			0.00005

Hudson River Sand	4.5	0.2			0.0091

18.1	0.336			0.03

Podunk Fine Sandy Loam	6.6	0.221	 	0.0012

20.2	0.371			0.0026

Leonardtown Silt Loam	9.0	0.316			0.0018

18.4	0.338			0.0021

Muck Soil	23.0	0.251			0.00076

59.0	0.321			0.00108

Yolo Clay	0.0	0.236			0.0014

29.0	0.72			0.0083

Granite Sandy Loam	0.0	0.291			0.0017

22.7	0.706			0.0071

Fine Calcareous Loam	0.0	0.175			0.00079

24.4	0.430			0.0048

Granitic Sand	0.0	0.269			0.00137

13.1	0.636			0.0108

Barns Loam	5.1	0.29			0.00041

26.0	0.35			0.00086

Chester Loam	2.0	0.32			0.00045

13.4	0.37			0.00087

Herman Sandy Loam	1.3	0.30			0.00049

13.4	0.37			0.00087

Kalkaska Loamy Sand	0.8	0.32			0.0006

5.7	0.37			0.00124

Northway Silt Loam	6.6	0.384			0.0013

22.5	0.636			0.0025

Fairbanks Silty Clay Loam	12.3	0.436			0.002

25.4	0.625			0.0028

Dakota Sandy Loam	1.9	0.269			0.00059

4.9	0.483			0.0054

Black Cotton Soil		0.336			0.00037

*	References

Rosenberg, N.J.  1974.  Microclimate:  The Biological Environment, p. 105.  Wiley -

Interscience, New York.

Kilmer, V.J.  1982.  Handbook of Soils and Climate in Agriculture.  CRC Press, Inc.  Boca

Raton, Florida.

TABLE 5-25.	HYDROLOGIC PROPERTIES BY SOIL TEXTUREa

Range of

Textural Properties

(Percent)		Water Retained at Water Retained at

Texture                                   		-0.33 Bar Tension -15.0 Bar Tension

Class	Sand	Silt	Clay		cm3 cm-3		cm3 cm-3

Sand	85-100	0-15	0-10		0.091b		0.033b

(0.018 - 0.164)c	(0.007 - 0.059)c

Loamy	70-90	0-30	0-15		0.125		0.055

Sand				(0.060 - 0.190)	(0.019 - 0.091)

Sandy	45-85	0-50	0-20		0.207		0.095

Loam				(0.126 - 0.288)	(0.031 - 0.159)

Loam	25-50	28-50	8-28		0.270		0.117

(0.195 - 0.345)	(0.069 - 0.165)

Silt Loam	0-50	50-100	8-28		0.330		0.133

(0.258 - 0.402)	(0.078 - 0.188)

Sandy Clay	45-80	0-28	20-35		0.257		0.148

Loam				(0.186 - 0.324)	(0.085 - 0.211)

Clay Loam	20-45	15-55	28-50		0.318		0.197

(0.250 - 0.386)	(0.115 - 0.279)

Silty Clay	0-20	40-73	28-40		0.366		0.208

Loam				(0.304 - 0.428)	(0.138 - 0.278)

Sandy Clay	45-65	0-20	35-55		0.339		0.239

(0.245 - 0.433)	(0.162 - 0.316)

Silty Clay	0-20	40-60	40-60		0.387		0.250

(0.332 - 0.442)	(0.193 - 0.307)

Clay	0-45	0-40	40-100		0.396		0.272

(0.326 - 0.466)	(0.208 - 0.336)

a Rawls, W. J., D. L. Brakensiek, and K. E. Saxton.  Estimation of Soil Water Properties.

Transactions ASAE Paper No. 81-2510, pp. 1316 - 1320.  1982.

b Mean value.

c One standard deviation about the mean.

TABLE 5-26.	DESCRIPTIVE STATISTICS AND DISTRIBUTION MODEL FOR FIELD

CAPACITY (PERCENT BY VOLUME)

Original Data

Stratum	Sample					CV	Distribution Model

(m)	Size	Mean	Median	s.d.	(%)	Transform	Mean	s.d.

Class A

0.0-0.3	52	11.8	9.4		9.2	78	ln	2.25	0.65

0.3-0.6	50	9.6	8.1		7.9	82	ln	1.99	0.73

0.6-0.9	42	7.3	5.9		5.8	79	ln	1.73	0.73

0.9-1.2	39	7.1	5.8		5.0	70	ln	1.73	0.71

Class B

0.0-0.3	456	19.5	19.1		8.3	42	sU	0.316	0.13

0.3-0.6	454	18.8	18.8		7.4	39	sU	0.311	0.12

0.6-0.9	435	18.7	18.7		7.1	39	sU	0.298	0.11

0.9-1.2	373	17.5	17.5		7.6	43	sU	0.288	0.12

Class C

0.0-0.3	371	22.4	22.5		7.8	35	sU	0.363	0.12

0.3-0.6	362	22.8	23.2		7.8	34	sU	0.369	0.12

0.6-0.9	336	22.7	22.9		8.6	38	sU	0.368	0.13

0.9-1.2	290	22.2	21.3		8.9	40	sU	0.359	0.13

Class D

0.0-0.3	230	24.1	24.2		9.1	38	sU	0.387	0.14

0.3-0.6	208	26.1	26.3		9.3	36	sU	0.419	0.14

0.6-0.9	178	25.0	25.6		8.2	33	sU	0.403	0.13

0.9-1.2	146	24.1	24.4		8.1	33	sU	0.390	0.12

CV   = coefficient of variation

s.d. = standard deviation

Source:  Carsel et al. (1988)

TABLE 5-27.	DESCRIPTIVE STATISTICS AND DISTRIBUTION MODEL FOR WILTING POINT (PERCENT BY VOLUME)

Original Data

Stratum	Sample					CV	Distribution Model

(m)	Size	Mean	Median	s.d.	(%)	Transform	Mean	s.d.

Class A

0.0-0.3	118	4.1	3.1		3.4	82	ln	1.83	0.64

0.3-0.6	119	3.2	2.3		2.4	75	ln	0.915	0.71

0.6-0.9	113	2.9	2.1		2.3	81	sB	3.32	0.88

0.9-1.2	105	2.6	1.9		2.3	87	sB	3.43	0.92

Class B

0.0-0.3	880	9.0	8.7		4.0	45	sU	0.150	0.066

0.3-0.6	883	9.4	9.3		4.3	46	sU	0.156	0.071

0.6-0.9	866	9.1	8.9		4.4	48	sU	0.151	0.072

0.6-1.2	866	8.6	8.4		4.6	53	sU	0.143	0.076

Class C

0.3-0.3	678	10.8	10.4		5.1	48	sU	1.63	0.62

0.3-0.6	677	12.2	12.1		5.6	46	sU	0.202	0.091

0.6-0.9	652	12.2	11.9		6.0	49	sU	0.201	0.096

0.9-1.2	582	11.8	11.5		5.7	48	sU	0.194	0.092

Class D

0.0-0.3	495	14.6	13.8		7.6	52	sU	1.26	0.76

0.3-0.6	485	16.9	17.0		7.3	43	sU	0.277	0.12

0.6-0.9	437	16.6	16.3		7.4	45	sU	0.271	0.12

0.9-1.2	401	15.7	15.1		7.6	48	sU	0.257	0.12

CV   = coefficient of variation

s.d. = standard deviation

Source:  Carsel et al. (1988)

TABLE 5-28.	CORRELATIONS AMONG TRANSFORMED VARIABLES OF ORGANIC MATTER, FIELD CAPACITY, AND WILTING POINT

Stratum		OM + WP		FC + OM			FC + WP

(m)	N		Corr.	N		Corr.	N		Corr.

Class A

0.0-0.3	118		0.738	52		0.624	51		0.757

0.3-0.6	119		0.630	49		0.404	49		0.759

0.6-0.9	111		0.487	42		0.427	42		0.811

0.9-1.2	98		0.456	38		0.170	39		0.761

Class B

0.0-0.3	877		0.545	459		0.609	455		0.675

0.3-0.6	870		0.372	446		0.384	450		0.639

0.6-0.9	844		0.375	419		0.336	429		0.714

0.9-1.2	780		0.392	347		0.412	370		0.762

Class C

0.0-0.3	673		0.495	369		0.577	370		0.745

0.3-0.6	664		0.473	355		0.409	361		0.775

0.6-0.9	627		0.457	321		0.434	334		0.784

0.9-1.2	543		0.434	264		0.456	289		0.751

Class D

0.0-0.3	488		0.538	228		0.496	226		0.847

0.3-0.6	472		0.434	201		0.454	204		0.845

0.6-0.9	420		0.456	171		0.369	174		0.782

0.9-1.2	384		0.415	137		0.106	145		0.687

OM = organic matter; WP = wilting point; FC = field capacity; N = sample size; Corr. = correlation.

Source:  Carsel et al. 1988.

TABLE 5-29.	MEAN BULK DENSITY (g cm-3) FOR FIVE SOIL TEXTURAL CLASSIFICATIONSa

Soil Texture	Mean Value	Range Reported

Silt Loams	1.32				0.86 - 1.67

Clay and Clay Loams	1.30				0.94 - 1.54

Sandy Loams	1.49				1.25 - 1.76

Gravelly Silt Loams	1.22				1.02 - 1.58

Loams	1.42				1.16 - 1.58

All Soils	1.35				0.86 - 1.76

a Baes, C.F., III and R.D. Sharp.  1983.  A Proposal for Estimation of Soil 	Leaching Constants for Use in Assessment Models.  J. Environ. Qual. 	12(1):  17-28.

TABLE 5-30.	DESCRIPTIVE STATISTICS FOR BULK DENSITY (g cm-3)

Stratum	Sample				CV

(m)	Size	Mean	Medium	s.d.	(%)

Class A

0.0-0.3	40	1.45	1.53	0.24		16.2

0.3-0.6	44	1.50	1.56	0.23		15.6

0.6-0.9	38	1.57	1.55	0.16		10.5

0.9-1.2	34	1.58	1.59	0.13		8.4

Class B

0.0-0.3	459	1.44	1.45	0.19		13.5

0.3-0.6	457	1.51	1.53	0.19		12.2

0.6-0.9	438	1.56	1.57	0.19		12.3

0.9-1.2	384	1.60	1.60	0.21		12.9

Class C

0.0-0.3	398	1.46	1.48	0.22		15.0

0.3-0.6	395	1.58	1.59	0.23		14.5

0.6-0.9	371	1.64	1.65	0.23		14.2

0.9-1.2	326	1.67	1.68	0.23		14.0

Class D

0.0-0.3	259	1.52	1.53	0.24		15.9

0.3-0.6	244	1.63	1.66	0.26		16.0

0.6-0.9	214	1.67	1.72	0.27		16.3

0.9-1.2	180	1.65	1.72	0.28		17.0

CV   = coefficient of variation

s.d. = standard deviation

Source:  Carsel et al. (1988)

TABLE 5-31.	DESCRIPTIVE STATISTICS AND DISTRIBUTION MODEL FOR ORGANIC MATTER (PERCENT BY VOLUME)

Original Data

Stratum	Sample					CV	Distribution Model

(m)	Size	Mean	Median	s.d.	(%)	Mean	s.d.

Class A

0.0-0.3	162	0.86	0.62		0.79	92	-4.53	0.96

0.3-0.6	162	0.29	0.19		0.34	114	-5.72	0.91

0.6-0.9	151	0.15	0.10		0.14	94	-6.33	0.83

0.9-1.2	134	0.11	0.07		0.11	104	-6.72	0.87

Class B

0.0-0.3	1135	1.3	1.1		0.87	68	-4.02	0.76

0.3-0.6	1120	0.50	0.40		0.40	83	-5.04	0.77

0.6-0.9	1090	0.27	0.22		0.23	84	-5.65	0.75

0.9-1.2	1001	0.18	0.14		0.16	87	-6.10	0.78

Class C

0.0-0.3	838	1.45	1.15		1.12	77	-3.95	0.79

0.3-0.6	822	0.53	0.39		0.61	114	-5.08	0.84

0.3-0.9	780	0.28	0.22		0.27	96	-5.67	0.83

0.9-1.2	672	0.20	0.15		0.21	104	-6.03	0.88

Class D

0.0-0.3	638	1.34	1.15		0.87	66	-4.01	0.73

0.3-0.6	617	0.65	0.53		0.52	80	-4.79	0.78

0.6-0.9	558	0.41	0.32		0.34	84	-5.29	0.82

0.9-1.2	493	0.29	0.22		0.31	105	-5.65	0.86

CV   = coefficient of variation

s.d. = standard deviation

Source:  Carsel et al. (1988)

a Johnson sB transformation is used for all cases in this table.

TABLE 5-32.	ADAPTATIONS AND LIMITATIONS OF COMMON IRRIGATION METHODS

Irrigation

Method	Adaptations	Limitations

Furrow	Light, medium-and fine-	Slopes up to 3 percent in

textured soils; row crops.	direction of irrigation; row

crops; 10 percent cross 		slope.

Sprinklers	All slopes; soils; crops.	High initial equipment cost; 		lowered efficiency in wind

and hot climate.

Flood	Light, medium, and heavy	Deep soils; high cost of land

soils.	preparation; slopes less

than 2 percent.

Source:  Adapted from Todd (1970).

TABLE 5-33.	WATER REQUIREMENTS FOR VARIOUS IRRIGATION AND SOIL TYPES

Typical Application Rate (Inches/Hour) by Sprinklers

Coarse	Light	Medium	Clay

Slope	Sandy	Sandy	Silt	Loam

(%)	Loam	Loam	Loam	Soils

Sprinkling	0-2	2.0	0.75	0.5	0.20

2-5	2.0	0.75	0.5	0.20

5-8	1.5	0.50	0.4	0.15

8-12	1.0	0.40	0.3

Source:  Adapted from Todd (1970).

TABLE 5-34.	REPRESENTATIVE FURROW PARAMETERS DESCRIBED IN THE LITERATURE

Manning's

Channel	Flow	Furrow	Bottom	Roughness

Reference	Location	Soil	Crop	Slope	Rate(m3/s)	Length(m)	Width(cm)	Coefficient

Elliott et al.	Colorado	Clay loam	Corn	0044	.001-.003	625	--	.02-.03

(1982)		Clay loam	Corn	.0092-.0095	.00085-.00096	425-450	--	.02-.03

Loamy sand	Corn	.0023-.0025	.003-.005	350	--	.02-.03

Hall (1956)	--	Medium	Corn	.005		200	--	.035

Fangmeier and	Arizona	Fine sandy	None	.01	.0004-.0018	9	--	.02-.04

Ramsey (1978)		loam	(test

furrows)

Karmeli et al.	Colorado	Clay loam	None	.0045	.0011	625	10-20	.01-.048

(1978)

TABLE 5-35.	FURROW IRRIGATION RELATIONSHIPS FOR VARIOUS SOILS, SLOPES, AND DEPTHS OF APPLICATION

| Soil Texture | Coarse | Medium | Fine |
| --- | --- | --- | --- |
| Max allowable | Depth of irrigation application (inches) |  |  |
| nonerosive |  |  |  |

| Slope furrow stream |  | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (percent) | (gpm) | Maximum allowable length of run (feet) |  |  |  |  |  |  |  |  |  |  |  |
| .25 | 40 | 500 | 720 | 875 | 1,000 | 820 | 1,150 | 1,450 | 1,650 | 1,050 | 1,500 | 1,750 | 2,140 |
| .50 | 20 | 345 | 480 | 600 | 680 | 560 | 800 | 975 | 1,120 | 730 | 1,020 | 1,250 | 1,460 |
| .75 | 13 | 270 | 380 | 480 | 550 | 450 | 630 | 775 | 900 | 580 | 820 | 1,000 | 1,150 |
| 1.00 | 10 | 235 | 330 | 400 | 470 | 380 | 540 | 650 | 760 | 500 | 750 | 850 | 990 |
| 1.50 | 7 | 190 | 265 | 330 | 375 | 310 | 430 | 530 | 620 | 400 | 570 | 700 | 800 |
| 2.00 | 5 | 160 | 225 | 275 | 320 | 260 | 370 | 450 | 530 | 345 | 480 | 600 | 675 |
| 3.00 | 3 | 125 | 180 | 220 | 250 | 210 | 295 | 360 | 420 | 270 | 385 | 470 | 550 |
| 5.00 | 2 | 95 | 135 | 165 | 190 | 160 | 225 | 270 | 320 | 210 | 290 | 350 | 410 |

TABLE 5-36.	SUITABLE SIDE SLOPES FOR CHANNELS BUILT IN VARIOUS KINDS OF MATERIALS

Material									Side slope

Rock										Nearly vertical

Muck and peat soils							¼:1

Stiff clay or earth with concrete lining				½:1 to 1:1

Earth with stone lining, or earth for large channels		1:1

Firm clay or earth for small ditches				1½:1

Loose sandy earth								2:1

Sandy loam or porous clay						3:1

Source:  Adapted from Chow (1959).

TABLE 5-37.	VALUE OF "N" FOR DRAINAGE DITCH DESIGN

Hydraulic radius (ft)						EN

less than 2.5						0.040 - 0.045

2.5 to 4.0							.035 - .040

4.0 to 5.0							.030 - .035

more than 5.0						.025 - .030

Source:  Adapted from U.S. Dept. of Agric. Soil Conservation Service.

TABLE 5-38.	REPRESENTATIVE PERMEABILITY RANGES FOR SEDIMENTARY MATERIALS

Hydraulic		Hydraulic

Conductivity		Conductivity

Material	(m/s)	Material	(m/s)

Clay	10-12 - 10-9 	Very fine sand	10-7 - 10-4

Silty clay	10-12 - 10-9	Find sand		10-6 - 10-3

Sandy clay	10-11 - 10-8	Medium sand		10-5 - 10-3

Silty clay loam	10-10 - 10-7	Coarse sand		10-5 - 10-2

Sandy loam sand	10-9  - 10-6	Gravel and sand	10-5 - 10-2

Silt	10-9  - 10-6	Gravel		10-5 - 10-2

Silt loam	10-9  - 10-6	Sandstone		10-6 - 10-3

Loam	10-9  - 10-6	Limestone*		10-7 - 10-4

Sandy loam	10-8  - 10-7	Shale			10-7 - 10-4

*	Excluding cavernous limestone.

Source:  Adapted from Todd (1970).

a	See also Table 5-40.

TABLE 5-39.	VALUES OF GREEN-AMPT PARAMETERS FOR SCS HYDROLOGIC SOIL GROUPS

SCS	Saturated Hydraulica	Suction

Hydrologic	Conductivity KS	Parameter HF

Soil Group	(cm hr-1)	(cm)

A	1.0  - 10.0					10

B	.60  - 1.0					10 - 20

C	.20  - 0.60					15 - 10

D	.005 - 0.20					20 - 150

Source:  Adapted from Brakensiek and Rawls (1983).

a	Also see Table 5-30.

5.3  VADOFT PARAMETERS

Input data for variably saturated flow simulations include the following:

(1)	System Geometry

●	Soil column dimensions (L)

(2)	Porous Medium Properties

●	Saturated hydraulic conductivity, Ks (LT-1)

●	Specific storage, Ss (L-1)

●	Effective porosity, ϕ

(3)	Constitutive Relationships for Variably Saturated Flow

●	Tabulated data of krw versus Sw, or values of parameters of analytic expressions for krw versus Sw

●	Tabulated data of Sw versus ψ, or values of parameters of analytic expressions for Sw versus ψ.

(4)	Initial and Boundary Conditions

●	Prescribed values of pressure head,  (L)

●	Prescribed values of nodal fluid flux

(infiltration rate), I (LT-1)

Input data for the transport model include the following:

(1)	System Geometry

●	Soil column dimensions (L)

(2)	Porous Medium Properties

●	Longitudinal dispersivity αL, (L)

●	Molecular diffusion coefficients, D* (L2T-1)

●	Effective porosity, ϕ

(3)	Properties of Solute Species

●	Decay coefficient, λ (T-1)

●	Retardation coefficient, R

(4)	Darcy Velocity, V (LT-1)

(5)	Water Saturation, Sw

(6)	Initial and Boundary Conditions

●	Prescribed value of concentration, co (ML-3)

●	Prescribed value of solute flux, Vco (ML-2T-1)

Guidance for certain of these parameters is given in the following paragraphs.

Saturated Hydraulic Conductivity - represents the rate at which a porous medium can transmit water under saturated conditions.  Table 5-40 gives representative values for various soil types.  Also note the values of the coefficient of variation in column three.  These CVs are for many soils nationwide that fall into this texture category.  CVs for a single soil are likely to be lower.  Jury (1985) gives a CV of 120% for this parameter, which may be more representative.  The most likely shape for the distribution is lognormal.

Soil-Water Characteristic Data - The user is allowed two options:  either to input these data as a set of paired functions (water saturation [Sw] versus relative conductivity [Krw] and pressure head [ψ] versus water saturation [Sw] or to input parameters of the analytic expressions for these functions in the code.  The parameterization of the latter functions is discussed here.

TABLE 5-40.	DESCRIPTIVE STATISTICS FOR SAT. HYDRAULIC CONDUCTIVITY (cm hr-1)

Hydraulic Conductivity (K)*

Soil Type			s	CV	n

Clay**	0.20		0.42	210.3		114

Clay Loam	0.26		0.70	267.2		345

Loam	1.04		1.82	174.6		735

Loamy Sand	14.59		11.36	77.9		315

Silt	0.25		0.33	129.9		88

Silt Loam	0.45		1.23	275.1		1093

Silty Clay	0.02		0.11	453.3		126

Silty Clay Loam	0.07		0.19	288.7		592

Sand	29.70		15.60	52.4		246

Sandy Clay	0.12		0.28	234.1		46

Sandy Clay Loam	1.31		2.74	208.6		214

Sandy Loam	4.42		5.63	127.0		1183

*	n = Sample size,  = Mean, s = Standard deviation, CV = Coefficient of variation (percent)

**	Agricultural soil, less than 60 percent clay

Source:  Carsel and Parrish (1988).

To provide a linkage for these parameters to widely known or easily obtained soils data (such as soil texture), Carsel and Parrish (1988) fit these analytic functions to data from soils all over the United States and tabulated corresponding parameter values by texture.  These are shown in Table 5-41.  The required parameters are α, β, and γ of the van Genuchten model (see Section 7).  Mean values of these parameters are shown along with CVs for each by soil texture.  Other parameters required to use these relationships are the air entry pressure head (ψa) and the residual water phase saturation (Swr).  The air entry pressure head is normally taken to be zero.  Values of the residual water phase saturation are given in Table 5-42 along with their respective CVs.  Table 5-43 from Carsel and Parrish (1988) shows the types of probability density functions used to fit the sample distributions of saturated hydraulic conductivity, residual water phase saturation, and van Genuchten parameters α and β.

Note that γ is related to β by the relationship:

γ = 1 - 1/β

In addition, Table 5-44 gives the correlations between these parameters by soil textural classification.

Specific Storage - For unsaturated zone flow, set the specific storage to 0.

Effective Porosity - Mean values of saturated water content (θs) and residual water content (θr) shown in Table 5-42 can be used to estimate effective porosity.  The saturation water content (θs) is equal to the total porosity of the soil.  The effective porosity can be roughly approximated as the difference of θs and θr in Table 5-43.  CVs for soil texture categories are also shown in Table 5-43.  According to Jury (1985) the normal distribution is an appropriate probability density function for this parameter.

Longitudinal Dispersivity  - (The user should refer to the discussion in Section 5.2 of the dispersion coefficient having units of cm2 day-1.)  Dispersion coefficients are calculated by the model as the product of the seepage velocity and the dispersivity input by the user.  In the absence of site-specific values it is recommended that the dispersivity be chosen as one-tenth of the distance of the flow path or:

α  = 0.1 xv

where

xv = the thickness of the vadose zone.

Molecular Diffusion  - See the discussion in Section 5.2.

TABLE 5-41.	DESCRIPTIVE STATISTICS FOR VAN GENUCHTEN WATER RETENTION MODEL PARAMETERS, α, β, γ (Carsel and Parrish 1988)

Parameter α, cm-1		Parameter ϐ			Parameter γ

Soil Type	X	SD	CV	N	X	SD	CV	N	X	CV	N

Claya 	0.008	0.012	160.3	400	1.09	0.09	7.9	400	0.08	0.07	82.7	400

Clay Loam	0.019	0.015	77.9	363	1.31	0.09	7.2	364	0.24	0.06	23.5	364

Loam	0.036	0.021	57.1	735	1.56	0.11	7.3	735	0.36	0.05	13.5	735

Loamy Sand	0.124	0.043	35.2	315	2.28	0.27	12.0	315	0.56	0.04	 7.7	315

Silt	0.016	0.007	45.0	88	1.37	0.05	3.3	88	0.27	0.02	 8.6	88

Silt Loam	0.020	0.012	64.7	1093	1.41	0.12	8.5	1093	0.29	0.06	19.9	1093

Silty Clay	0.005	0.005	113.6	126	1.09	0.06	5.0	374	0.09	0.05	51.7	374

Silty Clay Loam	0.010	0.006	61.5	641	1.23	0.06	5.0	641	0.19	0.04	21.5	641

Sand	0.145	0.029	20.3	246	2.68	0.29	20.3	246	0.62	0.04	6.3	246

Sandy Clay	0.027	0.017	61.7	46	1.23	0.10	7.9	46	0.18	0.06	34.7	46

Sandy Clay Loam	0.059	0.038	64.6	214	1.48	0.13	8.7	214	0.32	0.06	53.0	214

Sandy Loam	0.075	0.037	49.4	1183	1.89	0.17	9.2	1183	0.47	0.05	10.1	1183

= Mean, SD = Standard Deviation, CV = Coefficient of Variation, N = Sample size

aAgricultural Soil, Clay 60 percent

TABLE 5-42.	DESCRIPTIVE STATISTICS FOR SATURATION WATER CONTENT (θs) AND RESIDUAL WATER CONTENT (θr)

Saturation Water Content (θs) Residual Water Content (θr)

Statistic*

Soil Type		s	CV	n		s	CV	n

Clay**	0.38	0.09	24.1	400	0.068	0.034	49.9	353

Clay Loam	0.41	0.09	22.4	364	0.095	0.010	10.1	363

Loam	0.43	0.10	22.1	735	0.078	0.013	16.5	735

Loamy Sand	0.41	0.09	21.6	315	0.057	0.015	25.7	315

Silt	0.46	0.11	17.4	82	0.034	0.010	29.8	82

Silt Loam	0.45	0.08	18.7	1093	0.067	0.015	21.6	1093

Silty Clay	0.36	0.07	19.6	374	0.070	0.023	33.5	371

Silty Clay Loam	0.43	0.07	17.2	641	0.089	0.009	10.6	641

Sand	0.43	0.06	15.1	246	0.045	0.010	22.3	246

Sandy Clay	0.38	0.05	13.7	46	0.100	0.013	12.9	46

Sandy Clay Loam	0.39	0.07	17.5	214	0.100	0.006	6.0	214

Sandy Loam	0.41	0.09	21.0	1183	0.065	0.017	26.6	1183

*	n = Sample size,  = Mean, s = standard deviation,

CV = coefficient of variation (percent)

**	Agricultural soil, less than 60 percent clay.

TABLE 5-43.	STATISTICAL PARAMETERS USED FOR DISTRIBUTION APPROXIMATION

Soil	Hydrau-	Trans-	Limits of		Estimated*	Truncation Limits

Tex	lic	forma-	Variation		Standard	on Transformed ture**	Variable	tion	A	B	Mean	Deviation	D***   Variable

S	Ks	SB	0.0	70.0	-0.39387	1.15472	0.045

S	θr	LN	0.0	0.1	-3.11765	0.22369	0.053

S	α	SB	0.0	0.25	0.37768	0.43895	0.050

S	β	LN	1.5	4.0	0.97813	0.10046	0.063

SL	Ks	SB	0.0	30.0	-2.49047	1.52854	0.029

SL	θr	SB	0.00	0.11	0.38411	0.70011	0.034

SL	α	SB	0.00	0.25	-0.93655	0.76383	0.044

SL	β	LN	1.35	3.00	0.63390	0.08162	0.039

LS	Ks	SB	0.0	51.0	-1.26908	1.40000	0.036

LS	θr	SB	0.0	0.11	0.07473	0.56677	0.043

LS	α	NO	0.0	0.25	0.12354	0.04345	0.027

LS	β	SB	1.35	5.00	-1.11095	0.30718	0.070

SIL	Ks	LN	0.0	15.0	-2.18691	1.49414	0.046

SIL	θr	SB	0.0	0.11	0.47752	0.58156	0.073

SIL	α	LN	0.0	0.15	-4.09937	0.55542	0.083

SIL	β	SB	1.0	2.0	-0.37036	0.52557	0.104

SI	Ks	LN***	0.0	2.0	-2.20	0.7000	0.168	-2.564	-0.337

SI	θr	ND***	0.0	0.09	0.042	0.0145	0.089	0.013	0.049

SI	α	NO	0.0	0.1	0.01688	0.00611	0.252

SI	β	NO	1.2	1.6	1.37815	0.03729	0.184

C	Ks	SB	0.0	5.0	-5.75949	2.32884	0.122

C	θr	SU**	0.0	0.15	0.44537	0.28178	0.058	0.0065	0.834

C	α	SB**	0.0	0.15	-4.14805	1.29310	0.189	-5.01	0.912

C	β	LN**	0.9	1.4	0.00021	0.11800	0.131	0.00	0.315

SIC	Ks	LN	0.0	1.0	-5.68562	1.31421	0.205

SIC	θr	NO	0.0	0.14	0.06971	0.02337	0.058

SIC	α	LN	0.0	0.15	-5.65849	0.58445	0.164

SIC	β	SB	1.0	1.4	-1.28378	0.82074	0.069

SC	Ks	LN	0.0	1.5	-4.04036	2.01721	0.130

SC	θr	SB	0.0	0.12	1.72496	0.70000	0.078

SC	α	LN	0.0	0.15	-3.76810	0.56322	0.127

SC	β	LN	1.0	1.5	0.20209	0.07788	0.100

SICL	Ks	SB	0.0	3.5	-5.31256	1.61775	0.049

SICL	θr	NO	0.0	0.115	0.08871	0.00937	0.056

SICL	α	SB	0.0	0.15	-2.75043	0.60529	0.082

SICL	β	NO	1.0	1.5	1.23640	0.06130	0.082

CL	Ks	SB***	0.0	7.5	-5.87171	2.92220	0.058	-8.92	2.98

CL	θr	SU	0.0	0.13	0.67937	0.06005	0.061

CL	α	LN	0.0	0.15	-4.21897	0.71389	0.052

CL	β	SB	1.0	1.6	0.13248	0.72498	0.035

TABLE 5-43.	STATISTICAL PARAMETERS USED FOR DISTRIBUTION APPROXIMATION (continued)

Soil	Hydrau-	Trans-	Limits of		Estimated*	Truncation Limits

Tex	lic	forma-	Variation		Standard	on Transformed ture**	Variable	tion	A	B	Mean	Deviation	D***   Variable

SCL	Ks	SB	0.0	20.0	-4.03718	1.84976	0.047

SCL	θr	SB***	0.0	0.12	1.65387	0.43934	0.077	0.928	2.94

SCL	α	SB	0.0	0.25	-1.37920	0.82327	0.048

SCL	β	LN	1.0	2.0	0.38772	0.08645	0.043

L	Ks	SB	0.0	15.0	-3.71390	1.77920	0.019

L	θr	SB	0.0	0.12	0.63872	0.48709	0.064

L	α	SB	0.0	0.15	-1.27456	0.78608	0.039

L	β	SU	1.0	2.0	0.53169	0.09948	0.036

*	For distribution of transformed variables.

**	S = sand, SL = sandy loam, LS = loamy sand, SIL = silty loam, SI = silt, C = clay, SIC = silty clay, SC = sandy clay, SICL = silty clay loam, CL = clay loam, SCL = sandy clay loam, L = loam.

***	Truncated form of the distribution.

****	Kolmogorov-Smirnov test statistic.

Source:  Carsel and Parrish, 1988.

TABLE 5-44.	CORRELATIONS AMONG TRANSFORMED VARIABLES PRESENTED WITH THE FACTORED COVARIANCE MATRIX*

Ks	θr	α		β

Silt **(n = 61)

Ks	0.5349258	-0.0015813	0.0030541	0.0128700

θr	-0.204	0.0075771	0.0000021	-0.0145118

α	0.984	-0.200	0.0005522	0.0144376

β	0.466	-0.610	0.551	0.0133233

Clay (n = 95)

Ks	1.9614077	0.0701669	0.5645309	0.0475514

θr	0.972	0.0170159	-0.0798488	-0.0142394

α	0.948	0.890	0.1716520	0.0021973

β	0.908	0.819	0.910	0.0164640

Silty Clay (n = 123)

Ks	1.2512845	0.0082067	0.3143268	0.3674505

θr	0.949	0.0027392	0.0404171	-0.0858769

α	0.974	0.964	0.0608834	0.0660396

β	0.908	0.794	0.889	0.1305065

Sandy Clay (n = 46)

Ks	2.0172105	0.8827527	0.5391195	0.0756103

θr	0.939	0.3241979	0.0634106	0.0035688

α	0.957	0.937	0.1501651	-0.0010668

β	0.972	0.928	0.932	0.0178225

Sand (n = 237)

Ks	1.0370702	-0.1092256	0.3276629	0.0805436

θr	-0.515	0.1816914	0.2583835	-0.0471785

α	0.743	0.119	0.1429585	-0.0013674

β	0.843	-0.858	0.298	0.0167064

Sandy Loam (n = 1145)

Ks	1.6026856	-0.1529235	0.0372713	0.2108253

θr	-0.273	0.5378436	0.0174500	-0.1943369

α	0.856	0.151	0.0142626	0.0193794

β	0.686	-0.796	0.354	0.1084945

Loamy Sand (n = 313)

Ks	1.4754063	-0.2005639	0.0372713	0.2108253

θr	-0.359	0.5215473	0.0174500	-0.1943369

α	0.986	-0.301	0.0142626	0.0193794

β	0.730	-0.590	0.354	0.1084945

Silt Loam (n = 1072)

Ks	1.4754063	-0.02005639	0.5245489	0.3525548

θr	-0.359	0.5215473	0.0300399	-0.1696100

α	0.986	-0.301	0.0820163	0.2341768

β	0.730	-0.590	0.775	0.1583593

TABLE 5-44.	CORRELATIONS AMONG TRANSFORMED VARIABLES PRESENTED WITH THE FACTORED COVARIANCE MATRIX* (continued)

Ks	θr	α		β

Silty Clay Loam (n = 591)

Ks	1.6177521	0.0056509	0.5116521	0.0486478

θr	0.724	0.0053780	0.0475299	-0.0089569

α	0.986	0.777	0.0731704	0.0080399

β	0.918	0.549	0.911	0.0171716

Clay Loam (n = 328)

Ks	1.9200165	0.0395603	0.5886263	0.5417671

θr	0.790	0.0307122	-0.0619715	-0.1536351

α	0.979	0.836	0.1060875	0.0653030

β	0.936	0.577	0.909	0.1159401

Sandy Clay Loam (n = 212)

Ks	1.8497610	0.1020156	0.7838769	0.0766289

θr	0.261	0.3775754	0.1223451	-0.0305588

α	0.952	0.392	0.2198684	-0.0078559

β	0.909	-0.113	0.787	0.0155766

Loam (n = 664)

Ks	1.4083953	-0.0995016	0.6110671	0.0545016

θr	0.204	0.4775039	0.0727710	-.0545793

α	0.982	-0.086	0.0926351	0.0256843

β	0.632	-0.748	0.591	0.0288861

*	Entries in the lower triangular portion of the matrix are sample Pearson product-moment correlations given to three decimal places.  The diagonal and upper triangular entries form the triangular Cholesky decomposition of the sample covariance matrix.

**	n = Sample size.

Source:  Carsel and Parrish, 1988.

Pesticide Decay Coefficients  - See the discussion in Section 5.2.

Retardation Factors  - In VADOFT, in contrast to PRZM, the user inputs the retardation factor R instead of the distribution coefficient, KD (cm3 g-1).  The retardation factor is defined for saturated conditions in the input:

Kd ρ

R = 1 + ╶────╴	(5.5)

θs

and is adjusted internally for values of θ < θs.  In the above equation, ρ is the soil bulk density (g cm-3) and θs is the saturation water content (cm3 cm-3).  In making this calculation, the user should directly use the value for ρ, if known.  If necessary, ρ can be approximated according to:

ρ = 2.65 (1 - θs)	(5.6)

The CV of the retardation factor, R, can be computed knowing the uncertainties in Kd, ρ and θs (Taylor 1982).  The fractional uncertainties add to give an upper bound error on R (CVmax) or are combined as a root mean square for independent random errors.  Thus,

CVmax = (CVθs + CVKd + CVρ)	(5.7)

or

CV = 100 [(CVθs/100)2 + (CVKd/100)2 + (CVρ/100)2]½

The uncertainty in the value of Kd will depend upon whether it is measured, calculated as the product of Koc and % organic carbon, and whether the Koc is calculated from a surrogate parameter such as octanol water partition coefficient (Kow) or solubility (s).  Directly measured values would obviously have lower CVs.  Assuming that Kd is calculated from a measured soluble concentration, then it is possible that the CV would be on the order of 60 to 130% (Jury 1985).  For Kd derived from Kow or solubility, the CV could be on the order of 1000%.

SECTION 6

PESTICIDE ROOT ZONE MODEL (PRZM)

CODE AND THEORY

6.1  INTRODUCTION AND BACKGROUND (PRZM)

This section describes the theoretical background for a mathematical simulation model (PRZM) that has been developed and partially tested to evaluate pesticide leaching from the crop root zone under field crop conditions.

Following this short introduction, Section 6.2 describes the features and limitations of the model.  A description of the theory, including a detailed description of the equations solved, is provided in Section 6.3.  An outline of the numerical implementation techniques used by the model to apply the theory to the simulation of physical problems follows.  This section concludes with a discussion of testing results for new algorithms that have been added in this release.

6.1.1  Introduction

Pesticide leaching from agricultural fields as nonpoint source loads can lead to groundwater contamination.  Nonpoint source contamination is characterized by highly variable loadings, with rainfall and irrigation events dominating the timing and magnitude of the loading of pesticides leaching below the root zone.  The potentially widespread, areal nature of resulting contamination makes remedial actions difficult because there is no single plume emanating from a "point source" (the more common groundwater problem) that can be isolated and controlled.  In any case, a more prudent approach to prevention or reduction of groundwater contamination by pesticides must be based on understanding the relationships among chemical properties, soil system properties, and the climatic and agronomic variables that combine to induce leaching.  Knowledge of these relationships can allow a priori investigation of conditions that lead to problems, and appropriate actions can be taken to prevent widespread contamination.

Many investigators have studied the factors contributing to pesticide leaching.  These investigations have shown that chemical solubility in water, sorptive properties, volatility, formulation, and soil persistence determine the tendency of pesticides to leach through soil.  Similarly, the important environmental and agronomic factors include soil properties, climatic conditions, crop type, and cropping practices.  In short, the hydrologic cycle interacts with the chemical characteristics to transform and transport pesticides within and out of the root zone.  Vertical movement out of the root zone can result in groundwater contamination and is the problem that the model is designed to investigate.

Numerical models for the movement of solutes in porous media for steady-state, transient, homogenous, and multi-layered conditions have been previously developed.  Included in such studies have been linear and nonlinear sorption, ion exchange, and other chemical-specific reactions.  These investigations have proven valuable in interpreting laboratory data, investigating basic transport processes, and identifying controlling factors in transport and transformation.  As noted in a recent review of models for simulating the movement of contaminants through groundwater flow systems, however, the successful use of such models requires a great deal of detailed field data.  This unfortunate conclusion arises from the scaling problems associated with laboratory experiments and the traditional solution of the appropriate partial differential equations at points or nodes in a finite-difference or finite-element grid network.  Each spatial segment modeled must be properly characterized--a most expensive, if not impossible, task for many modeling problems.

Such problems in modeling pesticide leaching with existing procedures are discouraging when one considers the need to evaluate future problems arising from pesticides not yet widely distributed or used.  Models used to perform such evaluations should conform to the maximum possible extent to known theory, but must be structured to enable efficient analysis of field situations with minimal requirements for specialized field data.  In short, the goal is to integrate the essential chemical-specific processes for leaching with reasonable estimates of water movement through soil systems.  Data input requirements must be reasonable in spatial and temporal requirements and generally available from existing data bases.  This model attempts to meet these objectives.

6.1.2  Background

The Pesticide Root Zone Model (PRZM) (Carsel et al. 1984, Carsel et al. 1985) was selected as the code to provide the capability to simulate the transport and transformation of agriculturally applied pesticides in the crop root zone.  PRZM was initially designed for this purpose and has attained a degree of acceptability in both the regulatory community and in the agricultural chemical industry.  Therefore, its utility in accomplishing the objective of this model development effort is obvious.

6.2  FEATURES AND LIMITATIONS

6.2.1  Features

PRZM Release II is a one-dimensional, dynamic, compartmental model for use in simulating chemical movement in unsaturated soil systems within and immediately below the plant root zone (see Figure 6.1).  PRZM allows the user to perform simulations of potentially toxic chemicals, particularly pesticides, that are applied to the soil or to plant foliage.  Dynamic simulations allow the consideration of pulse loads, the prediction of peak events, and the estimation of time-varying mass emission or concentration profiles,

Figure 6.1

thus overcoming limitations of emission or concentration profiles, thus overcoming limitations of the more commonly used steady-state models.  Time-varying transport by both advection and dispersion in the dissolved phase or diffusion in the gas phase are represented in the program.

PRZM has two major components--hydrology and chemical transport.  The hydrologic component for calculating runoff and erosion is based on the Soil Conservation Service curve number technique and the Universal Soil Loss Equation.  Evapotranspiration is estimated from pan evaporation data, or by an empirical formula if input pan data are unavailable.  Evapotranspiration is divided among evaporation from crop interception, evaporation from soil, and transpiration by the crop.  Water movement is simulated by the use of generalized soil parameters, including field capacity, wilting point, and saturation water content.  Irrigation may also be considered.

Dissolved, adsorbed, and vapor-phase concentrations in the soil are estimated by simultaneously considering the processes of pesticide uptake by plants, surface runoff, erosion, decay, volatilization, foliar washoff, advection, dispersion, and retardation.  The user may elect to solve the transport equations using one of two finite-difference numerical solutions, the original backwards-difference implicit scheme featured in the first release, or a Method of Characteristics algorithm that greatly reduces numerical dispersion, but increases model execution time.

The hydrologic components of pesticide transport equations (i.e., moisture content and soil-water velocities) are decoupled, solved separately, and used to numerically integrate the equation in succeeding time steps.  Predictions are made on a daily basis.  Output can be summarized on a daily, monthly, or annual period.  A daily time series value for various fluxes or storages can be written to sequential files during program execution.

6.2.2  Limitations

There were severe limitations of the PRZM Release I Code, some that were obvious to the developers and some that were pointed out subsequently by model users.  These can be broken into four categories:

●  Hydrology

●  Soil hydraulics

●  Method of solution of the transport equation

●  Deterministic nature of the model

In Release II, many of these limitations to an extent, have been overcome.

Hydraulic computations are performed in PRZM on a daily time step; however, some of the processes involved (evaporation, runoff, erosion) are clearly among those that might be simulated on a finer time step to ensure greater accuracy and realism.  For instance, simulation of erosion by runoff depends upon the peak runoff rate, which is in turn dependent upon the time base of the runoff hydrograph.  This depends to some extent upon the duration of the precipitation event.  PRZM retains its daily time step in this release primarily due to the relative availability of daily versus shorter time step meteorological data.  A portion of this limitation has been mitigated, we hope, by enhanced parameter guidance.

The method of computing potential evapotranspiration using Hamon's formula, in the absence of some evaporation data, has also been retained.  Evapotranspiration from irrigated citrus in Florida was found to be substantially underpredicted when using this method to estimate potential evapotranspiration (Dean and Atwood 1985).  Users should check the model's hydrologic simulation carefully when using this option.

The capability to simulate soil temperature has been added to PRZM-2 in order to correct Henry's constant for the temperature occurring in various depths in the soil when performing vapor-phase calculations.  Removal of water by evaporation versus transpiration from the profile may have a pronounced effect on soil temperature.  This is due to the fact that more heat is removed during the process of evaporation because the energy necessary to vaporize water leaves the system, producing a cooling effect.  No differentiation is made between evaporation and transpiration in PRZM at this time.

In PRZM Release I, the soil hydraulics were simple--all drainage to field capacity water content was assumed to occur within 1 day.  (An option to make drainage time dependent was also included, but there is not much evidence to suggest that it was utilized by model users to any great extent).  This had the effect, especially in larger soil cores, of inducing a greater-than-anticipated movement of chemical through the profile.  While this representation of soil hydraulics has been retained in PRZM-2, the user has the option, with the linked modeling system, of coupling PRZM to VADOFT.  PRZM-2 is then used to represent the root zone, while VADOFT, with a more rigorous representation of unsaturated flow, is used to simulate the thicker vadose zone.  The difficulties in parameterizing the Richards equation for unsaturated flow in VADOFT is overcome by using the technique of van Genuchten to generate soil water characteristic curves using soil textural information.  For short soil cores, PRZM can obviously be used to represent the entire vadose zone.

The addition of algorithms to simulate volatilization has brought into focus another limitation of the soil hydraulics representation.  PRZM-2 simulates only advective, downward movement of water and does not account for diffusive movement due to soil water gradients.  This means that PRZM-2 is unable to simulate the upward movement of water in response to gradients induced by evapotranspiration.  This process has been identified by Jury et al. (1984) as an important one for simulating the effects of volatilization.  However, the process would seem less likely to affect the movement of chemicals with high vapor pressures.  For these chemicals, vapor diffusion would be a major process for renewing the chemical concentration in the surface soil.

Another limitation of the Release I model was the inadequacy of the solution to the transport equation in advection-dominated systems.  The backward difference formulation of the advection term tends to produce a high degree of numerical dispersion in such systems.  This results in overprediction of downward movement due to smearing of the peak and subsequent overestimation of loadings to groundwater.  In this new release, a new formulation is available for advection-dominated systems.  The advective terms are decoupled from the rest of the transport equation and solved separately using a Method of Characteristics (MOC) formulation.  The remainder of the transport equation is then solved as before, using the fully implicit scheme.  This approach effectively eliminates numerical dispersion, but with some additional overhead expense in computation time.  In low-advection systems, the MOC approach reduces to the original PRZM solution scheme, which is exact for velocities approaching zero.

The final limitation is the use of field-averaged water and chemical transport parameters to represent spatially heterogeneous soils.  Several researchers have shown that this approach produces slower breakthrough times than are observed using stochastic approaches.  This concern has been addressed by adding the capability to run PRZM in a Monte Carlo framework.  Thus, distributional, rather than field-averaged, values can be utilized as inputs that will produce distributional outputs of the relevant variables (e.g., flux to the water table).

6.3  DESCRIPTION OF THE EQUATIONS

The mathematical description of the processes simulated by PRZM are broken down in the following discussion into five categories:

●  Transport in Soil

●  Water Movement

●  Soil Erosion

●  Volatilization

●  Irrigation

The first three categories were simulation options previously available in PRZM Release I.  Since the capability to simulate ponding is new, the mathematical basis of the ponding algorithms is described in detail.  The final process, volatilization, was not available in the previous release of PRZM, and its theoretical basis is also described in detail.

6.3.1  Transport in Soil

The PRZM-2 model was derived from the conceptual, compartmentalized representation of the soil profile as shown in Figure 6.2.  From consideration of Figure 6.2, it is possible to write mass balance equations for both the surface zone and the subsurface zones.  Addition of the vapor phase and ponded water compartments in PRZM-2 require the consideration of additional terms.  The surface zone expressions for each of the dissolved, adsorbed, and vapor phases can be written as:

where

A		=	cross-sectional area of soil column (cm2)

Δz		=	depth dimension of compartment (cm)

Cw		=	dissolved concentration of pesticide (g cm-3)

Cs		=	sorbed concentration of pesticide (g g-1)

Cg		=	gaseous concentration of pesticide (g cm-3)

θ		=	volumetric water content of soil (cm3 cm-3)

a		=	volumetric air content of the soil (cm3 cm-3)

ρs		=	soil bulk density (g cm-3)

t		=	time (d)

JD		=	represents the effect of dispersion and diffusion of dissolved phase (g day-1)

JV		=	represents the effect of advection of dissolved phase (g day-1)

JGD	=	represents the effect of dispersion and diffusion in vapor phase (g day-1)

JDW	=	mass loss due to degradation in the dissolved phase (g day-1)

JDG	=	mass loss due to degradation in the vapor phase (g day-1)

JU		=	mass loss by plant uptake of dissolved phase (g day-1)

JQR	=	mass loss by removal in runoff (g day-1)

JAPP	=	mass gain due to pesticide deposition on the soil surface (g day-1)

JFOF	=	mass gain due to washoff from plants to soil (g day-1)

JDS	=	mass loss due to degradation of sorbed phase chemical (g day-1)

JER	=	mass loss by removal on eroded sediments (g day-1)

JTRN	=	mass gain or loss due to parent/daughter transformations

Equations for the subsurface zones are identical to Equations 6-1, 6-2, and 6-3 except that JQR, JFOF, and JER are not included.  JAPP applies to subsurface zones only when pesticides are incorporated into the soil.  For subsurface layers below the root zone, the term JU is also not utilized.

Note that terms representing phase transfers (e.g., volatilization) are neglected in Equations 6-1 through 6-3 because they cancel when the equations are added (see Equation 6-19 below).

Each term in Equations 6-1 through 6-3 are now further defined.  Dispersion and diffusion in the dissolved phase are combined and are described using Fick's law as

where

Dw		=	diffusion-dispersion coefficient for the dissolved phase, assumed constant (cm2 day-1)

Cw		=	dissolved concentration of pesticide (g cm-3)

θ   	=	volumetric soil water content (cm3 cm-3)

x		=	soil depth dimension (cm)

In a similar manner, dispersion and diffusion in the vapor phase are described by Fick's law as

where

Dg		=	molecular diffusivity of the pesticide in the air-filled pore space

(cm2 day-1)

Cg		=	vapor-phase concentration of pesticide (g cm-3)

a		=	volumetric air content (cm3 cm-3)

The dependence of the molecular diffusivity of the pesticide in air-filled pore space on the volumetric air content is described by the Millington-Quirk expression (Jury et al. 1983a)

where

a		=	the air-filled porosity (cm3 cm-3)

ϕ		=	total porosity (cm3 cm-3)

Da		=	molecular diffusivity of the chemical in air, assumed constant (cm2 day-1)

The mathematical theory underlying the diffusive and dispersive flux of pesticide in the vapor phase within the soil and into the overlying air can be found in the section describing volatilization.

The advective term for the dissolved phase, JV, describes the movement of pesticide in the bulk flow field and is written as

where

V		=	velocity of water movement (cm day-1)

Vapor-phase advection has not been included as a flux in the transport equation.  A number of researchers have indicated a consensus that vapor-phase advection is not likely to be significant for agricultural situations (W. Jury, W. Spencer, W. Farmer, L. Thibodeaux - personal communications, 1987).  Early studies of water vapor movement suggested that the fluctuation of barometric pressure at the soil surface could act as a pumping mechanism for vapor-phase advective transport (Fukuda 1955, Farrell et al. 1966, Scotter and Raats 1970).  However, using models for vapor emissions from landfills, Thibodeaux et al. (1982) found that atmospheric pressure fluctuations increased the total emission rate for benzene by only 15%, compared to constant pressure conditions.  Therefore, it appears to be a reasonable assumption at this time to neglect vapor-phase advection in modeling chemical migration for agricultural situations.

Degradation of a pesticide in or on soil may be due to such processes as hydrolysis, photolysis, and microbial decay.  If these processes follow pseudo first-order kinetics, the rate coefficients may be combined into a single decay coefficient.  Assuming the same rate constants for the solid and dissolved phases, we can write the rate of change of chemical out of each phase due to decomposition as:

where

Ks		=	lumped, first-order decay constant for solid and dissolved phases (day-1)

Kg		=	lumped, first-order decay constant for vapor phase (day-1)

Cs		=	solid-phase concentration of pesticide (g g-1)

Plant uptake of pesticides is modeled by assuming that uptake of a pesticide by a plant is directly related to transpiration rate.  The uptake is given by:

where

Ju		=	uptake of pesticide (g day-1)

f		=	the fraction of total water in the zone used for transpiration (day-1)

ε		=	an uptake efficiency factor or reflectance coefficient

(dimensionless)

Erosion and runoff losses as well as inputs to the surface zone from foliar washoff are considered in the surface layer.  The loss of pesticide due to runoff is

in which

JQR	=	pesticide loss due to runoff (g day-1)

Q		=	the daily runoff volume (cm3 day-1)

Aw		=	watershed area (cm2)

and the loss of pesticide due to erosion is

where

JER	=	the pesticide loss due to erosion (g day-1)

Xe		=	the erosion sediment loss (metric tons day-1)

rom		=	the enrichment ratio for organic matter (g g-1)

p		=	a units conversion factor (g tons-1)

Soil erosion is discussed in more detail in Section 6.3.3.

Pesticides can be applied to either bare soil if pre-plant conditions prevail or to a full or developing crop canopy if post-plant treatments are desired.  The pesticide application is an input mass rate that is calculated by one of the application/deposition models discussed in Section 7.1.  It is partitioned between the plant canopy and the soil surface, and the rate at which it reaches the soil surface is designated JAPP.

Pesticides applied to the plant canopy can be transported to the soil surface as a result of rainfall washoff.  This term, JFOF, is defined as:

where

E		=	foliar extraction coefficient (cm-1)

Pr		=	daily rainfall depth (cm day-1)

M		=	mass of the pesticide on the plant surface projected area basis (g cm-2)

The foliar pesticide mass, M, is further subject to degradation and losses through volatilization.  Its rate of change is given by

where

Kf		=	lumped first-order foliar degradation constant (day-1)

AF		=	application rate to the plant (g ha-1 day-1)

b		=	a units conversion factor (ha)

Adsorption and desorption in Equations 6-1 through 6-3 are treated as instantaneous, linear, and reversible processes.  Using this assumption, we can relate the sorbed phase concentration to the dissolved-phase concentration by:

where

Kd		=	partition coefficient between the dissolved and solid phases (cm3 g-1)

A similar expression can be developed to express the vapor phase concentration in terms of the dissolved-phase concentration as follows

where

KH		=	Henry's constant, i.e., distribution-coefficient between liquid phase and vapor phase (cm3 cm-3)

The transformation of parent to daughter is assumed to be first order and takes place according to

where

KTRN	=	the transformation rate constant (day-1)

When simulating an end-of-chain daughter, JTRN may also be a source term equal to the sum of the first-order transfers from any and all parents.

in which the superscript k denotes a parent compound.  For intermediate products, the solute transport equation may contain terms such as those shown in both Equations 6-18 and 6-19.  The transformation of parent to daughter compounds is discussed in detail in Section 6.5.4.  The section includes a description of the equations used to simulate this process.

Summing Equations 6-1, 6-2, and 6-3 and utilizing equations 6-16 and 6-17, produces the following expressions for the mass balance of pesticide in the uppermost soil layer:

Equation 6-20 is solved in PRZM-2 for the surface layer with fθ = 0, and an upper boundary condition that allows vapor phase flux upward from the soil surface to the overlying air.  This upper boundary condition is described more fully in the section on volatilization.  The lower boundary condition is one that allows advection, but no diffusion, out of the bottom of the soil profile.

6.3.2  Water Movement

Because V and θ are not generally known and not generally measured as part of routine monitoring programs, it is necessary to develop additional equations for these variables.  In the general case, Darcy's law can be combined with the continuity equation to yield the Richards equation (Richards 1931):

where

K(θ)	=	hydraulic conductivity at various heads (cm sec-1)

θ		=	soil water content (cm3 cm-3)

and

or, in simpler terms

where

θ	= soil water content (cm3 cm-3)

V	= soil water velocity (cm day-1)

Writing Equation 6-23 in an integrated backwards finite difference form yields

or

In these equations, t and t+1 denote the beginning and end of time step values, respectively, and i is the soil layer index.  These equations can be further simplified by substituting the nomenclature SW for θΔz so that

where

SW = soil water content (cm)

The velocities in Equation 6-26 are a function of inputs to the soil (precipitation, infiltration) and outflows from the soil (evapotranspiration, runoff).

Water balance equations are separately developed for (a) the surface zone, (b) horizons comprising the active root zones, and (c) the remaining lower horizons within the unsaturated zone.  The equations are:

Surface Zone

Root Zone

Below Root Zone

where

(SW)	=	soil water in layer "i" on day "t" (cm)

Ei		=	evaporation (cm day-1)

Ui		=	transpiration (cm day-1)

Ii		=	percolation out of zone i (cm day-1)

INF	=	infiltration into layer 1 (cm day-1)

Daily updating of soil moisture in the soil profile using the above equations requires the additional calculations for infiltration, evaporation, transpiration, and percolation.

Infiltration is calculated as

where, assuming a unit area of 1 cm2,

P		=	precipitation as rainfall, minus crop interception (cm day-1)

SM	=	snowmelt (cm day-1)

Q		=	runoff (cm day-1)

E		=	evaporation (cm day-1)

The calculations of precipitation, snowmelt, and runoff on a daily time step are described below.  The disaggregation of these values and the calculation of the change in the depth of ponding on a finer time step is included in  Sections 6.3.5.4 and 6.4.4 describing the simulation of furrow irrigation and ponded surface water.

Input precipitation is read in and pan evaporation and/or air temperature are inputs from which potential evapotranspiration (PET) is estimated.  Incoming precipitation is first partitioned between snow or rain, depending upon temperature.  Air temperatures below 0°C produce snow and may result in the accumulation of a snowpack.  Precipitation first encounters the plant canopy and once the interception storage is depleted, the remaining depth is available for the runoff or infiltration.

The runoff calculation partitions the precipitation between infiltrating water and surface runoff.  Infiltrating water may be ponded on the soil surface for a period of time before it infiltrates, but this ephemeral process is described in a following section.  Runoff is calculated by a modification of the USDA Soil Conservation Service curve number approach (Haith et al. 1979).  Snowmelt is estimated on days in which a snowpack exists and above freezing temperatures occur as

where

CM	=	degree-day snowmelt factor (cm °C-1 day-1)

T	=	average daily temperature (°C)

The precipitation and/or snowmelt are inputs to the SCS runoff equation written as

where S, the watershed retention parameter, is estimated by

where

RCN	= SCS runoff curve number

Curve numbers are a function of soil type, soil drainage properties, crop type, and management practice.  Typically, specific curve numbers for a given rainfall event are determined by the sum of the rainfall totals for the previous 5 days, known as the 5-day antecedent moisture condition.  In this release of PRZM, as in the original version, the curve numbers are continuously adjusted each day as a function of the soil water status in the upper soil layers.  These algorithms were developed and reported by Haith and Loehr (1979).

The daily evapotranspiration demand is divided among evaporation from canopy, ponded surface water, soil evaporation, and crop transpiration.  Total demand is first estimated and then extracted sequentially from crop canopy storage, ponded surface water, and then from each layer until wilting point is reached in each layer or until total demand is met.  Evaporation occurs down to a user-specified depth.  The remaining demand, crop transpiration, is met from the active root zone.  The root zone growth function is activated at crop emergence and increases stepwise until maximum rooting depth is achieved at crop maturity.

Actual evapotranspiration from a soil layer is estimated as:

where

ETi	= the actual evapotranspiration from layer 'i' (cm)

fdi		= depth factor for layer 'i'

WPi	= wilting point water content in layer 'i' (cm)

ETp	= potential evapotranspiration (cm)

This equation states that the transpiration from any layer 'i' is the minimum of the available water in layer 'i' or the demand remaining after extracting available water from layers above 'i' in the profile.

The depth factor, fdi, is internally set in the code.  It linearly weights the extraction of ET from the root zone with depth.  A triangular root distribution is assumed from the surface zone to the maximum depth of rooting, with the maximum root density assumed to be near the surface.  This algorithm essentially views the plant as a pump and assumes that it will expend the minimum energy possible in pumping.  As long as the soil water is equally available, water closest to the surface meets this criterion.

Evapotranspiration may also be limited by soil moisture availability.  The potential rate may not be met if sufficient soil water is not available to meet the demand.  In that case, PRZM-2 modifies the potential rate by the following equations.

where

FC		= soil moisture content at field capacity (cm)

WP		= soil moisture content at wilting point (cm)

SMFAC	= soil moisture factor

The SMFAC concept has been used in other similar water balance models (Haith et al. 1979, Stewart et al. 1976) and is internally set in the code to linearly reduce ETp when soil water becomes limited.  Finally, if pan evaporation input data are available, ETp is related to the input values as

where

PE	= pan evaporation (cm day-1)

Cp		= pan factor (dimensionless)

The pan factor is constant for a given location and is a function of the average daily relative humidity, average daily wind speed, and location of the pan with respect to an actively transpiring crop.

In the absence of pan evaporation data, ETp is estimated by

where

Ld		= possible hours of sunshine per day, in 12-hour units

SVD	= saturated vapor density at the mean air temperature (g cm-1)

SVD	= 0.622 SVP/(Rg Tabs)

where

SVP	= saturated vapor pressure at the mean absolute air

temperature (mb)

Rg		= dry-air gas constant

Tabs	= absolute mean air temperature (°K)

The final term in the water balance equations that must be defined is the percolation value, I.  Because the Richards equation is not solved in PRZM-2 utilizing soil water characteristic curves to predict water movement, PRZM-2 resorts to "drainage rules" keyed to soil moisture storages and the time available for drainage.  Two options are included.  Although these options are admittedly simplistic representations of soil moisture redistribution, they are consistent with the objectives of PRZM-2 and its intended uses.

6.3.2.1  Option 1--

Percolation, I, in this option is defined in the context of two bulk soil moisture holding characteristics commonly reported for agricultural soils--field capacity and wilting point.  Field capacity is a somewhat imprecise measure of soil water holding properties and is usually reported as the moisture content that field soils attain after all excess water is drained from the system under influence of gravity, usually at tensions of about 0.3 bar.  The difficulty with this concept is the fact that some soils will continue to drain for long periods of time, and thus field capacity is not a constant.  Admitting the lack of theoretical and physical rigor, we believe that the concept remains a useful measure of soil moisture capacity that has been successfully used in a number of water balance models (Haith et al. 1979, Stewart et al. 1976).  Wilting point is a function of both the soil and plants growing in the soil.  It is defined as the soil moisture content below which plants are unable to extract water, usually at tensions of about 15 bar.

Field capacity and wilting point are used operationally to define two reference states in each soil layer for predicting percolation.  If the soil water, SW, is calculated to be in excess of field capacity, then percolation is allowed to remove the excess water to a lower zone.  The entire soil profile excess is assumed to drain within 1 day.  The lower limit of soil water permitted is the wilting point.  One outcome of these assumed "drainage rules" is that the soil layers below the root zone tend to quickly reach field capacity and remain at that value.  When this condition is reached, all water percolated below the root zone will displace the water within the lower soil layer simulated, and so on.  There is no allowance for lateral water movement.  Water balance accounting in this manner should be most accurate for sandy soils in which water movement is relatively unimpeded and is least accurate for clay soils (Stewart et al. 1976).

6.3.2.2  Option 2--

The second option is provided to accommodate soils having low permeability layers that restrict the "free drainage" assumed in Option 1.  In the context of the field capacity reference condition, two things may occur.  First, conditions may prevail that raise the soil moisture levels above field capacity for periods of time because the water is "backed up" above a relatively impermeable layer.  Second, the excess water may not drain during the 1-day period assumed in Option 1.  To accommodate these conditions, two additional parameters are needed.  Maximum soil moisture storage, θs, is added to represent moisture contents under saturated conditions.  The drainage rate also must be modified to allow drainage to field capacity over periods in excess of 1 day (one time step).  The drainage rate is assumed to be a first-order function of the water content above field capacity and is modeled by

which has the solution

where

θ		= soil layer water content (cm3 cm-3)

θfc		= water content at field capacity (cm3 cm-3)

α		= drainage rate parameter (day-1)

In this equation, t and t+1 denote beginning and end of time step values, respectively, and i is the soil layer index.  The value t* denotes a value of time between the beginning and the end of the time step.  The variable θ* here denotes current storage plus any percolation from the next layer above, before the occurrence of any drainage from the current layer.  Because Equation 6-39 is solved independently for each layer in the profile, there is a possibility of exceeding the storage capability (saturation water content, θs) of a low-permeability layer in the profile if a more permeable layer overlies it.  At each time step, once redistribution is complete, the model searches the profile for any θi > θsi.  If this condition is found, the model redistributes water back into overlying layers, as if the percolation of additional water beyond that necessary to saturate the low-permeability layer had not occurred.  This adjustment is necessary due to the nature of Equation 6-39 and the fact that these equations for each layer are not easily coupled.  The difficulty in coupling the equations for the entire profile arises from the dichotomy that one of two factors limits percolation from a stratum in the profile:  either the rate at which that stratum can transmit water, or the ability of the stratum below it to store or transmit water.  This dichotomy leads to an iterative (or at least corrective) approach to the explicit solution of a system of equations for θi represented by Equation (6-39).  It should be noted, however, that the value of α selected by this approach is only relevant if the permeability of the soil materials, and not storage considerations in the profile (i.e., the presence of a water table), is the limiting factor for percolation of water.

6.3.3  Soil Erosion

Removal of sorbed pesticides on eroded sediments requires estimates for soil erosion.  The Modified Universal Soil Loss Equation (MUSLE) as developed by Williams (1975) is used to calculate soil loss:

where

Xe		= the event soil loss (metric tons day-1)

Vr		= volume of event (daily) runoff (m3)

qp		= peak storm runoff (m3 sec-1)

K		= soil erodability factor

LS		= length-slope factor

C		= soil cover factor

P		= conservation practice factor

a		= units conversion factor

Most of the parameters in Equation 6-40 are easily determined from other calculations within PRZM (e.g., Vr), and others are familiar terms readily available from handbooks.  However, the peak storm runoff value, qp, can vary widely depending upon rainfall and runoff characteristics.  A trapezoidal hydrograph is assumed in PRZM-2.  From the assumed hydrograph shape and the storm duration, a peak runoff rate is calculated.

The enrichment ratio, rom, is the remaining term that needs to be defined to estimate the removal of sorbed pesticides by erosion.  Because erosion is a selective process during runoff events, eroded sediments become "enriched" in smaller particles.  The sediment transport theory available to describe this process requires substantially more hydraulic spatial and temporal resolution than used in PRZM-2, leading to the adoption of an empirical approach (Mockus 1972).  The enrichment ratio for organic matter is calculated from

6.3.4  Volatilization

As volatilization was not available in the previous release of PRZM, its theoretical basis is discussed in detail here.  The following key processes have been identified as being important in volatilization algorithms to simulate vapor-phase pesticide transport within the soil/plant compartments:

●	Vapor-phase movement of the pesticide in the soil profile

●	Boundary layer transfer at the soil-air interface

●	Vertical diffusion of pesticide vapor within the plant canopy

●	Pesticide mass transfer between the plant (leaves) and the surrounding atmosphere

●	Soil temperature effects on pesticide volatilization

The discussion of the volatilization algorithms is presented in four parts:  influence of vapor phase pesticide in soil and volatilization flux, volatilization flux through the plant canopy, volatilization flux from plant surfaces, and soil temperature modeling and effects.  Figure 6.3 is a schematic of the pesticide vapor and volatilization processes considered in soil and plant compartments.

6.3.4.1  Soil Vapor Phase and Volatilization Flux--

The governing equations for chemical transport in the vapor phase were introduced previously in the description of transport in the soil.  Fluxes from the soil column in the vapor phase are summarized in that discussion by Equations 6-3, 6-5, and 6-9.  The terms in these equations are summed with the other flux terms to produce the transport Equation 6-20.  In addition to these enhancements, the upper boundary of PRZM-2 was changed from a zero-concentration boundary to a stagnant-layer boundary to allow diffusive transport upward from the soil to the overlying atmosphere.  This enhancement is discussed in detail below.

Surface boundary condition--When a pesticide is incorporated into the soil, the initial volatilization rate is a function of the vapor pressure of the chemical at the surface as modified by adsorptive interactions with the soil.  As the concentration at the surface of the soil changes, volatilization may become more dependent on the rate of movement of the pesticide to the soil surface (Jury et al., 1983b).

The soil surface layer can be visualized as a membrane that only allows water to pass through and keeps the solute behind.  Experimental results show that, within the top centimeter of the soil surface, the pesticide concentration can increase as much as 10-fold due to the accumulation of chemical at the surface layer, resulting in higher vapor density.  In order to describe these phenomena, Jury et al. (1983a, 1983b) proposed a boundary layer model that states that the controlling mechanism for pesticide volati-lization is molecular diffusion through the stagnant surface boundary layer.

The layer of stagnant air may or may not form a significant barrier to volatilization loss for a given pesticide, depending on a variety of factors.  In general, if the diffusion rate through the air layer is able to match the upward flux to the soil surface without having the surface concentration build up, then the stagnant layer is not acting as a barrier to loss and the volatilization flux will not depend strongly on the thickness of the volatilization flux will not depend strongly on the thickness of the boundary layer.  Conversely, if

the diffusion rate through the air is less than the flow to the surface by diffusion or mass flow, then the concentration at the soil surface will not be close to zero, and the thickness of the air layer will regulate the loss by volatilization.  In other words, the significance of the boundary layer model depends on the ratio of the magnitudes between the upward soil pesticide flux and the boundary layer diffusion flux.  Only downward, advective movement of water is treated in PRZM Release I.  In this case, the sources that contribute to the upward soil pesticide flux are only the diffusion processes in the vapor and dissolved phases, but not upward water advection.

The zero chemical concentration upper boundary condition in the first release was modified in accordance with Jury's boundary layer model.  The pesticide volatilization flux from the soil profile can be estimated as follows:

where

J1		=	volatilization flux from soil (g day-1)

Da		=	molecular diffusivity of the chemical in air (cm2 day-1)

A		=	cross-sectional area of soil column (cm2)

d		=	thickness of stagnant air boundary layer (cm)

Cg,1	=	vapor-phase concentration in the surface soil layer (g cm-3)

C,d	=	vapor-phase concentration above the stagnant air boundary layer (g cm-3)

The thickness of the stagnant boundary layer can be estimated using a water vapor transport approach (Jury et al. 1983a).  However, Wagenet and Biggar (1987) assumed a constant value of 5 mm for this thickness, which is consistent with the values estimated by Jury.  Consequently, the same assumption of a 5-mm thickness for the stagnant layer has been used here pending the results of further sensitivity analyses.  The value of C,d can take on a value of zero if the soil surface is bare or can be positive if a plant canopy exists.

6.3.4.2  Volatilization Flux Through the Plant Canopy--

In pioneering work on this topic, Parmele et al. (1972) discuss a number of micrometeorological techniques for calculating pesticide volatilization flux from observed aerial pesticide concentrations.  Their procedures are based on the assumption that the vertical diffusivity coefficient (Kz) for pesticide vapor is analogous to the vertical diffusivity for water vapor, energy, or momentum.  The pesticide volatilization flux can be computed by Fick's first law of diffusion, as follows.

where

Jz(Z)		=	pesticide flux at height Z (g m-2 s-1)

(dP/dZ)	=	pesticide concentration gradient (g m-2)

Kz(Z)	=	the vertical diffusivity at the height Z (m2 s-1)

The value of Kz depends on the turbulent flow of the atmosphere into which the pesticide vapor is dissipated.  Therefore, it is a function of the prevailing meteorological conditions and not of any physical or chemical property of the pesticide.

In order to apply these concepts, pesticide concentrations at two or more heights are required to estimate the pesticide gradient and the subsequent flux.  For the estimation of vertical diffusivity, more extensive meteorological information is also required.  All of these data requirements pose significant limitations for a predictive modeling approach.

In developing this PRZM-2 module, the following approaches are proposed to circumvent the intensive data requirements.  First, a relationship for Kz is derived as a function of height within the canopy.  Then one need only consider the pesticide concentration gradient (or a suitable surrogate) in order to compute the pesticide volatilization flux.

Estimation of Kz(Z)--Mehlenbacher and Whitfield (1977) present the following formula to compute Kz at various heights within the plant canopy.

where

Kz(Z)	=	thermal eddy diffusivity at height Z (m2 s-1)

Kz(ZCH)	=	thermal eddy diffusivity at canopy height (m2 s-1)

ZCH		=	canopy height (m)

Zo			=	roughness length (m)

D			=	zero plane displacement height (m)

k			=	von Karman's constant, 0.41

U*		=	friction velocity (m s-1)

ϕh			=	stability function for sensible heat

ψm(ϕm) 	=	integrated momentum stability parameter as a function of 	ϕm 		= stability function for momentum

UCH		=	wind velocity at the canopy height (m s-1)

For agricultural applications, the canopy height is used as a reference height for calculating U*.  The user is required to input the wind speed and the height where the measurement was made.  The wind speed at the canopy height (UCH) is computed based on the logarithm law.  The relationship is:

The friction velocity U* can be visualized as a characteristic of the flow regime in the plant canopy compartment in which the logarithmic velocity distribution law holds.  As shown in Equation 6-44, U* is calculated as a function of UCH, ZCH, Zo, D, and ψm.  Rosenberg (1974) describes Zo + D as the total height at which the velocity profile above the canopy extrapolates to zero wind velocity.  The values for both Zo and D can be estimated with the following equations presented by Thibodeaux (1979).  For very short crops (lawns, for example), Zo adequately describes the total roughness length, and little adjustment of the zero plane is necessary (i.e., D = 0).  D is assumed to be zero in the current code when ZCH is less than 5 cm.  For tall crops, Zo is related to canopy height (ZCH) by

In tall crops, Zo is no longer adequate to describe the total roughness length, and a value of D, the zero plane displacement, is needed.  For a wide range of crops and heights, 0.02 m < ZCH < 25 m, the following equation for D has been presented (Stanhill 1969).

This equation results from a linear regression analysis based on the published data for 19 different crops with limited data measured for the same crop at different growth stages.  Strictly speaking, both Zo and D should be evaluated from experimental observations.  In the calculation of Kz, the module uses these two equations for estimation of Zo and D, since there is no method available to justify any variations for crop type, row spacing, or canopy density.

With estimates of Zo and D, U* (friction velocity) can be estimated if the values of the stability parameters (ψm and ϕh) are known.  These two variables are closely related to Ri, the Richardson number, which is the measure of the rate of conversion of convective turbulence to mechanical turbulence.  It is defined as follows (Wark and Warner 1976).

where

g		=	acceleration of gravity (m sec-2)

T		=	potential temperature (°K)

Z		=	elevation (m)

U		=	wind velocity (m s-1)

Potential temperature is defined as the temperature that a parcel of dry air would acquire if brought adiabatically from its initial pressure to a saturated pressure of 1000 millibars (Perkins 1974).  In application of the model, the measured temperature is used in the Richardson number estimation as suggested by Rosenberg (1974).

The sign of Ri indicates the atmospheric condition, and its magnitude reflects the degree of the influence.  There are several different formulas for relating Ri to the atmospheric stability parameters; for these purposes, the sign of Ri is of greater concern than its magnitude.  When Ri is larger than 0.003, the atmosphere exhibits little vertical mixing, reflecting stable conditions; when the absolute value of Ri, |Ri|, is less than 0.003, neutral stability conditions exist (Oliver 1971); and when Ri is less than -0.003, convective mixing becomes dominant and atmospheric conditions are unstable.

To relate the atmospheric stability parameters to the Richardson number, Thom et al. (1975) proposed the following formulas based on the work by Dyer (1974) and Dyer and Hicks (1970).

For stable conditions -

For unstable conditions -

For neutral conditions -

The integrated momentum stability parameter, ψm, can be evaluated based on the following equation as derived by Lo (1977).

Under neutral conditions, ψm = 0 and the equation is not used.

In the application of these procedures, the calculations are performed as follows.

1)	Evaluate Richardson number from temperature and wind velocity gradients.

2)	Determine stability condition based on calculated Ri.

3)	Calculate ϕh and ϕm, based on the stability condition and associated Equations 6-51, 6-52, or 6-53.

4)	Calculate ψm, from Equation 6-54.

5)	Calculate Zo and D from canopy height using Equations 6-48 and 6-49.

6)	Estimate Kz(Z) by applying Equations 6-46, 6-45, and 6-44.

The resistance approach for the estimation of volatilization flux from soil-The calculation of the volatilization flux from the soil is based on a resistance-type approach.  For pre-plant pesticides, and time periods just after emergence and post-harvest, transport by volatilization from plant surfaces is much less than vapor phase transport by other mechanisms.  For those conditions in which the plant leaves do not act as significant sources or sinks for pesticide vapor, the resistances of the air for the whole plant compartment can be estimated as follows (Mehlenbacher and Whitfield 1977).

where

ΣR	=	total vertical transfer resistance (day cm-1)

Rbd	=	boundary layer resistance (day cm-1)

Rpc	=	plant canopy resistance (day cm-1)

The flux is calculated as follows.

where

Jpc		=	volatilization flux from plant canopy (g cm-2 day-1)

Cg,1	=	pesticide vapor concentration in top soil layer (g cm-3

For those conditions in which plants can act as significant pesticide sources or sinks, another approach must be taken.  The influences of plant canopy require the formulation for the surface boundary condition as described in the following two sections.

6.3.4.3  Volatilization Flux from Plant Surfaces--

A detailed description of the controlling factors for volatilization from plant surfaces has been presented by Taylor (1978).  He indicated that the distribution of the pesticide residues over the plant surface appeared to be  the dominant factor.  This, together with the influence of the microscale climate at the plant surface, makes accurate simulation of plant volatilization processes very difficult.

For organophosphate insecticides, Stamper et al. (1979) has shown that the disappearance rates from leaf surfaces can be estimated by a logarithmic or a first-order kinetics approach.  Similar observations for first-order kinetics were found for volatilization of 2,4-D iso-octyl ester from leaf surfaces by Grover et al. (1985).  Thus, a simple rate constant approach is possible that requires the user to input the first-order rate constant for volatilization.  The plant leaf volatilization flux can be estimated as follows.

where

Jpl	=	volatilization flux from the leaf (g cm-2 day-1)

M	=	foliar pesticide mass (g cm-2)

Kf	=	first-order volatilization rate (day-1)

A resistance type approach is also applicable for volatilization flux estimation from plant leaves.  The current code employs the first-order kinetics approach to calculate volatilization flux from plant leaf surfaces described above.  This approach, which requires the user to specify the first-order rates constant for plant leaf volatilization, was selected because it is consistent with the foliar fate model in PRZM Release I.

Average pesticide concentration in plant canopy--Volatilization flux from plant leaves (Jpl) will exist only after pesticide application to the plant foliage has been specified in the model input.  When a plant canopy exists, the average concentration in the air within the plant canopy can be estimated as follows.

where

C		=	average concentration in the air between the ground surface and the plant canopy height (g cm-3)

ΣR0.5	=	canopy resistance from half canopy height to the top of the canopy

Equation 6-60 then calculates the mean plant compartment pesticide concentration as the concentration at one-half of the canopy height.  This approach assumes a linear concentration gradient from ground surface to canopy height.

6.3.4.4  Soil Temperature Simulation--

Soil temperature is modeled in order to correct the Henry's law constant, KH, for temperature effects.  The interaction of its microclimate with the soil surface that results in a given soil temperature regime is complex and dynamic.  Soil surface configuration and plant residue cover, both affected by tillage, have significant impacts on soil heat flux and, therefore, soil temperature.  Studies of tillage and residue effects on soil temperature have been dominated by qualitative observations and site-specific measurements.  The lack of mathematical evaluation and supporting field data has limited the ability of researchers to predict, beyond qualitative terms, the tillage and residue effect on soil temperature for soil and climatic conditions other than those under which data have been collected.

The objective of the soil temperature model is to provide a scientifically sound and usable approach:  (i) to predict with reasonable accuracy the daily average soil temperatures at the soil surface and in and below the root zone, utilizing basic soil physical and thermal properties, and daily climatic measurements taken at weather stations; and (ii) to allow consideration of the residue, canopy, and tillage effects on soil temperature.

Several models are available to predict soil temperature under various soil surface conditions, but there are restrictions to the general use of these models because either they need large data bases that are not available at many places, or they are site specific.  Existing soil temperature models form two general groups:  (1) process-oriented models, which require detailed information on soil and surface characteristics, initial and boundary conditions, and inputs, and (2) semi- or non-process-oriented models, which often utilize weather station information and soil temperature information at one depth to develop empirical relationships.

Table 6-1 summarizes the key characteristics of the soil temperature models reviewed in this work.  For both the process and semi-process oriented models, the two primary components are estimation of soil surface (or upper boundary) temperatures and soil profile temperature utilizing the calculated or estimated surface temperature as the upper boundary condition.  A number of the models utilize the same procedure for calculating temperature in the soil profile (Gupta et al. 1981, Wagenet and Hutson 1987) and differ only in the procedures for specifying the surface boundary condition.

Van Bavel and Hillel (1975, 1976) developed a dynamic numerical procedure to link the process-oriented simulations of heat movement in the soil and the partition of heat and energy at the soil surface.  Soil surface temperature, To, is calculated as a factor in predicting evaporation from a bare soil.  Their technique utilized simultaneous solutions of seven equations with seven unknowns:  net radiative flux, evaporation rate, air sensible heat flux, soil sensible heat flux, surface soil temperature, Richardson's number, and the saturation humidity at the surface soil temperature.  Heat and water (liquid) flows are each coupled at the soil surface.  An iterative procedure was used at each update to find the proper soil surface temperature.  Soil temperatures were then estimated (Wierenga and de Wit 1970) by using these estimates of To as the surface boundary condition.  Inputs required for this model include solar radiation, air and dewpoint temperature, wind speed, initial soil temperature profile, and the surface roughness evaluated by its effect on the aerodynamic roughness parameter.  No comparisons were made between predicted and measured soil temperatures.  Thibodeaux (1979) describes a similar energy-balance procedure for calculating soil surface temperatures.

TABLE 6-1.	SUMMARY OF SOIL TEMPERATURE MODEL CHARACTERISTICS

Model/	Van Bavel	Thibodeaux	Gupta et al.	Parton	Cruse et al.	Hasfurther	Williams	Wagenet	Chen

Author(s)	and Hillel	(1979)	(1981,	(1984)	(1980)	and Burman	et al.	and Hutson	et al.

(1975)		'82, '83)			(1974)	(1983)	(1987)	(1983)

1) Type of Model:

a)	Process-Oriented	X	X					X	X	X

b)	Semi-Process-Oriented			X	X

c)	Non-Process-Oriented					X	X

2)	Heat Flow Process

a)	Conduction	X		X	X			X	X	X

b)	Convection									X

c)	Radiation	X	X			X		X		AT

3)	Upper Boundary Temperature

a)	Est. by Energy Partitioning	X	X

b)	Est. by Empirical Relationship			X	X	X		ME	AVE

4)	Soil Temperature Profile:

(Solving 1-D Heat Flow Eqn.

Using the Procedure of:)

a)	Hanks et al. (1971)			X	EX				X

b)	Wierenga and de Wit (1970)	 X*

c)	Curve Fitting					   X**	X	DD

5)	Input Data Required

a)	Daily Max and Min Air Temp.			X	X	X	X	X

b)	Daily Max and Min Soil			X

Surface Temperature

c)	Hourly Air Temperature	X	X	X						X

d)	Hourly Solar Radiation	X	X		XX	XX		XX

e)	Surface Albedo	X	X			X		X

f)	Wind Velocity	X	X			X

g)	Humidity/Dewpoint Temp.	X	X

h)	Canopy Shadow/Ht. of Veg.	X	X

i)	Soil Water Content	X		X		at 5 cm		X	X	X

j)	Soil Bulk Density	X		X		X		X	X	X

k)	Soil Mineral Composition	X		X		X			X	X

l)	Percentage Organic Matter	X		X		X			X	X

6)	Soil Surface Condition

a)	Residue Cover	X		X		X			X	100%

b)	Tillage Condition			X		X		X

c) Crop Canopy	X	X	X	X			X

7)	Time Step

a)	Hourly	X	X	X	X				X	X

b)	Daily					X	X	X	X	X

*	-	Horton et al. (1984) used a 2-D heat flow equation.	AVE	-	"Average" measured soil surface temperatures are used.

**	-	Regression equation is fitted for soil temp at 5-cm depth.	AT	-	Ambient air temperature is used as upper boundary temperature.

DD	-	Damping depth parameter is used to predict soil	XX	-	Total daily solar radiation.

temperature at different depths.	EX	-	Explicit Finite Difference Scheme.

ME	-	Simplified mathematical relationship involving solar

radiation, surface albedo, and daily min and max air temperatures.

For modeling soil profile temperatures, Hanks et al. (1971) used a numerical approxima-tion for the one-dimensional soil-heat flow equation.  This method requires the input of initial and boundary conditions, as well as the soil thermal conductivity and heat capacity as a function of depth and time.  Predicted root zone soil temperature profiles were within 1°C of observed values for a 3-day period, but this model needs estimated or measured soil surface temperatures as upper boundary condition.

Using the Hanks et al. (1971) procedure for the root zone, Gupta et al. (1981, 1982, 1983, 1984) developed a model for estimating hourly soil temperature by depth from meteorologic data.  Inputs needed for this model include hourly air temperature at the 2-m height; daily maximum and minimum soil temperatures; initial soil temperature with depth; and soil thermal diffusivity, which may be estimated from soil mineral composition, organic matter percentage, bulk density, and soil water content.  The upper boundary temperatures are estimated by a sine function.  The amplitude of the function is equal to the difference between daily maximum temperatures of air and soil surface or daily minimum temperatures of air and soil surface.  Empirical curves relating daily maximum air temperature to daily maximum soil surface temperature, and daily minimum air temperature to daily minimum soil surface temperature, were developed for different residue and tillage conditions for the specific application site.  These relationships provided a means of accounting for residue and tillage effects on soil temperature, but require site-specific data.

The soil temperature model in PRZM-2 is derived from a combination of the work by van Bavel and Hillel (1976) and Thibodeaux (1979) for estimating the soil surface/upper boundary temperature.  The soil profile temperature procedures were developed by Hanks et al. (1971) and applied by Gupta et al. (1981, 1982, 1983) and Wagenet and Hutson (1987).

Estimating upper boundary temperature--An energy balance procedure is used in PRZM-2 to estimate soil surface temperature (Thibodeaux 1979, van Bavel and Hillel 1976).  The same procedure is used in the POSSM model (Brown and Boutwell 1986), which employs PRZM-2 as a framework for PCB fate simulation.

The basic energy-balance equation with terms having units of cal cm-2 day-1 at the air/soil interface may be described as:

where

Rn		=	net radiation (positive downward)

Hs		=	sensible air heat flux (positive upward)

LEs	=	latent heat flux (positive upward)

Gs		=	soil heat flux (positive downward)

ΔTH	=	change in thermal energy storage in the thin soil layer (cal

cm-2 day-1)

The term ΔTH can be evaluated as:

where

ρb			= bulk density of soil (g cm-3)

d			= thickness of a thin, surface soil layer (cm)

s			= the specific heat capacity of soil (cal g-1 °C-1)

Ti,Ti+1	= the representative temperature for the surface layer at two consecutive 				time steps and can be represented as the average of temperatures at the 				top and bottom of the soil layers.

For evaluating the heat exchange across the air/soil interface, the thickness, d, can be set to a small value so that ΔTH may be neglected.  As a result, the right side of Equation

6-62 is set equal to zero.

Net radiation flux at any surface can be represented as:

where

Rn		= the net radiation flux (cal cm-2 day-1)

Rs		= incident short-wave solar radiation (cal cm-2 day-1)

Rsr		= reflected short-wave solar radiation (cal cm-2 day-1)

Rla		= incident long-wave atmospheric radiation (cal cm-2 day-1)

Rlar	= reflected long-wave atmospheric radiation (cal cm-2 day-1)

Rls		= long-wave radiation emitted by the soil (cal cm-2 day-1)

The terms Rs and Rsr include both the direct and diffuse short-wave radiation, and are related as follows.

where

α		= the albedo of the surface (dimensionless)

Therefore, the short-wave radiation component of the energy balance is

The incident short-wave radiation can either be measured directly using pyranometers or else calculated using a variety of available empirical relationships or nomographs.  The model requires input of a radiation time series, whether measured or calculated, in order to simulate soil temperature.

The albedo of a canopy-covered land surface can be estimated as:

where

α(t)	= albedo on day t

αc		= albedo of canopy cover (0.23 for vegetation)

C(t)	= canopy cover on day t (fraction)

αs		= albedo of soil surface (dimensionless)

Since the albedo of soil surface changes with the soil surface condition, it is defined by the user as 12 monthly values corresponding to the first day of each month; the albedo value for each day is interpolated between the neighboring monthly values.  For snow cover less than 0.5 cm, the surface albedo is estimated using Equation 6-67, and for snow cover above 0.5 cm, the surface albedo is set equal to the snow albedo value (0.80).

The incident long-wave atmospheric radiation, Rla, is represented as

where

ea		= emissivity of the atmosphere [dimensionless]

α		= the Stefan-Boltzmann constant [11.7*10-8 cal cm-2 °K-4 day-1]

Ta		= the air temperature [°K]

Wunderlich (1972) has proposed a correction to Equation 6-68 for the effects of cloud cover, which could increase Rla by up to 25 percent under overcast conditions.  However, this correction is not included in the model because it would require input of a cloud cover timeseries, and the effect on the calculated soil surface temperature would be small.

The emissivity of the atmosphere varies from a low of 0.7 to almost unity.  Numerous empirical relationships for estimating ea have been proposed (Salhotra 1986).  A simple reliable method is the use of Swinbank's formula:

The reflected long-wave radiation, Rlar, can be expressed as:

where

γ		=	the reflectivity of the surface for long-wave radiation [dimensionless]

The resulting net atmospheric long-wave radiation component becomes:

The long-wave radiation component emitted by the soil surface is represented in an analogous equation to the atmospheric component, as follows.

where

es		= infrared emissivity of soil (dimensionless)

Ts		=	soil surface temperature (°K)

Since the soil emissivity and reflectivity are related as es=1-γ, we can replace (1 - γ) in Equation 6-71 with es.

Combining the radiation components from Equations 6-66, 6-71, and 6-72, the net radiation flux is calculated as follows.

The evaporative heat flux, LEs, is estimated by:

where

μ		=	latent heat of vaporization/unit quantity of water

(580.0 cal g-1)

E		=	evaporation rate (cm day-1)

ρw		=	density of water (1.0 g cm-3)

The evaporation rate is obtained from the evapotranspiration (EVPOTR) subroutine of PRZM.  It is assumed that the calculated evapotranspiration from the top 5 cm of soil represents the potential evaporation energy loss at the air/soil interface.  However, only a fraction of the evapotranspiration loss calculated by PRZM contributes to this heat flux.  This fraction is estimated as the portion of the land surface not covered by vegetation, (i.e., 1.0 - canopy cover).

The sensible air heat flux, Hs, is given by:

where

ρa		=	air density (g cm-3)

=	(-0.0042 Ta + 1.292)10-3

Cpa	=	specific heat of air at constant pressure

(0.2402 cal g-1 °K-1)

h		=	heat transfer coefficient at air-soil interface (cm day-1)

Ta		=	the air temperature (°C)

The air density is computed based on the daily air temperature using a simple linear correlation Equation 6-73 developed from data in Thibodeaux (1979).  The heat transfer coefficient is given by:

where

K1		=	Von Karman's number (0.41)

Vz		=	wind velocity (cm day-1)

ZRH	=	reference height at which Vz is measured (m)

D		=	zero plane displacement (m)

Zo		=	roughness height (m)

Equation 6-76 is valid only when the air temperature does not vary greatly with height, as is often the case near sunrise or sunset or under cloudy skies or when canopy heights are relatively small.  It appears to be a reasonable approximation for most agricultural crops.  Correlations have been developed relating D and Zo to the canopy height as described previously in this section by Equations 6-48 and 6-49.

From the fundamental equation of heat conduction, the soil heat flux, Gs, is given by:

where

T1		=	temperature of the soil at bottom of layer 1 (°K)

Ts		=	soil surface temperature (°K)

λ1		=	thermal conductivity of layer 1 (cal cm-1 day-1 °K-1)

D1		=	thickness of layer 1 (cm)

Substituting Equations 6-71, 6-72, 6-73, and 6-75 into Equation 6-60, the following fourth-order equation in terms of Ts results.

The value of Ts at each time step is estimated by solving the above equation using an iterative solution based on the Newton-Raphson method.  The initial estimate of soil surface temperature is taken to equal measured air temperature, and Rn, LEs, Hs, and Gs are calculated as explained above.  The value for T1 is obtained from the previous time step.  These calculations are repeated until the difference between two consecutive estimates for soil surface temperature is less than the convergence criteria (set to 0.1°C).

Simulation of heat flow through soil profile--The soil profile temperature model is based on the one-dimensional partial differential equation describing heat flow in soils:

where

d		=	the thermal diffusivity.

The thermal diffusivity is equal to the ratio of thermal conductivity and heat capacity of the soil.  The procedures used to estimate soil thermal conductivity and heat capacity are taken from de Vries (1963).  They are calculated from basic soil properties--soil water content, mineral composition, texture, and thermal conductivity of the individual soil particles.  These parameters are either input or supplied by the model in the simulation.  The thermal diffusivity is given by:

where

d		=	thermal diffusivity of the soil layer (cm2 day-1)

λ		=	thermal conductivity of the soil layer

(cal cm-1 day-1 °C-1)

C		=	heat capacity per unit volume of the soil layer

(cal cm-3 °C-1)

Temperature effect--A detailed discussion of the temperature effect on the volatilization behavior of pesticides is presented by Streile (1984).  Two parameters that influence the vapor-phase transport in the soil profile are Henry's constant and the vapor diffusion coefficient.

The equation used to correct Henry's constant for temperature effects is (Streile 1984):

where

KH,1	=	Henry's constant at the reference temperature T1

ΔHap	=	partial molar enthalpy of vaporization from solution

(J mole-1)

The temperature effect on the vapor phase diffusion coefficient can be estimated from the Fuller correlation as presented in Liley and Gambill (1973).  However, it is not implemented in the code due to the general lack of information required to use it.

6.3.5  Irrigation Equations

PRZM-2 irrigation algorithms determine depths of irrigation water to be applied at the soil surface.  These depths are computed from the soil water deficit and are added as infiltration to the first PRZM soil compartment.  Above- and below-canopy sprinklers, flooding, and furrow irrigation can be simulated.  Methods for computing water application depths for each type of irrigation are described in the following paragraphs.

6.3.5.1  Soil Moisture Deficit--

Irrigation is triggered when the average root-zone soil moisture volume falls below a level fc defined by the user as a fraction of the available water capacity.  The soil moisture deficit, D, is then given by:

where

D		=	soil moisture deficit (cm)

z		=	average root-zone soil moisture content (cm3cm-3)

fc		=	average root-zone soil moisture content at field capacity (cm3cm-3)

Zr		=	root zone depth (cm)

D is the depth of water over the unit area that must be added to the soil by irrigation to bring the soil water content up to field capacity.

6.3.5.2  Sprinkler Irrigation--

Irrigation water from sprinklers may be applied either above or below the crop canopy.  When applied above the crop canopy, irrigation water is intercepted by the canopy and may run off when it reaches the soil surface.  The depth of water applied during a daily PRZM-2 time step by overcanopy sprinklers is estimated from the soil moisture deficit:

where

Da	= depth of irrigation water applied to the field (cm)

If		= crop canopy interception capacity (cm)

LF	= a factor specified by the user to allow for the practice in saline soils of

adding water to leach salts out of the root zone (fraction of Da)

The water depth Da is applied as precipitation above the crop canopy, and canopy interception is computed for the current crop in the PRZM-2 crop growth subroutines.  Sprinkler runoff from the soil surface is estimated using the SCS curve number approach, assuming that runoff characteristics of sprinkler water are similar to those of precipitation.  Water that does not run off infiltrates into the first PRZM-2 soil compartment.

Irrigation water applied below the crop canopy is not subject to canopy interception losses.  The depth of water applied by undercanopy sprinklers is therefore, is given by:

The irrigation water depth APDEP is applied as throughfall to the soil surface, and sprinkler runoff is estimated using the SCS curve number approach.

In some instances, the sprinkler system may be unable, due to hydraulic limitations, to deliver water at the rate needed to meet the required daily application depth.  In these cases, the sprinkler application depth Da is set equal to the maximum depth that the system can deliver.  The user, therefore, is required to input the maximum water application rate Rmax (cm hr-1) for the sprinkler system.

6.3.5.3  Flood Irrigation--

Flood irrigation, in this case, refers to the practice of flooding entire fields with irrigation water.  Flood-irrigated fields are diked around the edges to allow water to pond and infiltrate into the soil.  In the PRZM irrigation algorithm, it is assumed that this water ponds uniformly over the entire field.  The amount of water applied to the soil surface is then:

Since the field is assumed to be diked around the edges, no water is allowed to run off from the field.

6.3.5.4  Furrow Irrigation--

Furrow irrigation involves the release of water into numerous small channels that cut across the planted field.  Infiltration depths within furrows vary due to differences in times at which water reaches various locations down the furrow, with less water infiltrating at the downstream end (Figure 6.4).  Hydraulic characteristics of the furrow determine how quickly water moves down the channel, while soil characteristics determine the rate of infiltration once water reaches a location in the furrow.

The PRZM-2 furrow irrigation model computes daily infiltration depths at various locations down the length of the furrow.  This requires solution of the open channel flow equations of motion coupled with a soil infiltration model.  Model developers have made numerous attempts to solve the furrow-irrigation advance problem, ranging in complexity from empirical volume-balance solutions (Wilke and Smerdon 1965, Fok and Bishop 1965) to numerical solutions of the full open channel flow equations of motion (Bassett and Fitzsimmons 1974).  In general, solutions of the full equations of motion are too computationally intensive for this application, while simpler empirical models involve infiltration parameters that are not easily related to physical soil characteristics.

The PRZM-2 furrow advance model uses the kinematic wave simplification of the equations of motion coupled with the Green-Ampt infiltration model to determine furrow infiltration depths.  Kinematic-wave theory neglects inertial accelerations and assumes that the water surface slope is equal to the ground slope.  The equations of motion then reduce to:

where

Q		=	flow rate in the channel (m3 s-1)

A		=	cross-sectional area of flow (m2)

x		=	distance down the furrow (m)

q		=	volume infiltrated per unit length of channel (m3 m-1)

The flow area A is related to the flow rate Q by Manning's equation:

where

n		=	Manning's roughness coefficient

R		=	the hydraulic radius of flow (m)

S		=	the channel slope (vertical/horizontal)

Section 6.4.4 explains how the solution of the horizontal furrow irrigation equation is applied to PRZM-2.

6.4  NUMERICAL SOLUTION TECHNIQUES

This section describes the numerical techniques that are used to solve the differential equations introduced in the preceding section.  Section 6.4.1 discusses the two numerical techniques available to solve the chemical transport equations--a backwards-difference implicit scheme and a method of characteristics algorithm.  The additional terms and the adjustment in the upper soil boundary that are added into these transport equations to simulate volatilization are described in Section 6.4.2.  The numerical approximations used to calculate soil temperature are presented in Section 6.4.3 and the numerical solution for furrow infiltration depths are presented in Section 6.4.4.

6.4.1  Chemical Transport Equations

The second-order partial differential equation outlined in Section 6.3 must be solved with appropriate boundary conditions.  The calculations for moisture contents, air contents, pore velocities, erosion, and runoff are decoupled from, and solved in advance of, the transport equation.  The resulting values, treated as constant for each specific time step, are then used as coefficients in a discretized numerical approximation of the chemical transport equation.

Two techniques are currently available to solve the discretized chemical transport equation for the new dissolved pesticide concentration at the end of the time step.  The available techniques are:

●	A backward-difference, implicit scheme to simulate all chemical transport processes

●	A method of characteristics (MOC) algorithm that simulates diffusion, decay, erosion, runoff, and uptake by the backward-difference technique, but uses the method of characteristics to simulate advective transport

The user is allowed to select the desired solution technique in the input sequence.  Details of these techniques are provided below.  Results from test simulations are provided in Section 6.5.1.

Identical discretizations and initial and boundary conditions are used with both numerical simulation techniques.  A spatial and temporal discretization step is used equal to those applied in the water balance equations.  For boundary conditions at the base of the soil column, the numerical technique uses

in which the subscripts "i" refer to soil layer numbers.

This condition corresponds to a zero concentration gradient at the bottom of the soil profile.  The upper boundary condition is discussed in more detail in Section 6.4.2.

A backwards-difference solution algorithm was the only solution option available in the original PRZM model.  In this method, the first derivative in space, the advection term, is written as a backward difference (i.e., involves the difference C[i,j]-C[i-1,j]).  The second spatial derivative, the diffusion term, is centered in space (i.e., based on the terms C[i-1,j]+C[i+1,j]-2C[i,j]).  The time derivative is also calculated as a backward difference in the original code, (C[i,j]-C[i,j-1]).  The equations are then made implicit by writing each concentration for the (j+1)th time step.  The advantage of this numerical scheme is that it is unconditionally stable and convergent.  However, the terms truncated in the Taylor's series expansion from which the finite difference expression are formulated lead to errors that, in the advection terms, appear identical to the expressions for hydrodynamic dispersion.  In the simulation results, these terms manifest themselves as "numerical dispersion," which is difficult to separate from the physical dispersion that is intentionally simulated.  In systems exhibiting significant advection (i.e., high Peclet number), the artificial numerical diffusion may dominate the physical dispersion.  It can be larger by orders of magnitude, leading to difficulty in the interpretation of simulation results.

To minimize the effects of numerical dispersion in systems having high Peclet numbers, a method of characteristics solution was added as an option to PRZM-2.  This solution method avoids the backwards-difference approximation for the advection term and the associated numerical dispersion by decomposing the governing transport equation.  In advection-dominated systems, as the dispersion term becomes small with respect to the advection term, the advection-dispersion equation approaches a hyperbolic equation.  According to the MOC theory, advection of the solute can be simulated separately from the other processes governing the fate of that advected solute.  M. Baptista et al. (1984) state that no error is introduced by this decomposition provided that the advection equation is solved first by an explicit procedure, and the diffusion equation is solved next by an implicit technique.  This order was preserved in the PRZM-2 model by utilizing a new explicit algorithm for advection that is always called first, and is immediately followed by execution of a modified version of the existing implicit algorithm for simulation of other processes.  The advection algorithm employed was adapted from those described by Khalell and Reddell (1986) and Konikow and Bredehoeft (1978).  These techniques were modified to allow simulation of changes in saturation and adsorption of the pesticide and variable compartment size.

In the new explicit advection algorithm, in addition to the fixed grid system, a set of moving points is introduced.  These points can be visualized as carrying the chemical mass contained within a small region in space surrounding the point.  Initially, these points are uniformly distributed throughout the flow domain.  At each time interval, these moving points are redistributed according to the local solute velocity in each compartment.  New points may enter the top of the flow domain, while old points may move out the bottom.  When the moving points are transported in horizons where the compartment size is larger and numerical resolution is less, the points may be consolidated to conserve computational effort.  After the new locations have been assigned to each point, the average concentration in each compartment is computed based on the number and mass carried by the points contained within the compartment at that time.  This temporary average concentration is returned to the main program, and a subroutine that assembles the terms in the transport equation (without advection) is called.  Changes in concentration due to all other transport and transformation processes (diffusion, decay, sources, etc.) are calculated for each compartment exactly as in the original version of PRZM.  These values are then returned to the main program, and one transport step is complete.

When the MOC algorithm is called during the next time step, the exact location of each moving point has been saved.  The first task is to update the masses carried by each moving point using the changes calculated during the last time step.  Increases in mass are simply added equally to each point in the compartment, while decreases are weighted by the actual value at each point before subtraction to avoid simulating negative masses.  The updated moving points are then relocated and the two-step process is repeated again until the end of the simulation.

6.4.2  Volatilization

The numerical techniques discussed in section 6.4.1 are the basis of the simulation of chemical transport in all phases.  However, some modifications have been made to the upper boundary condition in order to model volatilization of chemical from the soil surface.

In order to simulate vapor-phase pesticide movement past the soil surface, the zero concentration upper boundary conditions used in the original PRZM code has to be modified.  Jury's boundary layer model (1983a, 1983b) has been incorporated into the PRZM-2 code.  The model states that the controlling mechanism for pesticide volatilization is molecular diffusion through the stagnant surface boundary layer.  The volatilization flux from soil profile can be estimated by:

where

J1		=	volatilization flux from soil (g day-1)

Da		=	molecular diffusivity of the chemical in air (cm2day-1)

Cg,1	=	vapor-phase concentration in the surface soil layer

(g cm-3)

C,d	=	vapor-phase concentration above the stagnant air boundary layer (= 0, for the no-canoy field condition) (g cm-3)

d		=	thickness of stagnant air boundary layer (cm)

This equation defines the new flux-type boundary condition for the volatilization simulation.  In order to incorporate the new flux-type boundary condition into the

PRZM-2 code, new mass balance equations were derived for the surface soil and stagnant air layers.  Figure 6.5(a) is a schematic of the top two soil layers and the stagnant surface boundary layer when no plant canopy exists.  Zero concentration is assumed for C,d under the no-canopy field condition.

A mass balance equation for the uppermost soil compartment is

where

Dg		=	molecular diffusivity of pesticide in air filled pore space (cm2day-1)

V		=	volume of the compartment (cm3)

A		=	area of the compartment (cm2)

a		=	volumetric air content (cm3 cm-3)

Kg		=	first-order reaction rate constant (day-1)

The first term of the right side of Equation 6-90 represents the gas diffusive flux into the surface soil layer, and the second term denotes the gas diffusive output as governed by the stagnant boundary layer above the soil surface.  By using backward implicit finite differencing, the following is derived.

where

n		=	time index

By substituting Equation 6-91 into the overall (i.e., all phases) mass balance equation for the uppermost soil layer, a flux-type upper boundary condition is obtained.  Figure 6.5(b) reflects the field situation when a plant canopy exists.  Zero concentration is now assumed to exist above the top of the canopy compartment.  The volatilization flux from the plant canopy is defined as follows.

where

Jpc		=	volatilization flux through the plant canopy (g cm-2 day-1)

ΣR	=	vertical transfer resistance (day cm-1, described in

Section 6.3.4.3)

C*		=	concentration above the plant canopy (assumed to be zero)

By carrying out a similar mass balance using finite differences, the boundary condition that describes the field with canopy existing is obtained.

6.4.3  Soil Temperature

Soil temperature is solved for numerically.  Section 6.3.4.4 describes the theoretical basis for the simulation of soil temperature.  The distribution of temperature within the soil profile is summarized by Equation 6-79.  This equation is solved numerically for soil temperature, T, as a function of depth, Z, and time, t, based on the input thermal diffusivity, d, for each soil compartment, and the following initial and boundary conditions.

Initial Condition:

Boundary Conditions:

where

T(z)	=	initial soil temperature in each soil compartment (°C)

Ts(t)	=	calculated soil surface temperature for each time step (°C)

TL(t)	=	lower boundary temperature condition at the bottom of the soil core (°C)

The lower boundary temperature is defined by the user as 12 monthly values corresponding to the first day of each month; the value for each day is interpolated between the neighboring monthly values.

The following numerical approximation used in the model is taken from Hanks et al. (1971).

Equation 6-96 is solved using a modified numerical solution procedure of Hanks et al. (1971), which involves the same finite difference technique and tridiagonal matrix solver (Thomas algorithm) used in PRZM (Carsel et al. 1984).

6.4.4  Furrow Irrigation

To simplify the algebra required to calculate the furrow infiltration volume as Manning's equation is substituted into the kinematic wave model (equation 6-86), Manning's equation is approximated as follows.

α and m are constants that are estimated by the model from the parameters of Manning's equation as follows.

where

A1, A2	=	cross-sectional areas (m2) at depths y1 and y2

Q1, Q2	=	flow rates (m3 s-1) computed from Manning's equation [(Equation 6-75)] at depths y1 and y2

y1		=	1 cm

y2		=	10 cm

The depths y1 and y2 were chosen to represent the range of depths likely to occur in furrows.

Substituting Equation 6-97 into Equation 6-86 produces:

No closed-form solution to the above equation is known when infiltration is time-variable.  Equation 6-88 therefore, is, solved for Q using the backwards-space, backwards-time finite-difference solution described by Li et al. (1975).  Writing Equation 6-100 in finite-difference form producers:

where

Q		=	flow rate at time k, station i

Δz		=	spatial step

Δt		=	time step

Infiltration volumes are computed using the Green-Ampt model:

where

I		=	infiltration depth at time k (m), station i

Ks		=	saturated hydraulic conductivity of the soil (m s-1)

H		=	ponded water depth (m)

Hs		=	suction parameter (m)

θ		=	available porosity (fraction)

I		=	total volume of infiltrated water (m)

The Green-Ampt model has long been accepted as a model of the advance of the wetting front through the soil column, and involves parameters that can be related to well-known soil properties.  The volume of infiltration is computed assuming I is an average infiltration depth for the channel at location i:

where

q		=	volume infiltrated at location i (m3 m-1)

W	=	current flow width at location i (m)

Furrow channels are assumed to be trapezoidal in shape.  Equation 6-87 is solved at each station at the end of each time step for the new flow rate Qn+1i+1.  Because the equation is non-linear with respect to Q, the new value of flow is found using second-order Taylor series iteration.  Given the flow rate in the furrow, infiltration depths at each location are then computed using the Green-Ampt model (Equation 6-90).

The PRZM-2 furrow irrigation model determines infiltration depths at various locations in the furrow.  Irrigation continues until the depth of water infiltrated at the downstream end of the furrow is sufficient to meet the soil moisture deficit SMDEF.  The depth of water applied as irrigation to the first PRZM-2 soil compartment is then set equal to either the average furrow infiltration depth or the infiltration depth at a specific location in the furrow, depending upon options selected by the user.  This depth of water then infiltrates through the root zone as determined by the PRZM-2 soil hydraulic algorithms.

6.5  RESULTS OF PRZM TESTING SIMULATIONS

This section includes the results of testing the two solute transport solution techniques and the volatilization algorithm.  Simulated results are compared with those from analytic solutions.  Sensitivity analyses also were performed to evaluate the effects of key model parameters on the prediction of volatization rates.  A test comparison of the model with field data from Georgia (soybeans) concludes the section.

The PRZM model has undergone additional performance testing with field data in New York and Wisconsin (potatoes), Florida (citrus), and Georgia (corn) (Carsel et al., 1985; Jones 1983; Jones et al., 1983).  The results of these tests demonstrate that PRZM is a useful tool for evaluating groundwater threats from pesticide use.  Please refer to these references for information regarding the further testing of PRZM-2 under field conditions.

6.5.1  Transport Equation Solution Options

Currently, two numerical solution options are available to the PRZM-2 user for the chemical transport equation.  As discussed in Section 6.4.1, the finite difference option (utilizing subroutine SLPST0) is unconditionally stable and convergent, but may result in excessive numerical dispersion in high Peclet number systems.  The method of characteristics algorithm (utilizing subroutines MOC and SLPST1) eliminates or reduces that numerical dispersion.  Two examples are provided that compare the alternate solutions methods at high Peclet number (greater than 5.0) and at low Peclet number (less than 0.5).

6.5.1.1  High Peclet Number--

Figure 6.6 presents the analytical solution (Hunt 1978) together with the SLPST0 and MOC/SLPST1 solutions at 6 days for the transport of a 69 mg cm-3 pesticide application in the uppermost compartment.  The physical parameters are as presented in the figure--notably the Peclet number is 5.1.  The following table details pertinent features of the simulation:

| Method | Location of Peak | Value of Peak (mg/cm3) | % Error at Peak | Runtime (sec) |
| --- | --- | --- | --- | --- |
| Analytical | 5.8 | 11.2 | -- | -- |
| SLPSTO | 4.5 | 5.07 | -54 | 88.5 |
| MOC/SLPST1 | 5.5 | 12.09 | +7 | 112.4 |

At this relatively high Peclet number, the SLPST0 algorithm shows excessive numerical dispersion, capturing only about half the amplitude of the peak concentration, while showing excessive mass in both tails.  In addition, the SLPST0 algorithm does not predict the location of the peak precisely.  (It is logged behind the location of the peak given by the analytical solution and the MOC/SLPST1 solution.)  The MOC/SLPST1 algorithm requires 27% more runtime, but errs by only 7% in the peak and shows good agreement in the tails.

6.5.1.2  Low Peclet Number--

Figure 6.7 illustrates the results of a SLPST0 and MOC/SLPST1 simulation 8 days after an incorporation of 69 mg/cm3 in the sixth compartment using the parameters listed.  The predicted concentrations at this lower Peclet number, 0.46, are very similar in the peaks and the tails, and apparently little additional resolution is gained from utilizing the MOC algorithm.  However, the additional computational burden associated with the MOC algorithm is only 7%.

6.5.2  Testing Results of Volatilization Subroutines

To test and validate the operation of the volatization algorithms, model results were compared with Jury's analytical solution (Jury et al., 1983a), and against field data for trifluralin from Watkinsville, GA.  Sensitivity analyses were also performed to evaluate effects of key parameters on model predictions.  The intent of this preliminary model testing was to evaluate model operation by comparing the results for the volatization flux from a soil surface application.

6.5.2.1  Comparison with Analytical Solution--

Jury et al. (1983a) presented a mathematical model for describing volatile loss and movement of soil-applied organic chemicals.  By making the following assumptions, they derived an analytical solution for evaluating the chemical concentration profile within the soil and the volatization flux at the soil surface:

1)	Uniform soil properties consisting of a constant water content, bulk density, liquid water flux (either upward, downward, or zero), and a constant organic carbon fraction

2)	Linear equilibrium adsorption isotherm

3)	Linear equilibrium liquid-vapor partitioning (Henry's law)

4)	Uniform incorporation of a quantity of chemical to a specified depth below the surface

5)	Pesticide loss by volatilization through a stagnant air boundary layer at the soil surface

6)	Infinite depth of uniform soil below the depth of incorporation

Assumptions 2 to 5 are satisfied by the current PRZM-2 code.  Assumption 6 defines zero concentration for the bottom layer, which is somewhat different from PRZM's zero gradient bottom boundary condition.  However, as long as no chemical reaches the bottom layer, these two types of boundary conditions produce identical results.  Our test runs for volatization were designed to satisfy this requirement.  In order to comply with assumption 1, the hydrological computation subroutines in PRZM were bypassed and replaced with a constant value for water flux.  A positive flux value indicates a leaching condition, whereas a negative flux value indicates an evaporating condition.  The hydrological subroutines in PRZM-2 are based on a moisture-routing method in which daily accounting of water inflow and outflow is recorded.  One limitation of the moisture-routing method is that it is unable to properly describe the upward movement of evaporating water.  Evaporation loss is removed from specific surface soil layers without accounting for movement between layers.

The pesticide 2,4-D was chosen as the test compound for our simulation; the input parameters are listed in Table 6-2 and were obtained from Jury et al. (1983a).  The test run results for daily volatilization flux are presented in Figures 2.8(a), 2.8(b), 2.9(a), and 2.9(b), corresponding to the four test cases listed at the bottom of Table 6-2.  Two different

Velocity = 1.82 cm/day	Delta x = 1 cm

Diff coef = 4.0 cm2/day	Delta t = 1 day

Retardation Coef = 11.74	Core Length = 20 cm

Decay = 0.1/day	Peclet = 0.46

soil compartment depths (DELX) of 1.0 and 0.1 cm were used to investigate the sensitivity of the volatilization algorithms to the spatial discretization in the surface soil horizon.

Figure 6.8(a) shows the steady state situation (i.e., no evaporation and no leaching) without any advective movement.  The daily volatilization flux values predicted by the two different DELXs are almost identical.  In this case, the magnitude of DELX is relatively unimportant.  The simulation results with a leaching rate of 0.01 cm day-1 are shown in Figure 6.8(b).  Because of the leaching influence, the predicted daily flux is smaller than the corresponding daily value shown in Figure 6.8(a).  The differences between the analytical solution and the PRZM-2 predictions are due to the finite difference solution technique and the occurrence of advective movement by leaching.  The simulation results using the smaller DELX (0.1 cm) more closely match the analytical solution results, and an even smaller DELX would have improved the agreement further.  The slope of both DELX curves is the same as the analytical solution, and the maximum differences (for the 1.0 cm DELX) from the analytical solution are 10% or less.

Figure 6.9 shows the simulation results under evaporating conditions with the upward advective velocity at 0.01 (Figure 6.9(a)) and 0.25 (Figure 6.9(b)) cm day-1.  The "wick effect" phenomenon (described in Section 6.3.4) leading to enhanced upward movement of the pesticide can be observed in these two figures.  The maximum daily flux occurs on the first day for the leaching conditions.  Depending on the magnitude of the evaporating water velocity, the maximum daily flux no longer occurs on the first day of the pesticide application.  Also the magnitude of the maximum daily flux is enhanced by the magnitude of the evaporating water velocity.  The effect of DELX becomes more critical as the influence of advective movement increases.  For simulations using a 1.0-cm DELX, Figure 6.9(a) shows stable numerical behavior with a small discrepancy when compared to the analytical solution result.  As the advective movement becomes larger, the numerical behavior becomes more unstable, as shown in Figure 6.9(b).  The smaller 0.1-cm DELX showed good agreement with the analytical solution for both test cases shown in Figure 6.9.

TABLE 6-2.	INPUT PARAMETERS FOR THE TEST CASES - ANALYTICAL SOLUTION

DG	Air diffusion coefficient	0.43 (m2 day-1)

DL	Water diffusion coefficient	4.3 x 10-5 (m2 day-1)

ϕ	Porosity	0.5

ρ	Bulk density	1.35 (kg m-3)

T	Temperature	25°C

foc	Organic carbon fraction	0.0125

θ	Water content	0.3

a	Air content	0.2

M	Pesticide applied	1 (kg ha-1)

L	Depth of incorporation	0.1 m

KH	Henry's constant for 2,4-D	5.5 x 10-9

Koc	Organic carbon partition

coefficient for 2,4-D	0.02 (m3 kg-1)

u	Decay coefficient for 2,4-D	4.62 x 10-2 (day-1)

l	Total depth of soil column	0.3 m

t	Simulation period	30 days

Jw	Water flux

E	Evaporation flux

Test case #1:  no evaporation and no leaching (Jw = E = 0)

Test case #2:  with leaching (Jw = 0.01 cm day-1)

Test case #3:  with evaporation (E = 0.01 cm day-1)

Test case #4:  with evaporation (E = 0.25 cm day-1)

Based on these test cases, it appears that a finer DELX, in the range of 0.1 to 0.5 cm, is needed for top soil layers when volatilization processes are simulated with PRZM-2.  However, this finer DELX requirements poses an additional computational burden for PRZM-2 applications due to the increase in the number of soil compartments.  To circumvent this burden, the PRZM-2 code was modified to allow a variable compartment depth, which allows the user to select a smaller DELX for the top horizon (or any other horizon) and a bigger DELX for the rest of the soil profile.  By selecting this variable compartment depth capability, a significant saving in CPU time may be achieved while a better representation is provided for calculation of the surface volatilization flux.  In conjunction with field data comparisons (presented below), the results of model runs and CPU time are presented for simulation runs both uniform and variable compartment depth.

6.5.2.2  Comparison with Field Data--

Preliminary model testing with field observations also was performed to assess the ability to predict the general magnitude of volatilization losses and daily fluxes under field conditions.  Based on a review of available volatilization field data sets, a USDA experimental watershed site in north-central Georgia was selected because of its use of a volatile pesticide (trifluralin), surface-applied to a major crop (soybeans), with a comprehensive micrometeorological and soil sampling plan.

The study site was located at Watkinsville, GA, on a 1.26-ha watershed comprised of Cecil soil (63.9% sand, 23.6% silt, and 12.5% clay) with 0.55% organic carbon, a pH of 6.5, and a slope of 3.0%.  Harper et al. (1976) present a detailed description of the site, the equipment, and the installation procedures required for collecting microclimate data.  They also summarize the method, assumptions, and calculations used for determining pesticide volatilization flux rates.  Trifluralin was surface-applied as a spray to a bare soil surface, using a ground sprayer equipped with flat-fan nozzles, at a rate of 1.12 kg/ha between 1220 and 1247 eastern daylight time (EDT) on 15 June 1973.

The field results shown in Table 6-3 were obtained from White et al. (1977).  The values in columns 2, 4 and 5 of Table 6-3 provide the cumulative volatilization flux, remaining pesticide in soil, and total cumulative decay losses, respectively.  A discrepancy is noted for the data in column 4 of Table 6-3; the pesticide remaining in soil at the 35th day is smaller than that at the 49th day.  This discrepancy is most likely due to sampling variations, although data were not available to establish accuracy limits on the data points.  Meteorological data required for applying PRZM to the site, which include daily precipitation and pan evaporation, were obtained from Smith et al. (1978).

The PRZM-2 input parameters for trifluralin and the Watkinsville site are listed in Table 6-4.  Two additional key parameters which influence the volatilization results are the decay rate and the adsorption partition coefficient.  The magnitude of the decay rate can be estimated from the data in column 5 of Table 6-3, assuming that decay accounts for all losses from the soil other than volatilization.  A value of 0.0206 per day for the first-order decay rate constant obtained from these data points is consistent with the value of 0.0198 per day used by Donigian et al. (1986) after reviewing the literature.  An initial value for Kd was obtained from the organic carbon content of 0.55% and an organic-carbon partition coefficient (Koc) value of 13,700, resulting in a Kd of 75 ml/g.  Figure 6.10 shows the results of sensitivity analyses runs for Kd and the decay rate; the observed data for trifluralin from Table 6-3 are also included for comparison.  Figure 6.10(a) shows a good representation of the observed cumulative volatilization curve.  Figure 6.10(b) shows that a value of 40 for Kd, and a decay rate of 0.02 per day provides the best representation of the decay rate values analyzed.

The simulation results for cumulative volatilization flux and cumulative pesticide decay are shown in Figure 6.11 for four different DELX combinations.  For these simulations, DELX values of 1.0, 0.5, 0.25, and 0.1 cm were chosen for the first horizon and 5-cm DELX for the rest of the profile.  The field data are also included in the figures for comparison.  Table 6-5 shows the total volatilization flux for each of the four combinations using variable DELX, as well as for a simulation using simulations, a constant 1.0-cm DELX throughout the whole soil profile.  The CPU requirements for each run are also included in Table 6-5.  The predicted total volatilization flux using the smallest DELX of 0.1 cm is closest to the field-measured value; the values for DELX of 0.25 cm and 0.50 cm are also quite close to the field value.  The saving of CPU time can be observed from Table 6-5.  The simulation requires 129 seconds using 1.0-cm DELX for the whole soil profile, compared with only 39 seconds for the simulation using 1.0 cm for the top horizon and 5.0 cm for the rest of the profile.  The results in Table 6-5 indicate that a DELX of 0.25 to 0.50 cm for the top horizon may be a reasonable compromise between simulation accuracy and CPU costs.

TABLE 6-3.	TRIFLURALIN VOLATILIZATION LOSSES, AMOUNTS REMAINING IN SOIL, AND ESTIMATED LOSSES VIA OTHER PATHWAYS FOR THE 120-DAY FIELD TEST

Cumulative Volatilized	Remaining*	Estimated

% of Total	% of Total	in Soil,	Other Losses,

Time, (day)	Applied	Applied	% Applied	% of Applied

Application	3.5	13.3	--	--

1	3.8	14.8	89	7.2

2	5.3	20.3	72	22.7

6	10.9	42.2	64	25.1

18	20.5	79.1	51	28.5

35	23.4	90.2	33	43.6

49	24.4	94.1	35	40.6

63	25.1	96.9	23	48.9

76	25.4	98.2	20	54.6

120	25.9	100.0	11	63.1

Source:  White et al. (1977).

*	Based on amount remaining in soil at a 0- to 7.5-cm depth as compared with an initial 1.0 μg/g level at application (rate was 1.12 kg/ha).

TABLE 6-4.	INPUT PARAMETERS FOR THE TEST CASES - WATKINSVILLE SITE

Simulation start date	14 June 1973

Simulation end date	31 December 1973

Trifluralin:	Henry's constant	6.7 x 10-3

Diffusion coefficient in air	0.43 m2 day-1

Application date	15 June 1973

Amount applied	1.12 kg ha-1

Incorporation depth	5 cm

Horizon	Thickness	DELX	Field	Wilting	Initial

(cm)	(cm)	Capacity	Point	Water Content

1	5	0.1	.207	.095	0.166

2	10	5.0	.207	.095	0.217

3	15	5.0	.339	.239	0.318

4	60	5.0	.320	.239	0.394

TABLE 6-5.	SIMULATION RESULTS OF USING DIFFERENT COMPARTMENT DEPTH (DELX)

Constant DELX 	           Variable DELX

Horizon	Depth	DELX	DELX	DELX	DELX	DELX

(cm)	(cm)	(cm)	(cm)	(cm)	(cm)

1	5	1.0	1.0	0.5	0.25	0.1

2	10	1.0	5.0	5.0	5.0	5.0

3	15	1.0	5.0	5.0	5.0	5.0 	Field

4	60	1.0	5.0	5.0	5.0	5.0	Value

Total	0.393	0.398	0.338	0.317	0.316	0.290

Volatilization

Flux (kg/ha)

CPU (Sec)	129	39	46	67	106

Figure 6.12(a) reveals significant differences between the observed pesticide decay and the simulated values during the first few weeks following application.  In fact, the observed data appear to indicate a much higher attenuation rate during the first few days following application, with a lower rate for the remaining period.  To better match the decay characteristics, and evaluate the potential impact on the volatilization simulation, a two-step decay procedure was used with a rate of 0.1 per day for 5 days following application and a rate of 0.01 per day for the remaining period.  The results of these simulations in terms of pesticide remaining in the soil, shown in Figure 6.12, indicate a much better agreement with the observed field values in Figure 6.12(b).  The impact of the two-step decay on both cumulative decay and volatilization flux is shown in Figure 6.13.  The cumulative pesticide decay shown in Figure 6.13(a) improves considerably (compared to Figure 6.11(b)), while the results for cumulative volatilization flux (Figure 2.13(b)) are slightly better than those in Figure 2.11(a).

6.5.2.3  Conclusions from Volatilization Model Testing--

The primary conclusions derived from this preliminary model testing are as follows.

1)	Comparisons with Jury's analytical solution indicate that the volatilization algorithms are operating correctly, and that, with a very small DELX (0.1 cm or less), the results are in excellent agreement.

2)	The preliminary field testing results with trifluralin in Watkinsville, GA, indicate good agreement between measured and predicted volatilization flux when measured decay rates and adjusted KD values are used.

3)	Small soil layer depths--in the range of 0.25 and 0.50 cm--are needed to provide the best presentation of volatilization flux at reasonable CPU times, based on the Watkinsville testing.

4)	A two-step decay rate best represents the attenuation behavior of trifluralin using a higher rate for the period immediately following application and a lower rate for the remaining period.

Further testing of the volatilization model should be performed to evaluate its capabilities for different compounds, different regions, and other crops.  In addition, the vapor transport and concentration calculations for the plant compartment should be tested with the additional data available from the Watkinsville site and from other field data sets (e.g., Grover et al. 1985, Willis et al. 1983).

6.5.3  Testing Results of Soil Temperature Simulation Subroutine

Preliminary testing of the simulation subroutine for the soil profile temperature was performed by comparing predicted values with values obtained by an analytical solution to the governing heat flow equation.  These testing results are discussed in this section.  Testing of the soil surface/upper boundary temperature simulation, estimated by the energy balance procedure in the model, was not performed due to problems in obtaining observed meteorological and soil temperature data for the Watkinsville, GA, test site.

An analytical solution presented in Kreysig (1972) for the classical one-dimensional heat flow partial differential equation (described in Section 6.3.4.4) was used to calculate changes in the soil temperature profile with time, due to a change in the upper boundary temperature.  In order to develop a valid comparison between the analytical and finite difference methods, three assumptions were made:

a)	Uniform properties throughout the soil profile

b)	Constant lower-boundary temperature

c)	Uniform initial temperatures throughout the profile

To compare the results of the analytical solution with the finite difference solution from the soil temperature model, the following parameters were used.

Depth of the soil profile	=	100 cm

Compartment thickness (DELX)	=	1.0 cm

Diffusivity of the soil profile	=	864 cm2 day -1

Upper-boundary temperature, T(o,t)	=	30°C

Lower-boundary temperature, T(L,t)	=	20°C

Initial temperature, T(x,o)	=	20°C

Figures 6.14 and 6.15 show the comparison of soil temperature profiles predicted by both the analytical solution and the finite difference soil temperature model after 1 day and 5 days of simulation.  In Figure 6.14 the finite difference solution is obtained by using a 1-hour time step, while in Figure 6.15 a 1-day time step is used.  The following observations are evident from these testing results.

1)	Comparison of the soil temperature profiles predicted by both methods indicate excellent agreement when the smaller, 1-hour time step is used in the finite difference procedure, as shown in Figure 6.14.

2)	The finite difference solution obtained by using the daily time steps deviates from the analytical solution by about 1°C, in the upper and middle portions of the soil profile (Figure 6.15).  This deviation is due to the assumption of a constant initial temperature profile and the abrupt change in the upper-boundary temperature from 20°C to 30°C for the first daily time step.

3)	As the steady-state condition is approached, irrespective of the time step used in the finite difference solution, the soil temperature profiles predicted by both methods are in good agreement (Figures 6.14(b) and 6.15(b)).

Table 6-6 shows that reducing the depth of the compartment from 1 cm to 0.1 cm does not produce any significant change in the finite difference solution.  These depths bracket the range of values for DELX (i.e., compartment thickness) likely to be used for the surface soil horizon.

These test results show that, for smaller time steps, the finite difference solution will be in complete agreement with the analytical solution.  For a daily time step as used in PRZM-2, under expected environmental conditions, with a non-uniform initial tempera-ture profile, non-uniform soil characteristics, and smaller daily changes in the upper-boundary temperature, the soil temperature profile estimated by the finite difference method used in the model is expected to be capable of providing close agreement with observed temperature profile data.  In addition to further testing of the soil profile temperature model with field data, the procedure to estimate the upper-boundary temperature should be tested to evaluate and demonstrate the validity of the entire soil temperature simulation model.

6.5.4  Testing of Daughter Products Simulation

The fate of pesticides in soils is a complex issue.  Many processes (i.e., volatilization, degradation, etc.) must be considered in order to adequately address this issue.  One of these processes, which has been largely neglected in pesticide leaching models, is that of the transformation of the parent compound to various toxic daughter products.  The tendency has been to lump all the toxic family into a "total toxic residue" and to model the fate of this composite as a single chemical.  This assumption may not be acceptable, especially if the daughters have very different decay rates or adsorption partition coefficients from the parent or from each other.

Algorithms have been included in PRZM-2 to simulate parent/daughter relationships.  An analytical solution to the decay and transformation model was derived to check the numerical model.

TABLE 6-6.	SIMULATED SOIL TEMPERATURE PROFILE AFTER ONE DAY FOR DIFFERENT COMPARTMENT THICKNESSES (TIME STEP = 1 DAY)

Depth (cm)	DELX = 1 cm	DELX = 0.1 cm

0.0	30.000	30.000

1.0	29.665	29.664

2.0	29.341	29.340

3.0	29.028	29.026

4.0	28.725	28.723

5.0	28.432	28.431

10.0	27.109	27.106

20.0	25.048	25.045

30.0	23.577	23.574

40.0	22.524	22.520

50.0	21.766	21.760

60.0	21.215	21.206

75.0	20.638	20.627

99.0	20.023	20.020

100.0	20.000	20.000

The system that was modeled is shown in Figure 6.16.  The Ci are dissolved concen-trations and the C are adsorbed concentrations.  The Ki are adsorption partition coefficients, the kj are decay and transformation rates in the dissolved species, the k are adsorbed phase decay coefficients and θ and ρ are the water content and soil bulk densities, respectively.  Notice that only the dissolved forms may be transformed from one toxic form to another.  A system of first order differential equations describing this system can be written as:

Making use of Ci Ki = Ci* we can reduce the six equations above to three equations in three unknowns, namely:

in which

These ordinary differential equations with constant coefficients can be solved analytically for C1, C2 and C3 using the initial conditions C1 = C when t = 0 and C2 = C3 = 0 at t = 0.  The solutions as given in Dean and Atwood (1985) are:

and

In PRZM-2, the equations are solved numerically as part of the general advection-dispersion equation for a solute in a porous medium by using an implicit scheme.  A new subroutine was added to set up the transformation (source and sink) terms for the system.  The relationship C1 → C2 → C3 may be modeled or the system can be configured for C1 → C2 and C1 → C3 or for independent C1, C2 and C3 simply by selecting zero or positive values for the appropriate transformation rate constants.

Figures 6.17 through 6.18 show the results of a series of tests performed on the numerical model and checked by the analytical model.  In these figures, the solid line represents the "true" or analytical solution, and the dashed line represents the approximate numerical solution.  In Figure 6.17, there was no decay of the dissolved phase chemicals and no adsorption of any species.  The rate of transformation from C1 to C2 was 0.2 day-1 and that from C2 to C3 was 0.5 day-1.  After 20 days nearly all the chemical is in form C3.  The numerical model traces the decay and formation of each constituent closely, being poorer in those regions where the rate of change of the concentrations are more rapid.  Figure 6.18 shows the same system with a decay rate of 0.01 day-1 in the dissolved phase.

Using the analytical model, the assumption of modeling the "total toxic residue" decay as a first-order process was tested. Adsorption coefficients for aldicarb, aldicarb sulfoxide and aldicarb sulfone in a Woburn sandy loam (K1 = 0.55, K2 = 0.16 and K3 = 0.185) and decay and transformation rate constants (k1 = 0.07, k2 = 0.55, k3 = 0.01, k4 = 0.031 and k5 = 0.0152) were taken from Bromilow et al. (1980).  A soil bulk density of 1.45, a water content of 0.27 cm3 cm-3 and an initial aldicarb parent mass of 100 mg were also used.  The model was run for 90 days and the results are shown in Figure 6.19.

The results show that the decay of the sum of the dissolved aldicarb concentrations does not follow first-order kinetics.  The reason for this is the conversion of aldicarb parent to aldicarb sulfoxide.  Because the sulfoxide has a lower partition coefficient, the dissolved concentration increases until most of this conversion is complete.  Once this happens, however, the sum of the sulfoxide and the sulfone concentrations does follow a first-order decay curve.

6.6  Biodegradation Theory and Assumptions

The biodegradation model is based on the:  Mathematical Model for Microbial Degradation of Pesticides in the Soil. Soil Biol. Biochem. V.14 pp.107-115 (Soulas 1982).  The soil is divided into two phases: the solid phase, consisting of the dry soil including the organic matter, and the aqueous phase dispersed within it, consisting of the soil moisture, various organic substrates, and all the biomass.  Some of the organic and inorganic components constituting the solid phase can adsorb the pesticide.  This adsorption is represented as a linear isotherm, instantaneous and without hysteresis.

The microbial population is divided into four groups.  The first two are responsible for the degradation of the pesticide.  These are the metabolizing and co-metabolizing populations.  The former corresponds to normal metabolic utilization, whereas the latter represents that fraction of the microflora which degrades without energy recovery.

The non-degrading population was divided into microorganisms that are sensitive to the lethal action of the chemical and those that are indifferent.

In the original development of the equations, all concentrations were expressed with respect to the soil solution.  Soulas (1982) reports that these concentrations are somewhat theoretical when considering the different biomasses and are not easy to evaluate by experiment.  Thus, all concentrations were expressed with respect to the weight of the moist soil.  For these biomasses, the simple proportionality

e

Xi = Ws Xi;    i = m, c, s, r

was chosen where

Xi = concentration of the Xi population in the moist soil

e

Xi = concentration of the Xi population in the soil solution

and

where

H = weight of the aqueous phase (soil solution)

P = weight of the solid phase (dry soil)

For the metabolizing population, growth is described by

This represents growth at the expense of both the pesticide (S) and the carbon (C) in the soil solution.  The population decreases as a result of a first-order death process with a death rate constant kdm.

For the co-metabolizing population,

This reflects growth only at the expense of soil carbon.  Allowance was also made for possible antagonistic effects by the non-degrading portion of the soil microflora.  These antagonisms were assumed to result only in a reduction of the growth rate of the co-metabolizing population.  Michaelis-Menten kinetics with non-competitive inhibition were used to simulate these conflicts.

For the sensitive population,

This includes a supplementary death term following second-order kinetics.

For the non-sensitive, non-degrading population,

This is the basic relation of growth term and death term.

The equation concerning the pesticide concentration,

has two parts.  The first term concerns the degradation due to the metabolizing population, while the second deals with the action of the co-metabolizing population.The equation for the concentration of carbon in the moist soil,

-is derived from the basis that the concentration is the difference between two reaction rates--the solubilization rate of carbon compounds from solid soil organic matter and the rate of microbial consumption.  It is assumed that soluble carbon in the soil solution is, in first approximation, sufficiently low to be neglected when compared to the saturation constant.

N.B.  There are some minor differences between the equations as developed by Soulas and as reported in his Appendix 3.  In addition, some slight changes were made to the equations to correct what were assumed to be some typographical errors.  These changes include:

Definitions:

Xi	=	Concentration of the Xi population in the moist soil (i = m, c, s, r)*

St	=	Pesticide concentration in the moist soil

Cw	=	Carbon concentration in the moist soil

μi	=	Maximum specific growth rate of the Xi population (i = sm, cm, c, s, r)*

Ki	=	Saturation constant of the Xi population (i = sm, cm, c, s, r)*

kdi	=	Death rate of the Xi population (i = m, c, s, r)*

Yi	=	True growth yield of the Xi population (i = sm, cm, c, s, r)*

k1	=	Second-order death rate of the Xs population

k2	=	Dissociation constant of the enzyme-substrate complex

Kin	=	Inhibition constant

In addition,

where

Kd = distribution coefficient

and

with

H = weight of soil solution (aqueous phase)

P = weight of dry soil (solid phase)

These equations are to be solved simultaneously, and the results used to determine the amount of pesticide in the soil that is degraded biologically over the timestep interval.

These equations are solved in PRZM-2 using a fourth-order Runge-Kutta method.

This subprogram uses the carbon concentration and the pesticide concentration in the moist soil of each compartment as input.  Using the populations of organisms in each compartment, which is saved between calls, the subprogram solves the degradation algorithm to determine the new pesticide amount, and thus the amount degraded, over the PRZM-2 time step.  Also, the changes to the organism populations are calculated and saved for use in the subsequent timestep.

SECTION 7

VADOSE ZONE FLOW AND TRANSPORT MODEL (VADOFT)

CODE AND THEORY

7.1  INTRODUCTION

VADOFT is a finite-element code for simulating moisture movement and solute transport in the vadose zone.  It is the second part of the two-component PRZM-2 model for predicting the movement of pesticides within and below the plant root zone and assessing consequent groundwater contamination.  The VADOFT code simulates one-dimensional, single-phase moisture movement in unconfined, variably saturated porous media.  The code considers only single-porosity media and also ignores the effects of hysteresis.  Transport of dissolved contaminants may also be simulated within the same domain.  Transport processes accounted for include hydrodynamic dispersion, advection, linear equilibrium sorption, and first-order decay.  VADOFT also simulates solute transformations in order to account for parent/daughter relationships.

7.2  OVERVIEW OF VADOFT

7.2.1  Features

7.2.1.1  General Description--

The VADOFT code can be used to perform one-dimensional modeling of water flow and transport of dissolved contaminants in variably or fully saturated soil/aquifer systems.  VADOFT can be operated as a stand-alone code or operated in conjunction with the root zone model, PRZM.  In the latter case, boundary conditions at the interfaces of the modeled domains are established via model linkage procedures.

7.2.1.2  Process and Geometry--

VADOFT performs one-dimensional transient or steady-state simulations of water flow and solute transport in variably saturated porous media.  The code employs the Galerkin finite-element technique to approximate the governing equations for flow and transport.  It allows for a wide range of nonlinear flow conditions, and handles various transport processes, including hydrodynamic dispersion, advection, linear equilibrium sorption, and first-order decay.  Steady-state transport can not be simulated when decay is considered.  Boundary conditions of the variably saturated flow problems are specified in terms of prescribed pressure head or prescribed volumetric water flux per unit area.  Boundary conditions of the solute transport problem are specified in terms of prescribed concentration or prescribed solute mass flux per unit area.  All boundary conditions may be time dependent.

7.2.1.3  Assumptions--

The VADOFT code contains both flow and solute transport models.  Major assumptions of the flow model are:

●	Flow of the fluid phase is one-dimensional and considered isothermal and governed by Darcy's law.

●	The fluid considered is slightly compressible and homogeneous.

●	Hysteresis effects in the constitutive relationships of relative permeability versus water saturation, and water saturation versus capillary pressure head, are assumed to be negligible.

Major assumptions of the solute transport model are:

●	Advection and dispersion are one-dimensional.

●	Fluid properties are independent of concentrations of contaminants.

●	Diffusive/dispersive transport in the porous-medium system is governed by Fick's law.  The hydrodynamic dispersion coefficient is defined as the sum of the coefficients of mechanical dispersion and molecular diffusion.

●	Adsorption and decay of the solute may be described by a linear equilibrium isotherm and a first-order decay constant.

●	Vapor transport can be neglected.

7.2.1.4  Data Requirements--

Data required for the simulation of variably saturated flow include values of the saturated hydraulic conductivity and specific storage of the porous media, the geometry and configuration of the flow region, as well as initial and boundary conditions associated with the flow equation.  Soil moisture relationships are also required.  These include relative permeability versus water phase saturation and capillary head versus water phase saturation.  These relationships may be supplied to the code using tabulated data or functional parameters.

Data required for the simulation of solute transport in variably saturated soil include dispersivity and porosity values, retardation and decay constants, Darcy velocity and water saturation values, as well as initial and boundary conditions associated with the transport equation.

7.2.2  Limitations

Major limitations of the VADOFT code are:

●	In performing a variably saturated flow analysis, the code handles only single-phase flow (i.e., water) and ignores the flow of a second phase (i.e., air) which, in some instances, can be significant.

●	The code ignores the effects of hysteresis on the soil moisture constitutive relations.

●	The code does not take into account sorption nonlinearity or kinetic sorption effects which, in some instances, can be important.

●	The code considers only single-porosity (granular) soil media.  It cannot handle fractured porous media or structured soils.

●	The code does not take into account transverse dispersion, which can be important for layered media.

7.3  DESCRIPTION OF FLOW MODULE

7.3.1  Flow Equation

VADOFT considers the problem of variably saturated flow in a soil column in the vadose zone of an unconfined aquifer.  The code solves the Richards' equation, the governing equation for infiltration of water in the vadose zone:

where

ψ	=	the pressure head (L)

K	=	the saturated hydraulic conductivity (LT-1)

krw	=	the relative permeability

z	=	the vertical coordinate pointing in the downward direction (L)

t	=	time (T)

η	=	an effective water storage capacity (L-1) defined as:

where

Ss	=	specific storage (L-1), Sw is water saturation

ϕ	=	the effective porosity.

Specific storage is defined by

Ss = ρg[ϕcf + (1 - ϕ) cs]	(7-2b)

where

cf	=	the fluid compressibility (LT2M-1)

cs	=	the solid skeleton compressibility (LT2M-1)

ρ	=	the fluid density ML-3), and g the gravitational acceleration (LT-2)

The initial and boundary conditions of the one-dimensional infiltration problem may be expressed as:

ψ(z,0) = ψi 	(7-3)

either

V(0,t) = I	(7-4a)

or

ψ(0,t) =  ψo	(7-4b)

either

ψ(L,t) = ψL	(7-5a)

or

V(L,t) = 0	(7-5b)

where

ψi	=	the initial pressure head value (L)

ψo	=	the pressure head at the upper boundary (L)

ψL	=	the pressure head at the lower boundary (L)

I	=	the rate of infiltration at the soil surface (LT-1)

L	=	the thickness of the vadose zone (L)

V 	=	the vertical Darcy velocity (LT-1) (defined by Equation 7-8).

The boundary condition in Equation 7-5b is valid because the bottom boundary of VADOFT allows fluid to exit.

To solve the variably saturated infiltration problem, it is also necessary to specify the relationships of relative permeability versus water saturation and pressure head versus water saturation.  Two alternative function expressions are used to describe the relationship of relative permeability versus water saturation.  These functions are given by Brooks and Corey (1966) and by van Genuchten (1976):

krw = Sen	(7-6a)

and

krw = Se1/2 [1 - (1 - Se1/γ)γ]2	(7-6b)

where

n and γ =  empirical parameters

Se	=	the effective water saturation defined as Se = (Sw - Swr)/(1 - Swr), with Swr being referred to as the residual water saturation.

The relationship of pressure head versus water saturation is described by the following function (van Genuchten 1976, Mualum 1976):

where

α and β =  empirical parameters

ψa	=	the air entry pressure head value (L)

Swr	=	the residual water phase saturation.

The parameters β and γ are related by γ = 1 - 1/β.

Descriptive statistical values for α, β, and γ have been determined by Carsel and Parrish (1987) for 12 soil classifications (see section 5).  Using the mean parameter values, the relationships of effective saturation versus capillary head and relative permeability versus effective saturation are plotted.  Logarithmic plots are shown in Figures 7.1 through 7.3.  To show more vividly the high degree of nonlinearities, the relationships of relative permeability versus effective saturation are also plotted on arithmetic scales and presented in Figures 7.4 through 7.6.  It is important that the finite element flow module be capable of handling such high nonlinearities to be successful in performing a Monte Carlo study of infiltration in the unsaturated zone.

Equation 7-1 is solved using the Galerkin finite element subject to the initial and boundary conditions given in Equations 7-3 through 7-5.  After the distributions of ψ and Sw have been determined, the Darcy velocity is computed from

∂ψ

V = - Kkrw(── - 1)	(7-8)

∂z

7.3.2  Numerical Solution

7.3.2.1  Numerical Approximation of the Flow Equation--

A numerical approximation of the one-dimensional flow equation in the vadose zone is obtained using a Galerkin finite-element formulation with spatial discretization performed using linear elements.  Time integration is performed using a backward finite difference approximation.  This leads to a system of nonlinear algebraic equations.  For a typical node i in the finite-element grid (see Figure 7.7), the equation may be expressed as

Figure 7.1

Figure 7.2

Figure 7.3

Figure 7.4

Figure 7.5

Figure 7.6

Figure 7.7

αiψk+1i-1 + βiψik+1 + γiψk+1i+1 = di	(7-9)

where k+1 is the current time level, and αi, βi, γi, and di are given by

and Δzi and Δtk are the spatial and time increments, respectively.  Note that the braces ({}) are used in the equations above (and below) to denote the value of the enclosed quantity at the element centroid.  The nonlinear system of equations is solved for each time step.  Three nonlinear schemes are provided in the VADOFT code.  The first scheme is a Picard type iteration scheme, the second scheme is a Newton-Raphson scheme, and the third is a Newton-Raphson scheme modified by Huyakorn (1988, personal communication).

In the Picard scheme, the matrix coefficients, αi, βi, γi, and di, are first evaluated using an initial  estimate  of  pressure  head  values, ψ.  The resulting system of linearized equations is then solved for ψ+1 using the Thomas algorithm.  Updating of the matrix coefficient is performed by recomputing values of nonlinear soil parameters.  Iterations are performed until the successive change in pressure head values is within a prescribed tolerance.

In the Newton-Raphson scheme, the nonlinear system of equations is treated by applying the Newton-Raphson technique (see Huyakorn and Pinder 1983, pp. 159-162) to Equation 7-9.  This leads to the following system of linearized algebraic equations.

(αi + α)r ψk+1i-1 + (βi + β)r ψik+1

+ (γi + γ)r ψk+1i+1 = (di + d)r	(7-11)

where superscript r is used to denote the r-th iterate; αi, βi, γi, and di are as defined previously; and α, β, γ, and d are given by

[Δzi-1 + Δzi]

where li = ─────────────

2

The initial solution and subsequent iterations of the Newton-Raphson scheme are performed in the same manner as that described for the Picard scheme.

7.3.2.2  General Guidance on Selection of Grid Spacings and Time Steps, and

the Use of Solution Algorithms--

In designing a finite-element grid for variably saturated flow simulations, one should select nodal spacings that will yield reasonable approximations to the expected moisture profiles.

In the analysis of the given variably saturated flow problem, small nodal spacings should be used in the zones where head gradients or moisture fronts are steep.  The nodal spacings may be gradually increased in the zone where no abrupt changes in hydraulic conductivities occur and the head gradients are gradually sloping.  The variably saturated flow simulation can be performed using either the Picard algorithm or one of the Newton-Raphson solution algorithms.  For one-dimensional cases where convergence difficulties are not expected, the efficiencies of these algorithms have been found to be similar.  For certain steady-state cases involving highly nonlinear soil moisture characteristics, the use of either of the Newton-Raphson algorithms is preferable, particularly when the Picard algorithm fails to converge within a reasonable number of iterations (say between 10 and 20).

7.4  DESCRIPTION OF THE TRANSPORT MODULE

7.4.1  Transport Equation

The governing equation for one-dimensional transport of a nonconservative solute species in a variably saturated soil takes the form

∂       ∂c       ∂c          ∂c

── (D──) - V── = θR(── + λc)	(7-13)

∂z       ∂z       ∂z          ∂t

where D is the apparent dispersion coefficient (L2T-1), c is the solute concentration (ML-3), θ is the volumetric water content (θ = ϕSw), R is the retardation coefficient, and λ is the first-order decay constant (T-1).  Note that the apparent dispersion coefficient is defined as

D = αLV + ϕD*, where αL is the longitudinal dispersivity, and D* is the effective molecular diffusion coefficient.

The initial and boundary conditions of the one-dimensional transport problem may be expressed as:

c(z,0) = ci	(7-14)

either

∂c

-D ── (0,t) = V (co - c)	(7-15a)

∂z

or

c(0,t) = co	(7-15b)

∂c

── (L,t) = 0	(7-16)

∂z

where ci is the initial concentration (ML-3), and co is the leachate concentration at the source (ML-3).

7.4.2  Numerical Solution of the Transport Equation

7.4.2.1  Numerical Approximation of the Transport Equation--

A numerical approximation of the one-dimensional transport equation is obtained using an upstream-weighted finite-element formulation with spatial discretization performed using linear elements.  Time integration is performed using a central finite-difference approximation.  This leads to a system of linear algebraic equations.  The equation corresponding to node i takes the form:

αi ck+1i-1 + βi ck+1i   + γi ck+1i+1 = di	(7-17)

where αi, βi, γi, and di are given by

αi = τα + {θR}i-1 Δzi-1/(6Δtk)	(7-18a)

βi = τβ + [{θR}i Δzi + {θR}i-1 Δzi-1]/(3Δtk)	(7-18b)

γi = τγ + {θR}i Δzi/(6Δtk)	(7-18c)

di = (τ - 1)(α c-1 + β c + γ c+1)

{θR}i-1                            {θR}i

+ ───────(c-1 + 2c)Δzi-1 + ─────(c+1 + 2c)Δzi	(7-18d)

6Δtk                              6Δtk

{D}i-1           (1 - ω)                λ{θR}i-1 Δzi-1

α = - ────── - ─────── {V}i-1 + ──────────────	(7-18e)

Δzi-1               2                         6

{D}i-1       {D}i       λ

β = (────── + ────) + ─ [{θR}i-1 Δzi-1 + {θR}iΔzi]

Δzi-1        Δzi        3

(1 - ω)                  (1 + ω)

+ ─────── {V}i-1 - ─────── {V}i	(7-18f)

2                         2

{D}i       (1 + ω)                λ{θR}i Δzi

γ = - ──── + ─────── {V}i + ──────────	(7-18e)

Δzi           2                        6

with τ and ω denoting the time weighting factor and the upstream weighting factor, respectively.

To obtain a second-order temporal approximation, the value of τ is set equal to 1/2.  This corresponds to using the Crank-Nicholson central difference time stepping scheme.  The upstream weighting factor ω is introduced in the above numerical approximation to curb numerical oscillations that may occur when the selected finite-element grid is not sufficiently refined for a given value of longitudinal dispersivity.  For each time step, the linear system of algebraic equations is solved using the Thomas algorithm.

Transport of a daughter species in a decay chain can also be handled by the VADOFT code.  In this case, the right side of the governing equation for single species transport (7-13) is modified by adding a source term accounting for transformation of parent components.  This source term is given by

np

m = -Σ ϕ Sw εℓ λℓ Rℓ cℓ	(7-19)

ℓ=1

where

subscript ℓ 	=	the parent species

np	=	the number of parent species

εℓ	=	the mass fraction of parent component that is transformed into the daughter species under consideration

The numerical solution of the modified transport equation can be performed in the same manner as that described previously for a single species.  The source term from Equation 7-19 is incorporated into the finite element matrix equation by adding d to the right side.  The term d is given by

1    np

d = ─ [ Σ εℓ λℓ Rℓ cℓk+1i  ] (Δzi + Δzi-1)(ϕ Sw)i	(7-20)

2   ℓ=1

In performing the solute transport analysis, the selection of nodal spacing

(Δz) and time step value (Δt) should follow the so-called Peclet number and Courant number criteria where possible.  These two criteria are given as follows.

Δz

── ≤ 4	(7-21)

αL

Vsol Δt/Δz ≤ 1	(7-22)

Vsol = V/θR	(7-23)

where

αL	=	the longitudinal dispersivity

Vsol	=	the solute velocity

V 	= 	Darcy velocity

θ 	=	water content

R 	=	retardation coefficient

The VADOFT code also provides the user with the option of using upstream weighting to curb numerical oscillations that may occur in solving the advective-dispersive transport equation.  The recommended value of ω, the weighing factor, is determined by using the following formulae:

ω = 1 - 4αL/ℓ,      ℓ > 4αL	(7-24)

ω = 0,              ℓ ≤ 4αL	(7-25)

where

αL	=	the longitudinal dispersivity

ℓ	=	the length of the element.

7.5  RESULTS OF VADOFT TESTING SIMULATIONS

Three sets of benchmark problems were used to test the VADOFT code.  The first set consists of two steady and transient problems designed to test the variably saturated flow component of the code.  The second set consists of four transient one-dimensional transport problems.  The third set consists of two coupled flow-transport problems.  Numerical results obtained from VADOFT are compared with analytical solutions and results obtained using two other finite-element codes, UNSAT2 and SATURN.  These test problems were simulated using VADOFT before it was linked in PRZM-2.

7.5.1  Flow Module (Variably Saturated Flow Problems)

7.5.1.1  Transient Upward Flow in a Soil Column--

This problem concerns transient, vertically upward moisture movement in a 20 cm long soil column.  The soil column is subject to zero pressure head at the base and zero flux at the top.  The initial distribution of pressure head is hydrostatic:  (t = 0) = -90 + z cm, where z is the depth below the top of the soil column.  Soil properties and discretization data used in the simulation are presented in Table 7-1.  The simulation was performed for 15 time steps with constant time step value of t = 0.01 d.  Numerical results given by the Picard and the Newton-Raphson schemes are virtually identical.  Both schemes require between 2 and 3 iterations per time step to converge to a head tolerance of 0.01 cm.  The simulation results obtained from VADOFT are compared with those obtained from UNSAT2 and SATURN (the two-dimensional finite-element codes described by Davis and Neuman [1983], and Huyakorn et al. [1984]) respectively.  Shown in Figures 7.8 and 7.9 are plots of distributions of pressure head and water saturation, respectively.  As can be seen, the results of VADOFT are in good agreement with the results of the other two codes.

7.5.1.2  Steady Infiltration in a Soil Column--

This problem concerns steady-state infiltration in a soil column.  The column is 550 cm in length and is subject to an infiltration rate of 4.07 cm day-1 at the top and zero pressure head at the bottom.  Soil properties used in the simulation are presented in Table 7-2.  Five cases of varying degree of nonlinearity of relative permeability function (krw = Sen) were simulated.  Both the Picard and the Newton-Raphson schemes were used in conjunction with a finite-element grid having constant nodal spacing, z = 10 cm.  The performance of the two iterative schemes are illustrated in Table 7-3.  Note that the Newton-Raphson scheme converges for all cases, whereas the Picard scheme fails to converge when the nonlinear exponent n exceeds 4.  Simulated distributions of pressure head and water saturation are shown in Figure 7.10 and 7.11, respectively.  These results of the VADOFT code are virtually identical to corresponding results obtained using the SATURN code.

7.5.2  Transport Module

7.5.2.1  Transport in a Semi-Infinite Soil Column--

This problem concerns one-dimensional transport of a conservative solute species in a saturated soil column of infinite length.  The solute is introduced into the column at the inlet section where z = 0.  The initial concentration is assumed to be zero, and the dimensionless constant inlet concentration is prescribed as 1.  Values of physical parameters and discretization data used in the numerical simulation are given in Table 3-4.  The finite-element grid representing the soil column was 400 cm in length.  The simulation was performed for 20 time steps.  Thus the duration of the simulation time of transport in the soil column was 50 hours.  For this duration, the selected grid length is sufficient to avoid the end boundary effect.  The numerical solution obtained from the VADOFT code was checked against the analytical solution of Ogata and Banks (1961).  Shown in Figure 7.12 and Table 7-5 are concentration values at t = 25 hours and t = 50 hours.  As can be seen, the numerical and analytical solutions are in excellent agreement.

7.5.2.2  Transport in a Finite Soil Column--

In this problem, downward vertical transport of dissolved contaminants in a soil column above the water table of an unconfined aquifer is considered.  The length of the soil column is 20 m and the Darcy velocity and water content are assumed to be constant and equal to 0.25 m day-1 and 0.25, respectively.  The initial concentration is zero, and water with dimensionless solute concentration of 1 enters the soil surface at a rate of 0.25 m day-1.  At the water table, a zero dispersive-flux boundary condition is assumed.  A list of physical parameter values and discretization data used in the simulation is provided in Table 7-6.  Two cases involving conservative and nonconservative species were simulated.  Results obtained from the VADOFT code are compared in Figure 7.13 and Table 7-7 with the analytical solution given by van Genuchten and Alves (1982).  There is excellent agreement between the numerical and analytical solutions for both cases.

7.5.2.3  Transport in a Layered Soil Column--

This problem concerns one-dimensional transport of a conservative solute species in a soil column consisting of three layers.  The initial concentration in the soil column is assumed to be zero, and the two boundary conditions prescribed are a unit concentration at the top and a zero dispersive flux boundary condition at the bottom.  A list of physical parameter values and discretization data used in the simulation is provided in Table 7-8.  Two cases corresponding to those considered by Shamir and Harleman (1967) were simulated.  Both cases have contrasting longitudinal dispersivity values among the three layers.  The dispersi-vity values of the second case are ten times those of the first case for the same layers.  The intention here is to test the numerical scheme used in the VADOFT code, as well as to check the validity of an approximate analytical solution presented by Shamir and Harleman (1967) and Hadermann (1980).  It should be noted here that the approxi-mate solutions by Shamir and Harleman (1967) and Hadermann (1980) are valid only for relatively small values of dispersivity.  Therefore, for a small dispersivity value, the solutions can be employed to verify the VADOFT code.  Then with appropriate discretization, the VADOFT code could be used to determine the validity of the analytical solutions at large dispersivity values.

TABLE 7-1.	SOIL PROPERTIES AND DISCRETIZATION DATA USED IN SIMULATING TRANSIENT FLOW IN A SOIL COLUMN

Parameter	Value

Length of soil column, L	20 cm

Saturated hydraulic conductivity, K	10 cm d-1

Porosity, ϕ			0.45

Residual water phase saturation, Swr	0.333

Air entry value, ψa	0.0 cm

Constitutive relations:

krw = (Sw - Swr)/(1 - Swr)

(ψ - ψa)/(ψr - ψa) = (1 - Sw/(1 - Swr)

where ψr = -100 cm.

Δz = 0.5 cm

Δt = 0.01 d

TABLE 7-2.	SOIL PROPERTIES USED IN SIMULATING STEADY-STATE

INFILTRATION

Parameter	Value

Length of soil column, L	550 cm

Saturated hydraulic conductivity, K	25 cm d-1

Porosity, ϕ			0.331

Residual water saturation, Swr	0.0

Air entry value, ψa	0.0 cm

Constitutive relations:

krw = Sen, n = 3, 4, 6, 8, 10

1

Se  = ────────────────

[1 + (α ψ - ψa )β]γ

where Se  = (Sw - Swr)/(1 - Swr), α = 0.014 cm-1, ψa = 0 cm,

β   = 1.51, γ = 0.338

Figure 7.8

Figure 7.9

TABLE 7-3.	ITERATIVE PROCEDURE PERFORMANCE COMPARISON

Number of Nonlinear Iterations

Newton-

Case	Raphson	Picard

n =  3	12	33

n =  4	13	56

n =  6	19	n.c.*

n =  8	27	n.c.

n = 10	31	n.c.

* No convergence.  Head tolerance = 0.0001 cm.  Grid spacing z = 10 cm.

TABLE 7-4.	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA USED IN SIMULATING ONE-DIMENSIONAL TRANSPORT IN A SEMI-INFINITE SOIL COLUMN

Parameter	Value

Darcy velocity, V	   1 cm hr-1

Porosity, ϕ  	0.25

Longitudinal dispersivity, αL 	   5 cm

Concentration at the source, co 	   1

Δz = 10 cm

Δt = 2.5 hr

Figure 7.10

Figure 7.11

Figure 7.12

TABLE 7-5.	CONCENTRATION PROFILE CURVES AT t = 25 hr AND t = 50 hr

SHOWING COMPARISON OF THE ANALYTICAL SOLUTION AND

RESULTS FROM VADOFT

Concentration Values

z		          t = 25 hr            	            t = 50 hr

Distance	Analytical	VADOFT	Analytical	VADOFT

(cm)

00.0	1.0000	1.0000	1.0000	1.0000

10.0	0.9997	0.9998	1.0000	1.0000

20.0	0.9983	0.9987	1.0000	1.0000

30.0	0.9945	0.9954	1.0000	1.0000

40.0	0.9854	0.9870	0.9999	1.0000

50.0	0.9662	0.9688	0.9999	0.9999

60.0	0.9313	0.9346	0.9996	0.9997

70.0	0.8745	0.8781	0.9991	0.9994

80.0	0.7924	0.7956	0.9981	0.9985

90.0	0.6858	0.6889	0.9960	0.9967

100.0	0.5619	0.5660	0.9921	0.9933

110.0	0.4321	0.4394	0.9854	0.9871

120.0	0.3099	0.3222	0.9743	0.9767

130.0	0.2060	0.2235	0.9570	0.9599

140.0	0.1264	0.1474	0.9313	0.9348

150.0	0.0713	0.0928	0.8953	0.8991

160.0	0.0369	0.0560	0.8475	0.8513

170.0	0.0175	0.0327	0.7872	0.7908

180.0	0.0075	0.0184	0.7151	0.7186

190.0	0.0030	0.0101	0.6331	0.6368

200.0	0.0011	0.0054	0.5447	0.5491

210.0	0.0003	0.0029	0.4541	0.4598

220.0	0.0000	0.0015	0.3660	0.3736

230.0			0.2845	0.2942

240.0			0.2129	0.2246

250.0			0.1532	0.1662

260.0			0.1058	0.1193

270.0			0.0701	0.0831

280.0			0.0444	0.0563

290.0			0.0270	0.0371

300.0			0.0157	0.0239

310.0			0.0087	0.0150

320.0			0.0046	0.0092

330.0			0.0000	0.0055

TABLE 7-6.	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA

USED IN SIMULATING ONE-DIMENSIONAL TRANSPORT IN A

FINITE SOIL COLUMN

Parameter	Value

Thickness of soil column, L	  20 m

Darcy velocity, V	0.25 m d-1

Water content, θ                 	0.25

Retardation coefficient, R	   1

Longitudinal dispersivity, αL 	   4 m

Source leachate concentration, co	   1

Case 1:

Decay constant, λ                  	   0 d-1

Case 2:

Decay constant, λ                  	0.25 d-1

Δz = 1.0 m

Δt = 0.5 d

Figure 7.13

TABLE 7-7.	CONCENTRATION PROFILE CURVES SHOWING COMPARISON OF THE ANALYTICAL SOLUTION AND VADOFT

Case 1:  λ = 0 d-1

Distance		          t = 5 d         		           t = 10 d        		           t = 20 d

z, (m)	Analytical	VADOFT	Analytical	VADOFT	Analytical	VADOFT

0.0	0.764	0.751	0.884	0.878	0.963	0.961

2.0	0.638	0.624	0.820	0.812	0.942	0.939

4.0	0.502	0.489	0.742	0.733	0.914	0.911

6.0	0.371	0.360	0.655	0.645	0.881	0.877

8.0	0.256	0.247	0.561	0.552	0.841	0.837

10.0	0.164	0.158	0.466	0.457	0.796	0.791

12.0	0.097	0.094	0.375	0.367	0.748	0.742

14.0	0 053	0.052	0.293	0.286	0.698	0.692

16.0	0.027	0.027	0.224	0.219	0.652	0.646

18.0	0.013	0.014	0.176	0.171	0.617	0.610

20.0	0.009	0.009	0.157	0.152	0.602	0.595

Case 1:  λ = 0 d-1

Distance		          t = 5 d         		           t = 10 d        		           t = 20 d

z, (m)	Analytical	VADOFT	Analytical	VADOFT	Analytical	VADOFT

0.0	0.593	0.588	0.615	0.613	0.618	0.617

2.0	0.416	0.411	0.449	0.447	0.453	0.452

4.0	0.283	0.279	0.326	0.325	0.333	0.332

6.0	0.186	0.182	0.236	0.234	0.244	0.243

8.0	0.116	0.113	0.169	0.167	0.179	0.178

10.0	0.069	0.067	0.119	0.118	0.131	0.131

12.0	0.038	0.037	0.083	0.083	0.096	0.096

14.0	0.020	0.019	0.057	0.057	0.071	0.071

16.0	0.009	0.009	0.039	0.039	0.053	0.053

18.0	0.004	0.004	0.028	0.028	0.042	0.042

20.0	0.002	0.002	0.024	0.024	0.038	0.038

Using the discretization data given in Table 7-8, the VADOFT code was run for 180 time steps.  Simulated breakthrough curves at the bottom end of the column (z = 86.1 cm) are presented in Figures 7.14 and 7.15 and in Tables 7-9 and 7-10.  As can be seen, the numerical solution of the VADOFT code compares very well with the analytical solution for case 1:  The small dispersivity case, where the analytical assumption of infinite ratio of layer thickness to layer dispersivity--i.e., each layer extends to infinity--is fairly accurate.  There is a slight discrepancy of the analytical solution from the numerical solution for case 2, where the analytical assumption is less accurate.

7.5.3  Combined Nonlinear Flow and Transport Modules

7.5.3.1  Transport During Absorption of Water in a Soil Tube--

This problem is selected to provide simultaneous testing of the flow and the transport modules of VADOFT.  The problem is depicted schematically in Figure 7.16.  A conservative solute species has a uniform initial concentration and moisture content.  The initial con-centration is assumed to be zero, and the inlet concentration co is assumed to be 1 ppm.  The solute is transported by dispersion and advection.  Note that the solute front and the wetting front advance at different rates.  The solute velocity, Vsol, was previously defined as Equa-tion 7-23.  The velocity of the wetting front is dependent upon the rate of water sorption into the soil, which is dependent on moisture diffusivity; thus, calculation of the wetting front velocity requires integration of the mass balance equation.  For the sake of convenience, all physical data pertaining to the geometry of the soil tube and the physical parameter values are kept the same as those used in the paper by Huyakorn et al. (1985).  The complete set of data is listed in Table 7-11.  The simulation was performed in two stages.  In the first stage, the transient water flow problem was analyzed to determine the distributions of Darcy velocity and water saturation for each time level.  These results are written on an output file.  In the second stage, the transient solute transport problem was analyzed to determine con-centration distributions using the velocity and water saturation data file obtained from the flow simulation.

The spatial and temporal discretization data used in running the VADOFT code are also given in Table 7-11.  Both the flow and the transport analyses were performed for 50 time steps.  Results of the flow analysis are plotted in Figure 7.17.  The water saturation profiles given by VADOFT compare well with those obtained using the semi-analytical solution of Phillip (1955) and the UNSAT2 finite-element flow code.  Results of the transport analysis are plotted in Figure 7.18.  The concentration distributions given by VADOFT also compare well with those obtained using the semi-analytical solution of Smiles et al. (1978) and the FEMWASTE finite-element transport code documented by Yeh and Ward (1981).

7.5.3.2  Transient Infiltration and Contaminant Transport in the Vadose Zone--

This problem, schematically depicted in Figure 7.19, involves variable infiltration and contaminant transport in a layered system in which layer permeabilities differ by more than two orders of magnitude.  The problem was chosen to demonstrate the capability of VADOFT to handle a higher nonlinear situation involving soil materials with sharp contrast in drainage properties.  Shown in Table 7-12 are values of physical parameters and discretization data used in the flow and transport simulations.  For the unsaturated flow simulation, the transient infiltration rates illustrated in Figure 7.20 were used.

Figure 7.14

Figure 7.15

TABLE 7-8.	VALUES OF PHYSICAL PARAMETERS USED IN THE SIMULATION OF TRANSPORT IN A LAYERED SOIL COLUMN

Value for Layer i

Parameter	Layer 1		Layer 2	Layer 3

Layer thickness, ℓi	25.48	30.31	30.31 cm

Seepage velocity, ui	0.127	0.123	0.121 cm s-1

Retardation coeff., Ri	  1.0	  1.0	  1.0

Decay constant, λi	    0	    0	    0 s-1

Source concentration, co	  1.0

Case 1:

Dispersivity, αLi	0.076	0.174	0.436 cm

Case 2:

Dispersivity, αLi	 0.76	 1.74	 4.36 cm

Δz = 0.6888 cm

Δt = 5 s

TABLE 7-9.	BREAKTHROUGH CURVES (at z = 86.1 cm) COMPUTED USING

THE ANALYTICAL SOLUTION AND VADOFT (CASE 1)

Concentration Values for Case 1

Time, t (s)	Analytical Solution	Numerical VADOFT

600	0.0204	0.0262

610	0.0361	0.0427

620	0.0596	0.0665

630	0.0923	0.0989

640	0.1354	0.1410

650	0.1887	0.1930

660	0.2514	0.2543

670	0.3217	0.3234

680	0.3971	0.3981

690	0.4748	0.4755

700	0.5518	0.5526

710	0.6255	0.6266

720	0.6935	0.6951

730	0.7544	0.7564

740	0.8072	0.8096

750	0.8517	0.8542

760	0.8881	0.8907

770	0.9172	0.9197

780	0.9400	0.9421

790	0.9573	0.9590

800	0.9704	0.9715

810	0.9800	0.9805

820	0.9870	0.9869

830	0.9919	0.9913

840	0.9950	0.9943

850	0.9970	0.9964

TABLE 7-10.	BREAKTHROUGH CURVES (at z = 86.1 cm) COMPUTED USING THE

ANALYTICAL SOLUTION AND VADOFT (CASE 2)

Concentration Values for Case 2

Time, t (s)	Analytical Solution	Numerical VADOFT

600	0.303	0.310

610	0.330	0.337

620	0.357	0.365

630	0.384	0.394

640	0.412	0.422

650	0.439	0.450

660	0.466	0.478

670	0.493	0.505

680	0.519	0.532

690	0.544	0.558

700	0.569	0.584

710	0.593	0.608

720	0.617	0.632

730	0.639	0.655

740	0.661	0.677

750	0.681	0.698

760	0.701	0.718

770	0.720	0.737

780	0.738	0.755

790	0.755	0.772

800	0.771	0.788

810	0.787	0.804

820	0.801	0.818

830	0.815	0.831

840	0.828	0.844

850	0.840	0.856

900	0.889	0.904

TABLE 7-11.	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA USED IN SIMULATING TRANSPORT IN A VARIABLY SATURATED SOIL TUBE

Parameter	    Value

Length of soil column, L	    20 cm

Saturated hydraulic conductivity, K	     1 cm d-1

Initial pressure head, ψi	-83.33 cm

Remaining flow parameters	   See Table 3-2

Initial concentration, ci	     0 ppm

Longitudinal dispersivity, αL	     0 cm

Molecular diffusion, D*	     1 cm2 d-1

Decay constant, λ	     0 d-1

Retardation coefficient, R	     1

Δz = 0.25 cm

Δt = 0.0025 d

TABLE 7-12.	VALUES OF PHYSICAL PARAMETERS AND DISCRETIZATION DATA

USED IN SIMULATING TRANSIENT INFILTRATION AND

CONTAMINANT TRANSPORT IN THE VADOSE ZONE

Material 1	Material 2

Property	(Sand)	(clay loam)

Saturated conductivity, K	713	6.24	 cm d-1

Porosity, ϕ	0.43	0.41

Residual Water Saturation, Swr	0.105	0.232

Air entry value, ψa	0.0	0.0 	cm

Soil moisture parameter, α	0.145	0.019	cm-1

Soil moisture parameter, β	2.68	1.31

Soil moisture parameter, γ	0.63	0.24

Longitudinal dispersivity, αL	1.0	1.0 	cm

Retardation coefficient, R	1.1	1.5

Decay coefficient, λ	0.00274	0.00274 d-1

Δz = 4 cm

Δt = 1 d

Figure 7.16

Figure 7.17

Figure 7.18

Figure 7.19

Figure 7.20

Figure 7.21

Figure 7.22

Figure 7.23

Figure 7.24

SECTION 8

UNCERTAINTY PREPROCESSOR

8.1  INTRODUCTION

In recent years, the use of quantitative models to assess the transport and transformation of contaminants in the environment has increased significantly.  Typically these models include a set of algorithms that simulate the fate of a contaminant within a medium (e.g., unsaturated zone, saturated porous media, air or a surface water body) based on a number of user-specified parameters.  These parameters describe the properties of the chemical, the transport medium, and the effects that man has on the system.

Unfortunately, the values of these parameters are not known exactly due to measurement errors and/or inherent spatial and temporal variability.  Therefore, it is often more appropriate to express their value in terms of a probability distribution rather than a single deterministic value and to use an uncertainty propagation model to assess the effect of this variability on the transport and transformation of the contaminant.

This section describes the Monte Carlo method of uncertainty propagation and a Monte Carlo shell that is coupled with the PRZM-2 model (subsequently referred to as the deterministic code in this report).  The composite code (i.e., the uncertainty shell coupled with the deterministic code) can be used for the quantitative estimate of the uncertainty in the concentrations at the monitoring point due to uncertainty in the (fate) model input parameters.

8.2  OVERVIEW OF THE PREPROCESSOR

The objective of the uncertainty analysis/propagation method is to estimate the uncertainty in model output (e.g., the concentration at a monitoring point) given the uncertainty in the input parameters and the transport and transformation model.  Alternatively stated, the objective is to estimate the cumulative probability distribution of the concentration at a receptor location given the probability distribution of the input parameters.  If Cw represents the concentration at the receptor, then

Cw = g(X)	(8-1)

where the function g represents the fate model and X represents the vector of all model inputs.  Note that some or all of the components of X may vary in an uncertain way, i.e. they are random variables defined by cumulative probability distribution functions.  Thus the goal of an uncertainty propagation method is to calculate the cumulative distribution function FCw(C) given a probabalistic characterization of X.  Note that FCw(C) is defined as:

FCw(C) = Probability (Cw ≤ C)	(8-2)

where C is a given output concentration.

8.2.1  Description of the Method

Given a set of deterministic values for each of the input parameters, X1, X2, . . . Xn, the composite model computes the output variable (e.g., a downgradient receptor well concentration Cw) as:

Cw = g (X1, X2, X3 . . . Xn)	(8-3)

Application of the Monte Carlo simulation procedure requires that at least one of the input variables, X1 . . . Xn, be uncertain and the uncertainty represented by a cumulative probability distribution.  The method involves the repeated generation of pseudo-random number values of the uncertain input variable(s) (drawn from the known distribution and within the range of any imposed bounds) and the application of the model using these values to generate a series of model responses i.e. values of Cw.  These responses are then analyzed statistically to yield the cumulative probability distribution of the model response.  Thus, the various steps involved in the application of the Monte Carlo simulation technique involve:

i)Selection of representative cumulative probability distribution functions for describing uncertainty in the relevant input variables.

ii)Generation of pseudo-random numbers from the distributions selected in (i).  These values represent a possible set of values for the input variables.

iii)Application of the model to compute the derived inputs and output(s).

iv)Repeated application of steps (ii) and (iii).

v)Presentation of the series of output (random) values generated in step (iii) as a cumulative probability distribution function (CDF).

vi)Analysis and application of the cumulative probability distribution of the output as a tool for decision making.

8.2.2  Uncertainty in the Input Variables

The parameters required by a transport and transformation model can be broadly classified into two different sets that exhibit different uncertainty characteristics.  These are:

●Chemical parameters.  Examples of these variables include the octanol-water partition coefficient, acid, neutral, and base catalyzed hydrolysis rate, soil-adsorption coefficient, Henry's Law Constant, etc.

●Media parameters.  Examples of these variables include the groundwater velocity, soil porosity, organic carbon content, dispersivity values, etc.

●Meteorological parameters.  Examples include precipitation, evaporation, solar radiation.

●Management parameters.  Examples include irrigation timing, pesticide application timing, well pumping rates, etc.

Uncertainty in chemical parameters primarily arises due to laboratory measurement errors or theoretical methods used to estimate the numerical values.  In addition to experimental precision and accuracy, errors may arise due to extrapolations from controlled (laboratory) measurement conditions to uncontrolled environmental (field) conditions.  Further, for some variables, semi-empirical methods are used to estimate the values.  In this case, errors in using the empirical relationships also contribute to errors/uncertainty in the model outputs.

Uncertainty in the second and third sets of parameters, identified above, may include both measurement and extrapolation errors.  However, the dominant source of uncertainty in these is the inherent natural (spatial and temporal) variability.  This variability can be interpreted as site-specific or within-site variation in the event that the fate model is used to analyze exposure due to the use and/or the disposal of a contaminant at a particular site.  Alternatively it can represent a larger scale (regional/national) uncertainty if the model is used to conduct exposure analysis for a specific chemical or specific disposal technology on a generic, nation-wide or regional basis.  Note that the distributional properties of the variables may change significantly depending upon the nature of the application.  Uncertainty in the fourth set of parameters may arise from a complex variety of factors including climate, sociology, economics, and human error.

Whatever the source of uncertainty, the uncertainty preprocessor developed here requires that the uncertainty be quantified by the user.  This implies that for each input parameter deemed to be uncertain, the user select a distribution and specifies the parameters that describe the distribution.

The current version of the preprocessor allows the user to select one of the following distributions.

i)		Uniform

ii)		Normal

iii)		Log-normal

iv)		Exponential

v)		Johnson SB distribution

vi)		Johnson SU distribution

vii)		Empirical

viii)		Triangular

Depending on the distribution selected, the user is required to input relevant parameters of the distribution.  The first requires minimum and maximum values.  The second and third distributions require the user to specify the mean and the variance.  The fourth distribution requires only one parameter - the mean of the distribution.  For the empirical distribution, the user is required to input the coordinates of the cumulative probability distribution function (minimum 2 pairs, maximum 20 pairs) which is subsequently treated as a piece-wise linear curve.  For the triangular distribution the user is required to input the minimum, maximum and the most likely value.  Finally, the Johnson SB and SU distribution requires four parameters -- mean, variance, and the lower and upper bounds.

In addition to the parameters of the distribution, the user is required to input the bounds of each model parameter.  These bounds may be based on available data or simply physical considerations, e.g., to avoid the generation of negative values.  Values generated outside these bounds are rejected.

Of the above eight distributions, the characteristics of the majority are easily available in the literature (Benjamin and Cornell 1970).  The triangular distribution has been discussed in Megill (1977).  Details of the Johnson system of distributions are presented in McGrath and Irving (1973) and Johnson and Kotz (1970).  Additional details for each of these distributions are presented in the following discussion.

In some cases, it may be desirable to include correlations among the variables.  For example, there may be correlation between hydraulic conductivity and particle size or between adsorption and degradation coefficients.  The uncertainty processor allows the generation of (linearly) correlated variables for cases where the underlying distribution of the variables is either normal and/or lognormal.

8.3  DESCRIPTION OF AVAILABLE PARAMETER DISTRIBUTIONS

The Monte Carlo shell has the ability to generate data from a number of probability distributions listed above.  A description of each of these distributions is provided in the following paragraphs, including parameters of the distributions, equations for the probability and cumulative density functions, and a brief discussion of the properties of each distribution.

8.3.1  Uniform Distribution

A uniform distribution is a symmetrical probability distribution in which all values within a given range have an equal chance of occurrence.  A uniform distribution is completely described by two parameters:  1) the minimum value (lower bound) A, and 2) the maximum value (upper bound) B.  The equation for the uniform probability density distribution of variable x is given by:

fu(x)	=	1/(B - A)	(8-4)

where

fu(x)	=	the value of the probability density function for x

The cumulative distribution F(x) is obtained by integrating Equation (8-4).  This yields the probability distribution:

Fu(x)	=	(x - A)/(B - A)	(8-5)

where

F(x)	=	the probability that a value less than or equal to x will occur

8.3.2  Normal Distribution

The term "normal distribution" refers to the well known bell-shaped probability distribution.  Normal distributions are symmetrical about the mean value and are unbounded, although values further from the mean occur less frequently.  The spread of the distribution is generally described by the standard deviation.  The normal distribution has only two parameters)--the mean and the standard deviation.  The probability density function of x is given by:

where

Sx	=	the standard deviation

mx	=	the mean of x

The cumulative distribution is the integral of the probability density function:

x

Fn(x) = ∫  fn(x)dx	(8-7)

-∞

The above integration must be performed numerically, but tables of numerically-integrated values of Fn(x) are widely available in the statistical literature.

8.3.3  Log-Normal Distribution

The log-normal distribution is a skewed distribution in which the natural log of variable x is normally distributed.  Thus, if y is the natural log of x, then the probability distribution of y is normal with mean my, and standard deviation Sy and a probability density function similar to Equation 8-10.  The mean and standard deviation of x (mx and Sx) are related to the log-normal parameters my and Sy as follows.

mx = exp[my + 0.5(Sy)2]	(8-8)

Sx2 = mx2 [exp(Sy2) - 1]	(8-9)

To preserve the observed mean and standard deviation of x, the parameters of the log-normal distribution (my and Sy) are selected such that the above relationships are satisfied.  Note that my and Sy do not equal the natural logs of mx and Sx, respectively.  Log-normal distributions have a lower bound of 0.0 and no upper bound, and are often used to describe positive data with skewed observed probability distributions.

8.3.4  Exponential Distribution

The probability density function for an exponential distribution is described by an exponential equation:

exp(-x/mx)

fe(x) =    ──────────	(8-10)

mx

where mx is the mean of x.  The cumulative distribution is given by:

Fe(x) = 1 - exp(-x/mx)	(8-11)

The exponential distribution is bounded by zero; the probability density function peaks at zero and decreases exponentially as x increases in magnitude.

8.3.5  The Johnson System of Distributions

The Johnson system involves two main distribution types--SB (log-ratio or bounded) and SU (unbounded or hyperbolic arcsine).  These two distribution types basically represent two different transformations applied to the random variable such that the transformed variable is normally distributed.  The specific transformations are:

where

ln	=	natural logarithm transformation

x	=	untransformed variable with limits of variation from A to B.

Y	=	the transformed variable with a normal distribution

Selection of a particular Johnson distribution for sample data set is accomplished by plotting the skewness and kurtosis of the sample data.  The location of the sample point indicates the distribution for the sample data.

For additional details of the Johnson system of distributions, the reader is referred to McGrath and Irving (1973) and Johnson and Kotz (1970).

8.3.6  Triangular Distribution

A triangular distribution is a relatively simple probability distribution defined by the minimum value, the maximum value, and the most frequent value (i.e., the mode).  Figure 8.1 shows an example triangular probability density function.  The cumulative distribution for values of x less than the most frequent value, xm, is given by:

(x - x1)2

F(x) =    ──────────────	(8-14)

(xm - x1)(x2 - x1)

where

x1	=	the minimum value

and

x2	=	the maximum value

For values of x greater than the most frequent value, the cumulative distribution is:

8.3.7  Empirical Distribution

At times it may be difficult to fit a standard statistical distribution to observed data.  In these cases, it is more appropriate to use an empirical piecewise-linear description of the observed cumulative distribution for the variable of interest.

Cumulative probabilities can be estimated from observed data by ranking the data from lowest (rank = 1) to highest (rank = number of samples) value.  The cumulative probabi-lity associated with a value of x is then calculated as a function of the rank of x and the total number of samples.  The cumulative probabilities of values between observed data can be estimated by linear interpolation.

8.3.8  Uncertainty in Correlated Variables

In many cases model input variables are correlated due to various physical mechanisms.  Monte Carlo simulation of such variables requires not only that parameters be generated from the appropriate univariate distributions, but also that the appropriate correlations be preserved in the generated input sequences.  The Monte Carlo module currently has the ability to generate correlated normal, log-normal, Johnson SB, and Johnson SU numbers; the procedures used are described in the following paragraphs.

Figure 8.1

The correlation coefficient is a measure of the linear dependence between two random variables and is defined as:

cov(x,y)

ρx,y =   ────────	(8-16)

ρx ρy

where

ρx,y	   	=	the correlation coefficient between random variables x any y

cov(x,y) 		=	the covariance of x and y as defined below

ρx, ρy		=	the standard deviation for x and y.

The covariance of x and y is defined as:

cov(x,y)		=	E [(x-mx)(y-mx)]

+∞

=	∫ ∫ (x-mx) (y-my) fx,y(x,y) dx dy	(8-17)

-∞

where

E		=	the expected value

mx, my		=	the mean of the random variables x and y

fx,y(x,y)		=	the joint probability distribution of x and y.

Note that the linear correlation coefficient between x and y can be computed using

To generate correlated random variables, three steps are required.  First uncorrelated, normally distributed random numbers are generated.  This vector is then transformed to a vector of normally distributed numbers with the desired correlation.  Finally, the normally distributed numbers are transformed to numbers with the desired distribution.

The transformation of uncorrelated to correlated normal numbers consists of multiplying the uncorrelated vector of numbers with a matrix B:

Y' = B e	(8-19)

where

e	=	the vector of uncorrelated, normally distributed random numbers.

B	=	and N by N matrix

Y'	=	a vector of standard normal deviates of mean zero and standard deviation of unity.

The matrix B is related to the variance-covariance matrix S as follows.

S = B BT	(8-20)

where BT is the transpose of the B matrix.  Since the normal variables Y' have means of zero and unit variances, the variance-covariance matrix is equivalent to the correlation matrix.

Thus, if the correlation matrix S is known, B can be found from Equation 8-20 by using a Choleski decomposition algorithm.  This algorithm will decomposes a symmetric positive definite matrix, such as S, into a triangular matrix such as B (de Marsily 1986, p. 381).

Having generated a vector of correlated normally distributed random numbers, the user can convert vector Y', through appropriate transformations, to the distribution of choice.  Thus for parameters Xi that have a normal distribution, the Y' numbers are transformed as follows.

Xi = mx + σx(Y)	(8-21)

For parameters that follow the lognormal distribution, the following transformation applies.

Xi = exp[(Y) (σln,i) + μln,i]	(8-22)

where

μln,i	=	the log mean of the ith parameter

σln,i	=	the log standard deviation of the ith parameter

For parameters with Johnson SB and SU distributions, the Y' are first transformed to normally distributed variables Y with mean My and standard deviation σy:

Yi = My + σy Y	(8-23)

Johnson SB numbers are then computed from Yi as follows.

Xi = (B exp(Yi) - A)/(1 + exp(Yi))	(8-24)

Johnson SU numbers are computed by:

Xi = A + (B-A)[exp(Yi) - exp(-Yi)]/2	(8-25)

Other distributions can be easily incorporated into the analyses at a later time when suitable transformations from the normal distribution can be found.  It is important to note that, in using this technique, the correlations are maintained in normal space, so if these correlations are estimated using actual data, the data should be transformed to a normal distribution before correlation coefficients are estimated.

For two correlated variables, one with a normal distribution (x2) and the other with a log normal distribution (x1), the following equation is used to transform correlations to normal space (Meija and Rodriguez-Iturbe, 1974).

σx1,x2[exp(σy12) - 1]½

σy1,y2 =    ──────────────────────	(8-26)

σy1

where

σy1,y2		=	the correlation coefficient between the two variables in the normal space

σx1,x2		=	the correlation coefficient between the two variables in the

arithmetic space

σy12		=	the variance of y1 derived from Equation (8-9)

If both x1 and x2 are log-normally distributed then the correlation coefficient is transformed using Meija and Rodriguez-Iturbe (1974):

where the relationships between Sx1(Sx2) and Sy1(Sy2) are given by Equations (8-8)

and 8-9.

Thus, for log-normal variables, the user enters the values of the correlation coefficients in log-normal space; Equations 8-26 and 8-27 are then used to transform the correlation coefficients into normal space.

No direct transformation of Johnson SB or SU correlations to normal correlations is currently known.  For these distributions, the user must supply the correlation coefficients between normal-transformed numbers.  This may be accomplished by first transforming Johnson SB and SU data to normal data using Equations 8-12 and 8-13.  The covariance matrix S is then derived using only normal, log-normal, and normal-transformed SB and SU data.

8.3.9  Generation of Random Numbers

Having selected the distribution for the various input parameters, the next step is the generation of random values of these parameters.  This requires the use of pseudo-random-number-generating algorithms for Normal and Uniform numbers.  Numerous pro-prietary as well as non-proprietary subroutines can be used to generate random numbers.  Many of these are comparable in terms of their computational efficiency, accuracy, and precision.  The performance of the algorithms included in this preprocessor has been checked to ensure that they accurately reproduce the parameters of the distributions that are being sampled (Woodward-Clyde Consultants 1988).

8.4  ANALYSIS OF OUTPUT AND ESTIMATION OF DISTRIBUTION QUANTILES

Model output generally will consist of a volume of data that represents a sample of outcomes.  Given the natural variability and the uncertainty of various model components, there will be variability in the output.  All of the factors that were allowed to vary within the model contribute to variability in model predictions.  Taken as a whole, the model output depicts possible events in terms of their relative frequency of occurrence.  Values produced by the model generally are treated as if they were observations of real field events.  In interpreting these values, it is important to maintain the perspective dictated by the design and scope of the study.

Model output can be analyzed in various ways depending upon current objectives.  Many features of the distribution may be characterized.  Quite often, for example, it is of interest to estimate certain quantiles or percentiles of the distribution.  Since the model output is treated as a sample from an unknown parent population, the methods of

statistical inference normally are used to estimate distribution parameters and to associate measures of uncertainty with these parameters.

One of the most frequently asked questions concerns the number of samples required for some given purpose.  In modeling, this translates into the number of model runs needed.  For the most part, since methods of basic inference are being applied in a Monte Carlo framework, resulting model output values are treated as observations forming a random sample.  The sample size required to estimate a given parameter depends on a number of factors.  These include the nature of the parameter that is being estimated, the form of the underlying distribution, the variability in the observations, the degree of precision and/or accuracy desired, the level of confidence to be associated with the estimate, and the actual statistical estimator used to provide the estimate.

Generally, if the output distribution is to be accurately characterized with respect to its many features, the number of model runs needed will be higher than if only a few parameters are to be estimated.  The simulation strategy should be determined by the issues addressed by the modeling effort.  It may be important, for example, to estimate the extreme upper percentiles of the output distribution.  In this case, the choice of simulation design should account for the relative difficulty of obtaining such estimates.  If it is not known exactly how the data will be utilized, then the problem becomes one of establishing a distributional representation that is as good as possible under the most extreme usage or estimation scenario.  For example, if only a distribution mean were to be estimated, the sample size required could be determined without concern for estimating, say, the 99th percentile.

8.4.1  Estimating Distribution Quantiles

In the following section, a summary is given for statistical techniques used to estimate distribution quantiles.  Many such methods are available to estimate a given percentile of an unknown distribution on the basis of sample data.  In the PRZM-2 code, four such methods can be used.  Among these are distribution-free or nonparametric techniques as described below.  Others include methods specific to certain distributions that assume a knowledge of the distributional form.  First, the point estimators are given, then the method for constructing a confidence interval is briefly described.

The order statistics of a sample are merely the ordered values denoted by x(1), x(2), ... , x(n), where n represents the sample size.  The empirical cdf can be defined simply as

⎧    0, if x(1) < x,

g(x) = ⎨  1/n, if x(i) ≤ x < x(i+1), for i=1, ... , n-1	(8-28)

⎩    1, if        x ≥ x(n).

Mathematically, g(x) is a step function, discontinuous at each value x(i).

By definition, the 100p-th percentile (i.e., the p-level quantile) is given by up where

p = Pr{ X < up }	(8-29)

If F(x) denotes the cumulative distribution function,

p = F(up) and up = F-1(p)	(8-30)

When only sample information is available, up is unknown, but it can be estimated by forming an appropriate function of the observations.

Nonparametric point estimates of up can be constructed as linear combinations of the order statistics.  In particular, each of Y1 through Y3 below is an estimator of up.  Let [z] denote the largest integer less than or equal to z.  Define

j = [np],       g = np - j	(8-31)

i = [np + 0.5], r = (np + 0.5) - i	(8-32)

k = [(n+1)p],   h = (n+1)p - k	(8-33)

Then,

Y1 = (1-h) X(k) + h X(k+1)	(8-34)

(X(j) + X(j+1))

Y2 =    ───────────────, if g=0	(8-35)

2

= X(j+1)         , if g>0

Y3 = (0.5+i-np) X(i) + (0.5-i+np) X(i+1)	(8-36)

= (1 - r) X(i) + r X(i+1)

In each of these definitions, only the values of n and p determine which order statistics are used in forming an estimate of up.  Thus, the estimators do not depend on the underlying distributions.  However, the relative performance of these estimators is dependent upon several criteria involving the level p, the sample size n, the type of parent distribution from which samples are drawn, estimator bias, and the mean squared error.  If the sample size is very large, the differences among the estimates are not very great.  Of the estimators available, the three shown above exhibit the best performance in relatively small samples (n≤50) from normal and lognormal distributions.

Another simple estimator used in the model is calculated by constructing the cdf of the output

F(x) = i/n	(8-37)

in which i is the rank of the outcome in the sample.  The specific quantile of interest is then determined by interpolation.

8.4.2  Confidence of up

Approximate confidence statements can be placed on up by selecting appropriate order statistics to serve as the upper and lower confidence bounds.  The rationale is given as follows.

For a given distribution, the value up is such that exactly 100p% of all values of this distribution are less than up, and 100(1-p)% exceed this value.  An individual value selected randomly from the distribution has probability p of being less than up.  In a random sample of size n from this distribution, the probability of not exceeding up remains constant for each individual element of the sample.  Thus, the number of values in the sample that are less than or equal to up is distributed binomially.  The probability that the random interval (X(j),  X(j+1)) will contain up is equivalent to the probability  that  exactly i of the n elements of the sample will be less than up.  Hence, this probability is

which is a simple binomial probability.

This expression can be calculated for each pair of consecutive order statistics X(i), X(i+1), for i=1, ..., n-1.  However, it is more convenient to deal with these several intervals by calculating cumulative probabilities of the form

For practical convenience, the normal approximation

F {[(i+0.5)-np]/√}	(8-40)

can be used, where F represents the cdf of the standard normal distribution.

All of this is utilized for determining two order statistics, denoted below with subscripts i and j, with the property

Pr{X(i) < up ≤ X(j)} = 1 - α	(8-41)

where 1-α is the predetermined confidence coefficient; typically, 1-α = 0.95.  Computationally, i and j can be determined by solving the equations

α/2 = F{[(i+0.5)-np]/√}	(8-42)

and

1 - α/2 = F{[(j+0.5)-np]/√}	(8-43)

This results in

i = (np-0.5) + √ F-1(α/2)	(8-44)

j = (np-0.5) + √ F-1(1-α/2)	(8-45)

where F-1 denotes the inverse cdf of the standard normal distribution (e.g., for 1-α = 0.90, F-1(1-α/2) = 1.645).  For example, with n=100, p=0.95, and 1-α=0.90, i=90 and j=98, so that (X(90), X(98)) forms the approximate 90% confidence interval on up.

Although the expressions for the confidence interval do not depend in any way on the underlying distribution, the expected width of the interval does.  In particular, it depends on the expected values of the order statistics involved.  In the example above, if the sample is from a standard normal distribution, up = 1.645 and the expected half-width of the interval is 0.349.  If the sample is from a lognormal distribution based on a standard normal, up = 5.180 and the expected half-width is 1.858.  Also, note that, in normal sampling, the expected confidence interval half-width for n=500 is 0.192 for the same estimate It was assumed that the initial condition corresponded to a hydrostatic pressure head distribution in the soil with pressure head values at the water table and the top of the soil equal to 0 and -420 cm, respectively.  The simulation was performed for 20 time steps using Δt = 1 d.Shown in Figures 7.21 through 7.23 are simulated profiles of water saturation, pressure head, and vertical Darcy velocity, respectively.  As expected, the two sand layers exhibit fast drainage response, whereas the intervening clay-loam layer exhibits slow drainage response.  This behavior is seen in Figure 7.21.  The pressure head and velocity profiles depicted in Figures 7.22 and 7.23 directly reflect the effect of temporal change in the infiltration rate.  Note that the values of Darcy velocity at the soil surface (Figure 7.23) are equal to the values of infiltration rate for the same time values.  Following the unsaturated flow simulation, the transport simulation was performed using the Darcy velocity file from the flow computation as an input file for the transport computation.  Concentration profiles determined by the code are plotted in Figure 7.24.  As illustrated, the contaminant front exhibits slow movement through the clay loam layer.

SECTION 9

REFERENCES

Anderson, E.A  1978.  Initial Parameter Values for the Snow Accumulation and Ablation Model.  In:  National Weather Service River Forecast System - User's Manual, Part IV.2.2.1.  NWS/NOAA, US Department of Commerce, Silver Springs, MD.

Avdonin, N.A.  1964.  Some Formulas for Calculating the Temperature Field of a Stratom during Termal Injection.  Izvestiya Vysshikh Uchebnykl Zavedenii Neft.  Gaz. 7(3) 37-41.

Bassett, D.L., and D.W. Fitzsimmons.  1974.  A Dynamic Model of Overland Flow in Border Irrigation.  Am. Soc. Ag. Eng. Paper No. 74-2529, St. Joseph, MI.

Battelle, Pacific Northwest Laboratories, and GeoTrans.  1988.  FASTCHEM Package Volume 2:  User's Guide to the EFLOW Groundwater Flow Code.  Electrical Power Research Institute, Palo Alto, CA.  EA-5870-CCM, Volume 2, Research Project 2485-2.

Bear, J.  1979.  Hydraulics of Groundwater.  McGraw-Hill, New York.

Benjamin, J.R. and C.A. Cornell.  1970.  Probability, Statistics, and Decision for Civil Engineers.  McGraw Hill Book Company.

Biggar, J.W. and D.R. Nielson.  1976.  Spatial Variability of the Leaching Characteristics of a Field Soil.  Water Res. Res., 12:78-84.

Brackensiek, D.L., and W.J. Rawls.  1983.  Green-Ampt Infiltration Model Parameters for Hydrologic Classifications of Soils.  In:  Proceedings of Special Conference on Advances in Irrigation and Drainage, American Society of Chemical Engineers, Jackson, Wyoming.

Bresler.  E.  1973.  Simultaneous Transport of Solutes and Water Under Transient Unsaturated Flow Conditions.  Water Res. Res., 9(4):975-986.

Bromilow, R.H., M. Richard and M. Leistra.  1980.  Pesticide Sci. II:389-395.

Brooks, R.H. and A.T. Corey.  1966.  Properties of Porous Media Affecting Fluid Flow.  ASCE J. Irrig. Drain Div. 92 (IR2):61-68.

Brown, S.M., and S.H. Boutwell.  1986.  Chemical Spill Exposure Assessment Methodology.  RP 2634-1.  Electric Power Research Institute, Palo Alto, CA.

CRC.  1981.  Handbook of Chemistry and Physics, 62nd edition.  CRC Press, Boca Raton, FL.

Carsel, R.F., and R.S. Parrish.  1988.  Developing Joint Probability Distributions of Soil-Water Retention Characteristics.  Water Resources Research, 24(5):755-769.

Carsel, R.F., R.S. Parrish, R.L. Jones, J.L. Hansen, and R.L. Lamb.  1988.  Characterizing the Uncertainty of Pesticide Leaching in Agricultural Soils.  Journal of Contam. Hydrol., 25:111-124.

Carsel, R.F., L.A. Mulkey, M.N. Lorber, and L.B. Baskin.  1985.  The Pesticide Root Zone Model (PRZM):  A Procedure for Evaluating Pesticide Leaching Threats to Ground Water.  Ecological Modeling, 30:49-69.

Carsel, R.F., C.N. Smith, L.A. Mulkey, J.D. Dean, and P. Jowise.  1984.  User's Manual for the Pesticide Root Zone Model (PRZM):  Release 1.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/3-84-109.

Chen, C.W., S.A. Cheriai, R.J.M. Hudson, and J.D. Dean.  1983.  The Integrated Lake-Watershed Acidification Study.  Vol. I.  Model Principles and Application Procedures.  Electric Power Research Institute, Palo Alto, CA.  Rep. No. EA-3221, Vol. 1.

Chow, V.T.  1959.  Open Channel Hydraulics.  McGraw-Hill, New York.

Cohen, P., O.L. Franke, and B.L. Foxworthy.  1968.  An Atlas of Long Island's Water Resources.  New York State Water Resources Commission Bull,  32:117.

Cruse, R.M., D.R. Linden, J.K. Radke, W.E. Larson, and K. Larntz.  1980.  A Model to Predict Tillage Effects on Soil Temperature.  Soil Sci. Soc. Am. J., 44:378-383.

Davis, L.A., and S.P. Neuman.  1983.  Documentation and User's Guide:  UNSAT2 - Variably Saturated Flow Model.  U.S. Nuclear Regulatory Commission Report, Washington, DC., NUREG/CR-3390.

Dean, J.D.  1979.  Determination of Irrigation Water Demand in Humid Climates.  Master's Thesis, University of Georgia, Athens, GA.

Dean, J.D., P.S. Huyakorn, A.S. Donigian Jr., K.A. Voos, R.W. Schanz, Y.J. Meeks, and R.F. Carsel. 1989.  Risk of Unsaturated/Saturated Transport and Transformation of Chemical Concentrations (RUSTIC).  Volume I: Theory and Code Verification.  U.S. Environmental Protection Agency, Athens, GA. EPA-600/3-89/048a.

Dean, J.D., P.S. Huyakorn, A.S. Donigian Jr., K.A. Voos, R.W. Schanz, Y.J. Meeks, and R.F. Carsel. 1989.  Risk of Unsaturated/Saturated Transport and Transformation of Chemical Concentrations (RUSTIC).  Volume II: User's Manual.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/3-89/048b.

Dean, J.D., A.M. Salhotra, E.W. Strecker, D.A. Gray, and P.H. Howard.  1988.  Aldicarb Exposure Assessment in Florida:  Further Simulation Studies.  U.S. Environmental Protection Agency, Athens, GA.  Unpublished report.

Dean, J.D., and D.F. Atwood.  1985.  Exposure Assessment Modeling for Aldicarb in Florida.  U.S. Environmental Protection Agency, Athens, GA.  EPA/600/3-85/051.

Dejonckheere, W., W. Steurbaut, O. Melkebeke, and R.H. Kips.  1983.  Leaching of Aldicarb and Thiofanox, and Their Uptake in Soils by Sugarbeet Plants.  Pesticide Sci., 14:99-107.

de Vries, D.A.  1963.  Thermal Properties of Soils.  In:  Physics of the Plant Environment,  W.R. van Wijk, (ed.).  J. Wiley and Sons, Inc., New York.

Donigian, A.S. Jr., C.S. Raju and R.F. Carsel.  1986.  Impact of Conservation Tillage on Environmental Pesticide Concentrations in Three Agricultural Regions.  U.S. Environmental Protection Agency, Washington, DC.  Unpublished report.

Dyer, A.J.  1974.  A Review of Flux-Profile Relationships.  Boundary-Layer Meteorology 7:363-372.

Dyer, A.J., and B.B. Hicks.  1970.  Flux-Gradient Relationships in the Constant Flux Layer.  Quarterly Journal of the Royal Meteorological Society 96:715-721.

Eagleson, P.S.  1978.  Climate, Soil and Vegatation.  The Distribution of Annual Precipitation Derived from Observed Storm Sequences.  Water Res. Res., 14(5):713-722.

Electric Power Research Institute.  1985.  A Review of Field Scale Physical Solute Transport Processes in Saturated and Unsaturated Porous Media.  EPRI Palo Alto, CA., EA-4190, Project 2485-5.

Elliott, R.L., W.R. Walker, and G.V. Skogerboe.  1982.  Zero Inertia Modeling of Furrow Irrigation Advance.  ASCE Journal of Irrigation and Drainage, 108(IR3):179-195.

Fangmeier, D.D., and M.K. Ramsey.  1978.  Intake Characteristics of Irrigation Furrows.  Transactions of ASAE, pp. 696-705.

Farrell, D.A., E.L. Greacen, and C.G. Gurr.  1966.  Vapor Transfer in Soil Due to Air Turbulence.  Soil Sci. 102:305-313.

Federal Register.  1984.  Proposed Guidelines for Exposure Assessments:  Request for Comments.  U.S. Environmental Protection Agency.  Vol. 49 (227), November 23.

Federal Register.  1986.  Hazardous Waste Management System:  Land Disposal Restric

tions.  USEPA, vol. 15. no. 9.

Fok, Y.S., and A.A. Bishop.  1965.  Analysis of Water Advance in Surface Irrigation.   ASCE of Irrig. and Drainage 91(IR1):99-116.

Freeze and Cherry.  1979.  Groundwater.  Prentice Hall, Englewood Cliffs, NJ.

Fukuda, H.  1955.  Air and Vapor Movement in Soil Due to Wind Gustiness.  Soil Science 79:249-258.

Grace, R.A. and P.S. Eagleson.  1966.  The Synthesis of Short-Time-Incremental Rainfall Sequences.  Report No. 91.  Massachusetts Institute of Technology, Cambridge, MA.

Grover, R., S.R. Shewchuk, A.J. Cessna, A.E. Smith, and J.H. Hunter.  1985.  Fate of 2,4-D Iso-Octyl Ester after Application to a Wheat Field.  J. Environ. Qual., 14:203-210.

Gupta, S.C., W.E. Larson, and R.R. Allmaras.  1984.  Predicting Soil Temperature and Soil Heat Flux under Different Tillage-Surface Residue Conditions.  Soil Sci. Soc. Am. J. 48:223-232.

Gupta, S.C., W.E. Larson, and D.R. Linden.  1983.  Tillage and Surface Residue Effects on Soil Upper Boundary Temperatures.  Soil Sci. Soc. Am. J. 47:1212-1218.

Gupta, S.C., J.K. Radke, W.E. Larson, and M.J. Shaffer.  1982.  Predicting Temperature of Bare and Residue-Covered Soils from Daily Maximum and Minimum Air Temperatures.  Soil Sci. Soc. Am. J. 46:372-376.

Gupta, S.C., J.K. Radke, and W.E. Larson.  1981.  Predicting Temperature of Bare and Residue-Covered Soils with and without a Corn Crop.  Soil Sci. Soc. Am. J. 45:405-412.

Hadermann, J.  1980.  Radionuclide Transport Through Heterogeneous Media.  Nuclear Technology 47:311-323.

Haith, D.A., and R.C. Loehr, eds.  1979.  Effectiveness of Soil and Water Conservation Practices for Pollution Control.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/3-79-106.

Hall, W.A.  1956.  Estimating Irrigation Border Flow.  Agric. Engineering, 37:263-265.

Hanks, R.J., D.D. Austin, and W.T. Ondrechen.  1971.  Soil Temperature Estimation by a Numerical Method.  Soil Sci. Soc. Am. Proc. 35:665-667.

Harper, L.A., A.W. White, Jr., R.R. Bruce, A.W. Thomas, and R.A. Leonard.  1976.  Soil and Microclimate Effects on Trifluralin Volatilization.  J. Environ. Qual. 5:236-242.

Hasfurther, V.R., and R.D. Burman.  1974.  Soil Temperature Modeling Using Air

Temperature as a Driving Mechanism.  Trans. ASAE 17:78-81.

Hiemstra, L.A.V. and R.L. Creese.  1970.  Synthetic Generation of Seasonal Precipitation.  Journal of Hydrology, 11:30-46.

Hunt, B.  1972.  Dispersive Sources in Uniform Ground-Water Flow.  ASCE Journal of the Hydraulics Division.  104(HYI).

Huyakorn, P.S., H.O. White, Jr., J.E. Buckley, and T.D. Wadsworth.  1988a.  VADOFT:  Finite Element Code for Simulating One-Dimensional Flow and Solute Transport in the Vadose Zone.  Project Report to Woodward-Clyde Consultants.  February.

Huyakorn, P.S., and J.E. Buckley.  1988b.  SAFTMOD:  Saturated Zone Flow and Transport Two-Dimensional Finite Element Model.  Project Report to Woodward-Clyde Consultants.  March.

Huyakorn, P.S., B.G. Jones, and P.F. Andersen.  1986a.  Finite Element Algorithms for Simulating Three-Dimensional Groundwater Flow and Solute Transport in Multilayer Systems.  Water Resour. Res. 22(3)361-374.

Huyakorn, P.S., E.P. Springer, V. Guvanasen, and T.P. Wadsworth.  1986b.  A Three-Dimensional Finite-Element Model for Simulating Water Flow in Variably Saturated Porous Media.  Water Resour. Res. 22(13):1790-1808.

Huyakorn, P.S., J.W. Mercer, and D.S. Ward.  1985.  Finite Element Matric and Mass Balance Computational Schemes for Transport in Variably Saturated Porous Media.  Water Resour. Res. 21(3):346-358.

Huyakorn, P.S., S.D. Thomas, and B.M. Thompson.  1984a.  Techniques for Making Finite Elements Competitive in Modeling Flow in Variably Saturated Porous Media.  Water Resour. Res. 20(8):1099-1115.

Huyakorn, P.S., A.G. Kretschek, R.W. Broome, J.W. Mercer, and B.H. Lester.  1984b.  Testing and Validation of  Models for Simulating Solute Transport in Ground-Water.  International Ground Water Modeling Center, Holcomb Research Inst., Butler, ID. HRI No. 35.

Huyakorn, P.S., and G.F. Pinder.  1983.  Computational Methods in Subsurface Flow.  Academic Press, New York, 473 pp.

Huyakorn, P.S., J.B. Kool, and T.D. Wadsworth.  1988.  A Comprehensive Simulation of Aldicarb Transport at the Wickham Site on Long Island.  In:  Validation of Flow and Transport Models for the Unsaturated Zone.  Wierenga P.J., and D. Bachelet (eds.).  Conference proceedings; May 23-26, 1988 Ruidoso, New Mexico.  Research Report 88-SS-04.  New Mexico State University, Las Cruces, NM.  545 pp.

Johnson, N.L., and S. Kotz.  1970.  Distributions in Statistics:  Continuous Univariate Distributions.  Houghton Mifflin Company, Boston, MA.

Jones, R.L.  1983.  Movement and Degradation of Aldicarb Residues in Soil and Ground Water.  Presented at the CETAC Conference on Multidisciplinary Approaches to Environmental Problems, November 6-9, Crystal City, VA.

Jones, R.L., P.S.C. Rao, and A.G. Hornsby.  1983.  Fate of Aldicarb in Florida Citrus Soil 2.  Model Evaluation.  Presented at the Conference on Characterization and Monitoring of Vadose (Unsaturated) Zone, December 8-10, Las Vegas, NV.

Jury, W.A., W.F. Spencer, and W.J. Farmer.  1984.  Behavior Assessment Model for Trace Organics in Soil:  III.  Application of Screening Model.  13:  573-579.

Jury, W.A., R. Grover, W.F. Spencer, and W.J. Farmer.  1983a.  Behavior Assessment Model for Trace Organics in Soil:  I. Model Description.  J. Environ. Qual. 12:558-564.

Jury, W.A., R. Grover, W.F. Spencer, and W.J. Farmer.  1983b.  Use of Models for Assessing Relative Volatility, Mobility, and Persistence of Pesticides and Other Trace Organics in Soil Systems.  In:  Harvard  Assessment of Chemicals,  J. Saxena, (ed.).  pp. 1-43.  Academic Press, New York.

Jury, W.A.  1985.  Spatial Variability of Soil Physical Parameters in Solute Migration:  A Critical Literature Review.  Electric Power Research Institute, Palo Alto, CA., Report No. EA-4228.

Karickhoff, S.W.  1984.  Organic Pollutant Sorption in Aquatic Systems.  J. Hydraul. Eng. (ASCE) 110:707-735

Karickhoff, S.W., D.S. Brown, and T.A. Scott.  1979.  Sorption of Hydrophobic Pollutants on Natural Sediments.  Water Res., 13:241-248.

Karmeli, D., L.J. Salazar, and W.R. Walker.  1978.  Assessing the Spatial Variability of Irrigation Water Applications.  U.S. Environmental Protection Agency.

Khalell, R., and D. Reddell.  1986.  MOC Solutions of Convective-Dispersion Problems.  Ground Water 24(6):798-807.

Konikow, L.F., and J.D. Bredehoeft.  1978.  Computer Model of Two-Dimensional Solute Transport and Dispersion in Ground Water.  Techniques of Water-Resources Investigations of the United States Geological Survey, Book 7, Chapter C2.

Kreysig, E.  1972.  Advanced Engineering Mathematics, pp. 430-434.  John Wiley and Sons Inc., New York, N.Y.

Lafleur, D.W., F.J. Pearson, and D.S. Ward.  1981.  Mathematical Simulation of Aldicarb Behavior on Long Island, Unsaturated Flow and Groundwater Transport.  INTERA Environmental Consultants, Inc.,  Houston, TX.

Lassey, K.R.  1982.  The Interception and Retention of Aerosols by Vegetation - I.  The Formulation of a Filtration Module.  Atmospheric Environment 16(1):13-24.

Lavy, T.L., F.W. Roeth, and C.R. Fenster.  1973.  Degradation of 2,4-D and Atrazine at Three Soil Depths in the Field.  J. Environ. Qual. 2(1):132-137.

Lester, B.H., P.S. Huyakorn, H.O. White, Jr., T.D. Wadsworth, and J.E. Buckley.  1986.  Analytical Models for Evaluating Leachate Migration in Groundwater Systems.  U.S. Environmental Protection Agency, Washington, DC.  Unpublished report.

Li, R., D.B. Simons, and M.A. Stevens.  1975.  Non-linear Kinematic Wave Approximation for Water Routing.  Water Resources Res. 11(2):245-252.

Liley, P.E., and W.R. Gambill.  1973.  Section 3:  Physical and Chemical Data.  In:  Chemical Engineers Handbook, P.H. Perry and C.H. Chilton (ed.).  McGraw Hill Publishing Co., New York.

Lo, A.K.  1977.  An Analytical-Empirical Method for Determining the Roughness Length and Zero-Plane Displacement.  Boundary-Layer Meteorology 12:141-151.

Lyman, W.J., W.F. Reehl, and D.H. Rosenblatt.  1982.  Handbook of Chemical Property Estimation Methods.  McGraw-Hill Publishing Co., New York.

M. Baptista, A.E., E.E. Adams, and K.D. Stolzenbach.  1984.  Eularian-Lagrangian Analysis of Pollution Transport in Shallow Water.  Massachusetts Institute of Technology, Cambridge, MA., Report 296.

Macalady, D.L. and N.L. Wolfe.  1985.  Effects of Sediment Sorption and Abiotic Hydrolysis.  I-Organophosphorothioate Esters.  J. Agric. Food Chem. 33:167-173.

Macalady, D.L. and N.L. Wolfe.  1984.  Abiotic Hydrolysis of Sorbed Pesticides.  In:  Treatment and Disposal of Pesticide Wastes.  R.F. Krueger and J.N. Seiber (eds.).  American Chemical Society, Washington, DC. ACS Symp. Ser. No. 250, pp. 221-244.

Matthews, G.A.  1978.  Pesticide Application Methods.  Longman, Inc.,  New York.

McGrath, E.J. and D.C. Irving.  1973.  Techniques for Efficient Monte Carlo Simulation, Volume II.  Random Number Generation for Selected Probability Distributions.  Report prepared for Office of Naval Research.  Project No. NR 366-076/1-5-72, Code 462.

Megill R. E.  1977.  An Introduction to Risk Analysis.  Petroleum Publishing Company, Tulsa, OK.

Mehlenbacher, L.A., and D.W.A. Whitfield.  1977.  Modeling Thermal Eddy Diffusivity at Canopy Height.  Boundary-Layer Meteorology 12:153-170.

Meija, J.M., and I. Rodriguez-Iturbe.  1974.  On the Synthesis of Random Field Sampling from the Spectrum:  An Application to the Generation of Hydraulic Spatial Processes.  Water Resour. Res. 10(4):705-712.

Mill, T., et al.  1982.  Laboratory Protocols for Evaluating the Fate of Organic Chemicals in Air and Water.  U.S. Environmental Protection Agency, Athens, GA.  EPA/600/3-82/022.

Miller, C.W. 1979.  In:  Proceedings of Symposium, Biological Implications of Radionuclides Released from Nuclear Industries, Vol. II.  Vienna, Austria.

Miller, G.C., V.R. Hebert and R.G. Zepp.  1987.  Chemistry and Photochemistry of Low-Volatility Organic Chemicals on Environmental Surfaces.  Environ. Sci. Technol. 21:1164-1167.

Mockus, V.  1972.  Estimation of Direct Surface Runoff from Storm Rainfall.  In:  National Engineering Handbook.  Section IV, Hydrology.  U.S. Soil Conservation Report NEH-Notice 4-102.  August.

Mualem, Y.  1976.  A New Model for Predicting the Hydraulic Conductivity of Unsaturated Porous Media.  Water Resour. Res. 12(3):513-522.

Nash, R.G.  1980.  Dissipation of Pesticides from Soils.  In:  CREAMS: A Field Scale Model for Chemicals, Runoff, and Erosion from Agricultural Management Systems, ed. W.G. Krisel, U.S. Department of Agriculture, Washington, DC.,  Res. Rep. No. 26.

Nash, R.G.  1974.  Plant Uptakes of Insecticides, Fungicides and Fumigants from Soils.  In: Pesticides in Soil and Water.  Soil Sci. Soc. Am., Madison, WI.

Neuman, S.P., R.A. Feddes, and E. Bresler.  1974.  Finite Element Simulation of Flow in Saturated-Unsaturated Soils Considering Water Uptake by Plants.  Hydrodynamics and Hydraulic Engineering Laboratory, Haifa, Israel,  Project No. ALO-SWC-77.

Neumen, S.P., and P.A. Witherspoon.  1969a.  Theory of Flow in a Confined Two-Aquifer System.  Water Resour. Res. 5(3):803-816.

Neuman, S.P., and P.A. Witherspoon.  1969b.  Applicability of Current Theories of Flow in Leaky Aquifers.  Water Resour. Res. 5(4):817-829.

Nielson, D.R., J.W. Biggar, and K.T. Erh.  1983.  Spatial Variability of Field Measured Soil Water Properties.  Hilgardia 42(7):214-259.

Nye, P.H.  1979.  Diffusion of Ions and Uncharged Solutes in Soils and Soil Clays.  Adv. in Agronomy 31:225-272.

Ogata, A., and R.B. Banks.  1961.  A Solution of the Differential Equation of Longitudinal Dispersion in Porous Media.  U.S. Geological Survey, Washington, DC., Professional Paper No. 411-A.

Oliver, H.R.  1971.  Wind Profiles in and above a Forest Canopy.  Quarterly Journal of the Royal Meteorological Society 97:548-553.

Papadopulos, I.S.  1965.  Nonsteady Flow to a Well in an Infinite Anisstropic Aquifer.  International Association of Scientific Hydrology Symposium of Dubrovnik.

Parmele, L.H., E.R. Lemon, and A.W. Taylor.  1972.  Micrometeorological Measurement of Pesticide Vapor Flux from Bare Soil and Corn under Field Conditions.  Water, Air, and Soil Pollution 1:433-451.

Parton, W.J.  1984.  Predicting Soil Temperature in a Shortgrass Steppe.  Soil Sci.  138(2):93-101.

Perkins, H.C.  1974.  Air Pollution.  McGraw-Hill, Inc., New York.

Phillip, J.R.  1955.  Numerical Solution of Equations of the Diffusion Type with Diffusivity Concentration-Dependent.  Trans. Faraday Soc. 51:885-892.

Pinder, G.F., and W.G. Gray.  1977.  Finite Element Simulation in Surface and Subsurface Hydrology.  Academic Press, London.

Rao, P.S.C., and R.J. Wagenet.  1985.  Spatial Variability of Pesticides in Field Soils:  Methods for Data Analysis and Consequences.  Weed Sci. 33:1-26.

Rao, P.S.C., et al.  1984.  Estimation of Parameters for Modeling the Behavior of Selected Pesticide and Orthophosphate.  U.S. Environmental Protection Agency, Athens, GA.  EPA 600/3-84-019.

Rawls, W.J.  1983.  Estimating Soil Bulk Density from Particle Size Analysis and Organic Matter Content.  Soil Sci. 135(2):123-125.

Richards, L.A.  1931.  Capillary Conduction of Liquids Through Porous Mediums.  Physics 1:318-333.

Rosenberg, N.J.  1974.  Microclimate:  The Biological Environment.  Wiley Interscience, New York.

Salhotra, A.M.  1986.  A Coupled Heat, Salt and Water Balance Model of Evaporation and Stratification in Saline Terminal Lakes:  An Application to the Dead Sea.  Ph.D. Dissertation.  Massachusetts Institute of Technology, Cambridge, MA.

Sauty, J.P.  1980.  An Analysis of Hydro Dispersive Transfer in Aquifers.  Water Resour. Res. 16(1):145-158.

Schnoor, J.L., C. Sato, D. McKechin and D. Sahoo.  1987.  Processes, Coefficients, and Models for Simulating Toxic Organics and Heavy Metals in Surface Waters.  EPA/600/3/3-87/015.  U.S. Environmental Protection Agency, Athens, GA.

Schwab, G.O., R.K. Frevent, T.W. Edminster, and K.K. Barnes.  1966.  Soil and Water Conservation Engineering.  J. Wiley and Sons, New York.

Scotter, D.R., and P.A.C. Raats.  1970.  Movement of Salt and Water near Crystalline Salt in Relatively Dry Soil.  Soil Sci. 109:170-178.

Shamir, U.Y., and D.R.F. Harleman.  1967.  Dispersion in Layered Porous Media.  J. of the Hydraulics Division, Amer. Soc. Civ. Engr. 93(HY5):237-260.

Smelt, J.H., M. Leistra, N.W.H. Houx, and A. Dekker.  1978.  Conversion Rates of Aldicarb and Its Oxidation Products in Soils.  I. Aldicarb Sulfone.  Pest. Sci. 9:279-285.

Smiles, D.E., J.R. Phillip, J.H. Knight, and D.E. Elrick.  1978.  Hydrodynamic Dispersion During Absorption of Water by Soil.  Soil Sci. Soc. Am. J. 42:229-234.

Smith, C.N., and R.F. Carsel.  1984.  Foliar Washoff of Pesticide (FWOP) Model:  Development and Evaluation.  J. Env. Sci. and Health.  B19(3):323-342.

Smith, C.N., R.A. Leonard, G.W. Langdale, and G.W. Baily.  1978.  Transport of Agricultural Chemicals from Small Upland Piedmont Watersheds.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/3-78-056.

Soil Conservation Service.  1972.  Hydrology.  Section 4, SCS National Engineering Handbook.  U.S. Department of Agriculture, Washington, DC.  NEH-Notice 4-102.

Spencer, W.F., W.A. Jury, and W.J. Farmer.  1984.  Importance of Volatilization as a Pathway for Pesticide Loss from Forest Soils.  In: Chemical and Biological Controls in Forestry.  American Chemical Society, Washington, DC. pp. 193-209.

Springer, E.P., and H.R. Fuentes.  1987.  Modeling Study of Solute Transport in the Unsaturated Zone.  U.S. Nuclear Regulatory Commission, Washington, DC., NUREC/CR-4615, LA-10730-MS, Volume 2.

Stamper, J.H., H.N. Nigg, and J.C. Allen. 1979.  Organophosphate Insecticide Disappearance from Leaf Surfaces:  An Alternative to First-Order Kinetics.  Environ. Sci. Technol., 13:1402-1405.

Stanhill, G.  1969.  A Simple Instrument for the Field Measurement of Turbulent Diffusion Flux.  J. Applied Meteorology 8:504-513.

Stewart, B.A., D.A. Woolhiser, W.H. Wischmeier, J.H. Caro, and M.H. Fere.  1976.  Control of Water Pollution from Cropland, Volume II:  An Overview.  U.S. Environmental Protection Agency, Athens, GA.  EPA-600/2-75-026b.

Streile, G.P.  1984.  The Effect of Temperature on Pesticide Phase Partitioning, Transport, and Volatilization from Soil.  Ph.D. Dissertation, University of California, Riverside.

Szeicy, G., G. Endrodi and S. Tajchman.  1969.  Aerodynamic and Surface Factors in Evaporation.  Water Resour. Res. 5(2):380-394.

Taylor, A.W.  1978.  Post-Application Volatilization of Pesticides under Field Conditions.  J. Air Pol. Control Assoc. 28:922-927.

Taylor, J.R.  1982.  An Introduction to Error Analysis.  University Science Books, Mill Valley, CA.

Thibodeaux, L.J., and H.D. Scott.  1985.  Air/Soil Exchange Coefficients.  In: Environmental Exposure from Chemicals, Vol. I., W.B. Neely and G.E. Blau (eds.), CRC Press, Inc., Boca Raton, FL.

Thibodeaux, L.J., C. Spencer, and L.M. Riley.  1982.  Models of Mechanisms for the Vapor Phase Emission of Hazardous Chemicals from Landfills.  J. Hazardous Materials 7:63-74.

Thibodeaux, L.J.  1979.  Chemodynamics: Environmental Movement of Chemicals in Air, Water, and Soil.  John Wiley & Sons, New York.

Thom, A.S., J.B. Stewart, H.R. Oliver, and J.H.C. Gash.  1975.  Comparison of Aerodynamic and Energy Budget Estimates of Fluxes over a Pine Forest.  Quarterly Journal of the Royal Meteorological Society 101:93-105.

Todd, D.K.  1970.  The Water Encyclopedia, Water Information Center, Port Washington, New York.

U.S. Department of Agriculture.  1980.  CREAMS:  A Field-Scale Model for Chemicals, Runoff and Erosion from Agricultural Management Systems.  W.G. Knisel, (ed.), United States Department of Agriculture, Washington, DC., Conservation Res. Rep. No. 26.

Varas, E.A. and R.K. Linsley.  1977.  Rainfall Synthesis with Scanty Data.  J. Hydrol. 34:235-249.

van Bavel, C.H.M., and D.I. Hillel.  1976.  A Simulation Study of Soil Heat and Moisture Dynamics as Affected by a Dry Mulch.  In:  Proceedings of 1975 Summer Computer Simulation Conference, San Francisco, California.  Simulation Councils, Inc., La Jolla, CA., pp. 815-821.

van Bavel, C.H.M. and D.I. Hillel.  1975.  Calculating Potential and Actual Evaporation from a Bare Soil Surface by Simulation of Concurrent Flow of Water and Heat.  Agri. Meteorol. 17:453-476.

van Genuchten, M. T., and W.J. Alves.  1982.  Analytical Solutions of the One-Dimensional Convective-Dispersive Solute Transport Equation.  U.S. Technical Bulletin No. 1661.  151 pp.

van Genuchten, M. T.  1976.  A Closed-form Equation for Predicting the Hydraulic Conductivity of Unsaturated Soils.  Soil Sci. Soc. J. 44:892-898.

van Genuchten, M.Th.  1978.  Mass Transport in Saturated-Unsaturated Media:  One Dimensional Solutions.  Princeton University, Princeton, NJ.  Res. Rep. 78-WR-11.

Wagenet, R.J., and J.W. Biggar.  1987.  Measurement and Interpretation of DBCP Fate in Agricultural Soils.  J. Environ. Qual. (in press).

Wagenet, R.J., and J.L. Hutson.  1987.  Leaching Estimation and Chemistry Model.  Cornell University, Ithaca, NY.

Wark, K., and C.F. Warner.  1976.  Air Pollution:  Its Origin and Control.  Harper and Row Publishers, New York.

Wehtje, G., L.N. Mielke, JH.R.C. Leavitt and J.S. Schepers.  1984.  Leaching of Atrazine in the Root Zone of an Alluvial Soil in Nebraska.  J. Environ. Qual. 13(4):507-513.

White, Jr., A.W., L.A. Harper, R.A. Leonard, and J.W. Turnball.  1977.  Trifluralin Volatilization Losses from a Soybean Field.  J. Environ. Qual. 6:105-110.

Wilke, O., and E.T. Smerdon.  1965.  A Solution of the Irrigation Advance Problem.  ASCE Journal of Irrigation and Drainage 91(IR3):23-24.

Williams, J.R., C.A. Jones, and P.T. Dyke.  1984.  A Modeling Approach to Determining the Relationship Between Erosion and Soil Productivity.  Trans. ASAE 27:129-144.

Williams, J.R.  1975.  Sediment Yield Prediction with Universe Equation Using Runoff Energy Factor.  In:  Present and Prospective Technology for Predicting Sediment Yields and Sources.  U.S. Dept. of Agriculture, Washington, DC.  ARS-S-40.

Willis, G.H., and L.L. McDowell.  1987.  Pesticide Persistance of Foliage.  In:  Reviews of Environmental Contamination and Toxicology, Vol. 100.  Springer-Verlag, New York.

Willis, G.W., L.L. McDowell, L.A. Harper, L.M. Southwick and S. Smith.  1983.  Seasonal Disappearance and Volatilization of Toxaphene and DDT from a Cotton Field.  J. Environ. Qual. 12:80-85.

Wilson, J.L., and P.J. Miller.  1978.  Two-Dimensional Plume in Uniform Groundwater Flow.  J. Hydraulics Div., ASCE 104(HY4).

Wischmeier, W.H. and D.D. Smith.  1978.  Predicting Rainfall Erosion Losses--A Guide to Conservation Planning.  Agriculture Handbook 537, U.S. Department of Agriculture,  Washington, DC., Agriculture Handbook 537.

Wolfe, N.L.  1990.  Abiotic Transformations of Toxic Organic Chemicals in the Liquid Phase and Sediments.  In:  Toxic Organic Chemicals in Porous Media.  Z. Gerstl, Y. Chen, U. Mingelgrin, and B. Yaron (eds.).  Springer-Verlag, New York, p. 136-147.

Wolfe, N.L.  1985.  Screening of Hydrolytic Reactivity of OSW Chemicals.  U.S. Environmental Protection Agency, Athens, GA.  Unpublished report.

Woodward-Clyde Consultants.  1988.  Background Document for EPA's Composite Landfill Model (EPACML).  U.S. Environmental Protection Agency, Washington, DC.  Unpublished report.

Woodward-Clyde Consultants.  1988.  Multimedia Exposure Assessment Model for Evaluating the Land Disposal of Hazardous Wastes.  U.S. Environmental Protection Agency, Athens, GA.  Unpublished report.

Yeh, G.T., and D.S. Ward.  1981.  FEMWASTE:  A Finite-Element Model of Waste Transport Through Saturated-Unsaturated Porous Media.  Rep. ORNL-5601, Oak Ridge National Laboratory, Oak Ridge, TN., Rep. ORNL-5601, 137 pp.

SECTION 10

APPENDICES

10.1  ERROR MESSAGES AND WARNINGS

The PRZM-2 code contains a number of error and warning messages that indicate either fatal or non-fatal routine conditions.  A list of the current error (fatal) and warning (non-fatal) conditions that are recognized by the code is given in Table 10-1.  Along with each message, troubleshooting approaches are described.  Error or warning messages originating in PRZM-2 (the main code) are numbered beginning with 1000; PRZM, 2000;

VADOFT, 3000; and the Monte Carlo module, 5000.  Note that error numbers less than 1000 may appear.  These numbers are being supplied by the FORTRAN compiler that was used to compile PRZM-2 and its associated modules.  These errors will probably be associated with reading input data; e.g., problems such as inappropriate characters in an input field that the code is attempting to interpret as an integer or a disk drive being unavailable for reading data.  Consult the compiler errors list for the exact cause.

Note also that, if the compiler used uses numbers in the range of 1000 to 5000 for these file access errors, an error number may appear that seems to be an EXESUP/PRZM/

VADOFT error.  The error message however, will not, correspond to the messages in Table 10-1.  The message will be something such as:  "Error in attempting to open file [<file name>]" or "Error in input....".  Again, check the compiler's run time error list for the exact cause.

When errors and warnings are reported in the output echo file, three lines of information are provided.  The first line reports the number and whether the condition was an error (fatal) or warning (non-fatal).  The second line supplies the associated message.  The third line supplies the subroutine trace of where the error occurred.  For example, the third line might be:  'PRZM2>INPREA>VADINP'.  This implies that the error occurred in the subroutine VADINP (the VADOFT input routine), which was called from subroutine INPREA, which was called from the PRZM-2 main program.  This third line will not appear if an error occurs in the routine INITEM, which is the routine to read the PRZM2.RUN file and initialize the simulation.

10.2  VARIABLE GLOSSARY

This section presents the major variables used in the PRZM-2 code.  Table 10-2 presents variables used in the EXESUP module, Table 10-3 presents PRZM variables, Table 10-4 presents VADOFT variables, and Table 10-5 presents variables used in the Monte Carlo module.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND

TROUBLESHOOTING APPROACHES

Error or

Warning				Troubleshooting Approach/Explanation

1010	Water table is above		The water table has accumulated to above

vadose zone				the top of the vadose zone.  Use higher

conductivities or increase the thickness

of the vadose zone.

1020	Water table is above	 	The water table is above the top of the

root zone.	 			root zone.  Use higher conductivities or

increase the thickness of the root zone.

1050	Zero or negative mass in		This is a warning only, the

VADOFT/PRZM nodes below	concentration values

the water table			in the VADOFT or PRZM nodes below the 						the water table will not be adjusted for

the current timestep.  If this warning

appears repeatedly, the VADOFT or PRZM

geometry might have to be adjusted.

1070	Error in the file name		An incorrect (or misspelled) identifier

input, line with....			was supplied for a file.

1090	Bad value [nnnn] for		The number of chemicals must be between

number of chemicals		1 and 3,inclusive.  Change the number in

the global data group of PRZM-2 input

file.

1092	Bad index [nnnn] of			An invalid index was provided for input

chemical				record EXESUP3 with ANAME =

'PARENT OF'.  Values less than 1 or

greater than NCHEM are not valid.

1100	Bad value [nnnn] for		Check input values.  Chemical 1 can have

chemical parent species		a parent of 0 only.  Chemical 2 can have 						a parent of 0 or 1.  Chemical 3 can have

a parent of 0, 1, or 2.

1190	Bad identifier reading		An invalid label appears in the global

global data [<value>]		data section (EXESUP) of the PRZM2.RUN 						input file.

1200	End date is before start		Check the 'START DATE' and 'END DATE'

date					records of PRZM2.RUN input file.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

1190	Bad identifier reading		An invalid label appears in the global

global data [<value>]		data section (EXESUP) of the PRZM2.RUN

input file.

1190	Bad identifier reading		An invalid label appears in the global

global data [<value>]		data section (EXESUP) of the PRZM2.RUN

input file.

1200	End date is before start		Check the 'START DATE' and 'END DATE'

date					records of PRZM2.RUN input file.

1202	End date and start date		Check the 'START DATE' and 'END DATE'

are the same				records of PRZM2.RUN input file.

1210	Unrecognized label			A record in the PRZM2.RUN file appears

[<label>] while attempting		after the 'ENDDATA' record before the

to read ECHO or TRACE		'ECHO' or 'TRACE' records.

1220	Echo level not defined;		No output echo level was specified in the 	set to 5 [or 1]			global parameter file.  The value was set 						to 5 if MONTE CARLO was not selected or 1 						if MONTE CARLO was selected.

1230	Trace level not defined; 		No subroutine trace level was specified in

set to 0				the global parameter file.  The value was 						set to the default value of 0.

1240	End of file on PRZM-2		Recheck the global data group of the

run file				PRZM-2 input file.  There is an error in

the input sequence; an option was set

which required more lines of data than

supplied.

1250	Error reading PRZM-2 		Error in reading PRZM-2 input data, most 	run file....				likely there are inappropriate characters 						in a data field that is attempting to be

interpreted as integer data.

1260	File type ['nn'] has			A file with the same unit number has been

already been specified		open while PRZM-2 is running.  Should

never occur in current version of PRZM-2.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

1270	Too many files requested 		The maximum number of files allowed

to be open at once			(defined in the include file IOUNITS.PAR) 						is too small a number for the (recently

modified) version of PRZM-2.  This error

should not appear in the current version

of PRZM-2.

1250	Error reading PRZM-2 		Error in reading PRZM-2 input data, most 	run file....				likely there are inappropriate characters 						in a data field which is attempting to be 						interpreted as integer data.

1260	File type ['nn'] has			A file with the same unit number has been

already been specified		open while PRZM-2 is running.  Should

never occur in current version of PRZM-2.

1270	Too many files requested 		The maximum number of files allowed

to be open at once			(defined in the include file IOUNITS.PAR) 						is too small a number for the (recently

modified) version of PRZM-2.  This error

should not appear in the current version

of PRZM-2.

1280	ENDFILE statement present	An input file, which is required for the

before file [nn] was			current PRZM-2 simulation configuration,

opened				has not been identified in the file group

of the PRZM-2 input file.

1290	Request to close file 		Should never occur in current version of

[nn] which was not open		PRZM-2. Implies that recent code

modifications have been made which did not 						properly account for which files were

open.

1300	Unknown unit number to		Implies that recent code modifications

open file				have been made which did not properly

account for which files were open.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

1310	Too many lines required		Should never occur in current version of

for Trace option			PRZM-2.  Implies that recent code

modifications have been made resulting in

a very high level of subroutine nesting.

1320	Argument [<value>] too	 	Attempt to take the exponential of too

large for EXP			large a real number.

1330	Negative or zero argument		Attempt to take the log of a zero or

[<value>]				negative argument.

1350	Single precision overflow		A mathematical operation resulted in a

number too large for the real value being

calculated.

1360	Negative argument			Attempt to take the square root of a

[<value>] to SQRT			negative number.  Subroutine trace

accompanying error message will show in

which routine the error occurred.

1390	Invalid index [nnnn] 		A bad index value in a VADOFT read,

in reading record			probably initial condition data.

[<record number>]

1400	Error reading PRZM data		Probable causes are inappropriate

characters in an input field for integer

or real reads.

1500	ENDDATA before starting		The label 'ENDDATA' appears in the global

end day was provided		day was provided parameters section of	 						PRZM2.RUN file before the record was

provided.

1510	ENDDATA before end day was	The label 'ENDDATA' appears in the global

provided				parameters section of PRZM2.RUN file

before the 'END DATE' record was provided.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

1530	ENDDATA before number of	The label 'ENDDATA' appears in the global

chemicals was provided		parameters section of PRZM2.RUN file (with

TRNSIM = 'ON') before the 'NUMBER OF 						CHEMICALS' record was provided.  The 						'NUMBER OF CHEMICALS' record is required 						for a transport simulation.

1540	ENDDATA before the parent 	The label 'ENDDATA' appears in the global 	of chemical n				parameters section of PRZM2.RUN file (with

was provided				TRNSIM = 'ON' and NUMBER OF 						CHEMICALS greater than 1) before the 						'PARENT OF n' record was provided.

1550	dd/mm/yy - Invalid START		An invalid date has been entered in the

(or END) DATE			global parameters section of the PRZM2.RUN

input file.  Check to see whether the month

being specified had the number of days

which is being implied (e.g., 31/02/88 is

not valid).

1560	End of file [<file			The end of the file specified was reached

identifier>] encountered 		while still attempting to read data.

1570	Monte Carlo simulation -		If an echo level greater than 3 is Echo

Level reset to 1			requested with Monte Carlo on, the echo

level will be reset to 1.  No action 						required.

2000	Simulation date (dd/mm/yy),	The meteorological data file is not 	meteorological date			aligned with the simulation data.  There

(dd/mm/yy) do not match		is probably a missing match record in the 						data file or the simulation start and end 						dates specified in PRZM2.RUN do not 						correspond to the dates in the 						meteorological data file.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

2010	Number of chemicals in		The value supplied to the PRZM input file 	PRZM [NN] <> number of		for the number of chemicals being 	chemicals in EXESUP [nn]		simulated does not agree with the number

supplied to the PRZM2.RUN input file.

2040	NPI [nnnn] + NEW [nnnn] is	Decrease the number of PRZM compartments 	greater than NPII [nnnn]		or increase the parameter NPII.  If the 						latter, in subroutine MOC recompile the 						code.  This error only occurs if the MOC 						rather than backward difference transport

solution technique is used.

2050	Solution for tridiagonal		If this message appears repeatedly, the

matrix not found, previous		PRZM problem definition geometry should be

day's values used			reevaluated.

2060	NDC [nnnn] is greater than	Change PRZM problem definition geometry so 	NC [nnnn]				that the input value of NDC is less than 						or equal to the parameter NC or change the 						value of NC and recompile.

2070	NCPDS [nnnn] is greater		Change PRZM problem definition geometry so 	than NC [nnnn]			that the input value of NCPDS is less than 						or equal to the parameter NC or change the 						value of NC and recompile.

2080	NAPS [nnnn] is greater		Change PRZM problem definition geometry so 	than NAPP [nnnn]			that the input value of NAPS is less than 						or equal to the parameter NAPP or change 						the value of NAPP and recompile.

2090	NHORIZ [nnnn] is greater		Change PRZM problem definition geometry so

than NCMPTS [nnnn]		that the input value of NHORIZ is less 						than or equal to the parameter NCMPTS or 						change the value of NCMPTS and recompile.

2100	NCOM2+1 [nnnn] is greater	Change PRZM problem definition geometry so 	than NCMPTS [nnnn]		that the input value of NCOM2 is less than 						the parameter NCMPTS or change the value 						of NCMPTS and recompile.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

2110	NPLOTS [nnnn] is greater		Reduce the number of requested plots.

than 7

2120	Sum of horizon thicknesses	Change PRZM problem definition geometry so 	exceeds depth			that the sum of horizon thickness is equal 						to the user supplied total depth.

2130	Soil profile description		Change PRZM problem definition file so

is incomplete, data			that profile data are supplied for the

available for xx.xx of		entire depth.

xx.xx cm

2140	Calculated field capacity		Either decrease the soil bulk density or

water content exceeds the		adjust the parameters for calculating

saturation value			field capacity water content (if THFLAG=1)

or lower the supplied value of field

capacity water content (if THFLAG=0).

2150	Application [nn] failed		The specified pesticide application did

to meet ideal soil			not meet soil moisture criteria before the 	conditions				WINDAY value expired.  Currently this 						error will halt execution.

2160	WINDAY [nn] for			The value for WINDAY, specified in the 	application [nn]			PRZM input sequence, causes overlap on a 	is too large				proceeding application date.  Reduce the 						value for WINDAY to a value lesser than 						the difference of application dates.

3000	Fatal error in HFINTP,		The current time in VADOFT exceeds the 	interpolation failed			supplied values of the interpolation time 						vector in attempting to interpolate head 						or flux values.  This error should not 						occur when running VADOFT in linked mode.  						If running VADOFT alone, increase the 						number of time periods of the 						interpolation time and head/flux vectors.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

3010	VARCAL - timestep nnn		The maximum number of time refinements was

solution fails to converge		exceeded due to non-convergence.  Relax

after nnn reductions		the converge criterion, change the 						iterative scheme or revise VADOFT 						parameters.

3020	Attempt to run VADOFT		The user has attempted to run VADOFT with 	w/PRZM on and ITRANS		PRZM on and ITRANS not equal to one.  Set 	.ne.1					ITRANS to 1 and make the appropriate 						changes to the VADOFT parameter file.

3030	Incorrect value for IMODL 	An incorrect value has been entered for

3040	in VADOFT input			IMODL in the VADOFT input file.  Check the 						values entered;  IMODL = 0 for transport, 						IMODL = 1 for flow.

3050	Requested value of NOBSND	The value entered for the number of

[nnnn] greater than			observation nodes in VADOFT (NOBSND)

MXPRT [nnnn]			exceeds the maximum (MXPRT).  Reduce the 						number of observation nodes or increase 						MXPRT in the PARAMETER statement.  If the 						latter, recompile the model.

3060	Transport simulation, 		The value of NVREAD supplied by the user 	NVREAD reset to 1			was reset to 1 since a transport 						simulation was requested; no action 						required.

3070	PRZM is on; IVSTED reset 	The value of IVSTED supplied by the user 	to 1					was reset to 1; no action required.

3080	PRZM is on; flow boundary		If PRZM is on and linked to VADOFT, a

conditions will be over-		prescribed flux b.c. will be used at the 	written				VADOFT top node.  Daily values of water 						and solute flux are generated by PRZM.  						Related boundary conditions in the VADOFT 						impact file are overwritten.  IBTND1 is 						set to 0; no action required.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

3090	PRZM is on; transient data	If PRZM is on, any transient flow data

at top node ignored			relevant to VADOFT's upper boundary is 						overwritten.  ITCND1 is set to 0; no 						action required.

3120	PRZM is on; transport		PRZM output will overwrite VADOFT upper 	boundary conditions will		boundary condition for solute transport.  						PRZM generates be overwritten daily volume 						of solute flux.  IBTNDI is set to 0.  No 						action required.

3130	PRZM is on; transient		If PRZM is on, any transient solute flux

data at top node ignored		data the user has input for the upper 						boundary in VADOFT is ignored.  ITCNDN is 						set to 0.  No action required.

3170	Invalid index [nnn] in		An invalid index (less than 1 or greater 	reading PINT			than the parameter NP) was supplied for an 						initial condition value.  Supply proper 						value.

3190	ITMGEN<>1 in linked mode,	The user is supplying output marker time 	results may be unpredict-		values that, potentially, could result in 	able					a read error of Darcy velocities during 						the VADOFT transport simulation.

3210	End of file reading			Check to see whether warning 3190 occurred

VADOFT Darcy velocities		prior to this fatal error.  Make necessary 						changes to VADOFT input file.

5000	Format error in reading		Check Monte Carlo input file.  Illegal

Monte Carlo input file		characters are in inappropriate data file 						columns.

5010	Premature end of			Check Monte Carlo input file.

Monte Carlo input file		Insufficient data lines have been provided 						given the users problem definition.

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

5020	Uniform random number		Random exponential distribution variates 	could not be generated for		could not be generated.  Probable cause is 	exponential distribution		inappropriate distribution parameters 						being supplied in the Monte Carlo input 						file.

5030	Cannot have a negative		A negative mean was calculated for a log 	mean for a log normal		normal distribution.  Check distribution 	distribution.  Mean			parameters supplied in the Monte Carlo

equals <value>			input file.

5040	Subroutine DECOMP		Monte Carlo solution matrix could not be

terminated, matrix BBT		decomposed.  Check distribution parameters

is not positive definite		supplied in Monte Carlo input file.

5050	The number of [MONTE CARLO	Too large a value was chosen for the 	RUNS] is greater than		number of Monte Carlo runs.  Reduce number 	maximum of <value>		in input file or change NRMAX in parameter 						file and recompile.

5060	The number of [MONTE CARLO	Reduce number in input file or change 	VARIABLES] is greater than	MCMAX and recompile.

maximum of <value>

5070	The number of [EMPIRICAL	Reduce number in input file or change NEMP 	DIST. DATA POINTS] is		and recompile.

greater than maximum of

<value>

5080	The number of [MONTE CARLO	Reduce number in input file or change NMAX 	OUTPUT VARIABLES] is		and recompile.

greater than maximum of

<value>

5090	The number of [DAYS IN		Reduce number in input file or change 	OUTPUT AVG. PERIOD] is	NPMAX and recompile.

greater than maximum of

<value>

TABLE 10-1.	PRZM-2 ERROR MESSAGES, WARNINGS, AND TROUBLESHOOTING APPROACHES (Continued)

Error or

Warning				Troubleshooting Approach/Explanation

5100	The number of [REQUESTED	Reduce number in input file or change 	OUTPUT CDFS] is greater		NCMAX and recompile.

than maximum of <value>

5110	First element for horizon		The PRZM horizon value provided for a

[<value>] not found			variable defined in the Monte Carlo input 						file is probably invalid (does not match 						the PRZM horizon/element number 						description provided in the PRZM input 						file).

TABLE 10-2.	EXESUP PROGRAM VARIABLES

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

BASEND	--	Scalar	Number of bottom PRZM node	EXESUP	--	M

within a given PRZM zone.

BOTFLX	cm day-1	Array	Water flux from VADOFT base	EXESUP	VADSTO	M

node for each timestep.

DAFLUX	q cm-2	Array	Dispersive-advective flux	EXESUP	PRZSTO	M

day-1	at each PRZM node in each

zone for each chemical

(positive).

DAVFLX	ppm cm	Array	Nodal values of dispersive	EXESUP	VADSTO	M

day-1		advective flux from VADOFT.

DISUNS	ppm	Array	Temporary storage of	EXESUP	--	M

(q cm-3)		VADOFT (or PRZM) nodal

concentrations for mass

correction computations.

EDAT	--	Array	Ending day, month, year of	EXESUP	--	M

PRZM simulation.

FLOSIM	--	Logical	Flow simulation indicator.	EXESUP	--	M

ICHEM	--	Scalar	Counter for number of	EXESUP	--	M

chemicals.

IDAYO	--	Scalar	Starting day of PRZM	EXESUP	--	M

simulation.

ILDLT	--	Scalar	Counter for PRZM or VADOFT	EXESUP	--	M

timesteps.

IMONO	--	Scalar	Starting month of PRZM	EXESUP	--	M

simulation.

IPRZM	--	Scalar	Counter for number of PRZM	EXESUP	--	M

zones.

IPZONE	--	Scalar	Counter for VADOFT zones.	EXESUP	--	M

IYRO	--	Scalar	Starting year of PRZM	EXESUP	--	M

simulation.

LLSTS	days	Scalar	Number of days in final	EXESUP	--	I

timestep.	INITEM		O

NCHEM	--	Scalar	Number of chemicals.	EXESUP	--	I

INPREA		I

INITEM		O

TABLE 10-2.	EXESUP PROGRAM VARIABLES (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

NDAYS	days	Scalar	Number of days in a	EXESUP	--	M

timestep minus one.

NLDLT	--	Scalar	Number of PRZM or VADOFT	EXESUP	--	I

timesteps.	INPREA		I

INITEM		O

NP	--	Scalar	Total number of nodes.	EXESUP	CONTR2	I

NPNARY	--	Array	Number of VADOFT nodes	EXESUP	--	M

in all VADOFT zones.

NPRZM	--	Scalar	Number of PRZM zones.	EXESUP	--	I

INPREA		I

INITEM		O

NPV	--	Scalar	Number of VADOFT nodes	EXESUP	--	I

in a given zone.

NPZONE	--	Scalar	Number of VADOFT zones.	EXESUP	--	I

INPREA		I

INITEM		O

NPZ	--	Scalar	Temporary storage for the	EXESUP	--	M

amount number of PRZM or

VADOFT nodes.

PINT	L	Array	VADOFT corrected values	EXESUP	VADSTO	M

M/L**3		of head or concentration.

PRZMON	--	Logical	PRZM on indicator.	EXESUP	--	I

INPREA		I

INITEM		O

PRZMPF	q cm-2	Array	Daily chemical flux from	EXESUP	PRZSTO	M

day-1		the base of PRZM.

PRZMWF	cm day-1	Array	Daily water flux from the 	EXESUP	PRZSTO	M

base of PRZM.

P2VWHT	--	Array	Weighting factors for trans-	EXESUP	ZONWHT	M

fer of water or chemical

flux from PRZM to VADOFT.

REDAT	--	Array	Ending day, month, year of	EXESUP	--	M

PRZM simulation within a

timestep.

TABLE 10-2.	EXESUP PROGRAM VARIABLES (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

RSDAT	--	Array	Starting day, month, year	EXESUP	--	M

of PRZM simulation within

timestep.

RSTFG	--	Scalar	PRZM restart flag, 1 if	EXESUP	--	M

first time through, 2

thereafter.

SAVCNC	ppm	Array	Concentrations at each	EXESUP	VADSTO	M

VADOFT node from previous

timestep.

SAVHED	cm	Array	Previous timestep VADOFT 	EXESUP	VADSTO	M

heads by node.

SDAT	--	Array	Starting day, month, year	EXESUP	--	M

of PRZM simulation.

TOPFLX	cm day-1	Array	Weighted water (or pesti-	EXESUP	VADSTO	M

(g cm-2		cide) flux leaving the

day-1)		base of PRZM.

TOWFLX	cm day-1	Array	Water flux from PRZM to top	EXESUP	VADSTO	M

of VADOFT for each timestep.

TRNSIM	--	Logical	Indicator for flow and	EXESUP	--	I

transport simulation.	INPREA		I

INITEM		O

VADFON	--	Logical	VADOFT on indicator.	EXESUP	--	I

INPREA		I

INITEM		O

VD2TC	--	Array	VADOFT correction factors	EXESUP	VADSTO	M

for converting from dissolved to

total solute concentration.

WHGT	--	Scalar	Temporary variable for	EXESUP	--	M

storing flux weighting

factors.

ZPESTR	g cm-2	Array	PRZM chemical flux by zone,	EXESUP	PRZSTO	M

day-1		compartment, time period,

and chemical.

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

A	day-1	Array	Lower Diagonal Element	SLPEST	PEST	O

of Solution Matrix (I-1)	TRDIAG		I

AAA	cm-1	Scalar	A Variable Used to	SLTEMP		M

Calculate the Average

Temperature Gradient

in the Top Compartment

ABSOIL	fraction	Scalar	Daily Value of Soil	SLTEMP		M

Surface Albedo

AD	day-1	Array	Soil Horizon Drainage	READ	HYDR	O

Parameter	ECHO		I

INITL		I

HYDR2		I

ADFLUZ	g cm-2	Array	Advective Flux of 	SLPEST	PEST	O

day-1		Pesticide	MASBAL		I

OUTPST		I

OUTTSR		I

ADS	mg kg-1	Array	Adsorbed Portion of 	OUTCNC

Pesticide in Each

Compartment

AFIELD	ha	Scalar	Area of Field	READ	HYDR	O

EROSN

AINF	cm	Array	Percolation Into Each	HYDROL	HYDR	O

Soil Compartment	HYDR1		I

HYDR2		I

AIRDEN	gm cm-3	Scalar	Density of Air at	SLTEMP		M

Ambient Temperature

AIRLMD	cal cm-1	Scalar	Thermal Conductivity	SLTEMP

day-1 °C-1	of Air

AKAY	--	Array	K-Factor in the Soil	SLTEMP		M

Thermal Conductivity

Equation

ALAMDA	cal cm-1	Array	Thermal Conductivity	SLTEMP

day-1 °C-1		of Soil Constituent

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

ALBEDO	fraction	Array	Soil Surface Albedo at	READ	MET	O

Start of Each Month	SLTEMP		I

AMXDR	cm	Scalar	Maximum Rooting Depth	READ	CROP	O

of Each Crop	INITL		I

PLGROW		I

ANETD	cm	Scalar	Minimum Depth from Which	READ	CROP	O

ET is Extracted Year	INITL		I

Around

ANUM	cm	Scalar	Total Available Water	EVPOTR

in Profile

APD	--	Scalar	Day of Month of Pesticide	READ

Application

APDEP	cm	Scalar	Depth of irrigation water	IRRIG	  --	O

applied to soil

APM	--	Scalar	Month of Pesticide	READ

Application

ATEMP	°C	Array	Air Temperature	Main		O

AVSTOR	cm3 cm-3	Scalar	Available Water Storage	HYDR2

AW	--	Scalar	Fraction of Soil Voids	EVPOTR

Occupied by Water

B	day-1	Array	Diagonal Element of 	SLPEST	PEST	O

BBB	°K cm-1	Scalar	A Variable Used to	SLTEMP		M

Calculate the Average

Temperature Gradient

in the Top Compartment

BBT	°C	Array	Bottom Boundary Temperature	READ	MET	O

at Start of Each Month	SLTEMP		I

BD	g cm-3	Array	Mineral Soil Bulk Density	SLTEMP	HYDR	I

BDFLAG	--	Scalar	Bulk Density Flag (0 =	READ		O

Whole Soil BD Entered,	ECHO		I

1 = Mineral BD and OC	INITL		I

Entered)

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

BT	m	Scalar	Bottom width of furrows	FURROW	IRGT	I

IRREAD		O

C	day-1	Array	Upper Diagonal Element of	SLPEST	PEST	O

Solution Matrix (I+1)	TRDIAG		I

CB	kg ha-1	Scalar	Cumulative Pesticide	OUTPST

Balance Error

CC	g	Array	Total mass associated 	MOC1 	PEST	M

with a moving point	INITL

CELLBG	--	Scalar	First location in a 	INITL		M

compartment

CEVAP	cm	Scalar	Current Daily Canopy	EVPOTR	HYDR	O

Evaporation Depth	MASBAL		I

OUTHYD		I

OUTTSR		I

CFLAG	--	Scalar	Conversion Flag for	READ	MISC	O

Initial Pesticide Input	INITL		I

CHANGE	g	Array	Change in total pesticide	MOC1		M

mass per compartment

CINT	cm	Scalar	Current Crop Interception	INITL	HYDR	O

Storage	HYDROL		I

EVPOTR		I

MASBAL		I

OUTHYD		I

OUTTSR		I

CINTB	cm	Scalar	Crop Interception From	PMAIN	HYDR	O

Previous Time Step	MASBAL		I

OUTHYD		I

CINTCP	cm	Array	Maximum Interception	READ	CROP	O

Storage of Each Crop	ECHO		I

PLGROW		I

CLAY	percent	Array	Percent Clay in Each Soil	SLTEMP	HYDR	I

Horizon

CONC	--	Alpha-	Flag for Output of Soil	PMAIN

numeric	Pesticide Concentration

Profile

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

CONDUC	cm day-1	Scalar	Canopy Conductance	MAIN	PEST	O

Including Boundary	SLPST0		I

Layer's Conductance	SLPST1		I

CONST	--	Scalar	Constant Values Used to	READ		O

Multiply Each Time Series	ECHO		I

Output	OUTTSR		I

CORED	cm	Scalar	Total Depth of Soil	READ	HYDR	O

Profile	ECHO		I

INITL		I

COVER	fraction	Scalar	Current Areal Cover of	SLTEMP	CROP	I

Crop Canopy

COUNT	--	Array	Number of moving points	MOC1		M

in a compartment

COVMAX	fraction	Array	Maximum Areal Coverage	READ	CROP	O

of Each Crop at Full	ECHO		I

Canopy Development	INITL		I

PLGROW		I

CN	--	Array	Runoff Curve Numbers for	READ	HYDR	O

Antecedent Soil Moisture	ECHO		I

Condition II	HYDROL		I

CNCPOND	g cm-3	Scalar	Concentration of 	MOC1 	PEST	I

pesticide in inflowing 	INITL

water

CNDBDY	cm day-1	Scalar	Boundary Layer's	MAIN		O

Conductance

CNDM	--	Array	Accumulated Number of 	PMAIN

Days in Each Month (With

and w/o Leap Year)

CNDMO	--	Array	Accumulated Number of	SLTEMP	MISC	I

Days in Each Month

CPBAL	g cm-2	Scalar	Cumulative Pesticide	MASBAL	PEST	M

Balance Error

OUTPST		I

CRC	day m-1	Array	Canopy Resistance	CANOPY		O

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

CRCNC	day m-1	Array	Canopy Resistance	MAIN	PEST	I

OUTPST		O

CTOT	g	Scalar	Concentration of 	MOC	-	M

consolidated points

CURVN	--	Scalar	Current Value of Runoff	HYDROL

Curve Number

CWBAL	cm	Scalar	Cumulative Water Balance	MASBAL	HYDR	M

Error	OUTHYD		I

D	m	Scalar	Zero Displacement Height	CANOPY		O

SLTEMP		M

DAIR	cm2 day-1	Scalar	Molecular Diffusivity	ECHO	PEST	I

in the Air	MAIN		I

READ		O

SLPST0		I

SLPST1		I

DAY	--	Alpha-	Flag for Daily Output of	PMAIN

numeric	Water or Pesticide

Summary

DELT	day	Scalar	Time Step	INITL	MISC	O

HYDR2		I

PLPEST		I

SLPEST		I

MASBAL		I

DELTA	°K	Scalar	Convergence Criteria 	SLTEMP		M

in the Newton-Raphson

Solution Technique

DELX	cm	Array	Compartment Thickness	SLTEMP	HYDR	I

DELXSQ	cm-2	Scalar	Compartment Thickness	INITL	HKYDR	O

Squared	SLPEST

DEN	-	Array	Point density.  The 	INITL	HYDR	M

number of points in the

horizon divided by the

depth of the horizon.

DENOM	cm	Scalar	Total Voids in the Soil	EVPOTR

Profile

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

DENOM	cm hr-1	Scalar	Available Water for Runoff	EROSN

During a Storm

DEPI	cm	Array	Depth of Pesticide	READ	PEST	O

Incorporation	ECHO		I

PESTAP

DFFLUX	g cm-2	Array	Diffusive/Dispersive	SLPEST	PEST	O

day-1		Flux of Pesticide Leaving	OUTPST		I

Each Soil Compartment	OUTTSR		I

DGAIR	cm2 day-1	Array	Molecular Diffusivity	SLPST0		I

in the Soil Air Pore	SLPST1		I

DGRATE	day-1	Array	First Order Decay Rate	ECHO	PEST	I

for Vapor-Phase Pesticide	INITL		I

READ		O

SLPST0		I

SLPST1		I

DIFFCH	m2 day-1	Scalar	Eddy Diffusivity at	CANOPY		O

Canopy Height

DIFFCO	cm2 day-1	Array	Diffusivity of Soil	SLTEMP		M

Compartment

DIFK	m2 day-1	Scalar	Eddy Diffusivity	CANOPY		O

DIN	cm	Scalar	Current Plant Canopy	PLGROW	HYDR	O

Interception Potential	HYDROL		I

OUTHYD		I

DISP	cm2	Array	Dispersion/Diffusion	READ	PEST	O

day-1		Coefficient	ECHO		I

INITL		I

SLPEST		I

DISS	mg l-1	Array	Dissolved Portion of	OUTCNC

Pesticide in Each

Compartment

DKFLUX	g cm-2	Array	Decay Flux of Pesticide	SLPEST	PEST	O

From Each Compartment	MASBAL		I

OUTPST		I

OUTTSR		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

DKRATE	day-1	Array	Pesticide Decay Rate in	READ	PEST	O

Each Soil Horizon	ECHO		I

INITL		I

SLPEST		I

DKRT12	day-1	Array	Transformation Rate from	ECHO	PEST	I

Parent Pesticide to First	READ		O

Daughter Product	INITL		O

PSTLNK		I

DKRT13	day-1	Array	Transformation Rate from	ECHO	PEST	I

Parent Pesticide to Second	READ		O

Daughter Product	INITL		O

PSTLNK		I

DKRT23	day-1	Array	Transformation Rate from	ECHO	PEST	I

First Daughter Product to	READ		O

Second Daughter Product	INITL		O

PSTLNK		I

DOM	--	Scalar	Number of Current Day of	SLTEMP	MISC	I

Month of Simulation

DPN	cm	Array	Layer Depth in Each	ECHO	HYDR	I

Horizon	READ		O

DT	hr	Array	Average Hours of Daylight	READ	MET	O

for a Day Falling in Each	ECHO		I

Month	EVPOTR		I

DVF	kg ha-1	Scalar	Daily Foliage Pesticide	OUTPST		O

day-1		Volatilization Flux

DW	Fraction	Scalar	Available porosity in soil	IRRIG	IRGT	O

column	FURROW

DX	m	Scalar	Spatial stop used in furrow	FURROW	IRGT	M

finite difference model	IRRIG		I

EF	kg ha-1	Scalar	Daily Erosion Flux	OUTPST

ELTERM	day-1	Scalar	Erosion Loss Term for 	EROSN	PEST	O

Pesticide Balance	SLPEST		I

EMD	--	Scalar	Day of Month of Crop	READ

Emergence	ECHO

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

EMM	--	Scalar	Month of Crop Emergence	READ

ECHO

EMMISS	fraction	Scalar	Infrared Emissivity of	READ	MET	O

Soil Surface	SLTEMP		I

EN	--	Scalar	Manning's roughness	FURROW	IRGT	I

coefficient for furrows	IRREAD		O

ENP	Kcal	Scalar	Enthalpy of Vaporization	KHCORR		I

mole-1

ENPY	Kcal	Array	Enthalpy of Vaporization	ECHO	PEST	I

mole-1				MAIN		I

READ		O

ENRICH	--	Scalar	Enrichment Ratio for	EROSN

Organic Matter

ERFLAG	--	Scalar	Erosion Flag (0= Not	READ	HYDR	O

Calculated, 1= Calculated)	PMAIN		I

ERFLUX	g cm-2	Scalar	Erosion Flux of Pesticide	SLPEST	PEST	O

From Soil Surface	MASBAL		I

OUTPST		I

EVAP	cm day-1	Scalar	Daily Evaporation from the	SLTEMP		M

Top 5 cm of Soil After

Adjusting for Crop

evapotranspiration

EXTRA	cm3 cm-3	Scalar	Extra Water Occurring in	OUTTSR		I

a Compartment Over the 	HYDR2

Allowed Saturation Amount

F	g cm-2	Array	Vector of Source Terms	SLPEST	PEST	O

day-1		for Each Compartment	TRDIAG		I

(Tri-diagonal Matrix)

F0/	kg ha-1	Scalar	Current Foliar Pesticide	OUTPST

Storage

FAIH	--	Scalar	Stability Function for	CANOPY		O

Sensible Heat

FAIM	--	Scalar	Stability Function for	CANOPY		O

Momentum

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

FAM	--	Scalar	Pesticide Application	READ	PEST	O

Flag (1= Soil, 2= Linear	ECHO		I

Foliar, 3= Exponential	PESTAP		I

Foliar)

FC	cm	Array	Field Capacity Water	INITL	HYDR	O

Depth in Soil Compartment	EVPOTR

FCV	--	Array	Regression Coefficients	THCALLC

for Prediction of Field

Capacity Soil Water

Content

FDAY	--	Scalar	Loop Limit, First Day	PMAIN

FEXTRC	cm-1	Scalar	Foliar Extraction Coef-	READ	PEST	O

ficient for Foliar Wash-	ECHO		I

off Model	PLPEST		I

FILTRA	m2 kg-1	Scalar	Filtration Parameter 	READ	PEST	O

for Exponential Foliar 	ECHO		I

Application Model	PESTAP		I

FIRST	-	Scalar	Index of first point	MOC	HYDR	M

under interface with

Ratio greater than 2

FL	kg ha-1	Scalar	Foliar Pesticide Decay	OUTPST

Loss

FLEACH	Fraction	Scalar	Leaching factor, as 	IRRIG	IRGT	I

fraction of soil moisture	IRREAD		O

deficit

FOLP0/	g cm-2	Scalar	Foliar Pesticide Storage	PLPEST	PEST	O

From Previous Time Step	MASBAL		I

OUTPST		I

PMAIN		I

FP	kg ha-1	Scalar	Current Daily Foliar	OUTPST

Pesticide Storage

FPDLOS	g cm-2	Scalar	Current Daily Foliar	PLPEST	PEST	O

Pesticide Decay Loss	MASBAL		I

OUTPST		I

OUTTSR		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

FPVLOS	g cm-2	Array	Daily Foliage Pesticide	MASBAL	PEST	I

day-1		Volatilization Flux	OUTPST		I

PLPEST		O

FPWLOS	g cm-2	Scalar	Current Daily Pesticide	PLPEST

Washoff Loss

FRAC	--	Scalar	Fraction of the Distance	READ

a Curve Number is Between

Increments of Ten

FRAC	--	Scalar	Fraction of the Current	PLGROW

Crop Growing Season

Completed

FRAC	--	Array	Number of Compartments	EVPOTR

Available to Extraction

of ET

FRACOM	--	Scalar	Fraction of Layer Attri-	INITL

buted to the Current

Horizon

FS	m	Array	Infiltration depth at each	FURROW	IRGT	O

station in furrow	IRRIG		I

FX1	°K4	Scalar	Fourth Order Energy	SLTEMP		M

Balance Equation in

Terms of Soil Surface

Temperature

FX2	°K3	Scalar	Derivative of Energy	SLTEMP		M

Balance Equation in

Terms of Soil Surface

Temperature

GAMMA	--	Array	Pesticide Uptake Effi-	PLGROW	PEST	O

ciency by Plant	SLPEST		I

GEE	Fraction	Array	Depolarization Factors of	SLTEMP		M

Soil Constituent in Three

Dimensions

GFLD	Fraction	Scalar	Depolarization Factor of	SLTEMP		M

Entrapped Air at Field

Capacity Water Content

GRADT	°C m-1	Scalar	Temperature Gradient	CANOPY		O

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

GRADW	day1	Scalar	Wind Speed Gradient	CANOPY		O

HAD	--	Scalar	Day of Month of Crop	READ

Harvest	ECHO

HAM	--	Scalar	Month of Crop Harvest	READ

ECHO

HEIGHT	cm	Scalar	Canopy Height	MAIN	CROP	I

OUTPST		I

PLGROW		O

SLTEMP		I

HENRY	cm3 cm-3	Scalar	Henry's Constant	KHCORR		I

HENRYK	cm3 cm-3	Array	Henry's Constant	ECHO	PEST	I

MAIN		I

READ		O

HF	m	Scalar	Green-Ampt Suction head	FURROW	IRGT	I

parameter	INFIL		I

IRREAD		O

HGT	m	Scalar	Thickness of Each Layer	CANOPY		O

in the Canopy

HORIZN	--	Array	Soil Horizon Number	READ	MISC	O

ECHO		I

INITL		I

OUTHYD		I

OUTPST		I

OUTCNC		I

HSWZT	--	Scalar	Hydraulics Flag (O= Free	READ		O

Draining Soils, 1= Res-	ECHO		I

tricted Drainage)	INITL		I

PMAIN		I

HTEMP	°C	Scalar	Average Air Temperature	CANOPY		O

HTITLE	--	Alpha-	Comment Line to Enter	READ

numeric	Information about Hydro-	ECHO

logy Parameters

HTMAX	cm	Array	Maximum Canopy Height	ECHO	CROP	I

PLGROW		M

READ		O

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

I	--	Scalar	Loop Counter	SLTEMP

KHCORR

CANOPY

IAPDY	--	Array	Julian Day of Pesticide	READ	MISC	O

Application	ECHO		I

PMAIN		I

IAPYR	--	Array	Year of Pesticide	READ	MISC	O

Application	ECHO		I

PMAIN		I

IARG	--	Array	Argument of Variable	READ	MISC	O

Identified by 'PLNAME'	ECHO		I

OUTTSR		I

IARG1	--	Scalar	Argument of Variable	OUTTSR

Identified by 'PLNAME'

IB	--	Scalar	Backward Loop Index	INITL

HYDR2

IBM1	--	Scalar	Counter	INITL

ICNAH	--	Array	Soil Surface Condition	READ	HYDR	O

After Harvest	ECHO		I

PLGROW		I

ICNCN	--	Array	Crop Number	READ	CROP	O

ECHO		I

INITL		I

ICROSS	--	Scalar	Number of horizon inter-	INITL,	HYDR	M

faces where points need	MOC

to be consolidated, i.e.,

Ratio greater than 2.

IDEL	--	Scalar	Number of points which	MOC	-	M

are consolidated

IDFLAG	--	Scalar	Flag to Identify if Soil	ECHO	MET	I

Thermal Conductivity and	READ		O

Heat Capacity are Input	SLTEMP		I

or Simulated in the Model	OUTCNC		I

IEDAY	--	Scalar	Ending Day of Simulation	READ	MISC	O

PMAIN		I

ECHO		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

IEDY	--	Scalar	Counter	INITL

IEMER	--	Array	Julian Day of Crop	READ	CROP	O

Emergence	ECHO		I

INITL		I

PLGROW		I

IEMON	--	Scalar	Ending Month of Simula-	READ	MISC	O

tion	ECHO		I

PMAIN		I

IEND	--	Scalar	Index of point at which	MOC	-	M

consolidation ends

IERROR	--	Scalar	Error Flag if Tri-	SLPEST

Diagonal Matrix Cannot	TRDIAG

be Saved

IEYR	--	Scalar	Ending Year of Simulation	READ	MISC	O

ECHO		I

PMAIN		I

IFIRST	--	Scalar	Flag to Print Output	OUTTSR

Heading and Initialize

Output Array

IHAR	--	Array	Julian Day of Crop Harvest	READ	CROP	O

ECHO		I

INITL		I

PLGROW		I

II	--	Scalar	Loop Counter	OUTPST

IJ	--	Scalar	Loop Counter	PMAIN

ILP	--	Scalar	Initial Level of Pesti-	READ	MISC	O

cide Flag (O= No Pesti-	ECHO		I

cide, 1= Initial Pesticide)

INABS	cm	Scalar	Initial Abstraction of	HYDROL	HYDR	O

Water from Potential	EROSN		I

Surface Runoff

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

INCROP	--	Array	Crop Growing in Current	READ	CROP	O

Cropping Period	ECHO		I

INITL		I

PLGROW		I

OUTHYD		I

OUTPST

INICRP	--	Scalar	Initial Crop Number if	READ	CROP	O

Simulation Starting Date	ECHO		I

is Before First Crop	INITL		I

Emergence Date

INTFC	--	Scalar	Whole Layer(s) Attributed	INITL

to the Current Horizon

IOUT	--	Scalar	Index of first point	MOC1		M

outside flow domain

IPEIND	--	Scalar	Pan Evaporation Indica-	READ	MET	O

tor Flag (O= Data Read	ECHO		I

In, 1= Calculated)

IPSCND	--	Scalar	Foliage Pesticide 	ECHO	CROP	I

Condition after Harvest:	PLGROW		M

1.	Surface Applied	READ		O

2.	Removed

3.	Surface Residue

IRTYPE	--	Scalar	Irrigation type flag:	IRRIG	IRGT	I

0=No irrigation	IRREAD		O

1=Flood irrigation

2=Furrow irrigation

3=Over-canopy sprinklers

4=Under-canopy sprinklers

ISCOND	--	Scalar	Surface Condition After	READ	HYDR	O

Harvest Corresponding to	ECHO		I

'INICRP'	PLGROW		I

HYDROL		I

EROSN		I

ISDAY	--	Scalar	Starting Day of Simula-	READ	MISC	O

tion		ECHO		I

INITL		I

PMAIN		I

ISDY	--	Scalar	Counter	INITL

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

ISMON	--	Scalar	Starting Month of Simu-	READ	MISC	O

lation	ECHO		I

INITL		I

PMAIN		I

ISTART	-	Scalar	Index of point at which	MOC	-	M

consolidation starts

ISTYR	--	Scalar	Starting Year of Simula-	READ	MISC	O

tion		ECHO		I

INITL		I

PMAIN		I

ITEM1	--	Alpha-	Hydrology Output Summary	READ	MISC	O

numeric	Indicator	ECHO		I

OUTHYD		I

ITEM2	--	Alpha-	Pesticide Output Summary	READ	MISC	O

numeric	Indicator	ECHO		I

OUTPST		I

ITEM3	--	Alpha-	Soil Pesticide Concentra-	READ	MISC	O

numeric	tion Profile Output	ECHO		I

Indicator	PMAIN		I

ITEMP	°C	Scalar	Mean Daily Temperature	EVPOTR	MISC	O

Rounded to Next Lowest

Whole Number

ITFLAG	--	Scalar	Soil Temperature Flag	ECHO	MET	I

MAIN		I

OUTCNC		I

READ		O

ITMP	--	Scalar	Number of Compartments	PESTAP

Pesticide is Applied to

When Incorporated

IY	--		Annual Loop Counter	PMAIN		I

PLGROW		I

OUTHYD		I

OUTPST		I

OUTTSR		I

OUTCNC		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

IYREM	--	Array	Year of Crop Emergence	READ	CROP	O

ECHO		I

INITL		I

PLGROW		I

IYRHAR	--	Array	Year of Crop Harvest	READ	CROP	O

ECHO		I

INITL		I

PLGROW		I

IYRMAT	--	Array	Year of Crop Maturation	READ	CROP	O

ECHO		I

INITL		I

PLGROW		I

J	--	Scalar	Loop Counter	PMAIN

READ

ECHO

INITL

PLGROW

OUTHYD

OUTPST

JJ	--	Scalar	Loop Counter	READ

JP1	--	Scalar	Counter (J+1)	READ

JP1T10	--	Scalar	Counter (JP1*10)	READ

JT10	--	Scalar	Counter (J*10)	READ

JULDAY	--	Scalar	Julian Day	PMAIN	MISC	O

PLGROW		I

OUTHYD		I

OUTPST		I

K	--	Scalar	Loop Counter	SLTEMP

KD	cm3 g-1	Array	Adsorption/partition	READ	PEST	O

Coefficient for Soil	ECHO		I

Compartment	INITL		I

KDCALC		O

PESTAP		I

SLPEST		I

MASBAL		I

OUTPST		I

OUTTSR		I

OUTCNC		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

KDFLAG	--	Scalar	Partition Coefficient	READ		O

Flag (O= Kd Read In,	ECHO		I

1= Kd Calculated)	PMAIN		I

KH	cm3 cm-3	Array	Henry's Constant at	MAIN	PEST	O

Current Time	SLPST0		I

SLPST1		I

KK	--	Scalar	Loop Counter	READ		I

KOC	cm3 g-1	Scalar	Organic Carbon Partition	KDCALC

-oc		Coefficient

KS	m/s	Scalar	Saturated hydraulic 	FURROW	IRGT	I

conductivity of soil	INFIL		I

IRREAD		O

L	--	Scalar	Loop Counter	SLTEMP

LAYERS	--	Scalar	Number of Layers in Canopy	CANOPY		O

LBTEMP	°C	Scalar	Daily Value of Bottom	SLTEMP		M

Boundary Temperature

LDAY	--	Scalar	Loop Limit (Last Day)	PMAIN

LEAP	--	Scalar	Additional Day Flag for	SLTEMP		I

Leap Year

LFREQ1	--	Scalar	Frequency of Soil Com-	READ	MISC	O

partment Reporting in	OUTHYD		I

Water Output Summary

LFREQ2	--	Scalar	Frequency of Soil Com-	READ	MISC	O

partment Reporting in	OUTPST		I

Pesticide Output Summary

LFREQ3	--	Scalar	Frequency of Soil Com-	READ	MISC	O

partment Reporting in	OUTCNC		I

Concentration Profile

Output Summary

LL	--	Scalar	Loop counter	MOC1

LOGD	--	Scalar	Logarithm of Zero	CANOPY		O

Displacement Height

LOGKOC	--	Scalar	Natural Log of Koc	KDCALC

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

LOGZO	--	Scalar	Logarithm of Roughness	CANOPY		O

Length

M	--	Scalar	Loop counter	MOC1

MAD	--	Scalar	Day of Month of Crop	READ

Maturation	ECHO

MAM	--	Scalar	Month of Crop Maturation	READ

ECHO

MASS	g	Array	Current pesticide mass	MOC1		M

in compartment

MASSO	g	Array	Total pesticide mass in	MOC1 	PEST	M

each compartment at	INITL

previous time step

MAT	--	Array	Julian Day of Crop	READ	MISC	O

Maturation	ECHO		I

INITL		I

PLGROW		I

MCFLAG	--	Scalar	Transport solution	ECHO	PEST	I

technique flag (0 =	READ

PRZM, 1= MOCPRZM)	PMAIN

MD	--	Scalar	Number of Day Read from	PMAIN

Meteorologic File

MDOUT	kg ha-1	Array	Monthly Pesticide Decay	OUTPST	ACCUM	M

from Each Compartment

MEOUTW	cm	Array	Monthly ET from Each Soil	OUTHYD	ACCUM	M

Compartment

MINPP	kg ha-1	Array	Monthly Advection/Disper-	OUTPST	ACCUM	M

sion Flux from Each

Compartment

MINPP1	kg ha-1	Scalar	Monthly Foliar Applied	OUTPST	ACCUM	M

Pesticide

MINPP2	kg ha-1	Scalar	Monthly Soil Applied	OUTPST	ACCUM	M

Pesticide

MINPW	cm	Array	Monthly Infiltration into	OUTHYD	ACCUM	M

Each Soil Compartment

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

MINPW1	cm	Scalar	Monthly Precipitation	OUTHYD	ACCUM	M

MINPW2	cm	Scalar	Monthly Snowfall	OUTHYD	ACCUM	M

MINTH	--	Alpha-	Flag for Monthly Output	PMAIN

numeric	Summary (for Either Water

or Pesticide)

MM	--	Scalar	Number of Month Read from	PMAIN

Meteorologic File

MNTHP1	--	Scalar	Current Month Plus 1	OUTHYD

(Month + 1)

MODFC	--	Scalar	Fraction Multiplier	INITL

MONTH	--	Scalar	Number of Current Month	SLTEMP	MISC	I

of Simulation

MOUTP	kg ha-1	Array	Monthly Pesticide Uptake	OUTPST	ACCUM	M

from Each Compartment

MOUTP1	kg ha-1	Scalar	Monthly Pesticide Washoff	OUTPST	ACCUM	M

Flux

MOUTP2	kg ha-1	Scalar	Monthly Pesticide Runoff	OUTPST	ACCUM	M

Flux

MOUTP3	kg ha-1	Scalar	Monthly Pesticide Erosion	OUTPST	ACCUM	M

Flux

MOUTP4	kg ha-1	Scalar	Monthly Foliar Pesticide	OUTPST	ACCUM	M

Decay Loss

MOUTP5	kg ha-1	Scalar	Monthly Pesticide Uptake	OUTPST	ACCUM	M

Flux from Profile

MOUTP6	kg ha-1	Scalar	Monthly Pesticide Decay	OUTPST	ACCUM	M

Flux from Profile

MOUTW	cm	Array	Monthly Exfiltration from	OUTHYD	ACCUM	M

Each Compartment

MOUTW1	cm	Scalar	Monthly Canopy Evapo-	OUTHYD	ACCUM	M

ration

MOUTW2	cm	Scalar	Monthly Thrufall	OUTHYD	ACCUM	M

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

MOUTW3	cm	Scalar	Monthly Runoff	OUTHYD	ACCUM	M

MOUTW4	cm	Scalar	Monthly Snowmelt	OUTHYD	ACCUM	M

MOUTW5	cm	Scalar	Monthly Evapotrans-	OUTHYD	ACCUM	M

piration

MOUTW6	MTonne	Scalar	Total Monthly Sediment	OUTHYD	ACCUM

Loss

MSTART	--	Scalar	Flag for Positioning	PMAIN

Meteorologic File

MSTR	cm	Array	Previous Month Storage	OUTHYD	ACCUM	M

of Water in Each Soil

Compartment

MSTR1	cm	Scalar	Monthly Canopy Inter-	OUTHYD	ACCUM	M

ception

MSTR2	cm	Scalar	Monthly Accumulation of	OUTHYD	ACCUM	M

Snow

MSTRP	kg ha-1	Array	Storage of Pesticide from	OUTPST	ACCUM	M

Previous Month in Each

Soil Compartment

MSTRP1	kg ha-1	Scalar	Storage of Foliar Pesti-	OUTPST	ACCUM	M

cide from Previous Month

MY	--	Scalar	Number of Year Read from	PMAIN

Meteorologic File

N	--	Scalar	Loop Counter	CANOPY

SLTEMP

NAPPC	--	Scalar	Pesticide Application	PMAIN	PEST	O

Counter	PESTAP		I

NAPS		Scalar	Number of Pesticide	READ	PEST	O

Applications in the	ECHO		I

Simulation	INITL		I

PMAIN		I

NBYR	--	Scalar	Beginning Year of Crop	INITL

Growth for Current Crop	PLGROW

(Loop Limit)

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

NCELL	--	Scalar	Compartment number in 	MOC1 		M

which a point is located	INITL

NCOM0/	--	Scalar	Number of Compartments	INITL	HYDR	O

from Which ET is Extracted	PLGROW		I

Year Round

NCOM1	--	Scalar	Current Number of Com-	PLGROW	HYDR	O

partments, that ET is	EVPOTR		I

Extracted From	OUTHYD		I

NCOM2	--	Scalar	Number of Compartments	SLTEMP	HYDR	I

in Soil Profile

NCOM2M	--	Scalar	Number of Compartments	INITL	HYDR	O

in Soil Profile Minus 1	SLPEST		I

(NCOM2 = 1)

NCOMRZ	--	Scalar	Number of Compartments	INITL	CROP	O

in the Root Zone	SLPEST		I

OUTHYD		I

OUTPST		I

NCP	--	Scalar	Number of Current Crop-	INITL	CROP	O

ping Period	PLGROW		I

NCPDS	--	Scalar	Number of Cropping	READ	CROP	O

Periods in the Simulation	ECHO		I

INITL		I

PLGROW		I

NCROP	--	Scalar	Number of Current Crop	INITL	CROP	O

PLGROW		I

HYDROL		I

EROSN		I

NDC	--	Scalar	Number of Different Crops	READ	CROP	O

in Simulation	ECHO		I

INITL		I

PLGROW		I

NDCNT	--	Scalar	Number of Days Since Crop	INITL	MISC	O

Emergence for Current	PLGROW		I

Crop

NDYRS	--	Scalar	Number of Years Between	INITL

Emergence and Maturation	PLGROW

of a Crop

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

NET	g	Array	Net change in mass due	MOC1		M

to advection

NEW	--	Scalar	Number of new points	MOC1		M

entering the flow domain

NEWK	cm3 cm-3	Array	Henry's Constant	KHCORR		O

NEXDAY	--	Scalar	Extra Day Added for Leap	PLGROW

Year

NEYR	--	Scalar	Ending Year of Crop	INITL

Growth for Current Crop	PLGROW

NHORIZ	--	Scalar	Total Number of Soil	READ	MISC	O

Horizons	ECHO		I

INITL		I

KDCALC		I

NLINES	--	Scalar	Numbers of Lines for	ECHO

Listing Initial Pesticides

in Profile (Loop Limit)

NM1	--	Scalar	Number of Compartments	TRDIAG

in Profile Minus 1

(NCOM2 - 1)

NOPRT	--	Scalar	Print Flag	OUTHYD

OUTPST

NPI	--	Scalar	Current number of	MOC1 	HYDR	M

moving points in soil	INITL

profile

NPLOTS	--	Scalar	Number of Time Series to	READ	MISC	O

be Output (Maximum of 7)	ECHO		I

PMAIN		I

OUTTSR		I

NRZCOM	--	Scalar	Current Number of Layers	PLGROW

in Root Zone

NSPACE	--	Scalar	Number of furrow stations	FURROW	IRGT	M

for finite difference	IRRIG		I

NSUM	--	Scalar	Cumulative Sum of Com-	EVPOTR

partment Numbers

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

NSUMM	--	Scalar	Termination Loop Index	OUTHYD

for Summary Output	OUTPST

NUM	--	Scalar	Number of Soil Compartment	KHCORR		I

NUM	--	Scalar	Initial number of moving	MOC1 	HYDR	I

points per compartment	INITL

NUMDYS	--	Scalar	Number of Days in a Month	SLTEMP		M

OC	percent	Array	Organic Carbon in Each	SLTEMP	PEST	I

Soil Horizon

OKH	cm3 cm-3	Array	Henry's Constant at	INITL	PEST	O

Previous Time	MAIN		I

SLPST0		I

SLPST1		I

ORGM	percent	Scalar	Organic Matter Content	INITL

of a Soil Horizon

OSNOW	cm	Scalar	Snow Accumulated at the 	PMAIN	HYDR	O

End of the Previous Time	HYDROL		I

Step	MASBAL		I

OUTPUT	--	Array	Output Array for Time	OUTTSR

Series

PA	kg ha-1	Scalar	Daily Foliar Pesticide	OUTPST

Application

PB	kg ha-1	Scalar	Pesticide Balance	OUTPST

PBAL	g cm-2	Scalar	Current Pesticide Balance	MASBAL	PEST	O

Error	OUTPST

PCDEPL	Fraction	Scalar	Fraction of available water	IRRIG	IRGT	I

capacity where irrigation 	IRREAD		O

is triggered

PCMC	--	Scalar	Partition Coefficient	READ	MISC	O

Model Flag (1= Karick-	KDCALC		I

hoff, 2= Kenega,

3= Chiou)

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

PCOUNT	--	Array	Number of points crossing	INITL,	HYDR	M

an interface with Ratio	MOC

greater than 2.

PESTR	g cm-2	Array	Total Pesticide in Each	READ	PEST	O

Soil Compartment	ECHO		I

INITL		I

PMAIN		I

PESTAP		I

MASBAL		I

OUTPST		I

PET	cm	Scalar	Total Daily Potential	EVPOTR

Evapotranspiration

PETP	cm	Scalar	Running Total of Avail-	EVPOTR

able Evapotranspiration

PEVP	cm	Scalar	Pan Evaporation	PMAIN	MET	O

EVPOTR		I

PFAC	--	Scalar	Pan Factor for ET	READ	MET	O

ECHO		I

EVPOTR		I

PI	--	Scalar	3.1415926	CANOPY

PLDKRT	day-1	Array	Foliar Pesticide Decay	READ	PEST	O

Rate	ECHO		I

PLPEST		I

PLNAME	--	Alpha-	Time Series Output Iden-	READ	MISC	O

numeric	tifier (Options Listed	OUTTSR		I

in User's Guide)

PLNTAP	g cm-2	Scalar	Pesticide Applied to Crop	PESTAP	PEST	O

Canopy	OUTPST		I

OUTTSR		I

PLVKRT	day-1	Array	Foliage Pesticide	ECHO	PEST	I

Volatilization Rate	PLPEST		I

READ		O

PNBRN	--	Array	Output Array for Time	OUTTSR

Series

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

PRECIP	cm	Scalar	Precipitation	PMAIN	MET	O

HYDROL		I

EROSN		I

MASBAL		I

OUTHYD		I

OUTTSR		I

PTEMP	g cm-3	Array	Temporary storage of	MOC1		M

total pesticide mass

per cc water after

advection step

PTITLE	--	Alpha-	Comment Line to Input	READ	MISC	O

numeric	Information About Pesti-	ECHO		I

cide Parameters

PVFLUX	g cm-2	Array	Daily Soil Pesticide	MASBAL	PEST	I

day-1		Volatilization Flux	OUTPST		I

OUTRPT		I

OUTTSR		I

SLPST0		O

SLPST1		O

PWIND	m day-1	Array	Wind Velocity	MAIN		O

Q	m3	Scalar	Runoff Volume	EROSN

QC1	cal cm-2	Scalar	Sensible Heat Flux Term	SLTEMP		M

day-1 °K-1

QEVF	cal cm-2	Scalar	Evaporation Heat Flux	SLTEMP		M

day-1

QGHF	cal cm-1	Scalar	Soil Heat Flux Term	SLTEMP		M

day-1 °K-1

QLW1	cal cm-2	Scalar	Atmospheric Longwave	SLTEMP		M

day-1 °K-4		Radiation Component Term

QLW2	cal cm-2	Scalar	Longwave Radiation Flux	SLTEMP		M

day-1 °K-1		Term Emitted by Soil

Surface

QO	m3/s	Scalar	Flow rate entering head of	FURROW	IRGT	I

furrow	IRREAD		O

QQP	m6 sec-1	Scalar	Runoff Energy Factor	EROSN

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

QS	m3/s	Array	Flow rate in furrow at each	FURROW	IRGT	M

downstream station

QSWR	cal cm-2	Scalar	Net Shortwave Radiation	SLTEMP		M

day-1		Flux Term

RATIO	--	Array	The ratio of point	INITL,	HYDR	M

densities between	MOC

adjacent horizons.

RETEAP	cm/hr	Scalar	Maximum rate of water that	IRRIG	IRGT	I

sprinklers can deliver	IRREAD		O

RF	kg ha-1	Scalar	Pesticide Runoff Flux	OUTPST

RINUM	--	Scalar	Richardson Number	CANOPY		O

RMULT	--	Scalar	Multiplication Factor for	OUTTSR

Time Series Output

RMULT1	--	Scalar	Multiplication Factor for	READ

Curve Number AMC I

RMULT3	--	Scalar	Multiplication Factor for	READ

Curve Number AMC III

RNSUM	--	Scalar	Converts NSUM to a Real	EVPOTR

Number

RNUM	ha cm-2	Scalar	Numerator of Peak Runoff	EROSN

Rate

RODPTH	--	Scalar	Number of Soil Compart-	HYDROL

ments that Affect Runoff

ROFLUX	g cm-2	Scalar	Runoff Flux of Pesticide	SLPEST	PEST	O

day-1		From Land Surface	MASBAL		I

OUTHYD		I

OUTTSR		I

RTR	day-1	Array	Transformation Term	PSTLNK	PEST	O

from Daughter Product	SLPST0		I

Consideration	SLPST1		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

RUNOF	cm	Scalar	Current Runoff Depth	HYDROL	HYDR	O

PMAIN		I

EROSN		I

SLPEST		I

MASBAL		I

OUTHYD		I

OUTTSR		I

RVEL	--	Array	Retarded solute velocity	MOC1		M

RZD	cm	Scalar	Maximum Root Zone Depth	INITL

for All Crops	OUTHYD

RZFLUX	g cm-2	Scalar	Dispersive/Advective Flux	SLPEST	PEST	O

of Pesticide Past the	OUTTSR		I

Bottom Root Zone Com-

partment

RZI	--	Scalar	Active Root Zone Flag	INITL	MISC	O

PLGROW		I

SA	kg ha-1	Scalar	Application of Pesticide	OUTPST

to the Soil

SAIM	--	Scalar	Integrated Momentum	CANOPY		O

Stability Parameter

SAND	percent	Array	Percent Sand in Each Soil	SLTEMP	HYDR	I

Horizon

SD	kg ha-1	Scalar	Sum of the Decay Fluxes	OUTPST

From All Compartments

in Soil Profile

SDKFLX	g cm-2	Scalar	Sum of the Decay fluxes	SLPEST	PEST	O

day-1		From All Compartments in	OUTPST		I

Soil Profile

SEDL	MTonne	Scalar	Erosion Sediment Loss	PMAIN	HYDR	O

day-1				EROSN		M

OUTHYD		O

SF	Fraction	Scalar	Slope of furrow channel	FURROW	IRGT	I

(vertical/horizontal)	IRREAD		O

SFAC	cm °C-1	Scalar	Snowmelt Factor	READ	MET	O

ECHO		I

HYDROL		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

SIGMA0	--	Scalar	Summation Variable Used to	SLTEMP		M

Calculate K Factor in the

Soil Thermal Conductivity

Equation

SIGMA1	cal cm-1	Scalar	Total Numerator Value in 	SLTEMP		M

°C day-1		the Soil Thermal Conduc-

tivity Equation

SIGMA2	--	Scalar	Total Denominator Value 	SLTEMP		M

in the Soil Thermal

Conductivity Equation

SJDAY	--	Scalar	Starting Day of Simulation	INITL

SLKGHA	kg ha-1	Scalar	Erosion Sediment Loss 	EROSN

day-1

SMDEF	cm	Scalar	Soil moisture deficit 	IRRIG	IRGT	O

requiring irrigation

SMELT	cm	Scalar	Current Daily Snowmelt	HYDROL	HYDR	O

Depth	EROSN

OUTHYD

SNOW	cm	Scalar	Snowpack Accumulation	SLTEMP	HYDR	I

Depth

SNOWFL	cm	Scalar	Current Snowfall Depth	HYDROL	MET	O

MASBAL		I

OUTHYD		I

OUTTSR		I

SOILAP	g cm-2	Array	Pesticide Applied to the	PESTAP	PEST	O

Soil		PMAIN		I

OUTPST		I

OUTTSR		I

SOL	mole	Scalar	Pesticide Solubility -	READ		O

fraction		Karickhoff Model	KDCALC		I

mg 1-1		Kenaga Model

umoles 1-1		Chiou Model

SOLRAD	cal cm-2	Scalar	Shortwave Solar Radiation	READ	MET	0

day-1				SLTEMP		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

SPESTR	g cm-3	Array	Dissolved Pesticide in	INITL	PEST	O

Each Soil Compartment	PMAIN		I

PESTAP		I

SLPEST		I

SPT	°C	Array	Temperature of Soil in	SLTEMP	MET	O

Each Compartment	MAIN		I

SPTEMP	g cm-3	Array	Temporary storage of	MOC1 	PEST	M

dissolved pesticide	SLPST1

mass per cc water after	INITL

advection step

SRC	g cm-3	Array	Source Term from Daughter	PSTLNK	PEST	O

day-1		Product Consideration	SLPST0		I

SLPST1		I

SRCFLX	g cm-2	Array	Source Flux of Pesticide	SLPST0	PEST	O

day-1		from Each Soil Compartment	SLPST1		O

OUTPST		I

STEMP	°C	Array	Soil Compartment 	KHCORR		I

Temperature

STEP1	--	Alpha-	Time Step of Water Output	READ	MISC	O

numeric	Summary	ECHO		I

OUTHYD		I

STEP2	--	Alpha-	Time Step of Pesticide	READ	MISC	O

numeric	Output Summary	ECHO		I

OUTPST		I

STEP3	--	Alpha-	Time Step of Concentration	READ	MISC	O

numeric	Profile Output	ECHO		I

Summary	OUTCNC		I

STITLE	--	Alpha-	Comment Line to Input	READ	MISC	O

numeric	Information About Soil	ECHO		I

Parameters

STK	°K	Scalar	Soil Surface Temperature	SLTEMP		M

in Kelvin Scale

STTDET	cm day-1	Scalar	Daily Evaporation from the	SLTEMP	MET	I

Top 5cm of Soil	EVPOTR		O

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

SU	kg ha-1	Scalar	Sum of the Uptake Fluxes	OUTPST

From All Soil Compart-

ments

SUMC	g	Array	Sum of mass in a 	MOC1		M

compartment

SUMXP	kg ha-1	Scalar	Sum of Soluble Pesticide	OUTPST

in Profile

SUPFLX	g cm-2	Scalar	Sum of the Uptake Fluxes	SLPEST	PEST	O

day-1		From All Soil Compart-	OUTPST		I

ments	OUTTSR		I

SV	kg ha-1	Scalar	Daily Soil Pesticide	OUTPST		O

day-1		Volatilization Flux

SW	cm	Array	Current Water Depth in	INITL	HYDR	O

Each Soil Compartment	HYDROL		I

EVPOTR		I

HYDR1		I

HYDR2		I

SLPEST		I

OUTTSR		I

T	--	Scalar	Fraction Compartment	INITL

Check

TA	day-1	Array	Lower Diagonal Element of	SLTEMP		M

Tridiagonal Matrix

TAPP	g cm-2	Array	Total Pesticide Applied	READ	PEST	O

Per Application	ECHO		I

INITL		I

PESTAP		I

TB	day-1	Array	Diagonal Element of	SLTEMP		M

Tridiagonal Matrix

TC	day-1	Array	Upper Diagonal Element of	SLTEMP		M

Tridiagonal Matrix

TCNC	g cm-3	Array	Average Pesticide	OUTPST		O

Concentration in Canopy

TCORR	mole	Scalar	Temperature Correction	KHCORR		M

cal-1		Factor

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

TEMP	°C	Scalar	Ambient Air Temperature	SLTEMP	MET	I

TEMPK	°K	Scalar	Air Temperature in	SLTEMP		M

Kelvin Scale

TEND	day	Scalar	Time required for point	MOC1		M

to move to compartment

boundary

TERM	--	Scalar	Exponential Pesticide	PLPEST

Washoff Term

TERM1	--	Scalar	Exponential Pesticide	PLPEST

Decay Term

TERM2	--	Scalar	Product of Washoff and	PLPEST

Decay Terms

TF	°C	Array	Vector of Previous Time	SLTEMP		M

Step Soil Compartment

Temperature

TFRAC	--	Scalar	Total Fraction of Com-	EVPOTR

partments Available for

Evapotranspiration

Extraction

THAIR	cm3 cm-3	Array	Volumetric Air Content	SLPST0		O

SLPST1

THCOND	cal cm-1	Array	Thermal Conductivity of	SLTEMP		M

day-1 °C-1		Soil Compartment

THEFC	cm3 cm-3	Array	Field Capacity Water 	SLTEMP	HYDR	I

Content for Each Soil

Horizon

THETAS	cm3 cm-3	Array	Soil Compartment Water	SLTEMP	HYDR	I

Content at Saturation

THETH	cm3 cm-3	Scalar	Soil Moisture Content Half	INITL	HYDR	O

Way Between Wilting Point	HYDROL		I

and Field Capacity in the

Top Soil Compartments

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

THETN	cm3 cm-3	Array	Soil Water Content at the	HYDR1	HYDR	O

End of the Current Day	HYDR2		O

for Each Soil Compartment	PMAIN		I

SLPEST		I

MASBAL		I

OUTHYD		I

OUTPST		I

OUTTSR		I

OUTCNC		I

THETO	cm3 cm-3	Array	Soil Water Content at the	SLTEMP	HYDR	I

End of the Previous Day

for Each Soil Compartment

THEWP	cm3 cm-3	Array	Wilting Point Water Content	SLTEMP	HYDR	I

for Each Soil Horizon

THFLAG	--	Scalar	Soil Water Content Flag	READ	MISC	O

(O= Field Capacity and	ECHO		I

Wilting Point are Input,	PMAIN		I

1= Field Capacity and

Wilting Point are

Calculated)

THKLY1	cm	Scalar	Thickness of Top	SLTEMP

Compartment

THKNS	cm	Array	Soil Horizon Thickness	READ	MISC	O

ECHO		I

INITL		I

HYDROL		I

THRUFL	cm	Scalar	Precipitation that Falls	HYDROL	MET	O

Past the Crop Canopy to	OUTHYD		I

the Soil Surface	OUTTSR		I

THZERO	cal cm-1	Array	Thermal Conductivity of	SLTEMP		M

day-10 C-1		Soil at Water Content

and Wilting Point

TITLE	--	Alpha-	Title of the Simulation	READ	MISC	O

numeric	(User Supplied)	ECHO		I

TLEFT	day	Scalar	Travel time left in	MOC1		M

current time step

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

TMPK	°K	Scalar	Soil Temperature	KHCORR		M

TNDGS	day	Array	Total Number of Days in	INITL	CROP	O

Each Growing Season	PLGROW		I

TOL	--	Scalar	Fraction Compartment	INITL

Check

TOP	--	Array	Location of top compart-	INITL,	HYDR	M

ment in horizon where	MOC

points are consolidated

TOT	day m-1	Scalar	Canopy Resistance	CANOPY		O

TOTAL	mg kg-1	Array	Total Pesticide in Each	OUTCNC

Compartment

TOTR	day m-1	Scalar	Total Canopy Resistance	CANOPY		O

TR	hr	Scalar	Duration of Average 	READ	MET	O

Erosive Storm Event	ECHO		I

EROSN		I

TRFLUX	g cm-2	Array	Transformation Flux of	SLPST0	PEST	O

day-1		Pesticide from Each Soil	SLPST1		O

Compartment	OUTPST		I

TS	cm3 cm-3	Array	Previous Soil Compartment	HYDR2

Water Content Minus

Evapotranspiration

TSRCFX	g cm-2	Array	Sum of the Source Flux	SLPST0	PEST	O

day-1		from All Compartments in	SLPST1		O

Soil Profile	OUTPST		I

TSW	cm	Scalar	Total Soil Water in	EVPOTR

Compartments Available

for Evapotranspiration

Extraction

TTHKNS	cm	Scalar	Total Thickness of Soil	INITL

Profile (For Computa-

tional Check)

TTRFLX	g cm-2	Array	Sum of the Transformation	SLPST0	PEST	O

day-1		Flux from All Compartments	SLPST1		O

in Soil Profile	OUTPST		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

TWLVL	cm cm-1	Scalar	Fraction of Water to Soil	HYDROL

Depth for Runoff

Calculation

TWP	cm	Scalar	Total Wilting Point Depth	EVPOTR

in Compartments Available

for Evapotranspiration

Extraction

U	--	Array	Upper Decomposed Matrix	TRDIAG

UBT	°C	Scalar	Upper Boundary or Soil	SLTEMP		M

Surface Temperature

UPF	kg ha-1	Scalar	Daily Pesticide Uptake	OUTPST

Flux in Profile

UPFLUX	g cm-2	Array	Uptake Flux of Pesticide	SLPEST	PEST	O

From Each Soil Compartment	OUTPST		I

UPTKF	--	Scalar	Plant Pesticide Uptake	READ	PEST	O

Efficiency Factor	ECHO		I

PLGROW		I

URH	m day-1	Scalar	Wind Velocity at Reference	CANOPY		I

Height	MAIN		O

USLEC	--	Array	Universal Soil Loss	READ	HYDR	O

Equation 'C' Factor	ECHO		I

EROSN		I

USLEK	--	Scalar	Universal Soil Loss	READ	HYDR	O

Equation 'K' Factor	ECHO		I

EROSN		I

USLELS	--	Scalar	Universal Soil Loss	READ	HYDR	O

Equation 'Ls' Factor	ECHO		I

EROSN		I

USLEP	--	Scalar	Universal Soil Loss	READ	HYDR	O

Equation 'P' Factor	ECHO		I

EROSN		I

USTAR	m day-1	Scalar	Friction Velocity	CANOPY		O

UTEMP	°C	Array	Air Temperature	CANOPY		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

UWIND	m day-1	Array	Wind Velocity	CANOPY		I

VAPLMD	cal cm-1	Scalar	Thermal Conductivity of	SLTEMP

day-10 C-1		Vapor in the Soil Pores

VAR1	kg ha-1	Scalar	Daily Advection/Disper-	OUTPST

sion Flux of Pesticide

Into a Compartment

VAR2	kg ha-1	Scalar	Daily Advection/Disper-	OUTPST

sion Flux of Pesticide

Out of a Compartment

VAR2D	cm	Scalar	Water Storage in a Single	OUTHYD

Compartment for the

Previous Day

VAR2M	cm	Scalar	Water Storage in a Single	OUTHYD

Compartment for the

Previous Month

VAR2RZ	kg ha-1	Scalar	Daily Advection/Disper-	OUTPST

sion Flux of Pesticide

Out of the Root Zone

VAR2Y	cm	Scalar	Water Storage in a Single	OUTHYD

Compartment for the

Previous Year

VAR3	kg ha-1	Scalar	Pesticide Storage in a 	OUTPST

Single Compartment for

the Previous Day

VEL	cm day-1	Array	Water Velocity in Each	HYDR1	HYDR	O

Soil Compartment	HYDR2		O

SLPEST		I

VHTCAP	cal cm-3	Array	Heat Capacity Per Unit	SLTEMP		M

°C-1		Volume of Soil

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

VLFLAG	--	Scalar	Advection flux flag	HYDR1 	HYDR	I

(0 = all soil water	PMAIN

velocities are zero,	HYDR2

1 = soil water velocity

is nonzero)

VOLCOR	--	Scalar	A Variable Used to Convert	SLTEMP

Weight Percents of Soil

Constituents to Volume

Fractions of Bulk Volume

WBAL	cm	Scalar	Current Water Balance	MASBAL	HYDR	O

Error	OUTHYD		I

WEIGHT	kg m-2	Scalar	Current Plant Dry Foliage	PLGROW	CROP	O

Weight	PESTAP		I

WF	kg ha-1	Scalar	Daily Pesticide Washoff	OUTPST

Flux

WFMAX	kg m-2	Array	Maximum Plant Dry Foliage	READ	CROP	O

Weight at Full Canopy	ECHO		I

INITL		I

WIND	cm sec-1	Scalar	Wind Speed	READ	MET	0

SLTEMP		I

MAIN		I

WLVL	cm	Scalar	Total Soil Water in the	HYDROL

Compartments that Affect

Runoff

WOFLUX	g cm-2	Scalar	Washoff Flux of Pesticide	SLPEST	PEST	O

day-1		From Plant Foliage	OUTPST		I

WP	cm	Array	Wilting Point Water Depth		HYDR	O

in a Soil Compartment	EVPOTR

WPV	--	Array	Regression Coefficients 	THCALC

for Prediction of Wilting

Point Soil Water Content

WTERM	g cm-2	Scalar	Current Daily Pesticide	PLPEST	PEST	O

Washoff Loss	SLPEST		I

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

X	g cm-3	Array	Dissolved Pesticide in	TRDIAG	PEST

Each Soil Compartment	SLPEST	PEST	O

MASBAL		I

OUTPST		I

OUTTSR		I

OUTCNC		I

PMAIN		I

XFRAC	Fraction	Scalar	Location in furrow where	IRRIG	IRGT	I

infiltration is to be used	IRREAD		O

in PRZM transport

calculations (as fraction

of total furrow length)

XL	m	Scalar	Length of furrows	IRRIG	IRGT	I

FURROW		I

IRREAD		O

XP	g cm-3	Array	Total Pesticide in	MASBAL

Each Soil Compartment

XVOL	fraction	Array	Volume Fraction of Soil	SLTEMP

Constituent

Y	--	Array	Intermediate Matrix Solu-	TRDIAG

tion Array

YDOUT	kg ha-1	Array	Annual Pesticide Decay	OUTPST	ACCUM	M

From Each Soil Compartment

YEAR	--	Alpha-	Flag for Annual Water and	PMAIN

numeric	Pesticide Summary Output

YEOUTW	cm	Array	Annual Evapotranspiration	OUTHYD	ACCUM	M

From Each Soil Compartment

YINPP	kg ha-1	Array	Annual Advective/Disper-	OUTPST	ACCUM	M

sive Flux Into Each Soil

Compartment

YINPP1	kg ha-1	Scalar	Annual Pesticide Applied	OUTPST	ACCUM	M

to Foliage

YINPP2	kg ha-1	Scalar	Annual Pesticide Applied	OUTPST	ACCUM	M

to Soil

YINPW	cm	Array	Annual Infiltration Into	OUTHYD	ACCUM	M

Each Soil Compartment

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

YINPW1	cm	Scalar	Annual Precipitation	OUTHYD	ACCUM	M

YINPW2	cm	Scalar	Annual Snowfall	OUTHYD	ACCUM	M

YOUTP	kg ha-1	Array	Annual Pesticide Uptake	OUTPST	ACCUM	M

From Each Soil Compartment

YOUTP1	kg ha-1	Scalar	Annual Pesticide Washoff	OUTPST	ACCUM	M

Flux

YOUTP2	kg ha-1	Scalar	Annual Pesticide Runoff	OUTPST	ACCUM	M

Flux

YOUTP3	kg ha-1	Scalar	Annual Pesticide Erosion	OUTPST	ACCUM	M

Flux

YOUTP4	kg ha-1	Scalar	Annual Foliar Pesticide	OUTPST	ACCUM	M

Decay Flux	OUTPST	ACCUM	M

YOUTP5	kg ha-1	Scalar	Total Annual Pesticide	OUTPST	ACCUM	M

Uptake Flux

YOUTP6	kg ha-1	Scalar	Total Annual Pesticide	OUTPST	ACCUM	M

Soil Decay Flux

YOUTW	cm	Array	Annual Exfiltration From	OUTHYD	ACCUM	M

Compartment

YOUTW1	cm	Scalar	Annual Canopy Evaporation	OUTHYD	ACCUM	M

YOUTW2	cm	Scalar	Annual Trufall	OUTHYD	ACCUM	M

YOUTW3	cm	Scalar	Annual Runoff	OUTHYD	ACCUM	M

YOUTW4	cm	Scalar	Annual Snowmelt	OUTHYD	ACCUM	M

YOUTW5	cm	Scalar	Total Annual Evapotrans-	OUTHYD	ACCUM	M

piration

YOUTW6	MTonne	Scalar	Total Annual Sediment	OUTHYD	ACCUM	M

Loss

YSTR	cm	Array	Previous Year Storage of	OUTHYD	ACCUM	M

Water in Each Soil Com-	OUTHYD	ACCUM	M

partment

YSTR1	cm	Scalar	Annual Canopy Interception	OUTHYD	ACCUM	M

TABLE 10-3.	PRZM PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATION (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

YSTR2	cm	Scalar	Annual Snow Accumulation	OUTHYD	ACCUM	M

YSTRP	kg ha-1	Array	Storage of Pesticide From	OUTPST	ACCUM	M

Previous Year in Each

Soil Compartment

YSTRP1	kg ha-1	Scalar	Storage of Foliar Pesticide	OUTPST	ACCUM	M

Z	Fraction	Scalar	Side slope of furrow	FURROW	IRGT	I

channel walls	IRREAD		O

(horizontal/vertical)

Z	--	Array	Location of moving 	MOC1 	HYDR	M

points	INITL

ZC	--	Array	Location of fixed 	MOC1 	HYDR	M

compartment center	INITL

ZCH	m	Scalar	Canopy Height	CANOPY		I

MAIN		O

SLTEMP		M

ZCTOT	--	Scalar	Concentration weighted	MOC	-	M

locations of consolidated

points

ZIN	--	Array	Temporary storage of	MOC1		M

new point locations

ZO	m	Scalar	Roughness Height	CANOPY		O

SLTEMP		M

ZRH	m	Scalar	Reference Height	CANOPY		I

MAIN		O

ZTOT	--	Scalar	Location of consolidated	MOC	-	M

Points

ZWIND	m	Scalar	Distance Above the Ground	READ		O

Where Wind Speed was	MAIN		O

Measured	SLTEMP		I

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

A	--	ARRAY	Left Diagonal of a	ASSEMF	ASOLV	M

Tridiagonal Matrix	ASSEMT

ASTORN	--	SCALAR	Value of A(NP) Where	ASSEMF	WORKA	M

NP=Number of Nodes	ASSEMT

BALCHK

B	--	ARRAY	Main Diagonal of a	ASSEMF	ASOLV	M

Tridiagonal Matrix	ASSEMT

BALSTO	--	ARRAY	Array Containing Mass	MAIN		M

Balance Information	BALCHK		O

BSTOR1	--	SCALAR	Value of B(1)	ASSEMF	WORKA	M

ASSEMT

BALCHK

BSTORN	--	SCALAR	Value of B(NP) Where	ASSEMF	WORKA	M

NP=Number of Nodes	ASSEMT

BALCHK

C	--	ARRAY	Right Diagonal of a	ASSEMF	ASOLV	M

Tridiagonal Matrix	ASSEMT

CORD	L	ARRAY	Nodal Coordinates	MAIN	CORD	I

VSWCOM

CSTOR1	--	SCALAR	Value of C(1)	ASSEMF	WORKA	M

ASSEMT

BALCHK

CTRFAC	--	ARRAY	Coordinate Transform-	CONVER	WORKN	M

ation Factors for 	DSWFUN

Different Soil Materials	MAIN

D	--	ARRAY	Right-Hand-Side Vector	ASSEMF	ASOLV	M

of a Tridiagonal Matrix	ASSEMT

DETAND	-	ARRAY	Nodal Storage Factor	ASSEMF	WELEM	M

DIS	L	ARRAY	Current Nodal Value of	MAIN	BSOLV	M

M/L**3		Head of Concentration	ASSEMF		O

BALCHK

VARCAL

VSWCOM

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

DLAMDA	1/t	SCALAR	Value of Decay Constant	MAIN	CONTR	M

for the Node Currently	ASSEMT

Being Evaluated	VARCAL

DLAMND	1/t	SCALAR	Nodal Value of Decay	MAIN		I

Constant	ASSEMT

BALCHK

VARCAL

DPKND	L/t	ARRAY	Nodal Values of Hyd.	ASSEMF	WELEM	M

Conductivity Increment

DPKRAV	L**2	SCALAR	Value of Rel. Perm. for	ASSEMF		M

Node Currently Being	PKWFUN

Solved

DSTOR1	--	SCALAR	The Value of D(1)	ASSEMF	WORKA	M

ASSEMT

BALCHK

DSTORN	--	SCALAR	The Value of D(NP)	ASSEMF	WORKA	M

Where NP = Number of	ASSEMT

Nodes	BALCHK

DTEPS	--	SCALAR	Time Step Tolerance	MAIN		M

Parameter

DTMARK	--	SCALAR	Marker Time Increment	MAIN		M

DX	--	SCALAR	DX = THL(I) NEL	MAIN		M

EL	L	SCALAR	Elemental Values for	MAIN		M

Finite-Element Element	ASSEMF

Length Formulation	ASSEMT

BALCHK

VARCAL

ETAND	--	ARRAY	Nodal Values of Fluid	ASSEMF	WELEM	M

Storage Factor	ASSEMT

BALCHK

FLX1	L**3/t	SCALAR	Value of Fluid Flux	MAIN	CONTR	M

Entering Node 1	ASSEMT

(for Flow FLX1 = 0.0)	HFINTP

VARCAL

FLXN	L**3/t	SCALAR	Value of Fluid Flux	MAIN	CONTR	M

Entering the Last Node	ASSEMT

(for Flow FLX1 = 0.0)	HFINTP

VARCAL

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

FVAL	-	ARRAY	Functional Coefficient	MAIN	MDATA	M

Values for the Soil	ASSEMT

Moisture Relationship	HFINTP

SWFUN

CONVER

DSWFUN

HAVE	L	SCALAR	Average Head Value	ASSEMF		M

SWFUN

DSWFUN

HCAP	L	ARRAY	Value of Pressure	MAIN	SWHDA	M

Head on Press. Head	ASSEMF

vs. Sat. Curve	INTERP

HCRIT	L	SCALAR	Critical Head Value	ASSEMF		I

SWFUN

DSWFUN

HDOBS	L	ARRAY	Head or Concentration	MAIN	DAOBS	M

M/L**3		of Observation Node			O

for Current Time

HINV	L	SCALAR	Default Value of Initial	MAIN		I

M/L**3		Head or Concentration

HTOL	L	SCALAR	Head Tolerance Allowed	MAIN	CONTR	I

for Nonlinear Solution	ASSEMF

VARCAL

DSWFUN

HVTM	L	ARRAY	Value of function	MAIN		M

corresponding to	HFINTP

Time Values(TMHV)

IBTND1	--	SCALAR	Last Node Boundary	MAIN		I

Condition Code (1=1st	ASSEMF

type, 0=3rd type)	ASSEMT

VARCAL

IBTNDN	__	SCALAR	Last Node Boundary	ASSEMF		I

Condition Code (1=1st 	MAIN

Type, 0=3rd type)	ASSEMT

VARCAL

ICONVG	--	SCALAR	Convergence Flag	MAIN		I

(1=Converged, 0=Not	VARCAL

Converged)

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

IHORIZ	--	SCALAR	Simulation Orientation	MAIN		I

Indicator (0=Vertical,

1=Horizontal)

IKALL	--	SCALAR	Time Stepping Scheme	MAIN		I

Indicator (1=Backward,

0=Central)

ILAYR	--	SCALAR	Current Layer Number	MAIN		I

IMAT	--	SCALAR	Counter Used in Looping	MAIN		I

with Respect to Materials	ASSEMF

ASSEMT

INTERP

PKWFUN

SWFUN

CONVER

DSWFUN

IMATL	--	ARRAY	Material Identifying	MAIN		I

Number for Current Layer

IMBAL	--	SCALAR	Mass Balance Computation	MAIN	CONTR	I

Indicating Parameter

IMOD	--	SCALAR	For Modified Newton	MAIN	CONTR1	I

Raphson Solution	DSWFUN

Procedure

IMODL	--	SCALAR	Simulation Identifier	MAIN	CONTR	I

(Flow or Transport)	BALCHK

VARCAL

INEWT	--	SCALAR	Nonlinear Iterative	MAIN	CONTR	I

Procedure Flag	ASSEMF

(1=Newton, 0=Picard)	VARCAL

INOCTS	--	SCALAR	Number of Computation	MAIN		I

Time Steps Required to	VARCAL

Simulate This Target

Time Step

INPFL	--	SCALAR	Unit Number for Input	MAIN		I

File

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

INTSPC	--	SCALAR	Initial Condition	MAIN		I

Specifier for Head

Conversion Convert

Initial Head Values

(1=Yes, 0=No)

IOBSND	--	SCALAR	Observation Node Index	WORKA		I

IPRCHK	--	SCALAR	Print Check Flag	MAIN		I

(Triggers Additional	ASSEMF

Diagnostic Output)	ASSEMT

BALCHK

VARCAL

CONVER

IPROP	--	ARRAY	Generated Material	MAIN	MDATA	I

Property Identifiers	ASSEMF

ASSEMT

IREP	--	SCALAR	Time Step Refinement	MAIN		M

Counter	VARCAL

IREPMX	--	SCALAR	Maximum Number of	MAIN		I

Nonlinear Solution 	VARCAL

Cycles

IRESOL	--	SCALAR	Maximum Number of	MAIN		I

Time Step Refinements	VARCAL

IRLTYP	--	SCALAR	Flag for the Type of	ASSEMF		I

Relative Function Being	INTERP

Evaluated

ITCND1	--	SCALAR	Node 1 Boundary	MAIN		I

Condition Flag	HFINTP

(1 = Transient,

0 = Steady State)

ITCNDN	--	SCALAR	Node 1 Boundary	MAIN		I

Condition Flag	HFINTP

(1 = Transient,

0 = Steady State)

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

ITER	--	SCALAR	Iterative Counter	MAIN		M

(Current Iteration	ASSEMF

Number)	ASSEMT

BALCHK

VARCAL

VSWCOM

ITMARK	--	SCALAR	Backup File Output 	MAIN		M

Indicator	VSWCOM

ITMFC	--	SCALAR	Marker Time Increasing	MAIN		M

Parameter	VSWCOM

ITMGEN	--	SCALAR	Marker Time Value	MAIN		I

Generation Indicator

ITRANS	--	SCALAR	Transient Steady-State	MAIN	CONTR	I

Flag (1=TR, 0=SS)	ASSEMF

VARCAL

ITSGN	--	SCALAR	Time Step Generation	MAIN		I

Indicator

ITSTH	--	ARRAY	Identifies Location of	MAIN		I

Previous Time Value of	HFINTP

Time Graph

IVSTED	--	SCALAR	Steady-State Velocity	MAIN		I

Field Indicator

KPROP	--	SCALAR	Flag for Perm-Saturation	MAIN	CONTR	I

and Pressure Head-	ASSEMF

Saturation Curves	VARCAL

(1=Functional,

0=Tabulated)

MARK	--	SCALAR	Flow Direction Flag	MAIN	CONTR	I

(1=Vertical,	ASSEMF

0=Horizontal)	VARCAL

VSHCOM

MM	--	SCALAR	Place Holder for Loop			M

Incrementer

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

MXMAT	--	SCALAR	Maximum Number of	MAIN		I

Materials Allowed	ASSEMF

(Due to the Size of	ASSEMT

Arrays)	INTERP

SWFUN

DSWFUN

MXNODE	--	SCALAR	Maximum Number of Nodes	MAIN		I

Allowed (Due to the Size	ASSEMF

of Some Arrays)	ASSEMT

BALCHK

TRIDIA

VARCAL

VSWCOM

MXTMV	t	SCALAR	Maximum Time Value to	MAIN		I

be Interpolated	HFINTP

NDCOUN	--	SCALAR	Material Number	MAIN		M

Temporary Counter

NDM1	--	SCALAR	Counter Minus One	MAIN		M

NDM1 = NDCOUN

NDOBS	--	ARRAY	Nodal Values of	MAIN	DAOBS	I

Observation Nodes

NE	--	SCALAR	Number of Elements in	MAIN	CONTR	I

the Linear Representation	VSWCOM

NEL	--	SCALAR	Storage Location for the	MAIN		M

Number of Finite Elements

in the Current Layer

NELM(I)

NELM	--	ARRAY	Number of Finite Elements	MAIN		I

in the Current Layer

NITMAX	--	SCALAR	Maximum Number of	MAIN	CONTR	I

Nonlinear Iterations 	VARCAL

Allowed per Time Step

NLAYRG	--	SCALAR	Number of Layers That	MAIN		I

Need to be Descritized

NMAT	--	SCALAR	Number of Soil Materials	MAIN		I

CONVER

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

NOBSND	--	SCALAR	Number of Observation	MAIN		I

Nodes in the Simulation

NONU	--	SCALAR	Nonuniform Initial	MAIN		I

Condition Indicator

NOWRIT	--	SCALAR	Restart Data Writing	MAIN		I

Indicator

NP	--	SCALAR	Total Number of Nodal	MAIN	CONTR	I

Points	ASSEMF

ASSEMT

BALCHK

TRIDIA

VARCAL

VSWCOM

NPIN	--	SCALAR	Number of Nondefault	MAIN		I

Initial Values

NPROB	--	SCALAR	Number of Simulations	MAIN		I

to be Made

NSTEP	--	SCALAR	Nodal Value Printout	MAIN	CONTR	I

Control Parameter	BALCHK

NTN1	--	SCALAR	Storage Location for 	MAIN		M

NTSNDH(1)

NTNP	--	SCALAR	Storage Location for	MAIN		M

NTSNDH(NP)

NTOMT	--	SCALAR	Number of Backup File	MAIN		I

Output Marker Time	VSWCOM

Values

NTS	--	SCALAR	Number of Time Steps	MAIN		M

in This Simulation

NTSNDH	--	ARRAY	Number of Time Values	MAIN		I

on the Time Graph	HFINTP

([1]=CONC, [2]=HEAD)

NUMK	--	ARRAY	Values of Permeability	MAIN	SWHDA	I

from the Permeability	ASSEMF

vs Saturation Table	INTERP

for Each Material

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

NUMP	--	ARRAY	Number of Pressure 	MAIN	SWHDA	I

Head vs. Saturation	ASSEMF

Values for Each Material	INTERP

NUMT	--	SCALAR	Time Step Incrementor	MAIN		I

NVPR	--	SCALAR	Velocity Printout Control	MAIN	CONTR	I

Parameter 	VSWCOM

NVREAD	--	SCALAR	Velocity Reading 	MAIN		I

Indicator

OUTFL	--	SCALAR	Output File Unit Number	MAIN		I

ASSEMF

ASSEMT

BALCHK

INTERP

VARCAL

VSWCOM

PCUR	L	ARRAY	Current Value of	ASSEMF	BSOLV	M

M/L**3		Pressure Head or	VARCAL

Concentration for the

Current Time Step

PINT	L	ARRAY	Initial Value of	MAIN	BSOLV	I

M/L**3		Pressure Head or	ASSEMF

Concentration	ASSEMT

BALCHK

VARCAL

PKND	L/t	ARRAY	Nodal Values of 	MAIN	WELEM	M

Hydraulic Conductivity	ASSEMF

VSWCOM

PKRW	L**2	ARRAY	Value of Relative 	MAIN	SWHDA	M

Permeability (on	ASSEMF

Perm. vs. Sat. Curve)	INTERP

PKWOUT	L**2	SCALAR	Relative Permeability	PKWFUN		M

Computed Using Function

Then Passed Back

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

PROP		ARRAY	Saturated Material	MAIN	MDATA	I

Properties (Flow or 	ASSEMF

Transport) Flow-	ASSEMT

Hydraulic Conductivity

Porosity, Specific

Storage Air Entry

Pressure Transport-

Dispersivity, Porosity,

Retardation Diffusion

QVTM	L**3/t	ARRAY	Volumetric Water Flux	MAIN		M

Values Corresponding	HFINTP

to Time Values

SLOPE	--	SCALAR	Slope of the Line	HFINTP		M

Between the Points	INTERP

Being Interpolated

SSWV	--	ARRAY	Value of Water Phase	ASSEMF	SWHDA	M

Saturation (on Press.	INTERP

Head vs Sat. Curve)

STMARK	t	SCALAR	Starting Marker Time	MAIN		M

Value

SWAVE	--	SCALAR	Average Water Saturation	ASSEMF		M

PKWFUN

SWDFI	--	ARRAY	Default Value of Water	MAIN		I

Saturation for the

Current Material

SWND	--	ARRAY	Current Water Saturation	MAIN		M

at the Node Being	ASSEMF

Evaluated	ASSEMT

VARCAL

VSWCOM

SWNDPT	--	ARRAY	Water Saturation for the	MAIN		M

Node at Previous Time	VSWCOM

Step

SWRKP	--	ARRAY	Temporary Working Array	CONVER	WORKN	M

SWV	--	ARRAY	Value of Water Phase 	MAIN	SWHDA	M

Saturation (on Perm.	ASSEMF

vs. Sat. Curve)	INTERP

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

TAP8	--	SCALAR	Unit Number for Restart	MAIN		I

File

TAP10	--	SCALAR	Unit Number of Flow-to-	MAIN	MDATA	I

Transport File (Darcy	VSWCOM

Vel. & Water Sat.)

TDIFF	t		TDIFF=TMCUR-TMVECX	MAIN		M

TERIFL	--	SCALAR	Unit Number for Input	MAIN		I

File

TEROFL	--	SCALAR	Unit Number for Output	MAIN		I

File

TFAC	--	SCALAR	Time Step Multiplier	MAIN		I

THETA	--	SCALAR	Value Used in the Time	MAIN		M

Stepping Scheme	ASSEMT

(Theta=0.5 for Central	BALCHK

Difference Scheme,	VARCAL

Theta=1.0 for Backward

Difference Scheme)

THETM1	--	SCALAR	Theta Minus One	MAIN		M

ASSEMT

BALCHK

VARCAL

THL	L	ARRAY	Thickness of Current	MAIN		M

Layer

TIN	t	SCALAR	Value of Initial Time	MAIN	CONTR	I

Step 	ASSEMF

ASSEMT

BALCHK

VARCAL

TIMA	t	SCALAR	Initial Time Value  of	MAIN	CONTR	I

the Simulation	VSWCOM

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

TIMAKP	t	SCALAR	Storage Location for	MAIN		M

the Value of Time Where			I

Iteration Computation

is Taking Place

TITLE	--	ALPHA-	Title of Simulation	MAIN		I

NUMERIC

ARRAY

TMACCU	L**3	SCALAR	Quantitative Storage	MAIN	CONTR	M

m		Water Volume or Solute		BALCHK

Mass

TMAX	t	SCALAR	Maximum Time Step Size	MAIN		I

TMCUR	t	SCALAR	Current Time Value	MAIN		M

VSWCOM

TMDCAY	m	SCALAR	Cumulative Solute Mass	MAIN	CONTR	M

Decay	BALCHK

TMFOMT	t	ARRAY	Time Values for Output	MAIN		I

to the Backup File	VSWCOM

TMHV	t	ARRAY	Time Values at the	MAIN		M

Interpolation Points	HFINTP

([1]=CONC, [2]=HEAD)

TMVEC	t	ARRAY	Values of Time Generated	MAIN		I

by the Code, to be Used	BALCHK		M

in the Simulation

TMVECX	t	SCALAR	Extra Time Value Due	MAIN		M

to the Reduction of a	BALCHK

Time Step When Solution	HFINTP

is not Converging	VARCAL

UWF	--	SCALAR	Value of Upstream	MAIN	CONTR	M

Weighting Factor for	ASSEMT

the Node Currently	VARCAL

Being Evaluated

UWFI	--	ARRAY	Value of Upstream-	MAIN	TPDEF	M

Weighting Factor for

the Current Material

TABLE 10-4.	VADOFT PROGRAM VARIABLES, UNITS, LOCATION, AND VARIABLE DESIGNATIONS (Continued)

Sub-	Common

Variable	Units	Type	Description	routine	Block	I,M,O

VALND1	--	SCALAR	Value of First Node	MAIN		M

(Depending on:  Type of	ASSEMF

Run & Type of Boundary	ASSEMT

HFINTP

VARCAL

VALNDN	--	SCALAR	Value of Last Node	MAIN		M

(Depending on:  Type of	ASSEMF

Run & Type of Boundary	ASSEMT

HFINTP

VARCAL

VDAR	L/t	ARRAY	Darcy Velocity for Each	MAIN		M

Node	ASSEMF		O

BALCHK

VARCAL

VSWCOM

VDARPT	L/t	ARRAY	Nodal Darcy Velocities	MAIN		M

at Previous Time	VSWCOM

VDFI	L/t	ARRAY	Default Value of Darcy	MAIN		I

Velocity for Current

Material

XX	--	SCALAR	The X value Passed in	INTERP		M

INTERP (to be Used in

the Interpolation)

YY	--	SCALAR	The Y Value Passed in	INTERP		M

INTERP (to be Used in

the Interpolation

TABLE 10-5.	MONTE-CARLO PROGRAM VARIABLES

Sub-

Variable	Units		Description		routine

BBT	Double		Correlation matrix for Monte-Carlo		Main program,

Precision		inputs.		READM, INITMC

CORR	Double		Array of correlation terms for		Main Program,

Precision		summary output variables.		STATIS, OUTPUT

Array

DECOM	Integer		Decomposed correlation matrix for		Main Program,

Monte-Carlo inputs.		INITMC, RANDOM

DIST	Real		Array storing empirical		Main Program,

Array		distributions.		READM, Random

IN2	Integer		Monte-Carlo input file number.		Main Program,

READM

IOUT	Integer		Monte-Carlo summary output file		Main Program,

unit number.		READM, OUTPUT

IOUT2	Integer		Output file unit number for 		Main Program,

results of each Monte-Carlo run.		STATIS

IRUN	Integer		Do loop counter for Monte-Carlo		Main Program,

runs.		STATIS

IVAR	Integer		Do loop counter for variable.		Main Program

number.

LARR	Integer		Array storing array addresses for		Main Program,

Array		random input variables.		READM, INITMC

MCMAX	Integer		Maximum possible number of random	Main Program

input variables.

MCVAR	Integer		Number of random input variables.		Main Program,

READM, INITMC,

RANDOM

NCMAX	Integer		Maximum possible number of		Main Program

variables for which cumulative

distributions can be plotted.

NDAT	Integer		Number of values in empirical		Main Program,

Array		distributions.		READM, RANDOM

NEMP	Integer		Maximum number of empirical		Main Program,

distribution value-probability		READM, RANDOM

pairs.

TABLE 10-5.	MONTE-CARLO PROGRAM VARIABLES (Continued)

Sub-

Variable	Units		Description		routine

NMAX	Integer		Maximum possible number of 		Main Program

variables for which summary

statistics can be printed.

NRMAX	Integer		Maximum number of Monte-Carlo		Main Program

runs allowed.

NRUNS	Integer		Number of Monte-Carlo Runs.		Main Program

READM, OUTPUT

NVAR			Number of summary output		Main Program

variables.

PNAME	Character		Input labels used to flag random		Main Program,

Array		input variables.		READM, INITMC

RMC	Real		Array of randomly-generated		Main Program,

Array		numbers.		RANDOM

SNAME	Character		Input labels used to flag summary		Main Program,

Array		output variables.		READM, OUTPUT

STAT	Double		Array of summary statistics for		Main Program,

Precision		output variables.		STATIS, OUTPUT

Array

VAR	Real		Array storing distribution		Main Program,

Array		parameters for random input		READM, INITMC,

variables.		RANDOM

XCDF	Real		Array storing values of selected		Main Program,

Array		variables for plotting cumulative		STATIS,OUTPUT

distributions.

XMC	Real		Array storing values of summary		Main Program,

Array		output variables.		STATIS

10.3  PRZM and VADOFT Example Input Files

The following pages contain examples using different options in PRZM and VADOFT.  Below each example file is a brief summary of the scenario illustrated.

1 CHEMICAL, 1 HORIZON, TEMP CORRECTION, BACKGROUND LEVELS HYDROLOGY PARAMETERS (CROP DATA FROM USDA NO.283 HANDBOOK)

0.72	0.03	0	15.000	1	1

0

1

1	0.00	20.0	80.000	1	86	78	82	0.0	0.0	0.0	60.0

1

110582	300982	151082	1

PESTICIDE TRANSPORT AND TRANSFORMATION AND APPLICATION PARAMETERS

1	1	0

ALDICARB

120582	0	1.0	1.00

1	1

SOILS PARAMETERS

20.0	0.3	0	0	0	0	0	0	1	1	0

4.3E03	1.0E-7	5.5E-3

0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.97 10.0

8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3

1

1	20.0	1.32	0.330	0.0	0.0

0.012	0.011	0.000

1.0	.330	.133	1.0	0.3

8.3	10.0	60.0	0.0	0.0

1	1

0.000	0.000	0.000	0.000	0.000	0.000			0.000		0.000

0.000	0.000	0.010	0.020	0.030	0.040			0.050		1.000

0.050	0.040	0.030	0.020

WATR	YEAR	1	PEST	YEAR	1		CONC		YEAR	1

5	YEAR

TUPX1	TSER	1.0E05

RZFX1	TSER	1.0E05

CHGT	TSER

PRCP	TSER

VFLX1	TCUM	1.0E05

SPECIAL ACTIONS

120682	KD	1	0.5

170682		SNAPSHOT

This PRZM input file represents a scenario where one chemical is applied and background levels are present at the bottom compartments of the root zone.  Volatilization is simulated through the entire root zone.  Plant uptake is simulated until crop harvest.  One soil horizon is specified of 20 cm with a compartment thickness of 1 cm.  Output is reported on a yearly basis for hydrology, flux, and concentration.  Special actions are implemented following chemical application.

1 CHEMICAL, NO TEMPERATURE CORRECTION, PRZM INPUT FOR ZONE 1

HYDROLOGY PARAMETERS (CROP DATA FROM USDA NO.283 HANDBOOK)

0.00	0.00	0	15.000	1	1

0

1

1	0.15	20.0	80.000	1	86	78	82	0.0	0.0	0.0	0.0

1

110582	300982	151082	1

PESTICIDE TRANSPORT AND TRANSFORMATION AND APPLICATION PARAMETERS

1	1	0

ALDICARB

120582	0	2.5	1.00

1	1

SOILS PARAMETERS

20.0	0.0	0	0	0	0	0	0	0	0	0

0.0E0	0.0E00	0.0E00

1

1	20.0	1.45	0.233	0.0	0.0

0.012	0.012	0.000

2.5	.233	.050	1.0	1

0	0

WATR	YEAR	1	PEST	YEAR	1		CONC		YEAR	1

3	YEAR

RFLX1	TSER	1.0E05

RUNF	TSER

INFL	TSER	12

This PRZM input file represents one chemical being applied 2.5 cm deep at a rate of 1.0 kg/ha.  The soil horizon is 20 cm deep with a compartment thickness of 2.5 cm.

An example of a basic sequence without any options.

3 CHEMICALS, 2 HORIZONS, EROSION, IRRIGATION, PRZM INPUT FOR ZONE 1

HYDROLOGY PARAMETERS (CROP DATA FROM USDA NO.283 HANDBOOK)

0.72	0.00	2	0.000	1	3

9.6	9.7	12.2	13.6	15.4	15.5

15.7	14.5	12.5	11.3	9.5	9.0

1

0.15	0.14	1.0	2.0	5.8

1

1	0.15	30.0	80.000	3	86	78	82	0.1	0.1	0.1	60.0

1

110582	300982	151082	1

PESTICIDE TRANSPORT AND TRANSFORMATION AND APPLICATION PARAMETERS

2	3	0

ALDICARB                                      ATRAZINE                       CARBOFURAN

120582	0	2.5	1.0	2.5	2.00	1.00	2.00

120682	0	2.5	1.0	2.5	1.00	0.00	1.00

1	1

SOILS PARAMETERS

45.0	0.3	0	0	0	0	0	1		1	1	0

4.3E3	1.0E-7	2.5E-7	1.4E-7			5.5E-5		5.5E-3					5.5E-5

3	0.25	0.55	.78

0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.97 10.0

8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3

2

1	15.0	1.45	0.233	0.0	0.0	0.0	0.0

0.012	0.000	0.000	0.010	0.010	0.000	0.015	0.015	0.000

0.5	.233	.050	1.0	.1	1.	.3

8.3	10.0	60.0	0.0	0.0

0.000	0.000	0.000

2	30.0	1.45	0.233	0.0	0.0	0.0	0.0

0.012	0.000	0.000	0.005	0.005	0.000	0.015	0.015	0.000

2.5	.233	.050	0.5	.1	.5	.1

8.3	10.0	60.0	0.0	0.0

0.000	0.000	0.000

0	0

WATR	YEAR	1	PEST	YEAR	1		CONC		YEAR	1

2	YEAR

RFLX1	TSER	1.0E05

RUNF	TSER

This PRZM input file represents 3 chemicals being applied at various incorporation depths  and various applications simultaneously.  Erosion losses are calculated.  Irrigation is triggered when water capacity falls below 55 percent during the cropping period.  Two soil horizons represent the 45 cm root zone with the first horizon occupying the first 15 cm and the second horizon the lower 30 cm.  Pesticide runoff flux and runoff depth are plotted to a time series file.

1 CHEMICAL, 2 HORIZONS, NO VOLATILIZATION, BIODEGRADATION, BACKGROUND LEVELS

HYDROLOGY PARAMETERS (CROP DATA FROM USDA NO.283 HANDBOOK)

0.00	0.00	2	0.000	1	3

9.6     9.7    12.2    13.6    15.4    15.5

15.7    14.5    12.5    11.3     9.5     9.0

0

1

1    0.00    45.0  80.000       3  50  50  50 0.0 0.0 0.0            60.0

1

110581  300981  151081       1

PESTICIDE TRANSPORT AND TRANSFORMATION AND APPLICATION PARAMETERS

2       1       0

ALDICARB

120281       0     0.5    0.00

120581       0     0.5    0.00

1       1

SOILS PARAMETERS

45.0     0.0   0   0   0   0   0   0   1   1   1

.005    .005    .005    .005    .001

0.2     0.4    0.35     0.4     0.3     0.1   .0025

0.01    0.02    0.01    0.01    10.0  1000.0

2.0     1.0     6.0     2.0     2.0

0.1     0.4     0.4     0.4     0.4

4.3E3  0.0E00  0.0E00

0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.15 0.97 10.0

8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3  8.3

2

1    15.0    1.50   0.350     0.0     0.0     0.0     0.0

0.5     0.5 .000001  .00001    0.05    0.05

0.000   0.000   0.000   0.000   0.000   0.000   0.000   0.000   0.000

2.5    .350    .150    0.06      1.

8.3    10.0    60.0     0.0     0.0

2    30.0    1.50   0.350     0.0     0.0     0.0    0.0

0.5     0.5 .000001  .00001    0.05    0.05

0.000   0.000   0.000   0.000   0.000   0.000   0.000   0.000   0.000

2.5    .350    .150    0.06      1.

8.3    10.0    60.0     0.0     0.0

1       0

8.8000  8.8000  8.8000  8.8000  8.8000  8.8000  8.8000  8.8000

8.8000  8.8000  8.8000  8.8000  8.8000  8.8000  8.8000  8.8000

8.8000  8.8000

WATR    MNTH       1    PEST    MNTH       1    CONC     DAY       1

3    YEAR

RFLX1   TSER          1.0E05

THET    TSER       2

INFL    TSER       2

This PRZM input file represents a scenerio where biodegradation is used.  Aldicarb is applied with application targeted for May 12, 1982.  With the FRMFLG option set, a window application date of 10 days has been specified to check for the ideal soil-moisture conditions around the target application date.  Solid, liquid, and gas phase degradation rates have been set to zero to observe only the decay resulting from biodegradation.

***********************************FLOW**************************************

1 CHEMICAL, 3 MATERIAL, VADOSE ZONE FLOW SIMULATION FOR ZONE 1

61	3	0	1	1	1	1	1	0	0

20	2	1	.01

1	1	1	1	0	1	2	1	0

0.0		1.0		1.0		1.0

1		0.0		1.0

3

1	20	1	40.0

2	20	2	40.0

3	20	3	40.0

0.00E00	0

0	1	0.0	0.0E00	0	0		0	0

7.12E02	.43E00	0.0E00	0.0E00

24.96E00	.43E00	0.0E00	0.0E00

1.06E02	.41E00	0.0E00	0.0E00

0.045E00	-1.0E00	0.145E00	2.68E00	0.626E00

0.078E00	-1.0E00	0.036E00	1.56E00	0.358E00

0.065E00	-1.0E00	0.075E00	1.89E00	0.470E00

5   10

YEAR

**********************************TRANSPORT**********************************

1 CHEMICAL, 3 MATERIAL, VADOSE TRANSPORT SIMULATION FOR ZONE 1

61	3	1	1	0	1

0	1	1	0	0	1	2	1

0.0		1.0		1.0		1.0

1		0.0		1.0

3

1	20	1	40.0

2	20	2	40.0

3	20	3	40.0

0.0E00		1

0	0	0.0		0.0	0	0	0	0

0.12E02	.43E00

1.480E00	0.0E00

0.12E02	.43E00

1.480E00	0.0E00

0.12E02	.41E00

1.480E00	0.0E00

1	1.0

1	0.0		1.0	0.0E00

1	0.001E00	0.0E00

2	0.0		1.0	0.0E00

2	0.005E00	0.0E00

3	0.0		1.0	0.0E00

3	0.004E00	0.0E00

1	1

5	10

YEAR

This VADOFT file represents a 1 chemical simulation with 61 nodes and 60 elements

at a depth of 120 cm.  Retardation and degradation are simulated.

***********************************FLOW**************************************

1 CHEMICAL, 3 MATERIAL, 91 NODES, VADOSE ZONE FLOW SIMULATION FOR ZONE 1

91	3	0	1	1	1	1	1	0	0

20	2	1		.01

1	1	1	1	0	1	2	1	0

0.0		1.0		1.0		1.0

1		0.0		1.0

3

1	20	1		40.0

2	20	2		40.0

3	50	3		120.0

0.00E00	0

0	1		0.0	0.0E00	0	0	0	0

7.12E02	.43E00	0.0E00	0.0E00

24.96E00	.43E00	0.0E00	0.0E00

1.06E02	.41E00	0.0E00	0.0E00

0.045E00	-1.0E00	0.145E00	2.68E00	0.626E00

0.078E00	-1.0E00	0.036E00	1.56E00	0.358E00

0.065E00	-1.0E00	0.075E00	1.89E00	0.470E00

5	10

YEAR

**********************************TRANSPORT**********************************

1 CHEMICAL, 3 MATERIAL, 91 NODES, VADOSE TRANSPORT SIMULATION FOR ZONE 1

91	3	1	1	0	1

0	1	1	0	0	1	2	1

0.0		1.0		1.0		1.0

1		0.0		1.0

3

1	20	1	40.0

2	20	2	40.0

3	50	3	120.0

0.0E00		1

0	0		0.0		0.0	0	0	0	0

1.20E00	.43E00

1.000E00	0.0E00

1.20E00	.43E00

1.500E00	0.0E00

1.20E00	.41E00

1.000E00	0.0E00

1	0.0

1	0.0			1.0	0.0E00

1	0.001E00		0.0E00

2	0.0			1.0	0.0E00

2	0.005E00		0.0E00

3	0.0			1.0	0.0E00

3	0.004E00		0.0E00

1	1

5	10

YEAR

This VADOFT input file represents 91 nodes and 90 elements at a depth of

200 cm.  Dispersion, retardation, and degradation are simulated.

***********************************FLOW**************************************

3 CHEMICAL, 2 HORIZON, 1 MATERIAL, VADOSE ZONE FLOW SIMULATION FOR ZONE 1

61	1	1	1	1	1	1	1	0	0

20	2	1		.01

1	1	1	1	0	1	2	1	0

0.0		1.0		1.0		1.0

1		0.0		1.0

2

1	20	1		50.0

2	40	1		80.0

0.00E00	0

0	1		0.0	0.0E00	0	0	0	0

7.12E02	.43E00	0.0E00	0.0E00

0.045E00	-1.0E00	0.145E00	2.68E00	0.626E00

5	10

YEAR

**********************************TRANSPORT**********************************

3 CHEMICAL, 2 HORIZON, 1 MATERIAL, VADOSE TRANSPORT SIMULATION FOR ZONE 1

61	1	1	1	0	1

0	1	1	0	0	1	2	1

0.0		1.0		1.0		1.0

1		0.0		1.0

2

1	20	1		50.0

2	40	1		80.0

0.0E00		2		0.0E00	2	0.0E00	2

0	0		0.0			0.0	0	0	0	0

0.00E00	.43E00

1.000E00	1.000E00	1.000E00	0.0E00	0.0E00	0.0E00

1		0.1	2	0.1

1		0.1	2	0.1

1		0.1	2	0.1

1		0.0	1.0	0.0E00

1	0.000E00	0.000E00	0.000E00	0.00E00	0.00E00	0.0E00

1	1

5	10

YEAR

This VADOFT input file represents 3 chemicals having initial concentrations at the top two nodes.  Dispersion, degradation, and dispersion are simulated over 2 horizons with a total depth of 130 cm.  21 nodes are placed at 2.5 cm distances from 20 elements and the remaining 40 nodes are placed at 2 cm distances from the remaining 40 elements.