*

*       <<<<<<<<  SWMM 4.3 STORAGE/TREATMENT DATA FILE >>>>>>>>>

*

*  This file last updated 4/22/94 by WCH.

*

*       This is an input data file to the SWMM 4.21 Storage/Treatment

*       (STRT) Block for modeling water quantity and quality.  All lines

*       with an asterisk in column 1 are comment lines and are ignored

*       by the program.

*

*       Input data are free format and may be up to 230 columns wide.

*       You must have a value for every data column even if the program

*       will not actually use a given value.  Alternatively, a slash (/)

*       may be used to indicate that remaining input values on a line

*       should be filled with zeroes.  There must be at least one

*       space or comma between every input value.  Alphanumeric data

*       should be enclosed in single quotes.

*

*       Caution!  Data lines that are "wrapped around" (continued on

*       two or more lines) should have a blank in column 1, unless a

*       card identifier is needed.

*

*       SWMM uses both U.S. customary units and metric units.  The

*       examples use feet, cfs, acres, inches and inches/hour.  If metric

*       is specified substitute meters, cms, hectares, millimeters and

*       millimeters/hour.  In the Storage/Treatment Block, if metric

*       units are specified, internal calculations are also done in

*       metric units.

*============================================================================

*	The SW card sets up the interface files to be used or created.

*	There is one output file (#9) which will contain the time series

*       of flows and pollutant loads for subsequent blocks.

*============================================================================

*  NBLOCK  JIN(1)  JOUT(1)

SW    1      8       9

*============================================================================

*   The MM card opens the scratch files to be used by different subroutines.

*   A certain number (1 for STORAGE/TREATMENT) may be required for each block.

*============================================================================

*  NITCH  NSCRAT(1) NSCRAT(2) NSCRAT(3) NSCRAT(4) NSCRAT(5) NSCRAT(6) NSCRAT(7)

MM   7       1         2          3        10         11        12       13

*============================================================================

*  NOTE: This file requires interface file TRANS1.DNT. This must either

*  be supplied or else generated as explained in GRAPH.DOC file.  This

*  example may be run without an interface file by setting NOTAPE = 1

*  in data group B1.

*============================================================================

*     The @ command is used to permanently save an interface or

*     scratch file.  This line should be placed before the first SWMM

*     block call.  The format of the @ command is as follows:

*============================================================================

*Column 1    Unit number of the       Name of the interface

*            interface file saved     file (any valid DOS filename)

*            or utilized

*

@             8                       'TRANS1.DNT'

***           9                       'STRT1.INT'

*============================================================================

$STORAGE      Call the STORAGE/TREATMENT block with a '$' in first column.

*============================================================================

*   Note: the input data below are essentially the same as found in

*   file SWDAT1.DAT and correspond to old Lancaster, PA, Stevens Avenue

*   example from past SWMM releases.

*============================================================================

*	Create title lines for the simulation.  There are two title lines

*       for the STRT Block.  Titles are enclosed in single quotes.

*============================================================================

*	A1 Line       :

*          TITLE2     :  Two lines with heading to be printed on output.

*============================================================================

A1  'Example Storage/Treatment Model, Lancaster, PA'

A1  'Connect to old Stevens Avenue interface file from SWDAT1.DAT'

*============================================================================

*	The 'B1' line is for control purposes.

*============================================================================

*	B1 Line       :

*          NOTAPE     :  Input data source

*                        = 0, Input is from an external input file,

*                        = 1, Input is supplied in Group J1,

*                        = 2, Input is from an external input file

*                             and Group J1.

*          JNS        :  External element number from the external

*                        block (e.g., JN, Group H1, in Transport Block)

*                        which routes flow to the S/T Block.  If

*                        NOTAPE = 1, the value of JNS is placed on

*                        the output file.

*          NDT        :  Total number of simulation time steps.

*          DS         :  Size of time step, seconds.  Required

*                        only if NOTAPE = 1.

*          NU         :  Number of storage/treatment units (Maximum = 5).

*          NP         :  Number of pollutants routed (Maximum = 3).

*         ICOST       :  Cost calculations performed?

*                        = 0, No

*                        = 1, Yes

*         METRIC      :  Metric input-output.

*                        = 0, Use U.S. customary units

*                        = 1, Use metric units.

*                             Metric input indicated in brackets [].

*          SAREA      :  Service area, acres [ha].  Required if NOTAPE = 1.

*============================================================================

*  NOTAPE  JNS  NDT     DS    NU    NP   ICOST  METRIC  SAREA

B1    2     50  50    60.0    1     1      0    0      165.0

*============================================================================

*       The C1 line defines the starting and ending date and time.

*============================================================================

*	C1 Line       :

*          IDATE      :  Date at beginning of simulation

*                       (6 digit number; year, month, day --

*                        e.g. March 10, 1979 = 790310)

*          TIME       :  Time at beginning of simulation

*                        (24 hour clock, e.g., 5:30 p.m. = 17.5)

*          ISUM       :  Summary print control parameter

*                        = 0, Print a summary at the end of

*                             the simulation only

*                        = 1, Print an annual summary and a summary

*                             at the end of the simulation

*                        = 2, Print monthly and annual summaries and

*                             a summary at the end of the simulation

*          IDET       :  Detailed print control parameter

*                        = 0, No detailed print of simulation results.

*                        > 0, Detailed print of results is provided

*                        at every time step that is a multiple of

*                        IDET (e.g., IDET = 2, gives a detailed report

*                        at every other time step) during specified

*                        periods (see below and group C2).

*          NPR        :  Number of detailed print periods.  Up to

*                        45 periods may be specified (See Group C2).

*                        Required only if IDET > 0.

*============================================================================

*   IDATE     TIME         ISUM      IDET       NPR

C1  731128     0.00          0         10         1

*============================================================================

*       The C2 line lists the NPR detailed print periods.

*              NPR (Group C1) periods must be specified.  Only date

*              to date periods may be used (e.g., 790720 to 790806).

*              Required only if IDET > 0 (Group C1).

*============================================================================

*	C2 Line       :

*          ISTART(1)  :  First detailed print period starting

*                        date (e.g., July 20, 1979 = 790720).

*          IEND(1)    :  First detailed print period ending

*                        date (e.g., August 6, 1979 = 790806).

*            .                       .

*                        Repeat for second period, etc. up to

*                        NPR Group C1 periods - place all

*                        ISTART and IEND pairs on a single line.

*            .                       .

*          ISTART(NPR):  Last detailed print period starting date.

*          IEND(NPR)  :  Last detailed print period ending date.

*                Note: Do not repeat line ID when reading in multiple

*                C2 lines.

*============================================================================

*   ISTART(1) IEND(1)

C2  731128    731128

*============================================================================

*    Use the D1 line for evaporation data.  Required only if there

*         are detention units.  (IDENT(I) = 1 for some units, see Group F2).

*============================================================================

*	D1 Line       :

*          E(1)       :  Evaporation rate, January in./day [mm/day].

*           .                              .

*          E(12)      :  Evaporation rate, December in./day [mm/day]

*============================================================================

*  EVAPORATION DATA

D1  .1   .1  .15  .25   .3  .6  .6  .7  .6   .4   .1  .1

*============================================================================

*       Data groups E1 - E6 define STRT pollutant characterization.

*            Require groups E1 - E6 only if NP > 0 on GROUP B1.

*============================================================================

*	E1 Line       :

*          IPOLL(1)   :  Pollutant 1 selector.  Required only if

*                        NOTAPE = 0 or 2 (Group B1).  The value

*                        selected depends on the order in which the

*                        pollutants were placed on the external input file.

*                        For example, if suspended solids was the third

*                        pollutant listed on the file and it was desired

*                        for use in the S/T block, then IPOLL(1) = 3.

*         NDIMI(1)    :  Dimensions for pollutant 1.  Required

*                        only if NOTAPE = 1 (Group B1).

*                        = 0, Dimensions are mg/l

*                        = 1, Dimensions are other/liter, e.g., MPN/liter

*                        = 2, Other concentration dimensions are

*                             used. (e.g., JTU, deg. C. pH)

*         IPART(1)    :  Particle size/specific gravity or

*                        settling velocity distribution parameter

*                        = 0, Distribution not used to

*                             characterize pollutant 1.

*                        = 1, Distribution used to characterize pollutant 1.

*         PNAME1(IN,1):  Pollutant 1 name.  Required only if

*                        NOTAPE = 1  (Group B1).

*         PUNIT1(IN,1):  Pollutant 1 dimension label.

*                        Required only if NOTAPE = 1 (Group B1).

*         IPOLL(2)    :  Pollutant 2 selector.  Required only if

*                        NP >_ 2 and NOTAPE = 0 or 2 (Group B1).  See above.

*         NDIMI(2)    :  Dimensions for pollutant 2.  Required only if

*                        NP >_ 2 and NOTAPE = 1 (Group B1).  See above.

*         IPART(2)    :  Particle size/specific gravity or settling

*                        velocity distribution parameter.  Required only

*                        if NP >_ 2 Group B1)  See above.

*         PNAMEI(IN,2):  Pollutant 2 name.  Required only if

*                        NP >_ 2 and NOTAPE = 1.  (Group B1)

*         PUNITI(IN,2):  Pollutant 2 dimension label.  Required only if

*                        NP >_ 2 and NOTAPE = 1. (Group B1)

*         IPOLL(3)    :  Pollutant 3 selector.  Required only if

*                        NP = 3 and NOTAPE = 0 or 2 (Group B1)  See above.

*         NDIMI(3)    :  Dimensions for pollutant 3.  Required only if

*                        NP = 3 and NOTAPE = 1 (Group B1).  See above.

*         IPART(3)    :  Particle size/specific gravity or settling

*                        velocity distribution parameter.  Required only

*                        if NP = 3. (Group B1).  See above.

*         PNAMEI(IN,3):  Pollutant 3 name.  Required only if NP = 3 and

*                        NOTAPE = 1 (Group B1).

*         PUNITI(IN,3):  Pollutant 3 dimension label.  Required

*                        only if NP = 3 and NOTAPE = 1 (Group B1).

*============================================================================

*  IPOLL NDIM IPART  PNAME    PUNIT IPOLL NDIM IPART PNAME  PUNIT IPOLL NDIM IPART PNAME  PUNIT

E1   1    0      0   'TSS '  'mg/l '    2     0    0   ' '    ' '    3    0      0    ' '   ' '

*============================================================================

*     Note:   GROUPS E2 - E6 are REQUIRED only if IPART (IP) = 1

*                            on group E1 for any pollutants.

*============================================================================

*	E2 Line       :

*          NVS        :  Classification parameter

*                        = 0, Particle size/specific gravity distribution

*                        is used to classify particles in waste stream.

*                        = 1, Settling velocity distribution is used.

*          NNR        :  Number of particle size ranges or settling

*                        velocities used to classify particles in waste

*                        stream (Max of 10).

*============================================================================

*     Use E3 data group to define particle size (if NVS = 0, group E2 or

*            settling velocity (if NVS = 1, group E2) range data.

*============================================================================

*	E3 Line       :

*          RAN(1,1)   :  Lower bound of size or velocity

*                        range 1, microns or ft/sec [cm/sec].

*          RAN(1,2)   :  Upper bound of size or velocity

*                        range 1, microns or ft/sec [cm/sec].

*             .                         .

*          RAN(NPR,1) :  Lower bound of size or velocity

*                        range NPR, microns or ft/sec [cm/sec].

*          RAN(NPR,2) :  Upper bound of size or velocity

*                        range NNR, microns or ft/sec [cm/sec].

*============================================================================

*            Use E4 data group to specific gravity data.

*                   Required only if NVS = 0 (Group E2).

*============================================================================

*	E4 Line       :

*          SPG(1)     :  Specific gravity for particles in size range 1.

*          SPG(2)     :  Specific gravity for particles in size range 2.

*             .                             .

*          SPG(NNR)   :  Specific gravity for particles in size range NNR.

*============================================================================

*            Use E5 data group waste stream temperature data.

*                   Required only if NVS = 0 (Group E2).

*============================================================================

*	E5 Line       :

*          TEMP(1)    :  Waste stream temperature, January, deg F. [C].

*             .                            .

*          TEMP(12)   :  Waste stream temperature, December, deg F. [C].

*============================================================================

*     Use E6 data group to define the fraction of each pollutant associated

*         with each particle size/specific gravity or settling velocity

*         range (Group E3).  Repeat these lines for each pollutant for which

*         IPART(IP) = 1 (Group E1).  Required only if NVS = 0 (Group E2).

*============================================================================

*	E6 Line       :

*          PSD(IP,1)  :  Fraction of pollutant IP in range 1.

*          PSD(IP,2)  :  Fraction of pollutant IP in range 2.

*                .                            .

*          Repeat for each range up to NNR (Group E2) ranges.

*============================================================================

*       REPEAT GROUPS F1 - I1 FOR EACH UNIT I.  THERE WILL BE NU (GROUP B1)

*              SETS.  THE UNIT NUMBER IS DICTATED BY THE ORDER IN WHICH

*                     THE SETS OF GROUPS F1 - I1 ARE READ.

*============================================================================

*	F1 Line       :

*         UNAME(I,ID) : Name of unit.

*============================================================================

*     UNAME(1,ID)

F1    ' STORAGE'

*============================================================================

*     Use line F2 to define the general unit parameters and flow directions.

*============================================================================

*	F2 Line       :

*          IDENT(1)   :  Detention modeling parameter

*                        = 0, Unit is the non-detention type

*                        = 1, Unit is the detention type

*          QMAX(I)    :  Maximum inflow (above which bypass

*                        occurs), cfs [cu.m/sec].

*          QRF(I)     :  Residual flow as a fraction of the inflow.

*                        Required only if IDENT(I) = 0.  Residual flows

*                        for detention units (IDENT(I) = 1) are determined

*                        in Groups H1, H3, and H6.

*          IDIREC(I,1):  Unit number to which bypass is directed

*                        (must be greater than 1).

*                        = 1 - 5, Downstream S/T unit

*                        = 100, Next block

*                        = 200, Ultimate disposal

*          IDIREC(I,2):  Unit number to which treated outflow is directed

*                        (must be greater than 1).  See above.

*          IDIREC(I,3):  Unit number to which residuals stream is directed

*                        (Must be greater than 1).  See above.   If

*                        IDRAW(I) = 0 (Group H1), set equal to any number > 1.

*============================================================================

*      IDENT   QMAX     RESIDUAL FLOW   IDIREC(1) IDIREC(2) IDIREC(3)

F2       1      125.       0.0             100       100       200

*============================================================================

*      Use data groups G1 - G4 to define Pollutant Removal.

*               Required only if NP > 0 on group B1.

*      REPEAT GROUPS G1 - G3 FOR EACH POLLUTANT FOR WHICH IPART(IP) = 0.

*============================================================================

*	G1 Line       :

*          RMX(I,IP)  :  Maximum removal fraction (<= 1.0).

*============================================================================

*      RMX

G1     1.0

*============================================================================

*     Define the removal equation variable group (Equation 7-1) on line G2.

*============================================================================

*	G2 Line       :

*        INPUT(I,IP,1):  Program variable for equation variable X1.

*                        = 0, Not used.

*                        For values = 1 - 7, see Table 7-3 in text.

*        INPUT(I,IP,2):  Program variable for equation variable X2.

*                        See above.

*             .                           .

*                        Repeat for each program variable Xi.

*============================================================================

*    REMOVAL EQUATION VARIABLES

G2     0  0  1  0  0  0  0  0  0  0  0

*============================================================================

*     Define Equation 7-1 coefficients on line G3.  The coefficients

*       must be consistent with the units used (see METRIC Group B1).

*============================================================================

*	G3 Line       :

*          A(I,IP,1)  :  Value of coefficient a1.

*          A(I,IP,2)  :  Value of coefficient a2.

*             .                      .

*          Repeat for each coefficient aj.

*============================================================================

*    EQUATION 7-1

G3    0 0 -0.003028 0 0 0 0 0 0 0 0 0.45 -0.45 0.0 0.0 1.0

*============================================================================

*     Define the Critical Particle Size or Settling Velocity on line G4.

*     Required only if IPART(IP) = 1 (Group E1) for any pollutant and

*          unit I is a non-detention unit, IDENT(I) = 0 (Group F2).

*============================================================================

*	G4 Line       :

*          PSC(I)     :  Critical particle size, microns, (if NVS = 0,

*                        Group E2) or settling velocity, ft/sec [cm/sec],

*                        (if NVS = 1, Group E2).

*============================================================================

*     PSC(I)

*G4    10.0

*============================================================================

*             Data Groups H1 - H8: Detention Unit Data

*             Required only if IDENT(I) = 1 (Group F2)

*============================================================================

*       Define the general detention unit parameters on line H1.

*============================================================================

*	H1 Line       :

*          IROUTE(I)  :  Pollutant routing parameter

*                        = 0, Plug flow mode is used.

*                        = 1, Complete mixing mode is used.

*                            (Note:  Particle size or settling velocity

*                             distribution is not routed through

*                             completely-mixed units.)

*                        [Note: user may encounter limit of 200 plugs

*                        (8/31/93) in use at one time.  Options are to

*                        increase S/T time step to reduce required number

*                        of plugs or to recompile Subroutine UNIT with

*                        larger limit.]

*          IOUT(I)    :  Treated outflow routing parameter

*                        = 0, The depth-treated outflow relationship is

*                        described by as many as 16 data pairs in Group H3.

*                        = 1, The depth-treated outflow relationship is

*                        described by a power equation in Group H4.

*                        = 2, The depth-treated outflow relationship is

*                        controlled by the pumps described in Group H5.

*          IDRAW(I)   :  Residuals stream draw-off scheme

*                        <_ -1, A residual stream is drawn off starting at

*                        every -IDRAW(I) time step (if possible).

*                        = 0, Residuals are never drawn off.

*                        >_ 1, A residuals stream (if available) is drawn off

*                        only after IDRAW(I) time steps of no inflow or

*                        treated outflow.

*          IRES(I)    :  Residual stream routing parameter

*                        Required only if IDRAW(I) not = 0

*                        = 0, The depth-residual flow relationship is

*                        described by as many as 16 data pairs in Group H3.

*                        = 1, The depth-residual flow relationship is

*                        described by a power equation in Group H6.

*============================================================================

*     PLUG FLOW ROUTING IS BEING USED

*     IROUTE    IOUT     IDRAW      IRES

H1       0         1         1         0

*============================================================================

*      Detention unit (plug flow only) parameters, required when pollutants

*  are characterized by a particle size/specific gravity or settling

*  velocity distribution.  Thus, the H2 line is required only if

*  IPART(IP) = 1, for any pollutant (Group E1) and IROUTE(I) = 0 (Group E2).

*============================================================================

*	H2 Line       :

*          ALEN(I)    :  Travel length for plug flow, ft [m].

*          AMAN(I)    :  Manning's roughness coefficent for

*                        detention unit surfaces.

*============================================================================

*    ALEN(I)   AMAN(I)

*H2    0.0      0.0

*============================================================================

*       Data for Sets of Depth, Surface Area, Volume,

*                Treated Outflow and Residual Flow

*

*   Each line contains a column for a unit depth and the corresponding

*   values of area, volume, treated outflow, and residual flow.  The

*   columns for treated outflow and residual flow may be left blank depending

*   on the values of IOUT(I) and IRES(I) in Group H1.  If no values for

*   volume are entered, the program estimates volume from the depth-surface

*   area relationship.  Order the data from the bottom of the unit

*   SDEPTH(I,1) = 0.0) to the maximum depth (including freeboard).

*                 There may be as many as 16 lines.

*============================================================================

*	H3 Line       :

*        SDEPTH(I,MM) :  A unit depth, ft [m].

*        SAREA(I,MM)  :  Surface area corresponding to the above

*                        depth, sq. ft [sq. m].

*        SSTORE(I,MM) :  Volume corresponding to the above depth

*                        cu. ft [cu. m].

*        SQQOU(I,MM)  :  Treated outflow at the above depth

*                        cfs [cu.m/sec].

*        SQQRS(I,MM)  :  Residuals stream flow at the above depth cfs[cu.m/sec].

*                        Occurs only when IDRAW(I) (Group H1) permits.

*============================================================================

*     DEPTH     AREA  VOLUME  TREATED OUTFLOW RESIDUAL FLOW

H3     0.0     1000.    0            0          5.

H3     8.0     1000.    0            0          5.

H3     8.5     1000.    0            0          5.

H3     9.0     1000.    0            0          5.

H3     9.5     1000.    0            0          5.

H3    10.0     1000.    0            0          5.

*============================================================================

*   Depth vs. Treated Outflow Power Equation (Equation 7-3) on line H4.

*

*   Outflow = C1*(Depth - D0)**C2

*

*   Required only if IOUT(I) = 1 (Group H1).  Coefficients must be

*         consistent with the units used. (See METRIC, Group B1.)

*============================================================================

*	H4 Line       :

*          C1         : Depth-treated outflow equation coefficient, C1.

*          D0         : Depth below which no treated outflow occurs, D0.

*          C2         : Depth-treated outflow equation coefficient, C2.

*============================================================================

*       C1        D0        C2

H4   66.66        8.       1.5

*============================================================================

*     Use the H5 data group to define any treated outflow pumping.

*             Required only if IOUT(I) = 2 (Group H1).

*============================================================================

*	H5 Line       :

*          DSTART(I,1):  Depth at which pumping rate QPUMP(I,1)

*                        begins, ft [m].

*          DSTART(I,2):  Depth at which pumping rate QPUMP(I,2)

*                        begins, ft [m].  Must be >_ DSTART(I,1).

*          QPUMP(I,1) :  Pumping rate when depth >_ DSTART(I,1)

*                        cfs [cu.m/sec].

*          QPUMP(I,2) :  Pumping rate when depth >_ DSTART(I,2)

*                        cfs [cu.m/sec].

*          DSTOP(I)   :  Depth below which all pumping stops,

*                        ft [m].  Must be <_ DSTART(I,1).

*============================================================================

*   DSTART(I,1) DSTART(I,2) QPUMP(I,1) QPUMP(I,2) DSTOP(I)

*H5        0.0         0.0         0.0        0.0      0.0

*============================================================================

*     Define the Depth vs. Residual Flow Power Equation (Equation 7-4)

*     on line H6.  Required only if IRES(I) = 1 (Group H1).  Coefficients

*     must be consistent with the units used (see METRIC, Group B1).

*============================================================================

*	H6 Line       :

*          C3         :  Depth-residual flow equation coefficient.

*          D1         :  Depth below which no residual flow occurs.

*          C4         :  Depth-residual flow equation coefficient.

*============================================================================

*    C3   D1   C4

*H6 0.0  0.0  0.0

*============================================================================

*   Use data group H7 to define Sludge Generation in Unit I.

*       Required only if I is a plug-flow detention unit (IROUTE(I) = 0,

*                Group H1) and NP > 0 (Group B1).

*============================================================================

*	H7 Line       :

*          NPSL(I)    :  Pollutant responsible for sludge generation.  Required

*                        only if a sludge depth warning message is desired.

*                        = 0, Not used.

*                        = 1, 2, or 3, Pollutant used to generate sludge

*                        volume (must correspond to the position in Group E1.

*          SLDEN(I)   :  Concentration of pollutant NPSL(I) in sludge.

*                        Required only if NPSL(I) >_ 1.  The dimensions

*                        used must be consistent with those indicated

*                        by NDIM(I) (Group E1).

*          SLDMAX(I)  :  Maximum sludge depth, ft [m].  A warning

*                        message is printed if this depth is exceeded

*                        by the accumulated sludge.  Required only if

*                        NPSL(I) >_ 1.

*============================================================================

*     NPSL   SLDEN  SLDMAX

H7      2    20000.    1.

*============================================================================

*     Use line H8 to define the initial conditions in detention unit I.

*============================================================================

*	H8 Line       :

*          WARN(I)    :  Total volume of water in unit at the

*                        start of the simulation, cu.ft [cu.m]

*

*   Note: The following concentrations must be given with dimensions

*         consistent with those entered in Group E1 (NDIM(IP)) if

*         NOTAPE = 1 (Group B1) or on the external input file if NOTAPE = 2.

*

*          PCO(I,1)   :  Concentration of pollutant 1 in the unit at

*                        the start of the simulation.  Used only if NP >= 1

*                        (Group B1) and WARN(I) > 0.0.

*          PCO(I,2)   :  Concentration of pollutant 2 in the unit volume

*                        at the start of the start of the simulation.

*                        Used only if NP >= 2 (Group B1) and WARN(I) > 0.0.

*          PCO(I,3)   :  Concentration of pollutant 3 in the unit volume

*                        at the start of the start of the simulation.

*                        Used only if NP = 3 (Group B1) and WARN(I) > 0.0.

*============================================================================

*    WARN      PCO(1)  PCO(2)   PCO(3)

H8   4000.       25.    0.0       0.0

*============================================================================

*    Use line I1 to enter Cost Data for Equations 7-5 to 7-8

*        (Capital Costs) and Equations 7-9 to 7-12 (Operation and

*         Maintenance Costs).  The coefficients must be consistent with

*         the units used (see METRIC, Group B1).  Required only if

*                        ICOST = 1 on GROUP B1.

*============================================================================

*	I1 Line       :

*          KPC(I,1)   :  Type of cost variable used in calculating

*                        initial capital cost.

*                        = 0, Not used.

*                        = 1, Maximum allowable inflow, QMAX(I),

*                          cfs [cu.m/sec] is used.

*                        = 2, Maximum inflow observed during simulation,

*                          QMAXS(I), cfs [cu.m/sec], is used.

*                        = 3, Maximum allowable storage, VMAX(I)

*                          cu.ft [cu.m] is used.  (Not applicable

*                          if IDENT(I) = 0, Group F2).

*                        = 4, Maximum storage observed during simulation,

*                          VMAXS(I), cu.ft. [cu.m] is used (Not applicable

*                          if IDENT(I) = 0  (Group F2).

*          CC(I,1)    :  Initial capital cost equation coefficient, a.

*          CC(I,2)    :  Initial capital cost equation coefficient, b.

*          KPC(I,2)   :  Type of cost variable used in calculating

*                        operation and maintenance costs.  See list for

*                        initial capital cost (above).

*          CC(I,3)    :  Operation and maintenance cost equation

*                        coefficient, d.

*          CC(I,4)    :  Operation and maintenance cost equation

*                        coefficient, f.

*          CC(I,5)    :  Operation and maintenance costs equation

*                        coefficient, h.

*==============================================================

*    KPC(I,1) CC(I,1) CC(I,2) KPC(I,2) CC(I,3) CC(I,4) CC(I,5)

*I1      0.0     0.0     0.0      0.0     0.0     0.0     0.0

*============================================================================

*                 Input Flow and Pollutant Time Series

*

*             REQUIRED ONLY IF NOTAPE = 1 OR 2 (GROUP B1)

*

*   All flows and concentrations are instantaneous values at the

*       indicated time.  A constant time interval is not required;

*       linear interpolation is used to obtain intermediate values of

*       flow and concentrations.  Hence, the difference between two time

*       entries, TCAR, should not be less than the time step, DS,

*       unless a step function input is desired.

*

*   The concentration units must be identical to those in Group E1

*      (NDIM(IP)) if NOTAPE = 1, Group B1, or on the external input

*       file if NOTAPE = 2.

*============================================================================

*	J1 Line       :

*          TCAR       :  Time of day, decimal hours, e.g., 6:30 p.m. = 18.5.

*                        If the first TCAR value is <= initial time,

*                        the program will read succeeding J1 data groups

*                        until TCAR >= initial time.  If simulation goes

*                        beyond one day, i.e., times > 24.0, then continue

*                        with times greater than 24.  I.e., TCAR should not

*                        be reset at beginning of day.

*          QCAR       :  Flow entering S/T plant (at unit 1), cfs [m3/sec].

*          PCAR(1)    :  Concentration of pollutant 1 entering S/T plant

*                        (at unit 1).  Used only if NP >= 1 (Group B1)

*                        and QCAR > 0.0

*          PCAR(2)    :  Concentration of pollutant 2 entering S/T plant

*                        (at unit 1).  Used only if NP >= 2 (Group B1)

*                        and QCAR > 0.0

*          PCAR(3)    :  Concentration of pollutant 3 entering S/T plant

*                        (at unit 1).  Used only if NP = 3 (Group B1)

*                        and QCAR > 0.0

*============================================================================

*   TCAR  = TIME IN HOURS

*   QCAR  = INSTANTANEOUS FLOW IN CFS AT TCAR

*   PCAR1 = INSTANTANEOUS SUSPENDED SOLIDS CONCENTRATION AT TCAR

*   TCAR    QCAR     PCAR(1)

J1   0.0    65.8      300.

J1   1.0    65.8      300.

*J1   2.0    77.1      220.

*J1   3.0     6.6      225.

*J1   4.0     0.1        5.

*J1   5.0     7.2      110.

*J1   6.0     0.8      140.

*J1   7.0    15.2      200.

*J1   8.0     2.8      300.

*J1   9.0     0.2      300.

*J1  10.0     1.0      5.0

*J1  50.0     1.0      5.0

*============================================================================

*               End your input data set with a $ENDPROGRAM.

$ENDPROGRAM